3GPP TSG-RAN WG4 Meeting # 114	R4-2501231

Athens, GR, 17th – 21st February, 2025

Title: 	Discussion on CSI-RS based L1 measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.27.2.2

Document for:	Discussion

Introduction

The revised R19 mobility WI was agreed in [1]. The objectives are duplicated as below.

### 4.1 Objective of SI or Core part WI or Testing part WI - Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

This contribution provides analysis on CSI-RS based L1 measurement.

Discussion

Know cell definition

It is agreed that RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells.

- Agreement: - RRM requirements for CSI-RS based L1-RSRP measurement only apply for known target cells

The status of known cell definition for CSI-RS based L1 measurement for candidate cells is duplicated from [2] as below. There are still three FFS, and we would like to provide analysis on them.

For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Consider SSB based L3 measurement as the pre-requisite condition only. E.g. no need to consider CSI-RS based L3 measurement as condition. - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

In last meeting, the baseline measurement procedure for CSI-RS based LTM measurement was almost settled. It is agreed that for FR2-1, after SSB based L3 measurement and report (i.e., step1), UE would perform SSB-based L1 measurement on the same candidate cell (i.e., step2), and then CSI-RS based L1 measurement would be performed on the same candidate cell (i.e., step3) where the CSI-RS shall be QCL-ed with SSB in step 2. To ensure the associated SSB in step 2 is detectable, the first FFS bullet of L1 measurement shall be kept.

Proposal 1: Keep L1 measurement for FR2-1 in the 2nd bullet.

The second FFS is regarding with the term of “detectable and measurable”. In the existing L1-RSRP measurement for serving cell, “measurable” is used for SSB/CSI-RS resources for L1 RSRP measurement. The reason is that no cell detection is considered for L1-RSRP measurement. We think the similar description can be inherited to candidate cells.

- 9.5.2 Requirements applicability The requirements in clause 9.5 apply, provided: - The CSI-RS or SSB or CSI-RS and SSB resources configured for L1-RSRP measurements are measurable. An SSB resource configured for L1-RSRP shall be considered measurable when for each relevant SSB the following conditions are met: - L1-RSRP related side conditions given in clauses 10.1.19.1 and 10.1.20.1 for FR1 and FR2, respectively, for a corresponding band, - SSB_RP and SSB Ês/Iot according to Annex B.2.4.1 for a corresponding band. A CSI-RS resource configured for L1-RSRP shall be considered measurable when for each relevant CSI-RS the following conditions are met: - L1-RSRP related side conditions given in clauses 10.1.19.2 and 10.1.20.2 for FR1 and FR2, respectively, for a corresponding band, - CSI-RS_RP and CSI-RS Ês/Iot according to Annex B.2.4.2 for a corresponding band. A CSI-RS and SSB resource configured for L1-RSRP shall be considered measurable when the measurable resource conditions are met for both CSI-RS resource and SSB resource.

Proposal 2: The CSI-RS from the target cell configured for L1 measurement remains measurable.

In R18 LTM, the SINR of SSB based L1-RSRP is -3dB. In R19, CSI-RS based L1-RSRP measurement is introduced. We don’t observe the justification for CSI-RS based L1-RSRP to use a different SINR condition than SSB based L1-RSRP.

Proposal 3: SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB.

Measurement procedure

In last meeting, the basic structure of the measurement procedure for CSI-RS L1 measurement on candidate cells was settled [2]. RAN4 agreed to have three steps: UE performs SSB based L3 measurement and report (i.e., step1), and then UE performs SSB-based L1 measurement on the same candidate cell (i.e., step2), after that CSI-RS based L1 measurement would be performed on the same candidate cell (i.e., step3). There are still one FFS and one bracket. In the following, we would like to further discuss on these.

- Issue 3-2: Measurement procedure <Agreement> - Agree the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements and reports at least before LTM configuration. - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions, or based on UE capability or based on network RRC configuration. FFS the details. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Depending on whether step 2 is skipped, the requirements in step 3 may be different. - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

The basic procedure of CSI-RS based L1 measurement on candidate neighbour cell shall be,

- Step 1 (gNB TX wide SSB beam, UE RX wide SSB beam): UE performs SSB based L3 measurement and acquires the SSB index information. UE reports the L3 SSB measurement results with SSB index to network.

- Step 2 (gNB TX wide SSB beam, UE RX fine SSB beam): UE performs SSB based L1-RSRP measurement on the candidate cell to refine the SSB based RX beam. Step 2 is what we specified for R18 LTM where UE sweeps RX beam based on SSB.

- Step 3 (gNB TX narrow CSI-RS beam, UE RX fine SSB beam): UE performs measurements on the configured CSI-RS resources of candidate cell where each CSI-RS resource is QCL-typeD with SSB for L1-RSRP measurement in step 2. Herein UE performs L1 CSI-RS based measurement without RX beam sweeping. Step 3 is similar as R15 CSI-RS based L1-RSRP measurement, the difference is step3 is on candidate cell and R15 CSI-RS based L1-RSRP measurement is on serving cell.

If step 2 is removed, the subsequent measurement procedure is: CSI-RS resources within a resource set are transmitted with different downlink spatial domain transmission filter, and UE sweeps Rx beam with finer CSI-RS level beam. The schematic diagram is shown in figure 1.

Figure 1. CSI-RS L1 measurement on neighbour cell with skipping step2 (NOT preferred)

At first glance, skipping step 2 (shown in Figure 1) can get the refiner RX beam, however some outstanding drawbacks and implementation restriction are observed:

1. Due to RX beam sweeping on CSI-RS resource of neighbour cell, the CSI-RS measurement delay of skipping step2 would be N times (N is UE Rx beam number) than that of keeping of step 2. In other words, the timeliness of obtaining CSI-RS measurement result would be worse.

Moreover the measurement delay of neighbour cell would be quite longer (N times) than that of serving cell where typical L1 measurement periodicity is 20ms for serving cell. As we know, in FR2 environment, the channel changes quickly. If UE is moving to the cell edge, due to different measurement periodicity, the RSRP on serving cell would react channel degradation faster than that on neighbour cell. It is most likely that UE will switch to neighbour cell. However when “neighbour cell” becomes serving cell, RSRP fluctuation of the current serving cell will be intense, and UE may quickly switch back. In a word, different measurement periodicity between neighbour cell and serving cell may result in “ping-pong” handover.

Observation 1: The CSI-RS based L1-RSRP measurement delay of skipping step2 would be N times (N is UE Rx beam number) than that of keeping of step 2.

Observation 2: Longer L1-RSRP periodicity of neighbour cell and short L1-RSRP periodicity of serving cell would result in “ping-pong” handover.

2. In field network, CSI-RS Repetition ON is extremely rare to be configured due to the large overhead burden. So in serving cell it is typically assumed CSI-RS resources repetition set to OFF. It means that P-3 is typically NOT executed in beam management procedure where P-3 is that CSI-RS resources is transmitted repetition ON and UE refines the Rx beam to CSI-RS level. So according to R15 CSI-RS L1-RSRP measurement requirements for serving cell, UE uses SSB level RX beam to perform CSI-RS measurement (QCL-D in qcl-InfoPeriodicCSI-RS is SSB).

- For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with - SSB for L1-RSRP measurement, or - another CSI-RS in resource set configured with repetition ON. - For periodic CSI-RS resources in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set) for Table 9.5.4.2-2, where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured with QCL-TypeD for all resources in the resource set.

If L1 CSI-RS measurement on neighbour cell is executed with RX beam sweeping, the RX beam would be refined to CSI-RS level beam. Then the RX beam gain on serving cell and on neighbour cell would be different. Therefore the RSRP comparison between serving cell and neighbor cell will be unfair.

Observation 3: For CSI-RS L1-RSRP measurement, RX beam gain on serving cell and on neighbour cell would be different if skipping step 2.

3. As per existing R15 CSI-RS L1-RSRP measurement requirements, the CSI-RS resource for L1 measurement shall be QCL-D with SSB index for L1 measurement (rather than SSB for L3 measurement, see requirements in above table). The reason is that the interaction between L1 measurement module and L3 measurement module may be not supported in UE implementation. Therefore it may encounter UE capability obstacle to directly use the L3 SSB information for CSI-RS L1 measurement, if step 2 is skipped.

Observation 4: It may have UE capability obstacle to directly use the L3 SSB information for CSI-RS L1 measurement, if step 2 is skipped.

Based on the above analysis, the measurement procedure

Proposal 4: Measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Step 2 can NOT be skipped for FR2-1.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

If CSI-RS resources of neighbor cell are set to Repetition ON, the CSI-RS resources in the resource set use the same downline spatial domain transmission filter (i.e, TX beam), and UE is supposed to sweep Rx beam. The intention of such kind of configuration is to train best RX beam for each TX beam. As discussed above, even for serving cell CSI-RS Repetition ON is extremely rare to be configured due the large overhead burden in practical network, we don’t think repetition ON configuration would be applied for CSI-RS L1 RSRP measurement on candidate cells.

Proposal 5: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Intra-frequency CSI-RS based L1 RSRP measurement definition

In last meeting, the definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement was discussed and consensus was reached [2]. In the following, we would like to discuss the three brackets.

- Issue 3-3: Definition of intra-frequency/inter-frequency for CSI-RS based L1 measurement <Agreement> - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of [active DL BWP], and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of [active DL BWP], and - It is applied for SCS = 60KHz - [the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).]

In RAN2#127bis meeting, RAN2 further agreed that R18 LTM CSI resource configuration is reused for CSI-RS based L1 measurement resource configuration on candidate cell.

<RAN2 agreements in RAN2#127bis>

For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

As per above RAN2 signalling design philosophy, the R19 L1 CSI-RS measurement resources configuration of candidate cells is to reuse existing NZP-CSI-RS-Resource structure in LTM-config rather than in MeasObjectNR. The NZP-CSI-RS-Resource is duplicated as below.

It is observed that the SCS and CP of CSI-RS resources are directly configured for neighbor cell. However the SCS and CP of CSI-RS resources of serving cell shall refer to that of active BWP. Therefore it is reasonable to compare SCS and CP type of CSI-RS resource of neighbor cell with that of active DL BWP.

According to the CSI-RS configuration on neighbor cell, one candidate neighbour cell can have multiple L1 CSI-RS resources where the center frequency and bandwidth of these CSI-RS resources can be different. In the same way, the L1 CSI-RS resource configuration in serving cell uses NZP-CSI-RS-Resource, and there may have also more than one NZP-CSI-RS resources with different center-frequency in the serving cell as well.

CSI-RS-ResourceMapping ::=          SEQUENCE {

frequencyDomainAllocation           CHOICE {

row1                                BIT STRING (SIZE (4)),

row2                                BIT STRING (SIZE (12)),

row4                                BIT STRING (SIZE (3)),

other                               BIT STRING (SIZE (6))

},

nrofPorts                           ENUMERATED {p1,p2,p4,p8,p12,p16,p24,p32},

firstOFDMSymbolInTimeDomain         INTEGER (0..13),

firstOFDMSymbolInTimeDomain2        INTEGER (2..12)                                                         OPTIONAL,   -- Need R

cdm-Type                            ENUMERATED {noCDM, fd-CDM2, cdm4-FD2-TD2, cdm8-FD2-TD4},

density                             CHOICE {

dot5                                ENUMERATED {evenPRBs, oddPRBs},

one                                 NULL,

three                               NULL,

spare                               NULL

},

freqBand                            CSI-FrequencyOccupation,

...

}

CSI-FrequencyOccupation ::=         SEQUENCE {

startingRB                          INTEGER (0..maxNrofPhysicalResourceBlocks-1),

nrofRBs                             INTEGER (24..maxNrofPhysicalResourceBlocksPlus1),

...

}

In our understanding, it is the consensus in last meeting that the intra-frequency CSI-RS L1 measurement definition depends on the center frequency of CSI-RS resource on neighbor cell and serving cell. So it is also reasonable to compare the centre frequency of the CSI-RS resources of the neighbour cell to the centre frequency of any of the CSI-RS resources of the serving cell.

Proposal 6: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).

Measurement with/without gap

One open issue is whether to support CSI-RS based L1 measurement with measurement gap.

- Issue 3-8: supported measurement type <Agreement> - CSI-RS based L1 measurement without gap is supported. - FFS on CSI-RS based L1 measurement with gap

The CSI-RS based L1-RSRP measurement on candidate can be outside active BWP, as the active BWP may be changed dynamically.  If RAN4 only define requirements for measurement without gap case, it would restrict network configuration.

Moreover gap based measurement is a typical scenario. Supporting completed mobility scenarios would be active for the prospect of feature application.

Proposal 7: Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

RTD assumption

As long as the intra-frequency definition for CSI-RS L1 measurement of neighbor cell is based on center frequency comparison, the term of “CSI-RS frequency layer” seems not complicated. We think if the center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources are the same, they are regarded as the same CSI-RS frequency layer.

Proposal 8: If the center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources are the same, they are regarded as the same CSI-RS frequency layer.

In R18 LTM, the capability of measurement with RTD>CP is introduced to support the scenario when RTD is larger than CP. If RAN4 agree to support the case where RTD is larger than CP between serving cell and neighbour cell on the same frequency layer, a similar capability is also needed, as the capability shall not be mandatory.

Proposal 9: If RAN4 agree to support the case where RTD is larger than CP between serving cell and neighbour cell on the same frequency layer, a UE capability is needed.

Applicability

When CSI-RS based L1-RSRP measurements are outside active BWP, measurement gap is needed. During measurement gap occasion, UE would retune to the centre frequency of the target CSI-RS resources to execute measurements. Within one measurement gap occasion, only the CSI-RS resources with the same center frequency, the same SCS and the same CP type are to be measured. We don’t think all CSI-RS resources outside BWP with different center frequency can be measured within one measurement gap occasion.

Proposal 10: Only the CSI-RS resources with the same center frequency, SCS and CP type can be measured within one measurement gap occasion.

Imitating R18 LTM CSI resource configuration structure, each CSI-RS resource on candidate cell can have flexible periodicity and time offset. From measurement perspective, it is expected that the measurement opportunity of the CSI-RS resource outside active BWP are covered in measurement gap. So there are some implicit restriction on CSI-RS resources configuration. e.g., on time domain the CSI-RS resources outside active BWP shall be aligned with measurement gap duration.

As we know, active BWP may be dynamically switched by DCI. While CSI-RS resource configuration is configured by RRC. So the same CSI-RS resource would be measured dynamically with measurement gap and without gap. So the restriction on CSI-RS resources outside active BWP shall be extended for within active BWP scenario. It shall be noted that such kind of restriction has no specification impact as these are up to network configuration.

Proposal 11: There are some implicit restrictions on CSI-RS resources configuration, e.g., the CSI-RS resources on time domain are supposed to be aligned with measurement gap. However this is network configuration, and there is no specification impact.

Conclusion

The contribution provides analysis on CSI-RS based L1 measurement. The following proposals are proposed:

Proposal 1: Keep L1 measurement for FR2-1 in the 2nd bullet.

Proposal 2: The CSI-RS from the target cell configured for L1 measurement remains measurable.

Proposal 3: SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB.

Observation 1: The CSI-RS based L1-RSRP measurement delay of skipping step2 would be N times (N is UE Rx beam number) than that of keeping of step 2.

Observation 2: Longer L1-RSRP periodicity of neighbour cell and short L1-RSRP periodicity of serving cell would result in “ping-pong” handover.

Observation 3: For CSI-RS L1-RSRP measurement, RX beam gain on serving cell and on neighbour cell would be different if skipping step 2.

Observation 4: It may have UE capability obstacle to directly use the L3 SSB information for CSI-RS L1 measurement, if step 2 is skipped.

Proposal 4: Measurement procedure for CSI-RS based LTM candidate cell measurements:

  - Step 1: SSB based L3 measurements and reports at least before LTM configuration.

  - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell

    - Step 2 can NOT be skipped for FR2-1.

    - Note: Step 2 is removed for FR1.

  - Step 3: CSI-RS based L1 measurements on the same candidate cell

    - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 2.

    - [Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.]

Proposal 5: Not to support CSI-RS resources with repetition ON in FR2 for CSI-RS based L1 RSRP measurement on candidate cell.

Proposal 6: A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

  - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of active DL BWP, and

  - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of active DL BWP, and

    - It is applied for SCS = 60KHz

  - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of any of the CSI-RS resources of the serving cell indicated for L1 measurement (if configured).

Proposal 7: Support L1 measurement with gap for CSI-RS based L1 measurement on neighbour cell.

Proposal 8: If the center frequency, SCS and CP length of the to-be-measured L1 CSI-RS resources are the same, they are regarded as the same CSI-RS frequency layer.

Proposal 9: If RAN4 agree to support the case where RTD is larger than CP between serving cell and neighbour cell on the same frequency layer, a UE capability is needed.

Proposal 10: Only the CSI-RS resources with the same center frequency, SCS and CP type can be measured within one measurement gap occasion.

Proposal 11: There are some implicit restrictions on CSI-RS resources configuration, e.g., the CSI-RS resources on time domain are supposed to be aligned with measurement gap. However this is network configuration, and there is no specification impact.

Reference

- RP-242356, Revised Work Item: NR mobility enhancements Phase 4

- R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1), Apple