3GPP TSG-RAN WG4 Meeting # 114	R4-2501233

Athens, Greece, February 17 – 21, 2025

Title: 	Discussion on general aspects in AIML mobility

Source: 	Huawei, HiSilicon

Agenda item:	7.21.1

Document for:	Discussion

# Introduction

The revised SID on AI/ML for mobility in NR agreed in [1]. For mobility enhancement, AI is introduced to predict the RRM measurement and event prediction. The identified use cases are AI based cell-level/beam-level measurement prediction, AI based HO failure/RLF prediction and AI based measurement event prediction. RAN4 is expected to evaluate testability, interoperability, and impacts on RRM requirements and performance.

Revised SID on AIML for mobility [1]: Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects: - AI/ML based RRM measurement and event prediction, - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2] - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2] - HO failure/RLF prediction (UE sided model) [RAN2] - Measurement events prediction (UE sided model) [RAN2] … - Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation

In RAN4#112bis meeting, RAN4 started measurement prediction study, and will carry out the discussion on measurement event prediction and HO failure/RLF prediction from 2025 if sufficient progress is achieved in RAN2 [2].

RAN4#112bis WF [2]: Issue 1-1-1: Priorities of different use cases Agreement: - RAN4 to start the discussion with RRM measurement prediction use-case from RAN4 #112bis meeting. Agreement: - RAN4 to only start the discussion of Measurement event prediction use-case and RLF use-case if sufficient progress in RAN2. - The earliest time to start the discussion for Measurement event prediction use-case is RAN4#114 meeting (Feb 2025). - The earliest time to start the discussion for RLF use-case is from RAN4#114bis meeting (Apr 2025).

This contribution focuses on general aspects in AI/ML mobility.

# 2. General aspects of AI mobility in RAN4

## 2.1 Scenario and sub-use case priorities

According to TR 38.744, scenarios 2, 3 and 4 are considered with high priority in RAN2. These scenarios are also selected to start the study in RAN4 at RAN4 #112bis. According to Moderator’s summary in RAN4 #113 [3], scenario and sub-use case priorities in RAN4 are suggested to be further discussed.

- Scenario priority

Table 5.2.1-1 in TR 38.744: Scenario number; Priority; Evaluation scenario; Target study goal; Methodology. 1; Low; FR1 to FR1 intra-frequency temporal domain case A; 2nd goal; TBD. 2; High; FR1 to FR1 intra-frequency temporal domain case B; 1st goal; Intra-cell. 3; High; FR1 to FR1 inter-frequency (frequency domain); 1st goal; Inter-cell. 4; High; FR2 to FR2 intra-frequency temporal domain case A; 2nd goal; Intra-cell. 5; Low; FR2 to FR2 intra-frequency temporal domain case B; 1st goal; TBD. 6; Middle; FR2 to FR2 intra-frequency spatial domain; 1st goal; Intra-cell. Moderator summary in RAN4 #113 [3]: Companies to discuss if Proposal 1 is agreeable. - Proposal 1: RAN4 to include FR2 to FR2 intra-frequency spatial domain scenario 6 into the scope of the Rel-19 study.

For scenario priority, it is observed that in FR2, the goal of handover performance improvement goes with the first priority in RAN2 compared to goal of measurement reduction. RAN4 can follow RAN2’s priority and start to study scenario 6 if RAN2 achieves sufficient progress. In addition, similar issues related to scenario 6 are under study in Rel-19 AI beam management, including testability of multiple AoAs for test setup. The ground truth definition and minimum requirement definition of L3-RSRP in other scenarios can also be reused in scenario 6. In order to avoid duplicate work, RAN4 can directly leverage the work from other scenarios if needed.

Proposal 1: RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress.

- Sub-use case priority

Moderator summary in RAN4 #113 [3]: - Proposals and Observations: - Proposal 1 (Nokia): RAN4 to consider the sub-use case 2 (L3 Cell-level measurement result(s) are predicted based on actual L3 cell-level measurement result(s)) as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction. - Proposal 2 (Ericsson): For intra-frequency intra-cell temporal prediction scenarios (scenario 2 and 4 agreed in RAN4#112bis), RAN4 to prioritize evaluation of impact of sub-use case 2 on RAN4 requirements. - Proposal 3 (Ericsson): Update agreement from RAN4#112bis on sub-use cases for cell level measurement prediction as: - Scenario 2, 3 and 4, the 3 sub-cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed. - Proposal 4 (Qualcomm): RAN4 to consider all three sub-use cases, with higher prioritization given to sub-use case 2 and 3. - Recommended WF: - Companies to further discuss above listed proposal.

According to Moderator’s summary in RAN4 #113 [3], some companies suggest to prioritize sub-use case 2 or sub-use case 2 and 3. In sub-use case 2, the model output is the prediction of L3 cell-level measurement result. In sub-use case 1 and 3, prediction of L3 cell-level measurement result needs to be derived based on model output. Though the eventual KPI is calculated regarding to L3 cell-level measurement prediction accuracy, requirement of prediction accuracy may be different due to different model inputs and outputs. NW may require different prediction accuracy or different side conditions for different purposes. Considering that RAN2 did not priority any sub-use case, it is suggested that RAN4 to consider all three sub-use cases.

Proposal 2: RAN4 to consider all three sub-use cases until RAN2 gives the priority.

## 2.2 Training and testing dataset

In RAN2, the AI model is trained by system level channel model and verified in terms of the accuracy of measurement prediction under the same channel model. However, in RAN4 legacy, only link level channel model is considered for defining test cases. There are two potential testing options for RAN4 in AI RRM measurement prediction.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy.  Pros and Cons need to be analyzed.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

There are two options for defining the testing and training dataset. One is using the channel model and the sampling rules. Another is that the dataset is specified or partially specified in a sample-by-sample manner.

Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

# 3. Conclusion

The contribution provides analysis on RRM measurement prediction in AI/ML mobility. The following proposals are proposed:

Proposal 1: RAN4 to start the study of other scenarios than scenarios 2,3,4 if RAN2 achieves sufficient progress.

Proposal 2: RAN4 to consider all three sub-use cases until RAN2 gives the priority.

Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy.  Pros and Cons need to be analyzed.

  - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4.

  - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4.

Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset.

  - Option 1: Use the channel model and the sampling rules.

  - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner.

# 4. Reference

- RP-242393, “Revised SID on AIML for mobility in NR”, 3GPP RAN#105, September 9-12, 2024.

- R4-2416855, “WF on AI/ML Mobility”, 3GPP RAN4#112bis, 14th – 18th October, 2024.

- R4-2418278, “Topic summary for [213][220] FS_NR_AIML_Mob”, 3GPP RAN4#113, 18th – 22th November, 2024.