3GPP TSG-RAN WG4 Meeting #114	R4-2501246

Athens, Greece, 17 – 21 February, 2025

Title: 	(NR_MG_enh-Core) Discussion on gap sharing for NCSG

Source: 	Huawei, HiSilicon

Agenda item:	4.5.2

Document for:	Discussion

Introduction

In RAN4#113, one company proposed [1] to add gap sharing definition for NCSG, but some companies commented that this is an issue that has not been discussed in R17.

In this paper, we will provide our views on gap sharing for NCSG.

Discussion

In R15, gap sharing is defined for measurement within MG. It is used to control how to distribute the MG occasions between different types of MOs, e.g. intra-frequency and inter-frequency MOs. For NR SA, it is defined as follows in clause 9.1.2.1a.

When network signals “01”, “10” or “11” with RRC parameter MeasGapSharingScheme [2] and the value of X is defined as in table 9.1.2.1a-1, and - Kintra = 1 / X * 100, - Kinter = 1 / (100 – X) * 100, When network signals “00” indicating equal splitting gap sharing, X is not applied. The RRC parameter MeasGapSharingScheme shall be applied to the calculation of carrier specific scaling factor as specified in clause 9.1.5.2.2. Table 9.1.2.1a-1: Value of parameter X for NR standalone measurement gap sharing measGapSharingScheme; Value of X (%). ‘00’; Equal splitting. ‘01’; 25. ‘10’; 50. ‘11’; 75. NOTE: It is left to UE implementation to determine which measurement gap sharing scheme in the table to be applied, when MeasGapSharingScheme is absent and there is no stored value in the field..

Gap sharing then impacts how CSSF is calculated as defined in e.g. clause 9.1.5.2.2 for NR SA.

For each measurement gap j used for a long-periodicity measurement defined above, Mintra,i,j = Minter,i,j = Mtot,i,j =0. The carrier specific scaling factor CSSFwithin_gap,i is given by: If measGapSharingScheme is equal sharing, CSSFwithin_gap,i= max(ceil(Ri×Mtot,i,j)), where j=0…(160/MGRP)-1 If measGapSharingScheme is not equal sharing and - measurement object i is an intra-frequency measurement object, CSSFwithin_gap,i is the maximum among - ceil(Ri×Kintra×Mintra,i,j) in gaps where Minter,i,j≠0, where j=0…(160/MGRP)-1 - ceil(Ri×Mintra,i,j) in gaps where Minter,i,j=0, where j=0…(160/MGRP)-1 - measurement object i is an inter-frequency or inter-RAT measurement object or NR PRS measurement on any one positioning frequency layer, CSSFwithin_gap,i is the maximum among - ceil(Ri×Kinter×Minter,i,j) in gaps where Mintra,i,j ≠0, where j=0…(160/MGRP)-1 - ceil(Ri×Minter,i,j) in gaps where Mintra,i,j=0, where j=0…(160/MGRP)-1 Where Ri is the maximal ratio of the number of measurement gap where measurement object i is a candidate to be measured over the number of measurement gap where measurement object i is a candidate and not used for a long-periodicity measurement defined above.

Coming to R17 NCSG, we agree that there was no specific discussion on gap sharing for NCSG, but we understand that CSSF for NCSG should be defined in same way as CCSF for MG. In clause 9.1.5.3.1, CSSF for NCSG is also defined with gap sharing and based on signalling measGapSharingScheme.

The carrier specific scaling factor CSSFwithin_ncsg,i is given by: If measGapSharingScheme is equal sharing, CSSFwithin_ncsg,i= max(Mtot,i,j), where j=0…(160/VIRP)-1 If measGapSharingScheme is not equal sharing and - measurement object i is an intra-frequency measurement object, CSSFwithin_ncsg,i is the maximum among - ceil(Kintra×Mintra,i,j) in NCSG occasions where Minter,i,j≠0, where j=0…(160/VIRP)-1 - Mintra,i,j in NCSG occasions where Minter,i,j=0, where j=0…(160/VIRP)-1 - measurement object i is an inter-frequency or inter-RAT measurement object, CSSFwithin_ncsg,i is the maximum among - ceil(Kinter×Minter,i,j) in NCSG occasions where Mintra,i,j ≠0, where j=0…(160/VIRP)-1 - Minter,i,j in NCSG occasions where Mintra,i,j=0, where j=0…(160/VIRP)-1

In RAN2 signaling, measGapSharingScheme can be configured to NCSG.

Technically, similar to MG, NCSG can be used for intra-frequency and inter-frequency MOs, so motivation to control gap sharing for MG also applies for NCSG. We do not see clear reason to define a new gap sharing scheme for NCSG, and our understanding has been that MG sharing scheme also applies for NCSG. We suggest to confirm this understanding in RAN4.

Proposal: Confirm that MG sharing scheme based on measGapSharingScheme also applies for NCSG.

We are open to discuss whether and how to update the spec.

Conclusions

In this paper we provided provide our views on gap sharing for NCSG.

Proposal: Confirm that MG sharing scheme based on measGapSharingScheme also applies for NCSG.

Reference

- R4-2418779, (NR_MG_enh-Core) CR on RRM requirements for NCSG, OPPO