3GPP TSG-RAN WG4 Meeting #114	R4-2501251

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on remaining issues for R18 MG Enhancement

Source: 	Huawei, HiSilicon

Agenda item:	5.12.1

Document for:	Discussion

Introduction

RRM requirements maintenance for R18 MG enhancement are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed.

- Deactivated SCell measurements with NCSG

- DRX in NFG operation

In this paper, we will provide our views on remaining issues in R18 MG enhancement.

Discussion

Deactivated SCell measurements with NCSG

< Agreement >: Agreement from RAN4 meeting#111: - For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association]. - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed. - For UE configured with 2 NCSG, deactivated SCells are measured with NCSG - If the association is provided, deactivated SCells are measured with NCSG according to gap association. - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case. Agreement from RAN4 meeting#112: - When UE switches measurement between NCSG and Type 2 MG, no additional processing delay is expected.. Issue 1-1-1: When the UE is configured with Concurrent gaps with NCSG, whether all deactivated SCells are measured within NCSG, regardless the gap association? - Further discuss in the next meeting. Issue 1-1-2: When the measurement of the measurement object is related to a deactivated SCell, and after this SCell is activated, and when UE is configured with the concurrent measurement gap and NCSG, what is the expected UE behaviour? - Further discuss in the next meeting

We support to remove the [] around “and gap association”. In our view, it could happen that the SCell MO is associated to a MG because when the SCell is activated the active BWP may not cover SSB. In this case, the SCell MO should also be measured within NCSG when the SCell is deactivated to avoid interruption outside NCSG, and it is in fact an important use case. It means the gap association will be ignored when the SCell is deactivated. We do not see any technical issue in it because no matter UE measures the SCell MO outside MG (R15) or within NCSG (R18), the gap association are both ignored when SCell is deactivated.

We also do not see the need to introduce extra processing delay for switching between NCSG and Type 1/2 MG. In R15, the switching between measurement outside MG and within MG could already happen following a BWP switching, and it is no different from switching between measurement outside MG and within NCSG (when SCell MO is measured outside MG when SCell is activated), or between measurement within MG and within NCSG (when SCell MO is measured within MG when SCell is activated).

Proposal 1: For UE configured with one NCSG and one Type 1/2 MG, all deactivated SCells are measured within NCSG, regardless of the reported UE capabilities and gap association.

After the SCell is activated, the SCell MO should be measured depending on whether MG/NCSG is needed.

- If the SSB is contained within the active BWP, or if the UE supports ‘nogap-noncsg’ for the SCell, the SCell MO should be measured outside MG/NCSG.

- If the SSB is outside the active BWP, and if the UE does not support ‘nogap-noncsg’ for the SCell, the SCell MO should be measured with MG/NCSG, and which MG or NCSG should be used depends on the configured gap association.

We understand above is already clear based on clause 9.2.1 of 38133.

Proposal 2: For UE configured with one NCSG and one Type 1/2 MG, after the SCell is activated

- If the SSB is contained within the active BWP, or if the UE supports ‘nogap-noncsg’ for the SCell, the SCell MO should be measured outside MG/NCSG.

- If the SSB is outside the active BWP, and if the UE does not support ‘nogap-noncsg’ for the SCell, the SCell MO should be measured with MG/NCSG, and which MG or NCSG should be used depends on the configured gap association.

No spec change is needed for clarifying the UE behavior.

DRX in NFG operation

Issue 2-1-1: Misalignment between DRX-on duration and SMTC for NFG measurements? - Background: - Agreement from RAN4#112 meeting: - Interruptions are not allowed during DRX ON duration - Exclude the time extended due to drx-inactivityTimer - if there is no SMTC occasion within a time period starting [4ms] before the starting point of the DRX ON duration and ending [4ms] after the ending point of the DRX ON duration. - Proposal: - Option 1: - Revise the value of the time period before and after the DRX-ON cycle for which the presence of SMTC allows interruptions during the DRX-ON period to 1 ms.

We support option 1. Option 1 will impose stricter limitation on UE implementation regarding where to retune RF chain around SMTC window. On the other hand, we agree with the analysis in [2] that the current value of 4ms will lead to a big limitation on NW side regarding where to place an interruption-free DRX ON duration in time domain. On this issue, we understand more weight should be given to NW concern because the interruption during DRX ON duration will also cause degradation to user experience at UE side, while there is more freedom for UE to plan the RF retuning to avoid interruption.

Proposal 3: Revise the value of the time period before and after the DRX ON duration for which the presence of SMTC allows interruptions during the DRX ON duration to 1 ms.

Conclusions

In this paper we provided provide our views on remaining issues in MG enhancement.

Proposal 1: For UE configured with one NCSG and one Type 1/2 MG, all deactivated SCells are measured within NCSG, regardless of the reported UE capabilities and gap association.

Proposal 2: For UE configured with one NCSG and one Type 1/2 MG, after the SCell is activated

- If the SSB is contained within the active BWP, or if the UE supports ‘nogap-noncsg’ for the SCell, the SCell MO should be measured outside MG/NCSG.

- If the SSB is outside the active BWP, and if the UE does not support ‘nogap-noncsg’ for the SCell, the SCell MO should be measured with MG/NCSG, and which MG or NCSG should be used depends on the configured gap association.

No spec change is needed for clarifying the UE behavior.

Proposal 3: Revise the value of the time period before and after the DRX ON duration for which the presence of SMTC allows interruptions during the DRX ON duration to 1 ms.

Reference

- R4-2420213, WF on RRM requirements for NR_MG _enh2, MediaTek.

- R4-2418884, Remaining aspects for NR_MG_enh2, Nokia