3GPP TSG-RAN WG4 Meeting #114	R4-2501253

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on remaining issues in R18 NTN core requirements

Source: 	Huawei, HiSilicon

Agenda item:	5.21.3

Document for:	Approval

Introduction

RRM requirements for R18 NTN maintenance are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussion, following issues needs to be further discussed.

- Te requirements for FR2-NTN

- Gradual timing adjustment

- Soft satellite switch

In this paper, we will provide our views on remaining issues in R18 NTN core requirements.

Discussion

Te requirements for FR2-NTN

In RAN4#113, CR [2] was endorsed. Table 7.1C.2-3 is copied below with all changes accepted. The Te for 120kHz is tightened from 10Ts to 7.5Ts, and the side conditions are FFS.

Table 7.1C.2-3: Te_NTN Timing Error Limit for mobile VSAT is served by GSO

| Frequency Range | SCS of SSB signals (kHz) | SCS of uplink signals (kHz) | Te_NTN |
| --- | --- | --- | --- |
| FR2-NTN | 120 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
|  | 240 | 60 | 13*64*Tc |
|  |  | 120 | 7.5*64*Tc |
| Note 1: Tc is the basic timing unit defined in TS 38.211 [6] Note 2: The value of Te_NTN for 120 kHz UL SCS assumes LOS, and GNSS signal power level above [TBD] dBm [and max acceleration of [TBD] m/s2] FFS other side conditions if needed. |  |  |  |

In our understanding, the new requirements of 7.5Ts corresponds to X+Y ~= 20m, where X is the UE GNSS error and Y is the satellite location error. Assuming Y = 5m, UE is required to meet a GNSS error X = 15m, which may be challenging in some scenarios, so GNSS related side conditions were discussed.

On GNSS signal power level, we suggest to refer to Table 6.2-1 of 38.171, which defines the signal power level for nominal accuracy for each GNSS system. As stated in 38.171, “nominal accuracy requirement verifies the accuracy of A-GNSS position estimate in ideal conditions”, we believe it is the most relevant conditions for a 15m GNSS error.

The other side conditions are related to UE movement. When UE is fixed or in uniform rectilinear motion, a good GNSS performance can be guaranteed, but it is challenging when UE is (de)accelerating or changing its moving direction as it is difficult to predict the location based on historical measurement.

For (de)acceleration, we suggest to consider additional moving distance due to (de)acceleration <= [5]m in one second. This translates into (de)acceleration a = 10 m/s2. In last meeting, some companies commented that it may be unreasonable to use instantaneous (de)acceleration as the side condition because the time duration of (de)acceleration can be rather short, causing minor impact to the actual moving distance. To address the comment, we suggest to use the velocity change in one second <= 10m/s to define the condition.

For moving direction change, we suggest to consider the moving distance change with and without direction change < = 15m. The allowed angular change depends on the velocity, e.g. if we consider aircraft moving at 900km/h, the allowed angular change would be much smaller compared to a car moving at 120km/h. We suggest to define the limit based on 120km/h assumption, as we understand vehicles moving at higher speed will have enhanced location system, i.e. they should be able to meet the 15m GNSS error without a limit on moving direction change. The angular limit is 15 degree for 120km/h velocity, i.e. the moving direction change within one second should be smaller than 15 degree.

Proposal 1: Define the following side conditions for Te_NTN for 120 kHz UL SCS.

- GNSS signal power level is>= those defined in Table 6.2-1 of 38.171

- UE velocity change within one second is <= 10m/s

- UE moving direction change within one second is <= 15 degree

Gradual timing adjustment

In last meeting, some companies raised concerns over the gradual timing adjustment requirements. The concern is that the timing change due to UE movement (UE location change) is subject to restrictions of Tp and Tq, and current Tq value does not account for high UE moving speed such as 900km/h.

Our understanding is that it is important to keep timing change due to UE movement (UE location change) is subject to restrictions of Tp and Tq unless RAN4 redefines the gradual timing adjustment framework. The reason is that NW may use TA command to correct the timing error caused by GNSS error at UE side, which may be random at each GNSS fix, and if timing change due to UE location change is not restricted by Tp and Tq, there can be large “timing jump” at GNSS fix, which causes NW commanded TA to be invalid.

To resolve the concern raised to not to break the existing principle, our proposal is to differentiate a) “maximum amount of the magnitude of the timing change” and b) “maximum aggregate adjustment rate per 200ms”. Currently, both a) and b) are limited by Tq.

- To keep existing principle (to avoid “timing jump” at GNSS fix), a) should be kept small, and we are open to discuss if current value of 2.5Ts is suitable or needs to be enlarged.

- To account for max UE moving speed as 900km/h, b) needs to be enlarged. Following same derivation as in R17 FR2 HST, b) need to be enlarged to 7.5Ts.

Proposal 2: RAN4 to define different values for the following gradual timing adjustment limits:

- maximum amount of the magnitude of the timing change: same as current Tq

- maximum aggregate adjustment rate per 200ms: enlarged to 7.5Ts

Soft satellite switch

### Issue 5-2-S: Soft Satellite switch FFS: - A UE that supports soft satellite switching shall mandatorily support parallelMeasurementWithoutRestriction-r17.

We do not see strong motivation to define the binding of two features. Having such a binding will limit the use of the soft switch feature. In any case, it is up to UE to decide how to report its capability – if a UE considers it not attractive to support soft switch without supporting parallel measurement, it can choose to indicate no support of soft switch, but there is no need to mandate such capability bindings in the spec.

On the other hand, even UE causes scheduling restriction, it does not mean NW will stop scheduling during the whole t-serviceStart to t-Service, e.g. NW may have its own estimation of PDD. For example, assuming the two satellites before and after switching are on the same plane, the largest PDD would occur when UE sees one satellite with 90-degree elevation angle and sees another satellite with small elevation angle such as 30 degree. For LEO600, this gives largest PDD as 2ms (distances to two satellites are 600km and 1200km). Even adding this uncertainty, the scheduling restriction is still shorter than SMTC periodicity, i.e. there is still gain from soft switching.

In last meeting, some companies mentioned that in worst case, NW will stop scheduling during the whole t-serviceStart to t-Service. In this case, the soft switch will be similar to hard switch in terms of interruption time when the duration between t-serviceStart and t-Service is not longer than 2 SMTC periods. In real deployment, we understand this would be the typical case (the duration between t-serviceStart and t-Service would be small).

Proposal 3: Do not define parallelMeasurementWithoutRestriction-r17 as pre-requisite for soft satellite switching capability.

RAN2#128 sent an LS [2] related to soft switching with the following question.

ACTION: RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

In our view, during soft switch UE maintains 2 timing for the serving cell: one timing for source satellite which UE applies before t-Service and another timing for target satellite which UE applies after t-Service.

The procedure mentioned in RAN2 LS essentially means that the offset of the SSB on which UE obtains the second timing is changed at t-Service. RAN2 question is whether the timing obtained with SSB offset #1 is maintained when the SSB offset is changed to offset #2, and our view is “yes”.

In our understanding, after obtaining the timing of a cell, UE knows where the start of SFN0 of the cell is, so UE knows where to receive every signal or channel that is referenced to SFN0. The timing is not related to a specific reference signal, e.g. after UE obtains timing based on SSB, it should be able to receive CSI-RS of the same cell. The timing is not related to specific reference signal parameter, e.g. if the time offset of TRS is re-configured, UE does not lose the timing of the cell and is still able to receive other signal or channel based on the obtained timing.

Based on above analysis, we suggest RAN4 to confirm with RAN2 that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service

Proposal 4: RAN4 to confirm the following to RAN2:

the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

A draft reply LS is provided in the Annex.

Conclusions

In this paper we provided provide our views on remaining issues in R18 NTN core requirements.

Proposal 1: Define the following side conditions for Te_NTN for 120 kHz UL SCS.

- GNSS signal power level is>= those defined in Table 6.2-1 of 38.171

- UE velocity change within one second is <= 10m/s

- UE moving direction change within one second is <= 15 degree

Proposal 2: RAN4 to define different values for the following gradual timing adjustment limits:

- maximum amount of the magnitude of the timing change: same as current Tq

- maximum aggregate adjustment rate per 200ms: enlarged to 7.5Ts

Proposal 3: Do not define parallelMeasurementWithoutRestriction-r17 as pre-requisite for soft satellite switching capability.

Proposal 4: RAN4 to confirm the following to RAN2:

the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

Reference

- R4-2420235, WF on RRM requirements for NR_NTN_enh, Moderator (Qualcomm Incorporated)

- R4-2420236, Corrections to UE transmit timing for Satellite Access, Eutelsat Group, Thales, European Space Agency, Sharp, CHTTL, Airbus

- R2-2410978, LS on soft satellite switch with re-sync, RAN2

Annex: draft reply LS on soft satellite switch with re-sync

3GPP TSG-RAN WG4 Meeting #114 R4-250xxxx Athens, Greece, 17 – 21 February, 2025 Title: reply LS on soft satellite switch with re-sync Response to: R2-2410978 Release: Release 18 Work Item: NR_NTN_enh-Core Source: RAN4 To: RAN2 Cc: RAN1 Contact Person: #### Name: Li Zhang - E-mail Address: zhangli164@huawei.com Attachments: - 1. Overall Description: RAN4 thanks RAN2 for the information in LS R2-2410978. Regarding the following question RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.. RAN4 reached the conclusion that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service. RAN4 respectfully asks RAN2 to take the above information into account in the future work for soft satellite switch with re-sync. 2. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account in the future work for soft satellite switch with re-sync. 3. Date of Next TSG-RAN4 Meetings: RAN WG4 Meeting #114-bis April 07 – April 11, 2025 TBD, China RAN WG4 Meeting #115 May 19 – May 23, 2025 Malta, MT