3GPP TSG-RAN WG4 Meeting #114	R4-2501256

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on remaining issues in core requirements for R18 positioning

Source: 	Huawei, HiSilicon

Agenda item:	5.22.1

Document for:	Discussion

Introduction

There is one remaining issue in core requirements for R18 positioning:

- End point of SL-PRS based RSTD measurement period requirements

The issue was discussed in RAN4#113 without conclusion. In this paper, we will provide our views on remaining issues in core requirements for R18 positioning.

Discussion

The issue of end point of SL-PRS based RSTD measurement period requirements was summarized in [1].

Issue 2-1: Clarification for SL-RSTD measurement period requirement - Proposals - Option 1: E/// - A clarification may be in the SL RSTD measurement period that the requirement applies, provided that the SL-PRS from the reference UE is available within TSL RSTD,total. - Recommended WF - Clarify the following in the SL-RSTD measurement period requirement. - SL-RSTD measurement period requirement applies provided that the SL-PRS from the reference UE is available within TSL RSTD,total. - Companies to check proposed clarification in R4-2419701.

The following was agreed in RAN4#110-bis.

#### Issue 1-1-1: End point of SL-PRS based RSTD measurement period requirements Agreements: - The SL RSTD measurement period ends after the UE has measured SL PRS resources from at least two different Tx UEs including target and reference UEs. - FFS whether any updates to the TS 38.133 are needed.

In our understanding, option 1 from last meeting means for a Tx UE (can be reference UE or other anchor UE), the measurement requirements apply only when the SL-PRS from the reference UE is available within the measurement period of this UE. Based on the definition of the measurement period, it further means the SL-PRS from the reference UE should occur no later than ΔSLproc after the last sample of SL-PRS from the Tx UE, otherwise there would be no requirement. ΔSLproc is a UE capability ranging from 20ms to 160ms.

minTimeAfterEndofSlotCarryActiveSL-PRS-Resources    ENUMERATED {ms20, ms30, ms40, ms50, ms80, ms100, ms160}

We do not support option 1 for clarifying the SL RSTD requirements as it imposes unnecessary restrictions on the applicability. Assume there are 3 Tx UEs, namely Tx UE#1, Tx UE#2 and Tx UE#3, transmitting SL-PRS at 0, 40ms and 80ms, respectively, and ΔSLproc for the Rx UE is 20ms. Technically, there is nothing wrong if the Rx UE selects Tx UE#3 as the reference UE, and the requirements should apply for each Tx UE. With option 1, however, the requirement would not apply for Tx UE#1 or Tx UE#2 for no technical reason. It is noted that selection of reference UE is up to UE implementation.

Another issue with option 1 is that it does really capture the agreement from RAN4#110-bis. Even with the proposed applicability, there are still per Tx UE requirements, but it is unclear when SL RSTD measurement period ends. To move forward, we suggest to simply capture the RAN4#110-bis agreement in the spec with some wording update (to make the wording aligned with existing spec).

Proposal: Capture the following texts based on RAN4#110-bis agreement to SL RSTD requirements.

The SL RSTD measurement period ends after the UE has measured SL-PRS resources from multiple anchor UEs including reference UEs and one or more other anchor UEs.

The text proposal is as below, and can also be found in our companion CR.

When the UE physical layer receives the last of SL-TDOA-ProvideAssistanceData and SL-TDOA-RequestLocationInformation from LMF or another UE via SLPP specified in TS 38.355 [37], the UE shall be able to perform multiple SL RSTD measurements based on SL-PRS from one or more other SL UEs (up to the UE capability specified in clause 12A.2.3), with each SL RSTD measurement based on SL-PRS from the reference UE and SL-PRS from another anchor UE, as defined in TS 38.215 [4]. The SL RSTD measurement for each anchor UE shall be performed during the measurement period TSL RSTD,total defined as: TSL RSTD,Total=s=1STSL RSTD,effect,s , where S is the number of samples per measured link, defined below: S = 1 for SL-PRS bandwidth>48 PRBs, S = 4 for SL-PRS bandwidth≤48 PRBs, and for each SL-PRS sample s of the target measured link, which is received within a slot where the UE receives SCI and the associated SL-PRS within its capabilities reported by UE via maxNumOfActiveSL-PRS-ResourcesInOneSlot-r18 and maxNumOfSlotsWIthActiveSL-PRS-Resources-r18 specified in TS 38.331 [2], TSL RSTD,effect,s is defined as: TSL RSTD,effect,s=ts+1-ts , for s<S, where ts+1 and ts are the beginning of the first slot of SL-PRS sample s+1 and SL-PRS sample s, respectively, TSL RSTD,effect,s=Tdur,s+ΔSLproc , for s=S, Tdur,s is the duration of the slot carrying SL-PRS sample s of the SL RSTD measurement, ΔSLproc is the processing time reported by the UE via minTimeAfterEndofSlotCarryActiveSL-PRS-Resources-r18 specified in TS 38.331 [2]. The SL RSTD measurement period ends after the UE has measured SL-PRS resources from multiple anchor UEs including reference UEs and one or more other anchor UEs. A UE may drop one or more SL PRS measurement samples if the number of active slots or the number of active resources per slot for the ongoing SL PRS measurement exceed the UE capabilities reported by UE via sl-PRS-CommonProcCapabilityPerBand-r18 specified in TS 38.331 [2]. For a single-sample measurement, the whole measurement may not be performed. If the synchronization reference source changes during TSL RSTD,total at the measuring UE, while the UE is performing the SL RSTD measurement, then the UE shall restart the SL RSTD measurement after the synchronization reference source change and shall send the measurement report during a measurement period, which can be longer than TSL RSTD,total. The requirements in this clause do not apply, when the synchronization reference source changes during TSL RSTD,total at the UE transmitting SL-PRS for the SL RSTD measurement. The requirements in this clause apply, provided that no SL-PRS symbols are dropped due to, e.g., selection or reselection of synchronization reference source according to clause 12.4 during the measurement period TSL RSTD,total. Otherwise, the measurement period can be longer. The requirements in this clause apply, provided that the reception of slots containing SL-PRS is not interrupted during the measurement period TSL RSTD,total. Otherwise, if the reception of the slots containing SL-PRS is interrupted, the measurement period can be longer.

Conclusions

In this paper we provided provide our views on remaining issues in core requirements for R18 positioning.

Proposal: Capture the following texts based on RAN4#110-bis agreement to SL RSTD requirements.

The SL RSTD measurement period ends after the UE has measured SL-PRS resources from multiple anchor UEs including reference UEs and one or more other anchor UEs.

Reference

- R4-2418269, Topic summary for [113][211] NR_pos_enh2_Part1, Moderator (Ericsson)