3GPP TSG-RAN WG4 Meeting #114	R4-2501261

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on performance part for less than 5MHz Ph2

Source: 	Huawei, HiSilicon

Agenda item:	7.4.4

Document for:	Discussion

Introduction

RRM core requirements for R19 less than 5MHz are closed in RAN4#113, and the Big draftCR is endorsed in [1]. Based on the work plan, the performance part of the WI starts from this meeting.

In this paper, we will provide our views on the performance part for less than 5MHz Ph2.

Discussion

The impacts of the WI to the RRM core requirements include

- SSB index reading in measurement

  - EMR with beam level reporting

  - SCell and PSCell measurement with beam level reporting (spec impact is for deactivated cell)

- HO with PSCell

  - Normal HO with PSCell (no spec change)

  - CHO with target MCG and SCG

- SCell and PSCell related procedure

  - SCell activation and SCG activation

  - PSCell addition, conditional PSCell addition, subsequent conditional PSCell addition

  - PSCell change (no spec change), conditional PSCell change, subsequent conditional PSCell change

It is clear that there is no impact to the accuracy requirements, so the performance work should focus on test cases for the defined core requirements.

We do not support to define test cases for 1 because currently there is no test case for beam level reporting in EMR, and there is no test case for SCell or PSCell measurement. We do not see strong need to test them for less than 5MHz if they are not tested with normal BW. Technically, since the SBI reading performance for 12 PRB SSB is verified for PCell and for inter-frequency measurement (via R18 test cases), we can safely assume UE will meet the requirements for SCell and PSCell including candidate SCell and PSCell (EMR) because the impact is same and the SINR side condition is same.

We support to define test case for 2a. There is test case for HO with PSCell for normal BW (A.6.3.1.17), and the delay performance to with a 3MHz PSCell cannot be verified via any R18 test case. Then we do not need to test 2b because the performance for PSCell preparation would be same no matter the PCell HO is directly triggered by HO command or conditional.

We support to define test case for 3a SCell activation. SCell activation is an important procedure and the delay performance is not verified with any R18 test case. We do not support to test SCG activation because the procedure is similar to SCell addition, and the impact of 12 PRB SSB on T∆ is also same.

For 3b and 3c, we suggest to pick up one SCell addition procedure and one SCell change procedure to test. Technically, the impact of 12 PRB SSB is independent of the PSCell related procedure, whether it is addition or change, or whether it is directly commanded, conditional or conditional subsequent. Considering the existing test cases, we suggest to test normal PSCell addition and subsequent conditional PSCell change.

Based on above discussions, the suggested test list is summarized in Proposal 1.

Proposal: Define the following list of test cases for less than 5MHz Ph2

- TC1: HO with PSCell (PSCell with 12 PRB SSB)

- TC2: SCell activation (SCell with 12 PRB SSB)

- TC3: PSCell addition (PSCell with 12 PRB SSB)

- TC4: subsequent conditional PSCell change (PSCell with 12 PRB SSB)

Conclusions

In this paper we provided provide our views on the performance part for less than 5MHz Ph2.

Proposal: Define the following list of test cases for less than 5MHz Ph2

- TC1: HO with PSCell (PSCell with 12 PRB SSB)

- TC2: SCell activation (SCell with 12 PRB SSB)

- TC3: PSCell addition (PSCell with 12 PRB SSB)

- TC4: subsequent conditional PSCell change (PSCell with 12 PRB SSB)

Reference

- R4-2420241, Draft Big CR for less than 5MHz RRM requirements in Rel-19, Intel Corporation, ZTE Corporation, Sanechips