3GPP TSG-RAN WG4 Meeting #114	R4-2501264

Athens, Greece, 17 – 21 February, 2025

Title: 	Simulation assumption and additional simulation results for LP-WUR measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.26.4.1

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to evaluate the link level performance of LP-WUR measurement. Part of simulation assumption was agreed in RAN4#112 in [2], and we have provided our initial simulation results for fading channel in [3].

In this paper, we will provide our views on the remaining issues for simulation assumption and also provide updated simulation results.

Discussion

Simulation assumption

Issue 2-1-1: SINR setting

In our view, RAN4 should use -3dB SINR as side condition of accuracy requirements for LR as baseline. The values provided by RAN1 are for different NF assumptions, and we believe RAN4 should take the worst-case assumption for defining requirements, which corresponds to -3dB SINR.

We understand RF session may take different assumptions for OOK based and OFDM based LR for defining the refsense requirements, but it does not mean RRM requirements should be defined with different side conditions. In particular, if we can assume NF for OFDM based LR will be smaller than that for OOK based LR, we can have 2 options:

- Option 1: define side condition for OOK based LR as -3dB and for OFDM based LR as -0.5dB

- Option 2: define side condition for OOK and OFDM based LR as -3dB

Option 1 assumes the coverage of both LRs is same, so OFDM based LR is only required to work in a better SINR side condition than OOK based LR. However, OFDM based LR is supposed to be more powerful than OOK based LR, and we should not relax its requirements for baseband measurement performance just because it has a better NF. Instead, it is more reasonable to assume OFDM based LR is able to work in the same SINR side condition as OOK based LR as in option 2, and the smaller NF can translate into better coverage, i.e. the feature can be more widely used in a cell.

Proposal 1: As baseline, RAN4 to use -3dB SINR as side condition of accuracy requirements for both OOK based and OFDM based LR.

Issue 2-1-4: On LP-SS sequence for simulation purpose

LP-SS measurement performance may be impacted by the binary sequence design, including the modulation scheme (OOK-1 and OOK-4 with different M values), the sequence length (L) and whether coding is used. It is noted that different combinations of {M,L} will translate into different number of OFDM symbols for LP-SS. In RAN1#119 the following was agreed regarding the parameters for LP-SS sequence.

Agreement For the length L of LP-SS binary sequence, limit the selection to the following: - M=1, L= {4, 6, 8} - M=2, L= {8, 12, 16} - M=4, L= {16, 32}

It can be seen that there are still many combinations, and we expect RAN1 will make further down selection the combinations and decide the set of sequences for each supported combination. At this stage, we suggest RAN4 to wait for RAN1 conclusion on the detailed sequence design because RAN4 requirements will be based on RAN1 agreed sequences, and even we agree some specific sequences for simulation purpose, we will need to re-run the simulations based on RAN1 agreed sequences to derive the requirements.

Proposal 2: RAN4 to wait for RAN1 conclusion on the detailed sequence design before deciding the exact sequence for simulation.

Issue 2-1-5: On ideal RSRP/RSRQ in simulation

In our view, the ideal RSRP should be based on the serving cell signal after fading channel. By definition, it is the received power on certain time or time/frequency resources, and the received power is impacted by the channel fading, so the ideal RSRP should be based on actual channel fading instead of statistic attenuation. Besides, we understand it has been the principle in R15 simulation to derive the normal RRM accuracy.

Similarly, for ideal RSRQ, the RSSI part should be based on the actual total received power at the specific time or time/frequency resources instead an assumed total received power.

Proposal 3: RAN4 to confirm that ideal RSRP is derived from the serving cell signal after the fading channel; ideal RSRQ is derived based on ideal RSRP divided by the actual RSSI.

Issue 2-1-6: On SCS in simulation

In our view, it is sufficient to only simulate 30kHz SCS for LR measurement, i.e. all the signals including LP-SS, SSB, NR data for the serving cell and interreference cell are with 30kHz SCS. The implication is that the requirements derived from the simulation of 30kHz SCS will also be applicable to 15kHz SCS. It should be fine assuming the sequence design and combination of {M,L} is agnostic to SCS as the resources for measurement is same. Although the impact of time and frequency error is different for 15k and 30kHz SCS, we believe it may not cause big performance difference for measurement. This is also aligned with the existing RSRP/RSRQ accuracy for MR measurement, which do not depend on SCS.

Proposal 4: RAN4 to only simulate 30kHz SCS for LR measurement. This can be revisited depending on the RAN1 agreement on the binary sequence design.

Issue 2-1-7: On OOK interference

Technically, we think the issue is valid because when the interfere is an OOK signal from neighbour cell, the impact of the interference on the measurement performance depends on the combination of the sequence used by the serving cell and neighbour cell. With 4 LP-SS sequences, there will be 16 combinations.

In our view, there are 2 approaches for defining the requirements:

- Alt 1: the requirements are applicable for all combinations

- Alt 2: the requirements are applicable for specific combinations

With Alt 1, the requirements should be based on the worst-case combination, and it should be up to each company to report results for the worst case. As each company may see different worst-case combination, there is no need to define the sequence combination as part of simulation assumption.

With Alt 2, RAN4 needs to define the sequence combination as part of simulation assumption. In general, such an approach is needed only when there is large performance gap with different sequence combinations.

As RAN1 has not agreed on the binary sequence detail, it would be difficult to evaluate the performance impact of sequence combination, so we believe it may be too early to make a choice between the two approaches. RAN4 should wait for RAN1 conclusion before deciding the combination of binary sequences for serving and neighbour cell.

Proposal 5: RAN4 to wait for RAN1 conclusion on the detailed sequence design before deciding whether to define sequence combination for serving and neighbour cells for simulation purpose.

  - Simulation results

In this section, we will provide updated simulation results for TDL-C channel.

The parameters selected for simulations are as follows.

- Serving cell SINR: -3dB

  - Serving cell SNR = -2.4dB, interfering cell SNR = -8.4dB, i.e. 6dB power difference

- Interference type: LP-SS, interfering cell is fully synchronous to serving cell, binary sequence for LP-SS is independent for serving cell and interfering cell

- Sample number: 1, 2, 3, 4

- Sampling rate: 7.68MHz

- ADC bit width: 8bit

- Filter: 3th Order Butterworth with 3.96MHz bandwidth

    - LP-SS performance

The results in this section are for OOK based LR, i.e. without utilizing the overlaid sequence on LP-SS. The parameters related to LP-SS used in the simulation are as follows.

- Number of OFDM symbols: 4

- OOK scheme: OOK-4 with M=4

- Binary sequence: length-8 M sequence with Manchester coding

- Overlaid sequence: ZC

Table 1: simulation results for LP-SS measurement accuracy (error in dB at max(5%,95%))

| AWGN | Sample number |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 |
| LP-RSRP | 3.73 | 2.77 | 2.77 | 2.71 |
| LP-RSRQ | 3.74 | 2.82 | 2.76 | 2.72 |

The simulation results show that for LP-SS, the measurement accuracy with the assumed LP-SS pattern cannot meet the target accuracy of 3.5 dB with single sample in TDL-C channel at -3dB SINR, but can meet the target accuracy with 2 or more samples in TDL-C channel at -3dB SINR.

Conclusions

In this paper we provided provide our views on the remaining issues for simulation assumption.

Proposal 1: As baseline, RAN4 to use -3dB SINR as side condition of accuracy requirements for both OOK based and OFDM based LR.

Proposal 2: RAN4 to wait for RAN1 conclusion on the detailed sequence design before deciding the exact sequence for simulation.

Proposal 3: RAN4 to confirm that ideal RSRP is derived from the serving cell signal after the fading channel; ideal RSRQ is derived based on ideal RSRP divided by the actual RSSI.

Proposal 4: RAN4 to only simulate 30kHz SCS for LR measurement. This can be revisited depending on the RAN1 agreement on the binary sequence design.

Proposal 5: RAN4 to wait for RAN1 conclusion on the detailed sequence design before deciding whether to define sequence combination for serving and neighbour cells for simulation purpose.

In this paper, we also provide updated simulation results.

Observation: For LP-SS, the measurement accuracy with the assumed LP-SS pattern cannot meet the target accuracy of 3.5 dB with single sample in TDL-C channel at -3dB SINR, but can meet the target accuracy with 2 or more samples in TDL-C channel at -3dB SINR.

Reference

- R4-2420100, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

- R4-2416861, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

- R4-2419145, Simulation assumption and additional simulation results for LP-WUR measurement, Huawei, HiSilicon