3GPP TSG-RAN WG4 Meeting #114	R4-2501265

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on RRM requirements for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	7.26.4.2

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on [1] the following issues need further discussion.

- General aspects

- LP-WUR measurement

- MR relaxation

- LP-WUR at CONNECTED mode

In this paper, we will provide our views on RRM requirements related to LP-WUR.

Discussion

General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

We do not support to consider Case #2.

- There is no clear necessity to require MR to measure the serving cell since LR is already measuring the serving cell.

  - In particular, the MR measurement will be with relaxation, so it is much more infrequent compared to the LR measurement.

  - Some companies commented that LR measurement may be less accurate than that of MR. In our view, however, it is both necessary and feasible to keep similar accuracy between LR and MR.

  - Some companies commented that MR measurement may be used by UE to assist LR measurement. We believe this can be left to UE implementation rather than mandated by requirements.

- There is additional power consumption due to MR measurement even with relaxation.

- It will complicate the spec since another threshold (coverage level) is needed to switch between Case #1 and Case #2.

Proposal 1: RAN4 not to support Case #2 for MR and LR measurement.

Issue 1-1-3: Measurement requirements to be specified for LP-WUR

RAN1#118-bis reached following agreements related to LP-SS.

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

Measurement of LP-SS by OFDM based LR is clearly supported according to the RAN1 agreement, so we support to define following requirements for different LR architectures.

- OOK based LR measurement on LP-SS

- OFDM based LR measurement on LP-SS

- OFDM based LR measurement on SSB

OFDM based LR can measure both LP-SS and SSB, so if only one of 2) and 3) is defined, it means that UE with OFDM based LR is mandated to measure a certain signal, which unnecessarily limits the flexibility of OFDM based LR. Using LP-SS for measurement has clear benefits compared to SSB, e.g. it allows OFDM based LR to perform sync/RRM and LP-WUS monitoring without RF re-tuning. Also, the measurement from LP-SS can better reflect the channel quality for LP-WUS reception compared to measurement from SSB which may be far away from LP-WUS in frequency domain.

In last meeting, some companies suggested not to define requirements for 2) because of RAN1 agreements. We would like to share our understanding of the RAN1 agreement.

- From the RAN1 agreements, the design of overlaid sequence(s) doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. We understand it means the sequence is not designed to optimize the performance of sync and RRM measurement, but it does not mean LP-SS with overlaid sequence cannot be used for sync and RRM. For example, the same LP-SS with overlaid sequence will be used by OOK based LR for sync and RRM measurement.

- From the RAN1 agreements, it is not intended to introduce new RAN4 requirements specific to overlaid sequences. Technically, OFDM based LR can utilize the overlaid sequence to achieve better sync/RRM performance compared to OOK based LR which can only use the envelop information. We understand RAN1 agreements mean RAN4 is not supposed to define specific requirements for sync/RRM for OFDM based LR to use the overlaid sequence, which are new compared to OOK based LR to use the envelop information.

Based on above discussion, it is reasonable to define requirements for OFDM based LR measurement based on LP-SS, and same requirements for LP-SS based measurement by OOK based LR can be re-used.

Proposal 2: RAN4 to define requirements for OFDM based LR measurement based on LP-SS, by re-using the requirements for OOK based LR measurement based on LP-SS.

Issue 1-1-8: LP-WUR operating carrier frequency

In RAN#106, the issue of LP-WUR operating carrier frequency was discussed and WF [2] was approved.

- For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Detailed discussions and final solution selection is left up to WGs.

The solution 1, 2 and 3 from [3] are re-produced below.

As noted in the last column of the table “Expected specification efforts”, in all 3 solutions the LR and MR are working in the same carrier frequency which is aligned with the assumption of RAN4 requirements. Therefore, we do not see any of the solutions will cause impact to RRM requirements. On the other hand, as the solution will be decided in RAN2, we suggest RAN4 to wait for RAN2 conclusion on the exact solution to be supported before discussing whether there is any RRM impact.

Proposal 3: RAN4 to wait for RAN2 conclusion on the exact solution to be supported for LP-WUR operating carrier frequency before discussing whether there is any RRM impact.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

In our understanding, the LR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring is depending on the entry condition of LP-WUS monitoring.

In RAN2#127 the following working assumption was made.

- Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

In RAN2#127-bis, the following agreement was made.

- If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

This means when NW configures LR threshold for the entry condition of LP-WUS monitoring or offloading, LR needs to be turned ON to perform serving cell measurement. However, we understand this is a transient period where LR measurement is needed only for evaluation of the entry condition (but not for mobility), we suggest RAN4 not to define requirements for this time period.

Proposal 4: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Issue 1-1-10: Considerations on higher priority frequency layer

In our view, UE should always perform measurement on higher priority carriers, if configured. Configuring higher property carriers is an important tool from NW perspective to manage the mobility of UEs in RRC_IDLE or RRC_INACTIVE, and it should not be totally compromised due to power saving. Otherwise, it may impact the usage of the LP-WUR feature, i.e. NW may not enable LP-WUR feature when it wants to manage the mobility of UEs in RRC_IDLE or RRC_INACTIVE.

In current spec, the requirements for higher priority carriers depends on whether serving cell condition:

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, higher priority carriers are measured according to requirements in clause 4.2.2.7, i.e. every (60 * Nlayers) s. This is already a relaxed requirement compared to the case below. We are open to discuss whether the requirements should be further relaxed.

- If Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, higher priority carriers are measured together with other carriers according to requirements in clause 4.2.2.4. When LR measurement is ON, the measurement should be relaxed based on agreement for MR relaxation.

Proposal 5: UE should perform measurement for higher priority frequency layers, if configured

- If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #1), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

- If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

Issue 1-1-11: RRM requirements for FR2

We support to prioritize defining RRM requirements for FR1 in this WI. Compared to FR1, an outstanding impact on RRM requirements of FR2 is Rx beamforming. It may consume a lot of RAN4 efforts to discuss the Rx beam sweeping factor, the Rx beamforming gain for a completely new Rx, etc. Considering the RAN4 workload, we suggest RAN4 to focus on defining RRM requirements for FR1 first.

Proposal 6: RAN4 to prioritize defining RRM requirements for FR1 in this WI.

Issue 1-1-12: LP-SS frequency domain relation with SSB

In our view, FDM between LP-SS and SSB is a valid NW configuration and should be supported. RAN4 may need to check whether there is any impact to RRM requirements from this configuration, e.g. whether UE can meet the measurement requirements for both MR and LR in Case 3. This depends on how SSB and LP-SS are configured. Since RAN1 has not concluded on supported LP-SS periodicity, we suggest RAN4 to further discuss the impact based on RAN1 conclusion.

Proposal 7: RAN4 to further discuss whether there is RRM impact due to the configuration of FDMed SSB and LP-SS.

Issue 1-1-13: Impact on Existing requirements

We understand the issue is related to Issue 1-1-9. Technically, we support to keep existing MR requirements unchanged before UE enters Case #1 or Case #3, but whether this is feasible depends on how UE is required to use LR for evaluating the entry condition of Case #1 or Case #3 as MR and LR cannot work at same time. We suggest to further discuss this issue after RAN4 concludes on Issue 1-1-9.

LP-WUR measurement

Issue 1-2-1: Accuracy requirements

We support to define accuracy requirements for LR measurement in the same way as cell reselection requirements, where accuracy is implicitly defined as margin (highlighted in yellow in the copied requirements). So far, dedicated accuracy requirement in clause 10 are all for reported measurement in RRC_CONNECTED, and we prefer to keep same principle for this WI.

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that: when rangeToBestCell is not configured: - the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

Proposal 8: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement, and reflect the accuracy performance as a margin in the core requirement.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

In last meeting, it was agreed that the periodicity for SSB based LP-WUR measurement delay requirements is based on LO periodicity, no matter if LP-SS is configured or not. One remaining issue if whether an upper bound is needed for the measurement periodicity. It is essentially a trade-off between power saving gain and how fast UE can exit LR when the condition for LR is not met.

Assuming the measurement period for LR measurement is n samples, and m measurements are needed for determining the exit condition is met (m=1 or m>1 is discussed in Issue 1-2-3), the total time between the time point when the channel condition gets insufficient for LR to the time point when UE determines to exit LR would be

Texit = n * Tsample + (m-1) * Tsample

where Tsample is the measurement periodicity. Assuming no upper limit is defined, UE may miss (n+m-1) LP-WUS since UE may not be able to decode LP-WUS under the channel condition. We understand n and m will be in the range of {1,2,3,4}, and it should not be big issue to user experience considering the probability of actual paging is low. Therefore, we suggest not to define upper limit for the measurement periodicity.

Another open issue is the measurement periodicity when LP-SS is configured and LP-SS periodicity is larger than LO periodicity. As RAN1 has not concluded on LP-SS periodicity, it is not clear whether this case exists or not, so we suggest to postpone the discussion for this case until LP-SS periodicity is clear in RAN1.

Proposal 9: Measurement periodicity for SSB is the LO periodicity, i.e. no upper limit is defined. Postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity.

Issue 1-2-2-1: Lower bound on LP-SS measurement periodicity

We support to define lower limit for LP-SS measurement periodicity. RAN4#112-bis agreed that LP-SS measurement periodicity shall be defined based on LP-SS periodicity. In our view, the agreement was made based on the assumption of LP-SS periodicity equalling to 320ms. However, RAN1 has considered LP-SS periodicity from 80ms to 10240ms.

In our view, smaller LP-SS periodicity like 80ms or 160ms is targeting for sync for LP-WUS decoding, but the LR measurement does not have to be so frequent. As discussed above for SSB measurement periodicity, even the measurement periodicity is LO periodicity, the impact to user experience is still acceptable. On the other hand, too frequent measurement will compromise the power saving gain of measurement offloading.

Proposal 10: Define lower bound for LP-SS measurement periodicity as 320ms.

Issue 1-2-2-2: Other on LP-WUR measurement interval for both LP-SS and SSB

We support to align measurement periodicity for LP-SS and SSB. We understand the reason why RAN4 needs to discuss case where LP-SS periodicity is larger than LO periodicity in Issue 1-2-2 is to check whether UE should still use LO periodicity as measurement periodicity for SSB when LP-SS periodicity is larger, i.e. some UE will measure LP-SS with LP-SS periodicity, so it may not be necessary for other UEs to measure SSB with a smaller LO periodicity. We share similar concern, and from our view it is desirable to define same measurement periodicity for SSB and LP-SS because they are serving the same purpose.

Proposal 11: RAN4 to discuss whether to align measurement periodicity for LP-SS and SSB.

Issue 1-2-3: On requirements for entry/exit criteria(threshold) evaluation for WUS paging monitoring/Fully Offloading (Case 1)/MR RRM relaxation (Case 3)

We support to define single set of evaluation requirements for the entry and the exit condition for all cases based on evaluation period which includes multiple measurement periods.

Evaluation of entry condition is not as time critical, and to make sure there is no ping-pong between entry and exit, we support to define evaluation requirements for the entry condition based on evaluation period. In last meeting, we proposed to define the evaluation of exit condition based on measurement period as slower exit of LR would result at higher probability of missing paging. On the other hand, some companies mentioned that exit condition should be also evaluated with multiple measurements because it may involve turning ON MR, which causes large power consumption. Considering the trade-off between power saving and the probability of missing paging, we suggest to define the evaluation of exit condition based on a small number of measurement periods.

In addition, to make sure the decision is not based on short term fast channel fading, we suggest each measurement should be based on filtering of at least 2 samples. This is also same as existing requirements.

Proposal 12: Define evaluation requirements for the entry and the exit condition for all cases based on evaluation period which includes [2] measurement periods. Each measurement is based on filtering of at least 2 samples.

Issue 1-2-3-1: On LP-WUR/MR evaluation requirement when both MR and LR threshold are configured

In our view, whether MR measurement, LR measurement or both MR and LR measurement is used for evaluating a particular entry or exit condition is discussed in other WGs. Also, it may depend on NW configuration, e.g. the entry condition of LP-WUS monitoring as RAN2#127-bis agreed.

On the other hand, for RAN4 we believe the requirements for LR and MR measurement period should be defined separately, regardless of how they are going to be used. For example, entry of LP-WUS monitoring may require both MR and LR measurement to be above certain threshold. In this case, the evaluation requirements of MR and LR apply independently, and UE enters LP-WUS monitoring when the later of the two evaluation periods ends.

Proposal 13: RAN4 to define LR and MR measurement period requirements separately.

Issue 1-2-4: On interruption related requirements for LP-WUR

In RAN1#118 the following is agreed related to PO monitoring after UE receives LP-WUS.

Agreement For the wake-up delay, consider the following options: Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 Above applies for IDLE/INACTIVE mode. Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring Agreement On the LO configuration for iDRX, offset value(s) between a LO and a reference PO/PF is/are configured. - FFS: one or multiple offset values, and if multiple offset values are supported, how a UE decides which value to use - FFS: the exact definition of the reference PF/PO and the detailed procedure for UE to determine the LO(s) corresponding to Option 1, 2, or 3 (if supported) - (Working Assumption) For each UE, the periodicity of LO is the same as its iDRX cycle. - If a UE receives a wake-up indication in a LP-WUS, consider the following alternatives - Alt 1: it monitors the PO associated with the offset. - Alt 2: it monitors the first PO after its reported wake-up delay. - Other alternatives are not precluded Note: The PO mentioned above refers to legacy PO configured for the UE.

In RAN1#118-bis the following is agreed related to PO monitoring after UE receives LP-WUS.

Agreement From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if - it does not detect a LP-WUS on the monitored LO - or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE.

In RAN1#119 the following is agreed related to PO monitoring after UE receives LP-WUS.

Agreement For the offset value(s) between an LO and a reference PO/PF, down-select between - Option 1-1 - Configurability between Option 1-1 and Option 1-2 - Option 2B-1 For future meetings: Consider the following alternatives for UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

In our view, it may be still too early to conclude whether RAN4 needs to define interruption requirements as it depends on how UE should monitor PO after it receives LP-WUS which has not been concluded in RAN1.

It is noted that if UE is not expected to receive any PO in a time period following RAN1 spec, e.g. during the time period between a LO and a reference PO/PF, then RAN4 does not need to define interruption for this time period.

Proposal 14: RAN4 to wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR.

Issue 1-2-5: On LP-WUR cell selection evaluation requirements for case 1

We do not see the need to define cell selection evaluation requirements for LR. In a reasonable deployment, when UE is close to cell edge where S criterion needs to be evaluated, UE should have already exited LR measurement, and MR measurement should have started.

Proposal 15: RAN4 not to define cell selection evaluation requirements for LR.

Issue 1-2-7: Accuracy for relaxed MR measurement

We support to re-use the legacy MR accuracy requirements for MR measurement with relaxation. Accuracy requirements do not depend on the measurement interval, i.e. same accuracy requirements apply regardless of whether the measurement is performed once per DRX cycle or per N DRX cycles. Here, by legacy MR accuracy requirements we mean the accuracy in clause 10 for CA/DC Idle Mode Measurements. We are also open to discuss other existing accuracy numbers in RRC_IDLE or RRC_INACTIVE.

Proposal 16: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation.

Issue 1-2-10: LP-WUS monitoring requirements while UE is not required to monitor paging

Issue 1-2-11: LP-WUS monitoring and MR relaxation requirements

We understand the issue is not in RAN4 scope. RAN1 has agreed that “if UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up”. It is a functional UE behaviour and should be defined in RAN1 or RAN2 spec. What RAN4 needs to do is to define requirements to make sure that UE makes timely evaluation of the entry or exit condition of LP-WUS monitoring, and this is already covered by Issue 1-2-3. Also, we understand the discussion in Issue 1-2-3 does not have any implication on whether conditions for LP-WUS monitoring, measurement offloading and MR relaxation are independent or not, which should be decided by RAN2.

MR relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3

Issue 1-3-3: On multiple scaling factors

On the delay requirements for relaxed MR measurement, as the relaxation is aimed at power saving, we believe the power saving gain should be the main factor to determine the relaxation factor. Figure 1 shows our evaluation on the power saving gain from SI for MR relaxed measurement, with different relaxation factors. It can be seen that the power saving gain reduces obviously when relaxation factor becomes smaller. With 10 times relaxation, the gain is 20% in ideal case for paging, and to achieve meaningful gain in realistic case, the relaxation factor needs to be > 20.

Figure 1: Power saving gain for MR relaxed measurement with different relaxation factors

The SI observations in 38.869 are reproduced below. It can be seen that when relaxation factor < 8, the average power saving gain is 4% and some companies including us observed negative gain. The reason why there could be negative gain is that the power consumption for transition (turning ON/OFF MR) is large when MR goes into ultra-deep sleep. In reality, the transition may be triggered by MR measurement (the frequency of which depends on the relaxation factor), channel condition change which triggers entry/exit condition to be fulfilled, the actual wakeup and false wakeup (when other UEs in the same subgroup are waken up), so it is important to limit the number of transitions by avoiding relaxation factor < 8.

- No MR RRM relaxed - Results in [8A-3], [8A-8], [8A-11], [8A-12], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: -211%, range: -624%~-5%), assuming MR enters ultra-deep sleep - Results in [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provides power saving gain 21%, assuming MR enters deep sleep - MR relaxed < 8 times - Results in [8A-1], [8A-3], [8A-8], [8A-9], [8A-15], [8A-16] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 4%, range: -18%~50%) - 8 times<= MR relaxed <=16 times - Results in [8A-1], [8A-3], [8A-4], [8A-8], [8A-9], [8A-11], [8A-12], [8A-13], [8A-15] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 40%, range: -18%~60%) - MR relaxed > 16 times - Results in [8A-1], [8A-3], [8A-4], [8A-8], [8A-11], [8A-13], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 60%, range: 22%~90%) - MR offload RRM to LR - Results in [8A-3], [8A-4], [8A-5], [8A-7], [8A-8], [8A-10], [8A-11], [8A-15], [8A-16], [8A-17] show that compared with i-DRX with or without PEI, LP-WUS provide mean power saving gain (average: 83%, range: 76%~94%)

Based on our evaluation and the observation from the SI, and considering that LR is new hardware targeting significant power saving, we suggest to define a single relaxation factor >= 16.

Proposal 17: Define a single MR relaxation factor which is >= 16.

LP-WUR at CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

It is true that the objective of LP-WUR at RRC_CONNECTED mode does not involve RAN4, but it may be too earlier to preclude any RRM impacts as the RAN1/2 discussions are ongoing, and there may be RRM impacts. RAN4 can first focus on the LP-WUR requirements in RRC_IDLE or RRC_INACTIVE, and postpone the discussion for RRC_CONNECTED mode until more RAN1/2 conclusions are available.

Proposal 18: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Conclusions

In this paper we provided provide our views on RRM requirements related to LP-WUR.

Proposal 1: RAN4 not to support Case #2 for MR and LR measurement.

Proposal 2: RAN4 to define requirements for OFDM based LR measurement based on LP-SS, by re-using the requirements for OOK based LR measurement based on LP-SS.

Proposal 3: RAN4 to wait for RAN2 conclusion on the exact solution to be supported for LP-WUR operating carrier frequency before discussing whether there is any RRM impact.

Proposal 4: RAN4 not to define LR measurement requirements before entering or after exiting LP-WUS monitoring or LR measurement offloading.

Proposal 5: UE should perform measurement for higher priority frequency layers, if configured

- If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #1), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed.

- If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements

Proposal 6: RAN4 to prioritize defining RRM requirements for FR1 in this WI.

Proposal 7: RAN4 to further discuss whether there is RRM impact due to the configuration of FDMed SSB and LP-SS.

Proposal 8: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement, and reflect the accuracy performance as a margin in the core requirement.

Proposal 9: Measurement periodicity for SSB is the LO periodicity, i.e. no upper limit is defined. Postpone the discussion for the case where LP-SS periodicity > LO periodicity until RAN1 has more agreements for LP-SS periodicity.

Proposal 10: Define lower bound for LP-SS measurement periodicity as 320ms.

Proposal 11: RAN4 to discuss whether to align measurement periodicity for LP-SS and SSB.

Proposal 12: Define evaluation requirements for the entry and the exit condition for all cases based on evaluation period which includes [2] measurement periods. Each measurement is based on filtering of at least 2 samples.

Proposal 13: RAN4 to define LR and MR measurement period requirements separately.

Proposal 14: RAN4 to wait for more RAN1 progress on PO monitoring before discussing the possible interruption requirements related to LR.

Proposal 15: RAN4 not to define cell selection evaluation requirements for LR.

Proposal 16: Legacy MR accuracy requirements in RRC_IDLE or RRC_INACTIVE are re-used for MR measurement with relaxation.

Proposal 17: Define a single MR relaxation factor which is >= 16.

Proposal 18: RAN4 to postpone the discussion on RRM impacts of LP-WUR at CONNECTED mode until more RAN1/2 conclusions are available.

Reference

- R4-2420100, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

- RP-243266, Way forward on Rel-19 LP-WUS separate band issues, RAN2 chair (InterDigital)

- RP-242656, On LR and MR operating in different bands in release 19, Vodafone, Vivo, Huawei, HiSilicon