3GPP TSG-RAN WG4 Meeting #114	R4-2501267

Athens, Greece, 17 – 21 February, 2025

Title: 	Additional simulation results for UE CLI measurement

Source: 	Huawei, HiSilicon

Agenda item:	7.23.3.1

Document for:	Discussion

Introduction

In RAN4#112-bis, link level simulation assumption is approved in [1] to evaluate the measurement performance of L1-SRS-RSRP, e.g. with different sample number, different timing error condition and different Es/Iot. In [2], we have provided our initial simulation results for basic setup, i.e. with SRS Es/Iot = 1dB and SRS PRB number = 48.

In this paper, we will provide additional simulation results for L1-SRS-RSRP.

Discussion

In this section, we will provide simulation results for the following configurations in [2].

- Number of SRS PRBs: 24 and 48

- SRS Es/Iot: 1dB and 3dB

- Channel: AWGN

Case 1

The measurement performance for Case 1 are shown in Table 1a and Table 1b for different Es/Iot levels.

Table 1a: L1-SRS-RSRP performance for Case 1 in AWGN @ 1dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -1.16 | 0.29 |
|  |  | 3 | -0.92 | -0.12 |
|  | 48 | 1 | -0.98 | 0.07 |
|  |  | 3 | -0.84 | -0.23 |
| 30 | 24 | 1 | -1.99 | -0.27 |
|  |  | 3 | -1.64 | -0.69 |
|  | 48 | 1 | -1.66 | -0.5 |
|  |  | 3 | -1.54 | -0.84 |
| 60 | 24 | 1 | -3.67 | -1.43 |
|  |  | 3 | -3.26 | -1.98 |
|  | 48 | 1 | -3.28 | -1.76 |
|  |  | 3 | -3.05 | -2.18 |
| 60 | 24 | 1 | -2.67 | -0.89 |
|  |  | 3 | -2.48 | -1.40 |
|  | 48 | 1 | -2.59 | -1.22 |
|  |  | 3 | -2.33 | -1.56 |
| 120 | 24 | 1 | -6.52 | -3.20 |
|  |  | 3 | -5.86 | -3.84 |
|  | 48 | 1 | -6.01 | -3.6 |
|  |  | 3 | -5.56 | -4.18 |

Table 1b: L1-SRS-RSRP performance for Case 1 in AWGN @ 3dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -1.07 | 0.08 |
|  |  | 3 | -0.90 | -0.22 |
|  | 48 | 1 | -0.9 | -0.08 |
|  |  | 3 | -0.80 | -0.31 |
| 30 | 24 | 1 | -1.81 | -0.41 |
|  |  | 3 | -1.61 | -0.77 |
|  | 48 | 1 | -1.65 | -0.67 |
|  |  | 3 | -1.51 | -0.93 |
| 60 | 24 | 1 | -3.84 | -1.60 |
|  |  | 3 | -3.23 | -2.09 |
|  | 48 | 1 | -3.20 | -1.90 |
|  |  | 3 | -3.03 | -2.27 |
| 60 | 24 | 1 | -2.72 | -1.08 |
|  |  | 3 | -2.43 | -1.52 |
|  | 48 | 1 | -2.49 | -1.36 |
|  |  | 3 | -2.31 | -1.63 |
| 120 | 24 | 1 | -6.37 | -3.38 |
|  |  | 3 | -5.80 | -4.04 |
|  | 48 | 1 | -5.88 | -3.82 |
|  |  | 3 | -5.51 | -4.29 |

Case 2

The measurement performance for Case 2 are shown in Table 2a and Table 2b for different Es/Iot levels.

Table 2a: L1-SRS-RSRP performance for Case 2 in AWGN @ 1dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.68 | 0.64 |
|  |  | 3 | -0.50 | 0.29 |
|  | 48 | 1 | -0.54 | 0.42 |
|  |  | 3 | -0.39 | 0.15 |
| 30 | 24 | 1 | -0.96 | 0.42 |
|  |  | 3 | -0.76 | 0.08 |
|  | 48 | 1 | -0.78 | 0.19 |
|  |  | 3 | -0.64 | -0.08 |
| 60 | 24 | 1 | -1.49 | 0.09 |
|  |  | 3 | -1.24 | -0.36 |
|  | 48 | 1 | -1.25 | -0.16 |
|  |  | 3 | -1.13 | -0.5 |
| 60 | 24 | 1 | -0.90 | 0.54 |
|  |  | 3 | -0.64 | 0.14 |
|  | 48 | 1 | -0.7 | 0.3 |
|  |  | 3 | -0.56 | 0.02 |
| 120 | 24 | 1 | -1.24 | 0.16 |
|  |  | 3 | -1.05 | -0.18 |
|  | 48 | 1 | -1.05 | -0.05 |
|  |  | 3 | -0.92 | -0.3 |

Table 2b: L1-SRS-RSRP performance for Case 2 in AWGN @ 3dB

| SCS (kHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.60 | 0.49 |
|  |  | 3 | -0.43 | 0.16 |
|  | 48 | 1 | -0.47 | 0.26 |
|  |  | 3 | -0.35 | 0.07 |
| 30 | 24 | 1 | -0.85 | 0.24 |
|  |  | 3 | -0.69 | -0.04 |
|  | 48 | 1 | -0.71 | 0.09 |
|  |  | 3 | -0.62 | -0.13 |
| 60 | 24 | 1 | -1.43 | -0.09 |
|  |  | 3 | -1.20 | -0.45 |
|  | 48 | 1 | -1.21 | -0.34 |
|  |  | 3 | -1.10 | -0.57 |
| 60 | 24 | 1 | -0.75 | 0.32 |
|  |  | 3 | -0.59 | 0.05 |
|  | 48 | 1 | 0.61 | 0.15 |
|  |  | 3 | -0.51 | -0.07 |
| 120 | 24 | 1 | -1.16 | 0.04 |
|  |  | 3 | -1.01 | -0.28 |
|  | 48 | 1 | -1.03 | -0.15 |
|  |  | 3 | -0.90 | -0.40 |

Conclusions

In this paper we provided our additional simulation results for L1-SRS-RSRP. We have made the following observations from the results for AWGN channel.

Observation 1: In Case 1, the gain of 48 PRB over 24 PRB is ~0.2 dB (with 3-sample); for 24 PRB, the gain of 3dB Es/Io over 1dB Es/Iot is ~0.1 dB.

Observation 2: In Case 2, the gain of 48 PRB over 24 PRB is ~0.2 dB (with 1-sample); for 24 PRB, the gain of 3dB Es/Io over 1dB Es/Iot is ~0.1 dB.

Reference

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson

- R4-2419148, Initial simulation results for UE CLI measurement, Huawei, HiSilicon