3GPP TSG-RAN WG4 Meeting #114	R4-2501269

Athens, Greece, 17 – 21 February, 2025

Title: 	Discussion on RRM requirements for RedCap UE in NTN

Source: 	Huawei, HiSilicon

Agenda item:	7.29.4.1

Document for:	Discussion

Introduction

RRM requirements for RedCap UE in NTN are discussed in RAN4#113, and the outcomes are captured in WF [1]. Based on the discussions, RAN4 needs to further discuss the following issues.

- RRM requirements

- Impact of 1Rx

- Impact of HD-FDD

- Other

In this paper, we will provide our views on RRM requirements for RedCap UE in NTN.

Discussion

RRM requirements

#### Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN?

We support to define requirements for cell reselection requirement for inter-RAT NR NTN to E-UTRA TN measurement. The requirements have been defined for normal UE in NTN, and we do not see clear reason not to define them for RedCap UE in NTN, especially considering for RedCap UE in TN there are also requirements for inter-RAT E-UTRA measurement. On the other hand, we understand this is not aligned with the agreement from RAN4#112, so we are also fine not to define requirements if there is no consensus to revert the earlier agreement.

Proposal 1: For (e)RedCap UE with FR1-NTN, define cell reselection requirement for inter-RAT NR NTN to E-UTRA TN measurement.

#### Issue 1-1-2: The Requirements to be further determined

For SDT, we do not support to define requirements related to TA validation. RSRP change is used reflect the between UE and BS, and when the change in distance is larger than a threshold, TA will be considered as invalid. This works for TN, but for NTN the change in distance is already compensated by UE in UE specific TA, so RSRP change is not a good metric for TA validation. It is also noted that for PUR in IoT NTN, requirements related to RSRP change based TA validation are not defined. This was confirmed by CR [2] in RAN4#113.

For MDT, we do not support to define requirements. We do not have MDT requirement for RedCap UE in TN, and there is no strong reason why RedCap UE performing MDT is important in NTN. Of course, this does not mean MDT cannot be configured to RedCap UE in NTN.

Proposal 2: For (e)RedCap UE with FR1-NTN,

- Do not define requirements related to TA validation in SDT

- Do not define requirements for MDT

#### Issue 1-2-1: Relaxed measurement

In last meeting, it is agreed to only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation). In our view, it means only 2 relaxation criteria ‘low mobility’ and ‘not at cell edge’ will be supported. For the thresholds, we can either re-use the existing thresholds defined for relaxed measurement in R17 NTN for normal UE, or we can define a new set of thresholds for RedCap UE.

We prefer to re-use the existing thresholds. As discussed last meeting, in NTN GSO scenario, it is difficult to use RSRP to determine how far the UE is from cell edge or use RSRP change to determine how fast the UE is moving, so it is difficult for NW to configure 2 sets of RSRP thresholds for normal UE and RedCap UE.

On the relaxation factor, we suggest to re-use the values in R17 RedCap relaxed measurement. The reason is that power saving is more important for RedCap UE than for normal UE.

Proposal 3: For relaxed measurement in RRC_Idle and RRC_Inactive mode for RedCap UE,

- Existing thresholds defined for relaxed measurement in R17 NTN are re-used

- When only one of ‘low mobility’ and ‘not at cell edge’ criteria is fulfilled, relaxation factor is 6

- When both ‘low mobility’ and ‘not at cell edge’ criteria are fulfilled, UE is not required to meet requirements for up to 4 hours

#### Issue 1-2-2: eDRX configuration

We do not support to define requirements for eDRX cycle >= 10.24s for LEO fixed cell scenario. Tdetect is 235.52s for eDRX cycle = 10.24s, and for larger eDRX cycle the detection time is most likely to be longer. Even the service time for a satellite can be longer than 4min in some scenarios, Tdetect is the time when 2 satellites (the current serving satellite and the next serving satellite) are covering the same area, and we do not think in real deployments it is likely to have 2 satellites covering same area for 4 min long.

Proposal 4: For LEO fixed cell scenario, do not define requirements for eDRX cycle >= 10.24s.

#### Issue 1-2-4: Multiple SMTCs

In current NTN requirements for normal UE, when multiple SMTCs do not overlap in time, there is no scaling in measurement period meaning UE has to support parallel measurement for those SMTCs. On the other hand, based on FG 25-1, NTN UE is mandatorily required to support 2 SMTCs per MO. Combining these 2 factors, it is observed that NTN UE is mandatorily required to support parallel measurement of 2 non-overlapping SMTCs.

In last meeting, some companies commented that parallel measurement of 2 non-overlapping SMTCs may not be an issue if the distance between the 2 SMTCs is large enough, and it can be considered as if NW configured a smaller SMTC period. We agree that using a larger proximity distance could be helpful, but it is difficult to keep the UE complexity not increased due to support of 2 SMTCs because the max proximity distance is 80ms (which resembles the case of 80ms SMTC period).

The reason is that cell search delay is lower bounded by 600ms, meaning UE is not expected to use every SMTC occasion for cell search when SMTC period is <= 80ms. With single SMTC of 80ms period, UE is required to do PSS/SSS detection over 1 set of SMTC within 600ms. With 2 SMTCs of 160ms period and 80ms proximity distance, UE is required to do PSS/SSS detection over 2 sets of SMTC within 600ms, which means the cell search activity is doubled within a 600ms period even with 80ms proximity distance.

It can be seen that supporting parallel measurement of 2 non-overlapping SMTCs will increase the UE complexity. As low complexity is important for RedCap UE, we prefer not to increase the measurement complexity to trade-off measurement delay for multiple STMCs, at least not as a mandatory requirement.

There are two ways to fully address the complexity issue:

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO,

- Option 2: scale measurement period with number of SMTCs no matter if the SMTCs collide or not.

We suggest RAN4 to discuss how to support multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

Proposal 5: RAN4 to discuss following options for defining requirements for multiple SMTCs

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO

- Option 2: scale measurement period with number of SMTCs no matter if the SMTCs collide or not

RAN4 also discussed the applicability of other measurement related features

- 25-2: Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell

- 25-5: Parallel measurements on multiple NGSO satellites within a SMTC

Supporting those 2 features means UE has the capability to handle multiple Doppler shifts for different satellites at the same time. As they also require increased complexity at UE, they are not likely to be supported by RedCap UE. We do not have strong view whether to define requirements for them or not, but slightly prefer to define for the following reasons.

- Those features are optional, so having the requirements does not mandate any UE implementation. There is no strong motivation to preclude them for RedCap UE in spec, e.g. some RedCap UE may choose to implement them.

- The requirements are already defined for normal UEs, so defining them for RedCap UE does not cause additional efforts, instead not defining them for RedCap UE would cause additional efforts.

Proposal 6: RAN4 to define requirements for FG 25-2 and 25-5.

#### Issue 1-2-5: The number of searchers and CSSF

We support to assume single searcher for defining CSSF for RedCap UE in NTN, as it is unreasonable to require a RedCap UE to support 2 searchers just because it supports NTN.

Proposal 7: Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

#### Issue 1-2-6: FH in PRS measurement for NW verified location requirements

We support not to consider FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN bands. FH in PRS measurement aims at improving measurement accuracy where the measurement is not restricted by the UE RF BW (20MHz) but can reach cell BW (100MHz). However, the cell BW in FR1-NTN bands are limited (~30MHz), so the gain from FH may be limited.

Proposal 8: Do not to consider FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN.

#### Issue 1-2-7: NCD-SSB

We support option 1 for defining requirements. In existing TN requirements, the term SSB is used and a clarification like “The SSB and SMTC in this section applies for both CD-SSB and NCD-SSB if it is not additional specified” is added. It is reasonable to follow the same approach with an updated clarification statement like in option 1. The problem with option 2 is that there will be many changes required across all the SSB related requirements, thus it is very error prone and make spec maintenance very difficult.

Proposal 9: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

#### Issue 1-2-8: Other features

For the features or requirements mentioned in P1 of Issue 1-2-8 that are not discussed in other issues,

- We support to define requirements for SMTC adjustment in Idle/Inactive as mandatory. In our view, this is a rather important functionality for RRM measurement in NTN in Idle/Inactive, and it does not require high complexity at UE side (mainly determining the PDD between the target cell and serving cell). Also, RAN4 has not defined UE capability for it.

- We suggest RAN4 not to further discuss location-based measurement report trigger. This is an optional feature for normal UE, and we think it is reasonable to define it as an optional feature for RedCap UE also, as the functionality mainly relies on GNSS. However, there is no requirement related to this feature in RAN4 spec, and the UE capability was not defined by RAN4. We do not think RAN4 should make decision on the applicability of this feature to RedCap UE.

- We support to apply the existing capability maxNumber-LEO-SatellitesPerCarrier-r17 to RedCap UE. With this capability, UE is mandatorily required to support measuring 2 LEO satellites per carrier, and we see it a reasonable requirement also for RedCap UE. The maximum number is 4, and we do not see the need to change it for RedCap UE as it is anyway optional.

Proposal 10: For the applicability of other NTN features to RedCap UE,

- RAN4 to define mandatory requirements for SMTC adjustment in Idle/Inactive

- RAN4 not to further discuss applicability of location-based measurement report trigger

- RAN4 to apply the existing capability maxNumber-LEO-SatellitesPerCarrier-r17 to RedCap UE

Impact of 1Rx

#### Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

RAN4 agreed to define enhanced cell reselection measurement requirements for 1Rx and 2Rx RedCap UE. Comparing the normal requirements and enhanced requirements for normal UEs, Tdetect is tightened most. Considering cell detection is the most complex part is cell reselection measurement, and power saving is important for RedCap UE, we suggest to relax Tdetect for RedCap UE. On the exact value, we suggest to extend Tdetect by [2] samples, and we are open to other views.

Tmeasure and Tevaluate are also tightened in enhanced requirements, but to us the enhanced requirements are still acceptable for RedCap in terms of power saving. After all, enhanced cell reselection measurement is an optional feature, and further relaxing the enhanced requirements will make it not much different from the normal requirements.

We suggest to apply the same enhanced requirements for 1Rx and 2Rx UEs, following the same principle as normal requirements, where the relaxation for 1Rx UE is reflected in the measurement accuracy.

Proposal 11: To define enhanced cell reselection requirements for RedCap UE,

- Re-use the requirements for normal UE except that Tdetect is extended by [2] samples

- Same delay requirements for 1Rx and 2Rx UEs.

#### Issue 2-1-5: Satellite switching with re-sync

First, we support to define requirements for soft switching for 1Rx UE. The concern raised up by companies from last meeting was that the extended Tsearch will “increase the measurement uncertainty”. We do not see an issue here – UE will perform the cell search and DL sync towards the new satellite regardless of the configuration of how long the time period from t-seviceStart to t-service is, i.e. from NW side configuration of t-seviceStart does not need to account for 1Rx UE, and from UE side the gain of soft switch is still there (interruption time between SSB occasions can be saved).

Proposal 12: Define requirements for soft satellite switching for 1Rx UE.

Next, we support to define the extension of Tsearch for 1Rx UE based on SSB periodicity (instead of SMTC periodicity). In existing requirements, Tsearch is already based on SSB periodicity, i.e. first SSB occasion regardless of whether the SSB occasion is within SMTC window or not.

- Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

Technically, SMTC window is still used by UE to determine the time location of search window for the SSB from new satellite, but it does not mean UE needs to use SMTC periodicity to do the search. In satellite switch scenario, the source and new satellites will transmit SSB with same periodicity, so UE knows SSB periodicity of the new satellite from the serving cell.

Proposal 13: Define the extension of Tsearch for 1Rx UE based on SSB periodicity.

#### Issue 2-4-1: L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

#### Issue 2-4-2: Network verified UE location for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

#### Issue 2-5-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

We support the WF in [1] for the above issues, as they clearly capture all the impacts of 1RX in TN, and we do not see any issue to re-use them for RedCap in NTN.

Proposal 14: Adopt the WF [1] for following requirements for RedCap UE with FR1-NTN bands

- L1/L3 Measurement requirements

- Network verified UE location

- L1/L3 measurement accuracy

Impact of HD-FDD

#### Issue 3-1-1: General HD-FDD related impact

P1a lists all the impacted requirements due to HD-FDD support in TN, and we do not see any issue to re-use them for RedCap in NTN. One exception is “MAC-CE/DCI/RRC based uplink spatial relation switch delay”, for which there is no requirement for NTN. Another exception is “Scheduling availability of UE during L1-RSRP measurement”, which is related to RAN1 discussion on TA mismatching as we will discuss next.

Proposal 15: Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE during L1-RSRP measurement

There is ongoing discussion in RAN1 related to TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB. Agreements that are relevant to RRM requirements in our view are copied below.

Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13.

It can be seen that there are possibilities that semi-static DL reception of CSI-RS and PRS is de-prioritized compared to UL transmission, e.g. when NW indicates UL overriding DL. In this case, the measurement period of CSI-RS and PRS based measurement may be impacted if the CSI-RS and PRS resources collide with UL transmission based on actual TA used by the UE because of the punctured measurement occasions. For such cases, we suggest RAN4 to clarify that the measurement period requirements apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission. Another alternative is to define the extended measurement period, but considering that such cases are anyway “semi-error” cases that NW would try to avoid, we do not prefer to spend much efforts to define exact requirements for them.

In existing requirements for scheduling availability of UE during L1-RSRP measurement, it is specified that CBD evaluation is prioritized over UL transmission. This may not be always true based on the latest RAN1 agreement. Our suggestion is to remove those 2 paragraphs because the UL/DL prioritization is anyway defined in RAN1 spec. This is why we list “Scheduling availability of UE during L1-RSRP measurement” as an exception for re-using the TN requirements.

- 9.5B.6.1 Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1 For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1. - 9.5B.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 - For HD-FDD RedCap UE, scheduling restrictions apply for transmission on PUCCH/PUSCH/SRS during the CBD evaluation period, as CBD evaluation is prioritized over UL transmission. The CBD evaluation period equals the measurement period of TL1-RSRP_Measurement_Period_SSB_RedCap in case L1-RSRP measurement is performed on SSB as defined in clause 9.5B.4.1, or TL1-RSRP_Measurement_Period_CSI-RS_RedCap in case L1-RSRP measurement is performed on CSI-RS and/or SSB as defined in clause 9.5B.4.1.

Proposal 16: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

#### Issue 3-2-1: The impact on NR Handover due to HD-FDD

We support P1 in WF [1]. Technically, P1, P2 and P3 are very similar but P1 is more accurate.

On P4, we do not see specific issue for HD-FDD UE. This is because based on RAN1 agreement, SSB measurement is always prioritized over UL transmission, so the cell search and DL sync time for HD-FDD UE would be same as FD-FDD UE and TDD UE.

Proposal 17: Adopt P1 in WF [1] for the impact on HO due to HD-FDD.

Other

#### Issue 4-1-1: Specification structure

We support option 1 to define requirements for (e)RedCap UE with FR1-NTN bands. In our view, this is a reasonable way to make the spec clear while avoid too much duplication.

Proposal 18: Use new sections and section number with new suffix X in the specification to capture requirements for RedCap UE in NTN. Utilize the reference method for the requirements that can be reused.

Conclusions

In this paper we provided provide our views on RRM requirements for RedCap UE in NTN.

Proposal 1: For (e)RedCap UE with FR1-NTN, define cell reselection requirement for inter-RAT NR NTN to E-UTRA TN measurement.

Proposal 2: For (e)RedCap UE with FR1-NTN,

- Do not define requirements related to TA validation in SDT

- Do not define requirements for MDT

Proposal 3: For relaxed measurement in RRC_Idle and RRC_Inactive mode for RedCap UE,

- Existing thresholds defined for relaxed measurement in R17 NTN are re-used

- When only one of ‘low mobility’ and ‘not at cell edge’ criteria is fulfilled, relaxation factor is 6

- When both ‘low mobility’ and ‘not at cell edge’ criteria are fulfilled, UE is not required to meet requirements for up to 4 hours

Proposal 4: For LEO fixed cell scenario, do not define requirements for eDRX cycle >= 10.24s.

Proposal 5: RAN4 to discuss following options for defining requirements for multiple SMTCs

- Option 1: make it optional for RedCap UE to support 2 SMTC per MO

- Option 2: scale measurement period with number of SMTCs no matter if the SMTCs collide or not

Proposal 6: RAN4 to define requirements for FG 25-2 and 25-5.

Proposal 7: Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Proposal 8: Do not to consider FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN.

Proposal 9: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause that the requirements are only applicable to CD-SSB.

Proposal 10: For the applicability of other NTN features to RedCap UE,

- RAN4 to define mandatory requirements for SMTC adjustment in Idle/Inactive

- RAN4 not to further discuss applicability of location-based measurement report trigger

- RAN4 to apply the existing capability maxNumber-LEO-SatellitesPerCarrier-r17 to RedCap UE

Proposal 11: To define enhanced cell reselection requirements for RedCap UE,

- Re-use the requirements for normal UE except that Tdetect is extended by [2] samples

- Same delay requirements for 1Rx and 2Rx UEs.

Proposal 12: Define requirements for soft satellite switching for 1Rx UE.

Proposal 13: Define the extension of Tsearch for 1Rx UE based on SSB periodicity.

Proposal 14: Adopt the WF [1] for following requirements for RedCap UE with FR1-NTN bands

- L1/L3 Measurement requirements

- Network verified UE location

- L1/L3 measurement accuracy

Proposal 15: Adopt TN requirements as baseline for clarifying the requirements listed in P1a for HD-FDD RedCap UE except for

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE during L1-RSRP measurement

Proposal 16: Clarify that the requirements for CSI-RS and PRS apply provided that no CSI-RS and PRS resource is dropped due to collision with UL transmission during the measurement period.

Proposal 17: Adopt P1 in WF [1] for the impact on HO due to HD-FDD.

Proposal 18: Use new sections and section number with new suffix X in the specification to capture requirements for RedCap UE in NTN. Utilize the reference method for the requirements that can be reused.

Reference

- R4-2420104, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- R4-2420185, CR on core maintenance for R18 IoT NTN, MediaTek inc.