3GPP TSG-RAN WG4 Meeting # 114			                                         R4-2501272

Athens, Greece, 17 - 21 February, 2025

Title: 					Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Source: 	Huawei, HiSilico

Agenda item:	7.17.2.2

Document for:	Discussion

Introduction

The RRM Phase 5 FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC discussion, as outlined in WF[1], was one of the topics during RAN4 #113. Key contributions from companies are detailed in WF[1]. In this paper, we present our perspectives and contributions regarding FR2-1 L3 measurement delay, focusing on optimising the CSSF outside gap in CA/DC scenarios.

Discussion

UE measurement procedure

There were three options in the WF[1] captured in the following.

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF Option 1 (CATT, Apple, HW, Ericsson): The enhanced CSSF apply to all the 3 aspects and do not define the priority. Option 2 (Xiaomi, MTK, ZTE): the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: Aspect 1: SSB based Intra-frequency measurement without MG, including: TPSS/SSS_sync_intra and TSSB_measurement_period_intra CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 Option 2a(ZTE): To be more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered. Option 3 (Nokia, Samsung): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion.

For the enhancement of CSSF, all the aforementioned aspects are integral and equally crucial components. There are two points for this is detailed as follows:

- Prioritising any single part would necessitate the creation of individual specification content, which could complicate future maintenance and expansion.

- Given that the solution has not yet been fully discussed or finalised, isolating and prioritising one aspect may not align with potential solutions that could arise.

Proposal 1: The optimisation of CSSF for L3 measurement delay reduction in CA/DC should take into account all three aspects.

Candidate Solutions

In this section, the approach for utilising the CSSF outside the gap will be presented from two perspectives: the 2-searcher solution and the 3-searcher solution.

2-searcher solution 1

The following content has been extracted from the last meeting’s WF[1].

[Wayforwad]: FFS the following solution: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation - FFS whether indication is per-UE or per-band - Other options are not precluded. - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

In our view, this solution should be based on NW configuration. The key reason is that in certain scenarios, serving cell measurement needs to be triggered by the NW, such as for improving serving cell mobility, especially when the serving cell's quality deteriorates. Typically, when the NW initiates handover measurement, the UE must measure the CCs on each configured band, which is time-consuming in FR2.

At this stage, multiple MOs exist on one band, allowing the UE to search for a better cell, aligning with this solution’s scenario. However, there is a distinction between letting the UE determine how many MOs it can measure and allowing the NW to configure the number of MOs per band. Below is our analysis:

On one hand, if this solution allows UE implementation to select one MO per band for measurement during handover, the UE may provide suboptimal measurement results to the NW for each CC because of the interference, potentially leading to degraded handover performance.

On the other hand, if the NW configures one MO per band to facilitate the selection of a new serving cell for handover, three key advantages emerge:

- The NW can determine which MO should represent the MO measurement result for that band.

- The NW can determine when to indicate UE to measure only one serving CC per band.

- The NW can dynamically adjust whether the UE measures one CC per band or all MOs per band.

Based on these three points, this approach enables the NW to configure the appropriate MO for the serving CC per band, accelerating the measurement process when multiple CC measurement results are needed. In summary, the NW, based on the trend of measurement results from UE feedback, determines both how many CC measurement results are required for a band and how many MOs the UE should measure. This allows the NW to instruct the UE to quickly measure one CC per band and select a new serving cell. A prerequisite for this solution is that, before the NW configures a single MO to represent all CCs in a band, the NW must have previously configured multiple CCs in that band at least once. The rationale for this is that interference levels experienced by the UE may differ across CCs. When interference is present, the measurement result from one CC may not accurately represent the results from all CCs in the band. Therefore, the NW must first collect a full set of measurement results from all CCs in the band, which will support the configuration of a single serving MO per band.

Overall, in our perspective, this solution is NW can configure one serving CC's MO per band if it requires the UE to measure multiple serving CCs per band. By indicating one MO per band, the signalling cost is lower compared to a multiple-MO configuration.

Observation 1: NW control is able to decrease the signalling cost because the UE only need to report one CC per band.

Observation 2: NW can configure one serving CC's MO to represent this band measurement result if the UE is required to measure multiple serving CCs per band as the legacy.

When the measurement results are stable, the NW enables the CSSF enhancement, allowing the UE to measure only one serving CC per band if multiple serving CCs exist in the same band. If the measurement results deteriorate, the NW disables the CSSF enhancement, reverting to the legacy process which lets the NW determine the MO of serving CC per band.

Proposal 2: NW indicate to enable UE to activate this CSSF enhancement in specific scenarios. Otherwise, NW and UE will follow the legacy to perform serving and neighbor cell measurements.

Proposal 3: RAN4 need to clarify whether Solution 1 will affect the legacy neighbour measurement process.

2-searcher solution 2

The following content has been extracted from the last meeting’s WF[1].

[Wayforward]: FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1: - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement. - FFS whether network indication is needed - Details is FFS - FFS on whether and how PCC and PSCC resource sharing is supported

This solution applies when the NW configures multiple MOs for SCells across multiple bands. At this stage, SCCs are in different bands—for example, three SCCs in three separate bands. Therefore, this solution does not overlap with 'Solution 1'.

Observation 3: Solution 2 is needed and it is applicable when the SCCs are in separate bands.

If the NW prioritizes certain SCCs for measurement but does not reduce the number of MOs for all SCCs, the overall measurement period remains unchanged. Consequently, simply altering the order of SCC measurements will not increase the measurement speed of any specific SCC.

Observation 4: Only changing the order of SCC measurement is not able to increase the particular SCC’s measurement speed.

About changing the PCC or PSCC sharing their searcher ratio, our view is that the PCC PSCC resource sharing solution will let the PCC, PSCC and SCCs’ measurement period change completely. This may cause the Rx beam scheduling problem.

Proposal 4: Sharing the PCC or PSCC resource may result in the Rx beam scheduling problem.

3-searcher solution

In the last meeting, the following ad hoc agreement about 3-searcher scenarios is captured in WF[1]:

Agreement: - after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios.

Here, we would like to briefly summarize the technical background of our 3-searcher solution. When the per-FR GAP is configured in FR1 + FR2 CA or NR-DC, and all MOs are outside the GAP, the UE can trigger the 3-searcher solution. The key point here is that, in the FR1 + FR2 scenario, UE has the capability to trigger the third searcher based on the legacy requirement.

As explained above, the FR1 + FR2 scenario is feasible. If we extend the scenario to FR1 only, the UE’s measurement capability will be less affected compared to the FR1 + FR2 scenario, and the 3-searcher solution can be achieved without limitations. For the details of the solution, the third searcher will be used to speed up the SCC measurement. The searchers allocation is PCC occupied one searcher and all SCC share two searchers.

Regarding the last meeting agreement, the 3-searcher solution will be limited to a specific scenario and the FR1 is included in the scope. Our original solution aimed to utilise the 3-searcher solution to the FR1 + FR2 CA/NR-DC scenario. It can also compromise to limit the solution to FR1 only. In the FR1 only scenario, the 3-searcher solution is PCC occupied one searcher and all SCC share two searchers.

Proposal 5: Based on the technical analysis, the 3-searcher solution is able to be used to the UE that supports“per-FR GAP”-like capability. The 3-searcher scenario can be FR1 only scenario and FR1 + FR2 scenario. It can also compromise to define the 3-searcher solution in FR1 only scenario.

Proposal 6: In scenario FR1 only, the 3-searcher solution is PCC occupied one searcher and all SCC share two searchers.

Conclusions

Proposal 1: The optimisation of CSSF for L3 measurement delay reduction in CA/DC should take into account all three aspects.

Observation 1: NW control is able to decrease the signalling cost because the UE only need to report one CC per band.

Observation 2: NW can configure one serving CC's MO to represent this band measurement result if the UE is required to measure multiple serving CCs per band as the legacy.

Proposal 2: NW indicate to enable UE to activate this CSSF enhancement in specific scenarios. Otherwise, NW and UE will follow the legacy to perform serving and neighbor cell measurements.

Proposal 3: RAN4 need to clarify whether Solution 1 will affect the legacy neighbour measurement process.

Observation 3: Solution 2 is needed and it is applicable when the SCCs are in separate bands.

Observation 4: Only changing the order of SCC measurement is not able to increase the particular SCC’s measurement speed.

Proposal 4: Sharing the PCC or PSCC resource may result in the Rx beam scheduling problem.

Proposal 5: Based on the technical analysis, the 3-searcher solution is able to be used to the UE that supports“per-FR GAP”-like capability. The 3-searcher scenario can be FR1 only scenario and FR1 + FR2 scenario. It can also compromise to define the 3-searcher solution in FR1 only scenario.

Proposal 6: In scenario FR1 only, the 3-searcher solution is PCC occupied one searcher and all SCC share two searchers.

References

- R4-2420093 WF on RRM requirements for NR_RRM_Ph5_Part1