3GPP TSG-RAN WG4 Meeting #114	R4-2501290

Athens, Greece, 17th – 21st, February 2025

Source: 	Ericsson

Title:  	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Agenda Item:	7.17.2.2

Document for:	Discussion

# Introduction

As per WI of NR Radio Resource Management (RRM) enhancements (WID in [1]) for Rel-19, In last meeting 3GPP TSG RAN WG4 Meeting #113, the outcome of the debate, encompassing consensuses and unresolved issues, were documented in WF [2]. Following the discussions, we herein in this contribution present our views and proposals on the remaining open issues relevant to core requirements.

# Discussion

## Topic #2-2: L3 measurement delay reduction by optimizing CSSF for UE not in multiple-Rx simultaneous reception mode

Issue 2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement in RAN#112 meeting: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”. - Option 1 (CATT, Apple, HW, Ericsson): The enhanced CSSF apply to all the 3 aspects and do not define the priority. - Option 2 (Xiaomi, MTK, ZTE): the following aspect to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Option 2a(ZTE): To be more concentrated, prioritize the intra-frequency measurement without gap could make the discussion more efficient. But when re-design the CSSF factor, both inter-frequency and inter-RAT measurement without gap should also be considered. - Option 3 (Nokia, Samsung): To start from SSB-based intra-frequency and inter-frequency measurements without MG(aspect 1 and 2) to simplify the CSSF optimization discussion. - Recommended WF - [Moderator]: As a compromise, is option 3 agreeable?

In practical deployments, E-UTRA continues to play a key role in CA/DC configurations. We encourage the enhancement of measurement delay taking LTE into account by the chance. We’d like to understand the reason for the concern regarding aspect 2 and aspect 3, since we don’t see any technical issue preventing the same solution from being applicable for aspect 1, 2 and 3.

Proposal 1: Regarding UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF, we recommend including Aspects 1, 2, and 3 in the scope, since the same solution can deal with all aspects.

Proposal 2: Regarding Aspect 2 and 3, we suppose that it has practical use cases, and don’t observe any technical issue preventing the application. However, we’re open to Option2 and Option3 if the reason is the worry about the workload.

Issue 2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

- Option 1 (Nokia): To focus the discussion on the CSSF optimization solutions in SA scenarios. The same principle can be applied to update CSSF tables in other scenarios afterwards. - Option 2 (Ericsson): We suggest covering SA, EN-DC and NE-DC in the scope without deprioritization any of them. - Recommended WF - TBA

Similar to the last issue, LTE shall not be neglected since its application in real deployments.

Proposal 3: We suggest to cover SA, EN-DC and NE-DC in the scope without deprioritization of any of them.

Issue 2-4: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Issue 2-4-1: Tentative solution 1: measure one serving CC per band

[Wayforwad]: FFS the following solution: - If network indicates to enable this CSSF enhancement, UE only needs to measure one serving CC per band if multiple serving CCs are in the same band and UE is allowed to determine the CC in the band to be measured, - Reuse the legacy principle for neighbour cell measurement - Measure PCC if PCC in band or - Measure PSCC if PSCC in band or - Measure SCC with neighbor cell MO or - UE implementation - FFS whether indication is per-UE or per-band - Other options are not precluded. - otherwise, UE will follow the R15 spec to perform serving and neighbor cell measurements.

The basis of Solution1 is such an assumption that the channel properties and quality, e.g., due to co-location, on serving cells in one band are similar, such that the NW and UE may rely on only one CC’s measurement result for the estimation of all CCs in the same band.

For CSSF optimization, a UE can measure only one CC per band by reusing the rule in Rel-15. The fact that NW is unaware of which CC is being assessed raises concerns. As a result, NW was unable to obtain complete information regarding the overall channel quality for the entire band or all of the band's CCs. In case that the channel quality similarity throughout the entire band is invalid, it could ruin network performance, e.g., adopting improper CC(s) for the CA/DC high throughput use case.

Such that, our opinion is NW at the least shall be aware of the measured CC.

Proposal 4: For multiple MO configuration, solution1 is defined as:

- NW indicates the intended CC(s) to be measured, if more than one serving CC are in the same band.

Another underlying issue is single MO configuration, it hasn’t been clarified in previous meetings whether or not single MO configuration shall be enhanced as well in this WI. Actually, the solution basis: ‘similar channel properties on serving cells in one band’ also can be applied under single MO configuration, and the measurement delay requirements on all CCs obtain similar enhancement.

Proposal 5: For single MO configuration, if there are more than one carrier in a FR2 band,

- Both UE and NW assume the same signal quality on the carriers in the same band.

- All carriers in the same band should be believed as known carriers after measurement the configured MO.

Issue 2-4-2: Tentative solution 2: change the searcher occupancy ratio of measurement among the measured CCs

[Wayforward]: FFS: whether solution 2 is needed if solution 1 is agreed in issue 2-4-1: - UE may change the searcher occupancy ratio of SCC measurement to speed up measurement. - FFS whether network indication is needed - Details is FFS - FFS on whether and how PCC and PSCC resource sharing is supported

The CSSF rule is derived from the assumption of two searchers, and it subtly identifies the ways in which searchers are used in the following:

- One searcher is dedicated for PCC,

- The other searcher is shared by all SCCs.

  - For DC, the cell search/L3-measurement engine is dedicated for PSCC with 50% of the overall time, and for the other 50%, the engine is equally shared by the rest of the CCs.

  - For CA, half of the other searcher (50%) is dedicated for SCC where neighbour cell measurement is required, which is prioritized than other SCC(s), the remaining half of the searcher is shared among FR1 SCCs and FR2 SCCs where neighbour cell measurement is not required.

Solution1 in the aforementioned issue is unable to resolve the measurement delay problem in a network configuration with several bands set up and just a small number of carriers (e.g., 1) in each band. In contrast, this network setup can only be used with Solution2.

As reference, there are some typical multi-carrier configurations in real deployment as follows:

- For FR1 only, 5 bands/6 CCs combinations include:

  - CA_n25A-n41C-n66A-n71A-n77A

  - CA_n25A-n41(2A)-n66A-n71A-n77A

  - CA_n25(2A)-n41A-n66A-n71A-n77A

- For FR1+FR2, 3 bands combination includes:

  - DC_n25A-n41A-n258(G-J)

Furthermore, as prospective research, 3GPP shall be aware of the configurations that may add further combinations in the future.

Observation 1: Multiple bands’ scenario in real field cannot be solved by the solution (i.e., Solution 1) by optimization within one band.

As per Solution2, by (re-)arranging the measurement ratio among CCs, it seems possible to further improve the CSSF. This may optimize the measurement delay of one or a few CCs, but not all of them. We certainly think it's significant from the standpoint of scheduling and network configuration.

To enhance/shorten certain CC(s)’s measurement delay, especially for multiple bands, searcher occupancy ratio needs to be changed. The following options are at least one way to optimize measurement delay on the intended SCC(s):

- Option 1: Borrow searcher resource of the 1st searcher for PCC.

- Option 2: Borrow searcher resource of the 2nd searcher for PSCC/SCCs.

- Option 3: Option 1 and Option 2 both are applied.

We understand that there exist some concerns that reducing PCC’s measurement delay is critical and shall not be relaxed/prolonged by reducing searcher occupancy ratio. But that said, NW has the knowledge when PCC’s measurement delay can be relaxed or not based on measurement reporting, so NW may enable it only when the channel quality on PCC is qualified and stable, for example.

Proposal 6:  To optimize measurement delay on intended SCC(s),  the SCC borrows a% of PCell searcher resource.

- NW indicates the intended carrier(s).

- ‘a’ is provided by NW

One example of intended SCCs is listed in the below table (for sake of simplification, only one scenario is demonstrated here).

Table 1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 intended SCC | CSSFoutside_gap,i for FR1 unintended SCC |
| --- | --- | --- | --- |
| FR1 only CA | (1+NPCC_CSIRS + NPCC_CCA_RSSI/CO) /(1/(1-a)) | (Nint SCC_SSB- + +Y1+Z1+2x NintSCC_CSIRS + Nint SCC_CCA_RSSI/CO)/(1/a) | (NSCC_SSB- Nint SCC_SSB ) +(Y-Y1) + (Z-Z1)+2x (NSCC_CSIRS- NintSCC_CSIRS)+ (NSCC_CCA_RSSI/CO -Nint SCC_CCA_RSSI/CO) |
| Y1, Z1: the intended SCC number out of Y, Z. Y1, Z1 =0 if invalid., Nint SCC_SSB: the intended SCC number out of NSCC_SSB NintSCC_CSIRS: the intended SCC number out of NSCC_CSIRS Nint SCC_CCA_RSSI/CO: the intended SCC number out of NSCC_CCA_RSSI/CO |  |  |  |

Proposal 7:  To optimize measurement delay on intended SCC(s), the SCC borrow a% from Pcell and b%from Scells.

- NW indicates the intended carrier(s).

- ‘a’, ‘b’ are provided by NW.

- One example of intended SCCs is listed in the below table(for sake of simplification, only one scenario is demonstrated here)

Table 2: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 intended SCC | CSSFoutside_gap,i for FR1 unintended SCC |
| --- | --- | --- | --- |
| FR1 only CA | (1+NPCC_CSIRS + NPCC_CCA_RSSI/CO) /(1/(1-a)) | (Nint SCC_SSB- + +Y1+Z1+2x NintSCC_CSIRS + Nint SCC_CCA_RSSI/CO)/(1/(a+b)) | ((NSCC_SSB- Nint SCC_SSB ) +(Y-Y1) + (Z-Z1)+2x (NSCC_CSIRS- NintSCC_CSIRS)+ (NSCC_CCA_RSSI/CO -Nint SCC_CCA_RSSI/CO))/(1/(1-b)) |
| FR2 only intra band CA | N/A | N/A | N/A |
| Y1, Z1: the intended SCC number out of Y, Z. Y1, Z1 =0 if invalid., Nint SCC_SSB: the intended SCC number out of NSCC_SSB NintSCC_CSIRS: the intended SCC number out of NSCC_CSIRS Nint SCC_CCA_RSSI/CO: the intended SCC number out of NSCC_CCA_RSSI/CO |  |  |  |

Issue 2-4-3: Tentative solution 3: 3 searchers for CSSF

Agreement: RAN4 to study the scenario of using 3 searchers for CSSF reduction. The scenario is: - FFS: FR1 scenario (CA and NR-DC) only. - FFS: FR1 scenario (CA and NR-DC) and FR1 + FR2 scenario (CA and NR-DC) only. - FFS: All scenarios.

Proposal 8: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule,

- Use a dedicated searcher (the 3rd searcher) for speeding up the prioritized  carriers(s), which is provided by NW.

- If not configured, the 3rd searcher may be equally shared by multiple Scells, same as the second searcher.

# Conclusion

Observation 1: Multiple bands’ scenario in real field cannot be solved by the solution (i.e., Solution 1) by optimization within one band.

Proposal 1: Regarding UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF, we recommend including Aspects 1, 2, and 3 in the scope, since the same solution can deal with all aspects.

Proposal 2: Regarding Aspect 2 and 3, we suppose that it has practical use cases, and don’t observe any technical issue preventing the application. However, we’re open to Option2 and Option3 if the reason is the worry about the workload.

Proposal 3: We suggest to cover SA, EN-DC and NE-DC in the scope without deprioritization of any of them.

Proposal 4: For multiple MO configuration, solution1 is defined as:

- NW indicates the intended CC(s) to be measured, if more than one serving CC are in the same band.

Proposal 5: For single MO configuration, if there are more than one carrier in a FR2 band,

- Both UE and NW assume the same signal quality on the carriers in the same band.

- All carriers in the same band should be believed as known carriers after measurement the configured MO.

Proposal 6:  To optimize measurement delay on intended SCC(s),  the SCC borrows a% of PCell searcher resource.

- NW indicates the intended carrier(s).

- ‘a’ is provided by NW

One example of intended SCCs is listed in the below table (for sake of simplification, only one scenario is demonstrated here).

Table 1: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 intended SCC | CSSFoutside_gap,i for FR1 unintended SCC |
| --- | --- | --- | --- |
| FR1 only CA | (1+NPCC_CSIRS + NPCC_CCA_RSSI/CO) /(1/(1-a)) | (Nint SCC_SSB- + +Y1+Z1+2x NintSCC_CSIRS + Nint SCC_CCA_RSSI/CO)/(1/a) | (NSCC_SSB- Nint SCC_SSB ) +(Y-Y1) + (Z-Z1)+2x (NSCC_CSIRS- NintSCC_CSIRS)+ (NSCC_CCA_RSSI/CO -Nint SCC_CCA_RSSI/CO) |
| Y1, Z1: the intended SCC number out of Y, Z. Y1, Z1 =0 if invalid., Nint SCC_SSB: the intended SCC number out of NSCC_SSB NintSCC_CSIRS: the intended SCC number out of NSCC_CSIRS Nint SCC_CCA_RSSI/CO: the intended SCC number out of NSCC_CCA_RSSI/CO |  |  |  |

Proposal 7:  To optimize measurement delay on intended SCC(s), the SCC borrow a% from Pcell and b%from Scells.

- NW indicates the intended carrier(s).

- ‘a’, ‘b’ are provided by NW.

- One example of intended SCCs is listed in the below table(for sake of simplification, only one scenario is demonstrated here)

Table 2: CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 intended SCC | CSSFoutside_gap,i for FR1 unintended SCC |
| --- | --- | --- | --- |
| FR1 only CA | (1+NPCC_CSIRS + NPCC_CCA_RSSI/CO) /(1/(1-a)) | (Nint SCC_SSB- + +Y1+Z1+2x NintSCC_CSIRS + Nint SCC_CCA_RSSI/CO)/(1/(a+b)) | ((NSCC_SSB- Nint SCC_SSB ) +(Y-Y1) + (Z-Z1)+2x (NSCC_CSIRS- NintSCC_CSIRS)+ (NSCC_CCA_RSSI/CO -Nint SCC_CCA_RSSI/CO))/(1/(1-b)) |
| FR2 only intra band CA | N/A | N/A | N/A |
| Y1, Z1: the intended SCC number out of Y, Z. Y1, Z1 =0 if invalid., Nint SCC_SSB: the intended SCC number out of NSCC_SSB NintSCC_CSIRS: the intended SCC number out of NSCC_CSIRS Nint SCC_CCA_RSSI/CO: the intended SCC number out of NSCC_CCA_RSSI/CO |  |  |  |

Proposal 8: For 3 searcher scenario, on top of keeping the legacy two (the first and the second) searchers sharing rule,

- Use a dedicated searcher (the 3rd searcher) for speeding up the prioritized  carriers(s), which is provided by NW.

- If not configured, the 3rd searcher may be equally shared by multiple Scells, same as the second searcher.

# References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator)

- R4-2420093, WF for NR_RRM_Ph5, Apple