# 3GPP TSG-RAN WG4 Meeting #114	R4-2501301

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.26.4.2

Source: 	Ericsson

Title:	Discussion on LP-WUR RRM requirement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 had discussed the LP-WUS/WUR WI and made the following agreement [1]. In this contribution, we continue the discussion on the RRM impact of LP-WUR operation.

- General aspects

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Based on LP-WUS WID, different scenarios to support the RRM relaxation shall be defined as below:

- MR further relaxation with LR

- MR offloading by LR

In last RAN4 meetings, RAN4 agreed the case #1, #3, but the other MR relaxation case is FFS.

RAN4 to further discuss case #2 to #4: RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #1 Fully offloading case; Off; Off: FFS the condition and the details; ON. #2 Relaxed case a; On with relaxation measurement; Off; ON. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. #4 Relaxed case c; Off; On, FFS the condition and the details; ON.

When both NW and UE supports LP-WUS feature, different scenarios are valid by the NW using different thresholds and the UE will need to adapt its measurement behaviour to the respective zones. Adapting herein comprises turning ON/OFF the MR or LR receiver chains.

In legacy release, when neighbour cell measurement relaxation criteria were introduced, UE may perform neighbour cell measurements from normal measurement, relaxed measurement till stop the measurement. Now in Rel-19 LP-WUS, different serving cell measurement relaxation scenarios will be introduced. As shown in the figure below, different thresholds for neighbour cell measurement and serving cell measurement can be introduced for different serving cell and neighbour cell combinations.

Figure 1 Serving and neighbour cell measurement scenarios

There are still FFS for some scenarios. When UE enters LP-WUS mode, UE will use LR to monitor the serving cell quality without MR to achieve the power saving gain. Due to UE does not to combine the measurement results across MR and LR, it seems scenario 2 is not useful.

RAN4 #112 Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility. Recommendations: Suggest to discuss issues related to higher priority frequency layers under 1-1-11. Continue discuss case 2 and other issues.

Proposal 1: RAN4 not to introduce scenario 2 ‘both MR and LR for serving cell measurement but neighbour cell measurement is OFF’.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading (case 1) case/RRM relaxation (case 3) case)

In RAN2#127 meeting, RAN2 made the WS on the criteria for entry/exit criteria.

RAN2#127 LP-WUS entry/exit condition - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

From our understanding, such working assumption is aligned with the pricinples to guarantee UE to enter the LP-WUR mode carefully. In other words, before UE entering LR-based RRM relaxation or offloading, LR may already wake up during a transition period once NW configures both LR and MR criteria. We don’t think there is any issue from UE side to wake up LR a little early and we clearly see the benefits to avoid the ping-pang between the LR and MR. Thus, we propose to confirm RAN2’s WS. Thus, when NW configures both MR and LR criteria, UE needs to wake up LP-WUR earlier before LP-WUS monitoring. However, when UE wakes up the LR receiver to monitor the LR status is up to UE implementation. As a compromise, we’re fine not to define the requirement when UE is at legacy status.

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) - Proposals - P1: LR status is ON (xiaomi) - P2: LP-WUR can be ON or OFF for serving cell measurement. If the threshold for LP-WUS monitoring and LP-WUR RRM measurement are same the LP-WUR can be OFF, else if the threshold for LP-WUS monitoring is higher than for the LP-WUR RRM measurement the LP-WUR can be ON. (LG) - P3-1: LP-WUR is ON for serving cell measurement, but when and how to turn on LR for serving cell measurement is up to UE implementation. (Apple) - P3-2: When a LP-based threshold is configured, at legacy case LP-WUR needs be “ON” at least some duration early before comparing with a LP-based threshold. (vivo Huawei Ericsson) - No need define any LR measurement requirements when a UE is at legacy case (vivo Huawei)

Proposal 2: When NW configures both MR and LR criteria, UE needs to wake up LP-WUR earlier before LP-WUS monitoring. No LR measurement requirement will be defined for such scenario.

Issue 1-1-10: Considerations on higher priority frequency layer

Another remaing issue related to scenario is about higher priority relaxation. In our understanding, legacy RRM relaxation has already relaxed the higher priority measurement. We don’t think any further higher priority relaxation criteria shall be introduced.

Issue 1-1-11: Considerations on higher priority frequency layer - Proposals - P1: higher priority layer neighbor cell measurement could also be OFF for case #1. (Apple vivo) - P2: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor. (ZTE)

Proposal 3: High priority measurement relaxation criteria shall be the same as Rel-16 RRM relaxarion.

Issue 1-1-12: LP-SS frequency domain relation with SSB

To ensure NW scheduling flexibility, it should be possible to have flexible placement of LP-SS resources in frequency and time.

LP-SS frequency domain relation with SSB - Proposals - P1: RAN4 to further discuss whether there is RRM impact due to the configuration of FDMed SSB and LP-SS. (Huawei) - P2: For LP-SS and SSB FDMed multiplex, RAN4 to wait RAN1’s progress (Ericsson)

Considering in IDLE mode, MR should be used to monitor the CD-SSB in sync raster and LP-SS is unnecessary to also occupy the sync raster frequency. Since both SSB and LP-SS are broadcasted periodically, it is beneficial if the LP-SS and SSB can be set with FDM such that it allows the gNB to go into micro sleep mode to help with NW energy saving. Thus, we propose RAN4 to also consider LP-SS and SSB are FDM scenario together with TDM scenario.

Proposal 4: For LP-SS and SSB FDMed multiplex, RAN4 to wait RAN1’s progress.

- Detail LP-WUR requirements

Issue 1-2-1: Accuracy requirements

In RAN4 #111 meeting, RAN4 spent a long time to discuss how to define the accuracy requirement. Two possible way forwards were proposed.

- Define a new LP-SS accuracy directly in performance section

- Follow the legacy IDLE mode requirements to reflect accuracy in reselection margin

Issue 1-2-1: Accuracy requirements - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (xiaomi LG CMCC CT ZTE Apple Ericsson Huawei MTK) - P2: If “using a margin to reflect accuracy performance of LP-WUR in core specs”, is used for LP-WUR, for MR, the following options could be considered (vivo) - Option 1: For MR, margin is also introduced when MR serving cell measurement result comparing with a threshold, this margin will be based on legacy MR accuracy requirements. - Option 2: For MR, margin is not also introduced when MR serving cell measurement result comparing with a threshold, as that of the legacy specs. - P3: Conclude final accuracy requirement details after simulations have been collected and compared (Nokia)

We prefer the second option to reuse the same approach where the legacy SSB-based measurements in IDLE mode are used indirectly as reselection margin other than creating a new LP-SS performance part for IDLE/INACTIVE mode. The NR SSB based performance requirements were also defined because the SSB performance can be verified in CONNECTED mode, but LP-SS is only used in IDLE mode which cannot be verified due to no measurement reporting.

Proposal 5: No dedicated accuracy requirement in the performance section is defined for LP-SS based measurement and reflect the accuracy performance as a margin in the IDLE/INACTIVE mode core requirement.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements

In last meeting, RAN4 spent a long time to discuss the periodicity of SSB based LP-WUR and made the following agreements. The remaining issue is how to handle the case LO periodicity is smaller than LP-SS periodicity. In our understanding, it’s not reasonable to configure LO periodicity smaller than LP-SS periodicity. Therefore, we’re fine with no requirement is defined if LP periodicity is smaller than LP-SS periodicity.

Issue 1-2-2: Periodicity for SSB based LP-WUR measurement delay requirements Agreement: If LP-SS is not configured, periodicity for SSB based LP-WUR measurement delay requirements - based on LO periodicity - FFS whether to define an upper bound - Note: DRX cycle and LO periodicity are the same based on RAN1 agreement. Agreement: When LP-SS is configured, apply the same agreement as for when LP-SS is not configured - The above agreement applies if LO periodicity is equal to or larger than LP-SS periodicity, FFS if LO periodicity is smaller than LP-SS periodicity.

Proposal 6: No requirement is defined if LO periodicity is smaller than LP-SS periodicity.

Issue 1-2-2-2: Other on LP-WUR measurement interval for both LP-SS and SSB

Some companies also propose to discuss the measurement filtering.

Issue 1-2-2-2: Other on LP-WUR measurement interval for both LP-SS and SSB - Proposals - P1: RAN4 to define the minimum measurement interval in LP-WUR as 160ms for both LP-SS and SSB (Ericsson) - P2: The baseline periodicity of measuring LP-SS or SSB should be a multiple of LP-SS periodicity that is decided based on the corresponding DRX configuration (QC)

In legacy, the DRX case, the filtering interval is at least DRX/2. We think it’s reasonable to define the measurement filtering based on the periodicity of LP-SS. In LP-WUR, the typical LP-SS peridocity is 320ms. To achieve the comparable performance, we propose to also use 320/2ms as measurement interval.

The UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by, at least eDRX_IDLE cycle/2, if the UE is configured with eDRX cycle ≤ 10.24s; otherwise DRX cycle/2.

Proposal 7: RAN4 to define the minimum measurement interval in LP-WUR as 160ms for both LP-SS and SSB.

Issue 1-2-3: LR based serving cell evaluation

In legacy IDLE mode requirement, no serving cell evaluation relaxed criteria I is introduced. It means serving cell evaluation is always on from UE side. In Rel-19, when UE enters the LR mode, we believe the serving cell evaluation should also be kept. To avoid negative impact on paging or cell re-selection when using WUR, the following entry/exit pricinples should be followed.

- The entry should be made carefully and delay is not critical.

- The exit should be made fast because LP-SS measurement and mobility detection may delay for the exit condition to be fulfilled.

Thus, RAN4 needs to define the LP-WUR based serving cell evaluation requirement. The evaluation requirements can be defined as follow.

- For the exit condition based on single measurement period;

- For the entry condition based on evaluation period which includes multiple measurement period; Each measurement should be based on filtering of at least 2 samples.

Proposal 8: RAN4 to define LP-WUR based exit/entry evaluation requirement as follow.

- For the entry condition based on N times consecutive evaluation periods. UE needs to fulfill N times entry condition then enter the LP mode. (Such as N=2)

- For the exit condition based on evaluation period with M measurement periods to meet the accuracy requirement, then UE exits the LP mode. M depends on the simulation.

Issue 1-2-5: On LP-WUR cell selection evaluation requirements for case 1

As we mentioned before, it cannot guarantee the quality is always good enough when UE is in the LR mode even if NW configures a very tight condition to stay in purely LR mode case 1. UE may directly trigger the cell selection procedure in LR mode. Thus, RAN4 should define the LP-WUR based cell selection evelaution requirement.

Issue 1-2-5: On LP-WUR cell selection evaluation requirements for case 1 - Proposals - P1: No need to define the LR-WUR based cell selection criterion evaluation in case 1. (Apple vivo Huawei MTK) - P2: Define LP-WUR based serving cell selection criterion S evaluation requirement in case 1. (CATT Ericsson) - P2-1：The existing MR based evaluation requirements can be the baseline (CATT)

Proposal 9: RAN4 to define LP-WUR based serving cell evaluation requirement in case 1.

Issue 1-2-4: On interruption related requirements for LP-WUR

In last meeting, RAN4 discussed the related interruption requirement. Companies believed that RAN4 can directly reuse RAN1’s wake up delay value as the interruption requirement. However, in last RAN1 meeting, RAN1 made a new option to consider the time sync, delay in RAN4 later. Thus, RAN4 can wait RAN1’s further progress regarding wake-up delay capability report related to interruption requirements for LP-WUR.

RAN1 #119 meeting For future meetings: Consider the following alternatives for UE capability report on the wake-up delay: - Alt 1: For the 3 candidate values for the wake-up delay capability report, support {[80ms], [500ms] and [900ms]}. - The reported values assume SSB periodicity of 20ms, where [80ms] assumes [3] SSBs needed for synchronization, [500ms] and [900ms] assume [5] SSBs needed for synchronization. - FFS: translation of wake-up delay for different SSB periodicities - Alt 2: For the wake-up delay capability report, UE reports a pair of values for main radio ramp up and the number of SSBs needed for synchronization (D, NSSB). Support the following 3 candidate capabilities: {[(20ms, 3), (400ms, 5), (800ms, 5)]}. - Alt 3: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The number of SSBs needed for synchronization is defined in specs, and different number of SSBs can be defined for different UE capability on the main radio ramp up time. - 3 candidate values for the main radio ramp up time: {[20ms, 400ms, 800ms]} - The number of SSBs needed for synchronization is defined as [3, 5, 5] for [20ms, 400ms, 800ms] main radio ramp up time, respectively. - Alt 4: Modify the previous agreement on UE capability report on the wake-up delay to UE capability report on the main radio ramp up time. The time needed for synchronization will be defined by RAN4 requirements later.

Proposal 10: RAN4 to wait RAN1’s further progress regarding wake-up delay capability report related to interruption requirements for LP-WUR.

- MR RRM requirement

In RAN2 #127 meeting, RAN2 has made the working assumption that both MR and LR can be used to evaluate the entry condition.

RAN2#127 LP-WUS entry/exit condition - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

When UE meets the LP-WUS entry condition, UE will enter the LP-WUS state. However, at the same time, UE may still need to evaluate the neighour cell if the serving cell quality is not better than the threshold SIntraSearch, SnonIntraSearch. RAN4 agreed to introduce the scenario 3 which means UE has to perform neighbour cell measurement when UE has already entered the LP-WUS. The issue will be which kind of RRM relaxation can be applied in scenario 3.

In our understanding, the Sintrasearch can be configured quite conservatively, i.e. basically requiring the UE to measure intra-frequency neighbouring cell in the complete cell. This is typically motivated in urban environments where cell coverage does not follow a perfect circle but is rather irregular, and the UE may detect a better cell suddenly around the corner. We expect no offloading scenario 4 in this typical deployment. In other words, the power saving from LR mainly come from the RRM relaxation (scenario 3).

Observation 1: NW may always configure the neighbour cell measurements in irregular deployment.

Example of coverage in urban environment:

On one hand, UE is expected to achieve better power saving gain with a larger relaxation factor in some good deployments. On the other hand, to avoid mobility issue, it is preferred a quite conservative relaxation factor to neighbour cell measurements. At the same time, considering the LR measurement instead of MR, UE can still have chance to achieve better power saving to relax the serving cell measurements.

We understand that MR serving cell measurement can be further relaxed since UE will use LR to monitor the serving cell quality, but for the neighbour cell relaxation, it should be more careful. In legacy R16, neighbour cell measurement relaxation has already been introduced based on cell center and low mobility criteria in RRM relaxation WI. In that scenario, UE uses the serving cell measurements to monitor the cell quality and mobility. However, in Rel-19, UE will only use LR to monitor the serving cell quality. We have concerns to further relax the neighour cell measurement due to LR introduced since LR is used for power saving other than enhancing the measurement capability.

Observation 2: Both NW and UE need to tradeoff between the power saving and mobility.

In our understanding, NW has good knowledge of cell deployment and which kind of RRM relaxation can be achieved. NW can configure different relaxing factors in different deployment. Thus, we propose to introduce the NW controlled measurement relaxation. The relaxation factor should at least include 3 which means the same neighbour cell relaxation criteria as legacy.

In last meeting, some companies mentioned small scaling factor will impact the power saving gain from UE side. If NW has concern on the MR relaxation, NW can turn off the LR. However, typically, most 80% UEs are staying in the urban scenario which has complex network deployment as the figure shown above. It means the intra-frequency measurement for cell reselction is always needed. To achieve the power saving gain(mainly coming from serving cell relaxation), it should guarantee at least one small scaling facor controlled by NW. Typically, in rual scenario, NW can choose the larger scaling factor

Proposal 11: RAN4 to introduce NW controlled serving/neighbour cell measurement relaxation. The relaxation factor can be indicated by NW from one of the following values: 4, 8, 16 etc.

In last meeting, we compromised to introduce the scenario 3, but we still have concern on whether NW will configure LR based serving cell measurement with neighbour cell measurement at the same time. Thus, to allow the flexibility of network’s configuration, we propose that NW can disable such scenario 3’s relaxation scenario based on the configured threshold. How to disable such scenario depends on RAN2’s criteria design.

Proposal 12: Whether LR based serving cell RRM relaxation case is valid is fully up to NW’s configuration, such as NW can enable/disable any RRM relaxation scenario based on the configured thresholds.

- RRC-CONNECTED mode

RAN4 shall focus on developing the RRM requirements for IDLE and INACTIVE mode during this meeting since not many agreements have been reached in other WGs for CONNECTED mode. Discussions on CONNECTED mode shall therefore be postponed until any agreements are achived in other WGs regarding, and if so RAN4 can check the RRM requirements for those at later stage.

Proposal 13: Discussions on LP-WUS CONNECTED mode requirements are postoponed until more progress is achieved in other WGs.

- Others

Carrier frequency

In RAN4 meetings, RAN4 discussed the LS from RAN1 and made the conclusion that RAN4 assumed LR and MR are operating on the same carrier frequency as baseline.

In last RAN Plenary, the LP-WUS scope was also discussed. In Rel-19, a standardized solution (e.g. solution 1, 2, and 3 in RP-242656) for intra-frequency case can be defined[2]. Inter-frequency solutions (e.g. solution 4) will not be considered. It could be seen that no RAN4 spec. impact is expected even if RAN2 introduced new solution 1,2,3[3].

RAN Meeting #106 RP-243266 - For idle/inactive mode, consider a standardized solution for intra-frequency case (e.g. solution 1, 2, and 3 in RP-242656). For Rel-19, inter-frequency solutions (e.g. solution 4) will not be considered. - Detailed discussions and final solution selection is left up to WGs. RAN Meeting #106 RP-242656 Solution 1: MR is prioritized (via SIBs) to camp on the same band as LR (System information, Paging, RACH on LR band) - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline. - Frequency priority configured for LP-WUS capable UE in SIB Solution 2: MR is prioritized to camp on the same band as LR (System information, Paging on LR band; and RACH on another MR band based on reselection criteria) - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline - Frequency priority configured for LP-WUS capable UE in SIB or dedicated signalling. - UE needs to perform cell reselection following the legacy frequency priority after waking up. Solution 3: MR is not prioritized to camp on the same band as LR but can be redirected via dedicated signalling depending on LR band load (up to gNB implementation) - Mainly in RAN2 since same carrier frequency is assumed in RAN4 as baseline - Frequency priority configured for LP-WUS capable UE in dedicated signalling.

Observation 3: No RAN4 spec. impact is expected since same carrier frequency is assumed in RAN4 as baseline.

Proposal 14: RAN4 to insist on only handle the MR and LR in the same carrier frequency to define the requirment in Rel-19.

EMR

In last meeting, some companies mentioned to discuss UE’s behaviour in EMR. In legacy IDLE/Inactive mode RRM requirement, the relaxation is not allowed when EMR is configured on that frequency layer and T331 is running. However, RAN4 never discussed the UE’s behaviour when NW configures EMR in MR relaxation and offloading scenarios. Thus, we support RAN4 to discuss UE’s behaviout of MR relaxation with EMR.

Issue 1-5-1: LR based RRM with EMR in IDLE/Inactive mode - Proposals - P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple) - P2: The principle of P1 is reasonable. The issue can be discussed later (vivo) - P3: The methodology of EMR requirement definition under each Cases can refer to the methodology of high priority layer measurement requirement definition. (CMCC) - P4: Discuss the UE behaviour while MR relaxation is applied to idle-mode measurements. (Nokia) - If T331 timer is configured UE shall not enter LP-WUS mode (MR relaxation / offloading) (e.g. for the first 300 seconds in idle-mode) - RAN4 to also discuss if the rel-18 idle-mode behavior follows the same principle where UE shall keep the MR ON.

Proposal 15: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact.

- Summary

In this contribution, we have discussed the LP-WUS operation. Based on the discussions, we have made following proposals and observations:

Observation 1: NW may always configure the neighbour cell measurements in irregular deployment.

Observation 2: Both NW and UE need to tradeoff between the power saving and mobility.

Observation 3: No RAN4 spec. impact is expected since same carrier frequency is assumed in RAN4 as baseline.

Proposal 1: RAN4 not to introduce scenario 2 ‘both MR and LR for serving cell measurement but neighbour cell measurement is OFF’.

Proposal 2: When NW configures both MR and LR criteria, UE needs to wake up LP-WUR earlier before LP-WUS monitoring. No LR measurement requirement will be defined for such scenario.

Proposal 3: High priority measurement relaxation criteria shall be the same as Rel-16 RRM relaxarion.

Proposal 4: For LP-SS and SSB FDMed multiplex, RAN4 to wait RAN1’s progress.

Proposal 5: No dedicated accuracy requirement in the performance section is defined for LP-SS based measurement and reflect the accuracy performance as a margin in the IDLE/INACTIVE mode core requirement.

Proposal 6: No requirement is defined if LO periodicity is smaller than LP-SS periodicity.

Proposal 7: RAN4 to define the minimum measurement interval in LP-WUR as 160ms for both LP-SS and SSB.

Proposal 8: RAN4 to define LP-WUR based exit/entry evaluation requirement as follow.

- For the entry condition based on N times consecutive evaluation periods. UE needs to fulfill N times entry condition then enter the LP mode. (Such as N=2)

- For the exit condition based on evaluation period with M measurement periods to meet the accuracy requirement, then UE exits the LP mode. M depends on the simulation.

Proposal 9: RAN4 to define LP-WUR based serving cell evaluation requirement in case 1.

Proposal 10: RAN4 to wait RAN1’s further progress regarding wake-up delay capability report related to interruption requirements for LP-WUR.

Proposal 11: RAN4 to introduce NW controlled serving/neighbour cell measurement relaxation. The relaxation factor can be indicated by NW from one of the following values: 4, 8, 16 etc.

Proposal 12: Whether LR based serving cell RRM relaxation case is valid is fully up to NW’s configuration, such as NW can enable/disable any RRM relaxation scenario based on the configured thresholds.

Proposal 13: Discussions on LP-WUS CONNECTED mode requirements are postoponed until more progress is achieved in other WGs.

Proposal 14: RAN4 to insist on only handle the MR and LR in the same carrier frequency to define the requirment in Rel-19.

Proposal 15: RAN4 to discuss the UE’s behaviour of MR relaxation with EMR impact.

- References

[1]. R4-2420100, “WF for RRM requirements for LP-WUS/WUR”, vivo

[2]. RP-243266, “Way forward on LP-WUS”, RAN2 chair (InterDigital)

[3]. RP-242656, “On LR and MR operating in different bands in release 19”, Vodafone, Vivo, Huawei, HiSilicon