# 3GPP TSG-RAN WG4 Meeting #114	R4-2501304

# Athens, GR, 17th – 21st February, 2025

Agenda Item:	7.25.2.1

Source: 	Ericsson

Title:	Discussion on OD-SSB in Rel-19 NES

Document for:	Discussion

# Introduction

In last RAN4 meeting, the OD-SSB based deactivated SCell measurement and SCell activation known condition were agreed.

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we continue to discuss on the topic on OD-SSB operation of Rel-19 NES WI from RAN4 perspective.

- General

L1 measurement for deactivated Scell

In last meeting, there is an open issue about how to understand RAN1 agreement for L1 measurement.

Issue 1-1-2: L1 measurement before SCell activation - Proposals - Option 1: L1 measurements are not supported for deactivated SCell. - Option 2: Wait RAN1’s further clarification on the applicable scenario. Issue 1-1-3: L1 measurement after SCell activation - Proposals - Option 1: CMCC - Unified design between L1 and L3 measurement is preferred, regarding whether OD-SSB or AO-SSB or both will be used for measurement, and when/how to perform the measurement. - Option 2: CATT, Ericsson, Xiaomi - RAN4 to define OD-SSB based L1 measurement requirements based on RAN1 progress. - Option 3: LG, Apple - RAN4 to update the L1 measurement requirement based on OD-SSB. - TSSB = the periodicity of the activated OD-SSB configured for L1-RSRP measurement.

In our understanding, RAN1’s agreement is applied to the L1 measurement after receiving SCell activation command other than before SCell activation. There is no such L1 measurement mechanism for deactivated Scell today. Thus, we don’t support such L1 measurement for deactivated Scell, but it’s fine to define OD-SSB based L1 measurement during unknown Scell activation and after the Scell activated.

Proposal 1: L1 measurements are not supported for deactivated Scell. Instead, RAN4 can define OD-SSB based L1 measurement during unknown Scell activation or after the Scell activated.

OD-SSB measurement accuracy requirement

There is another open issue about OD-SSB measurement accuracy. In our understanding, OD-SSB is a specific type of SSB. It doesn’t change any SSB physical design, such as the sequence, componenets. Thus, we don’t think any new measurement accuracy requirement is needed for such OD-SSB measurement.

Issue 1-1-4: OD-SSB accuracy - Proposals - Option 1: CMCC, Ericsson, Qualcomm, China Telecom, Apple - The legacy measurement accuracy is applied for OD-SSB based measurement. - Recommended WF - Moderator suggests the group to check whether the following proposal can be agreed. - The legacy measurement accuracy is applied for OD-SSB based measurement.

Proposal 2: Legacy measurement accuracy requirements should also apply for OD-SSB.

Neighbour cell measurement

In last meeting, some companies proposed to clarify UE’s measurement behaviour for neighbour cells. RAN4 had already agreed UE to perform fast measurement in a measurement window when OD-SSB is indicated. After that, UE will perform deactivated SCell’s measurement based on legacy requirement.

Issue 1-1-7: OD-SSB impact on neighbor cell measurement clarification - Proposals - Option 1: Nokia - RAN4 to further clarify the OD-SSB based neighbour cell measurement as follow. - If there is an OD-SSB neighbor cell on a serving CC with legacy SCell, the existing measurement requirement based on SMTC shall apply to the OD-SSB neighbor cell only if OD-SSB is activated on the cell. - If there is OD-SSB SCell on the serving CC, the existing measurement requirement for neighbor cells still apply to the legacy neighbor cells on the CC. - Option 2: Huawei - For neighbor cell measurement when UE performs deactivated SCell L3 measurement based on OD-SSB, RAN4 to considering following two options: - UE is not required to perform neighbor cell measurement - UE performs neighbor cell measurement based MeasCycle. - Recommended WF - Moderator suggests the group to discuss the issue with the following options. - Option 1: The existing measurement requirement based on SMTC shall apply to - the OD-SSB neighbor cell only if OD-SSB is activated on the cell - other neighbour cell - Option 2: UE is not required to perform neighbor cell measurement - Option 3: The existing measurement requirement based on MeasCycleSCell shall apply. - Other option is not precluded

In legacy, UE will always perform neighbour cells measurement based on MeasCycleSCell. When OD-SSB based fast mode measurement introduced, it only focused on serving cell other than neighbour cell. UE can still perform neighbour cell measurement based on MeasCycleSCell. However, UE doesn’t know whether there is any OD-SSB based neighbour cell or not. Thus, UE shall always follow legacy requirement based on MeasCycleSCell for the neighbour cell measurements regardless of OD-SSB based neighbour cell exists or not.

Proposal 3: UE shall always follow legacy requirement based on MeasCycleSCell for the neighbour cell measurements regardless of OD-SSB based neighbour cell exists or not.

- OD-SSB based deactivated SCell measurement

## 3.1 Cell identification component

In last meeting, there is an open issue about whether UE needs to always perform cell identification to obtain coarse timing before deactivated Scell activation. The motivation to always require additional cell indentification is due to OD-SSB is not always transmitted.

Issue 2-1-6: OD-SSB based deactivated SCell measurement components - Proposals - Option 1: vivo - RAN4 to clarify the deactivated SCell measurement components includes both cell identification and measurement.

However, we think at least for the following cases UE doesn’t need to always perform cell identification. Firstly, for always-on SSB transmitted in case 2 means UE has performed the cell identification based on always-on SSB and doesn’t need to perform cell identification when OD-SSB is indicated. Secondly, in Case 1, if the OD-SSB SCell being measured is contiguous to an active serving cell in the same band, UE can refer on the active serving cell’s timing as legacy. No cell identification is needed. Thirdly, if UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is less than Xs and then the OD-SSB is transmitted again, cell identification is also unnecessary.

Proposal 4: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [X]s and then the OD-SSB is activated again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

## 3.2 Case 1: No always-on SSB on the cell

In last meeting, RAN4 made the following agreements about OD-SSB based deactivated SCell measurement in Case 1. The OD-SSB based fast L3 measurement framework has agreed, but some details are still FFS.

Issue 1-2-3: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1 – How to perform measurement after time window within one time OD-SSB activation/deactivation? Agreement: - After the fast time window, the UE shall meet the legacy deactivated SCell measurement requirement as long as OD-SSB is activated/transmitted from UE’s perspective. - The agreement can be revisited based on further RAN1 agreement about OD-SSB deactivation mechanism

OD-SSB based deactivated SCell measurement requirement

In last meeting, RAN4 discussed the OD-SSB based deactivated SCell measurement requirement in fast mode as follow.

Issue 1-2-1: OD-SSB based deactivated SCell measurement – measurement periodicity in fast mode measurement Agreement: For the case of CSSF = 1: - OD-SSB periodicity regardless of measCycleSCell and DRX cycle. - FFS whether a lower bound will be specified. FFS for the other cases of CSSF. Issue 1-2-2: OD-SSB based deactivated SCell measurement – CSSF in fast mode measurement - Option 1: The same CSSF as legacy - Option 2: - OD-SSB based L3 measurement is prioritized over legacy L3 measurement, if cell identification is needed for OD-SSB-based L3 measurements. - RRM requirements on PSCC and other SCCs are not applicable during the time-window. - Legacy L3 measurement on other SCC are not impacted, if OD-SSB based cell identification is not needed. Other option is not precluded.

To maximize the NES gain, NW is expected to extend the muting duration before SCell activation as much as possible. When NW indicates the OD-SSB transmission for deactivated Scell measurement, UE shall follow the OD-SSB periodicity to perform measurement. The OD-SSB based deactivated Scell measurement delay could be ceil(5 x Kp) x OD-SSB periodicity.

Proposal 5: The OD-SSB based fast L3 measurement and cell identification for deactivated Scell should be as follow.

  - UE to prioritize the measurement for Scells with activated OD-SSB in the fast window

  - The measurement within the fast window is based on OD-SSB periodicity regardless of measCycleSCell and DRX cycle

  - Lower bound is not needed for measurement within the fast window

Multiple Layers measurement

When NW configures different measurement objects for multiple frequency layers’ meaurements, the following scenarios need to be considered.

Scenario 1: One legacy measurement object with AO-SSB and one OD-SSB SCell measurement

As we mentioned before, to maximize both the NW and UE’s power saving gain, the duration of fast window shall be as short as possible. Thus, UE shall prioritize the OD-SSB based SCell’s measurement and deprioritize the legacy SCell’s measurement. The CSSF of the OD-SSB equals to 1. Longer delay of legacy SCell is expected.

Figure 1. Legacy measurement object with fast measurement window of OD-SSB based deactivated SCell

Observation 1: When OD-SSB based deactivated SCell measurement colliding with the frequency layer with AO-SSB, the CSSF of OD-SSB equals 1. Longer delay is expected for the frequency layer being measured.

Scenario 2: Two OD-SSB SCells measurement with non-overlapping fast window

When considering multiple OD-SSB SCells, one of the typical scenarios is NW configures multiple OD-SSBs with non-overlapping. For example, NW indicates OD-SSB transmission for SCell 1 and no transmission for other SCells. After UE finishing the first SCell’s OD-SSB measurement, NW deactivates the OD-SSB for SCell 1 and activates the OD-SSB for SCell 2. In our understanding, the CSSF for each OD-SSB SCell should still be 1 if the OD-SSB fast windows are non-overlapping among the OD-SSB SCells.

Figure 2. Non-overlapping Fast Measurement Windows of OD-SSB based deactivated SCell

Observation 2: When two OD-SSB based deactivated SCell measurement fast windows are non-overlapping, the CSSF for each OD-SSB based SCell equals 1.

Scenario 3: Two OD-SSB SCells measurement with overlapping fast window

However, when the OD-SSBs’ fast window are overlapping, UE had to perform measurement for these OD-SSBs in parallel. For example, the CSSF equals 2 in the following figures.

Figure 3. Overlapping Fast Measurement Windows of OD-SSB based deactivated SCell

Observation 3: When two OD-SSB based deactivated SCell measurement fast windows are overlapping, the CSSF for each OD-SSB based SCell equals 2.

Proposal 6: The OD-SSB based deactivated SCell measurement and cell identification delay requerement in fast mode are

  - FR1 measurement period: ceil(5 x Kp) x OD-SSB periodicity x CSSFOD-SSB

  - FR1 PSS/SSS detection period: ceil(5 x Kp) x OD-SSB periodicity x CSSFOD-SSB

  - FR2 measurement period: ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x OD-SSB periodicity x CSSFOD-SSB

  - FR2 PSS/SSS detection period: ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x OD-SSB periodicity x CSSFOD-SSB

Proposal 7: Longer measurement delay of other frequency layers with AO-SSBs only are expected due to OD-SSBs based fast measurement window.

Proposal 8: The CSSF of OD-SSB based deactivated SCell shall only account the number of frequency layers with activated OD-SSBs within its fast measurement window.

- The CSSF equals 1 if non-overlapping with other OD-SSBs transmission during the fast measurement window.

- The CSSF equals N+1 if overlapping with N other layres’ OD-SSB fast measurement window.

Time window

Sub-issue 3&4: How to define the time window and whether the time window is configured or pre-defined? Agreement: The time window is based on the number of OD-SSB samples and UE provided one valid report. - The uncertainty time should also be considered, such as T_min, T_processing and T_uncertainy - UE should have enough number of OD-SSB samples to achieve the measurement requirement. - The number of OD-SSB is FFS. - FFS whether UE measurement reporting is needed

In our understanding, when UE finished the first deactivated Scell measurement and reporting, UE is allowed to fallback to the legacy deactivated measurement considering the UE power issue. If NW decides to activate the Scell fast, NW can indicate the Scell activation command immediately when NW receives the deactivated measurement report. The time window can be defined from UE performing OD-SSB based fast measurement until the first time UE reporting the target OD-SSB based de-activated Scell measurement results. Once the NW deactivates the OD-SSB before the time window expires, UE is not required to measure the deactivated Scell.

Proposal 9: The time window is defined from UE performing OD-SSB based measurement in a fast mode until the time point = min(Treport, Tminimum requirement period).

- Treport is the latest valid OD-SSB based L3 measurement reporting occasion

- Tminimum OD-SSB meas. period is as follow.

        - A detected cell: OD-SSB based measurement period

        - A newly detectable cell: OD-SSB based cell identification period

However, after UE finishing the first time OD-SSB based measurement, UE may not transmit the L3 measurement report due to the event condition isn’t fulfilled. Thus, NW had to continue transmitting the OD-SSBs until the time window based on the defined minimum measurement requirement. Therefore, it’s better to introduce an OD-SSB based L3 measurement reporting. Once NW indicates the OD-SSBs, UE shall report the OD-SSB based L3 measurement reporting.

Proposal 10:  RAN4 to discuss whether to introduce OD-SSB based L3 measurement reporting.

UE behaviour when OD-SSB further activation/reconfiguration

In last RAN1 meeting, RAN1 had already agreed to introduce OD-SSB deactivation mechanism, and the detail is FFS.

RAN1 #119 Agreement For a cell supporting on-demand SSB SCell operation, support at least the following options to deactivate on-demand SSB transmission from a UE perspective. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Deactivation by RRC is up to RAN2 - FFS: Which scenario Option 1 is used - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - FFS: Whether Option 4, 4a is needed in addition to Option 2 - FFS: Whether the value of N can be implicitly determined using a timer

Thus, it’s valid to discuss UE’s behaviour when OD-SSB is further activated or reconfigured after OD-SSB deactivation.

Sub-issue 2: What’s the expected minimum interval(T2, stage 4) between multiple times OD-SSB activation/deactivation(reconfiguration)? Sub-issue 5: What’s the UE behaviour when OD-SSB further activation/reconfiguration after T2?

In last meeting, some companies proposed to define a minimul time interval T2 which means NW doesn’t need to re-activate the OD-SSB within T2 since UE can still believe the SCell as a known cell if UE reports measurement report before. In our understanding, RAN4 can define a time interval T2, if NW re-activates the OD-SSB within T2, UE only needs to follow legacy deactivated SCell measurement requirement without cell identification. After T2, the cell will be an unknown cell. UE needs to perform cell identification to acquire the timing information again, but UE can perform the cell identification based on fast mode.

Proposal 11: After OD-SSB deactivation, RAN4 to define a time interval T2 for OD-SSB reactivation which equals to the cell detectable time [X]s.

- When NW re-activates/re-configures the OD-SSB transmission within T2, UE follows legacy deactivated SCell measurement requirement.

- Otherwise, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

## 3.3 Case 2: With always-on SSB on the cell

In last meeting, RAN4 discussed OD-SSB based deactivated SCell measurement in Case 2. We proposed to follow the OD-SSB based deactivated SCell measurement procedure in Case 1. At the same time, there is another compromised solution which is to apply the Case 1 approach only in unknown SCell condition.  UE should follow the legacy requirement if the serving cell is already known. In our understanding, the main purpose to introduce the fast measurement window in case 1 is to speed up the deactivated SCell measurement to consider the SCell being activated as a known cell. When the serving cell is already a known cell, UE doesn’t need to keep fast measurement similar as the conclusion in case 1(UE falls back to legacy deactivated SCell measurement outside fast measurement window). Thus, we propose to follow the compromise solution(option 3) to only apply the fast mode when the serving cell is unknown in Case 2.

Issue 1-3-1: Scenario 1 - OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered - Option 1: UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 1-1: The measurement periodicity is OD-SSB periodicity - Option 1-2: The measurement periodicity is based on combining OD-SSB and AO-SSB - Option 2: UE always follow legacy requirements based on measCycleSCell - Option 3: The same approach in Case 1 can be reused if the serving cell is unknown. UE follows legacy requirements based on measCycleSCell if the serving cell is already known.

Proposal 12: In Case 2, if the serving cell is unknown, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. Otherwise, UE will follow legacy deactivated SCell measurement requirement.

Scenario

In RAN4 #112bis meeting, RAN4 made the agreements to support scenario 1 about the relation between AO-SSB and OD-SSB as follow.

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies.

In last meeting, RAN1 replied RAN4 LS to clarify the valid sceanarios between always-on SSB and OD-SSB as follow[2].

R1-2410917 Reply to Q1 (What is the relation in terms of periodicity between always-on SSB and OD-SSB?): - The periodicity of on-demand SSB is one of 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms. - The periodicity of on-demand SSB can be configured separately from the periodicity of always-on SSB. - RAN1 is discussing what is the relation between periodicity of always-on SSB and periodicity of on-demand SSB and it has been identified that the main use case is that the periodicity of on-demand SSB is equal to or smaller than that of always-on SSB. Reply to Q2 (What is the relation in terms of time location between always-on SSB and OD-SSB?): - RAN1 understands the time location of OD-SSB in Q2 refers to the time location of possible OD-SSB burst. - RAN1 is still discussing the relation in terms of time location between always-on SSB and OD-SSB. Reply to Q3 (What is the relation in terms of frequency location between the always-on SSB and OD-SSB?): - The frequency location of on-demand SSB is the same as the frequency location of always-on SSB at least for the case where always-on SSB is not CD-SSB. RAN1 is discussing the frequency location of OD-SSB for the case where always-on SSB is CD-SSB. Reply to Q4 (What is the spatial relation between the always-on SSB and OD-SSB?): - SS/PBCH blocks with the same SSB indexes for always-on SSB and on-demand SSB are quasi co-located with respect to Doppler spread, Doppler shift, average gain, average delay, delay spread, and when applicable, spatial RX parameters. - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - When a signal/channel is configured to be QCLed with a SSB index, the signal/channel is QCLed with the same SSB index of always-on SSB and on-demand SSB (if transmitted) with the same QCL parameters according to existing specifications - Applies at least for the case when the centre frequency locations of always-on SSB and OD-SSB is same - At least the case where SSB indices within on-demand SSB burst are identical to SSB indices within always-on SSB burst is supported. RAN1 is discussing whether to support the case where SSB indices within on-demand SSB burst can be subset of SSB indices within always-on SSB burst.

Based on RAN1 reply LS, a new valid use case about the relation of AO-SSB and OD-SSB is AO-SSB and OD-SSB are with same SSB carrier frequency, same periodicity and different time offset.

Proposal 13: In case 2, RAN4 to further support a new scenario 4 based on RAN1 reply LS.

- Scenario 4: AO-SSB and OD-SSB are with same SSB carrier frequency, same periodicity and different time offset

Combine OD-SSB and AO-SSB

In Case 2, NW transmits the legacy always-on SSB together with the OD-SSB as follows: (Green SSB is legacy always-on SSB, Orange SSB is the OD-SSB). Before NW indicating the OD-SSB transmission, UE is assumed to use legacy always-on SSB to perform deactivated SCell measurement. When NW indicates the OD-SSB transmission, it's reasonable to combine AO-SSB and OD-SSB in measurement fast window when AO-SSB and OD-SSB are on the same SSB carrier frequency with same spatial relation(scenario 1 and 4). Otherwise, there is no reason to additionally transmit the OD-SSB in these scenarios.

Figure 4. OD-SSB based deactivated SCell measurement and reporting(Case 2)

Observation 4: UE can combine the measurement results if the always-on SSB and OD-SSB are on the same frequency with the same spatial relation.

However, in last meeting, some UE vendors had the concern on UE complexity and UE behaviour if considering combining the OD-SSB and AO-SSB. As a compromise, we propose to only apply the combined requirement in the case if the periodicity of the union of AO-SSB and OD-SSB equals any predefined SSB periodicity. For example, UE can follow the combined AO-SSB and OD-SSB requirement in sub-case 1 and 2, but UE will only follow the OD-SSB based requirement in sub-case 3.

Proposal 14: In Case 2, the measurement requirement is defined based on combining the AO-SSB and OD-SSB if the periodicity of the union of AO-SSB and OD-SSB equals any defined SSB periodicity(10ms, 20ms, 40ms, 80ms, 160ms) and

- AO-SSB and OD-SSB are on the same frequency with the same spatial relation

- Whether and how to combine AO-SSB and OD-SSB is up to UE implementation

- OD-SSB based SCell activation

In last meeting, RAN4 agreed to follow the legacy known/unknown Scell activation princinples. Based on current known condition in SCell activation, the UE needs to send a valid L3 measurement report (MR) for the SCell being activated. Thus, during the time period where the Scell is deactivated, UE needs to perform deactivated SCell measurement. After NW receiving the L3 MR, the SCell can be believed as a known SCell. When NW indicates the SCell activation command, UE is assumed to perform SCell activation based on known SCell requirement. In other words, the SCell is an unknown SCell if no L3 MR is transmitted to NW.

RAN4 needs to discuss the impacts due to OD-SSB for these Scell activation requriements.

Aspect 2: OD-SSB uncertainty

In last meeting, the issue related to aspect 1: which SSB will be used in Scell activation has been agreed. The SCell activation requirement is based on OD-SSB periodicity if OD-SSB activation indication is no later than SCell activation command.

Issue 1-4-5: SCell activation requirement – OD-SSB periodicity Agreement: - In Case 1, the SCell activation delay requirement is defined based on OD-SSB periodicity. - Existing SCell activation delay requirement shall apply based on OD-SSB periodicity. - In Case 2, RAN4 to define the SCell activation delay requirement based on OD-SSB periodicity if the time instance receiving OD-SSB activation indication is no later than the time instance when receiving the SCell activation command. Otherwise, RAN4 to define the requirement based on always-on SSB for SCell activation. - This agreement is valid under the assumption of the same spatial relation for OD-SSB and always-on SSB [with the same SSB index], and this can be revisited based on RAN1 LS reply. - Note: T_firstSSB_max for intra-band co-located scenario is discussed in another issue.

The remaining issue is how to update the uncertainty time in the SCell activation requirement. Currently, in legacy requirement, TFirstSSB, TFirstSSB_MAX are introduced to define the uncertainty to receive the first complete SSB burst after HARQ processing and MAC-CE parsing time. We think a similar uncertainty shall also be introduced in OD-SSB based SCell activation. UE needs to wait for the first complete OD-SSB burst after HARQ processing and MAC-CE parsing time. RAN4 to introduce TFirstOD-SSB, TFirstOD-SSB_MAX  to the end of the first actual transmitted complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX. TFirstOD-SSB_MAX  shall be based on max SSB periodicity among active serving cells other than SMTC.

Proposal 15: RAN4 to introduce the SSB transmission uncertainty duration TFirstOD-SSB, TFirstOD-SSB_MAX which is the time to the end of the first actual transmitted complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX.

- TFirstOD-SSB_MAX is based on max SSB periodicity other than SMTC.

Aspect 4: known cell condition

In last meeting, there is an agreement about the known cell condition as follow.

Issue 1-4-1: The known cell condition – measurement reporting Agreement: - In case 1, known/unknown condition should be defined based on only OD-SSB. - In case 2, known/unknown condition should be defined based on either always-on SSB or OD-SSB. - Note 1: whether the always-on SSB and OD-SSB can be combined is discussed in another issue. - Note 2: the agreement is valid at least for the scenario with the same frequency for always-on SSB and OD-SSB. - Note 3: RAN4 to further clarify the relation for the SSB measured for known cell condition and the SSB used in SCell activation procedure.

Currently, the known cell condition is as follow.

FR1 SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. FR2 For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class1 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes.

Currently, the known cell condition is assumed the same SSB is used to perform measurement and Scell activation. Considering OD-SSB, the known cell condition shall be updated based on different OD-SSB and AO-SSB relations.

- Case 1: Deactivated SCell Measurement – AO-SSB; SCell activation-OD-SSB

- Case 2: Deactivated SCell Measurement – AO-SSB+OD-SSB; SCell activation-OD-SSB

- Case 3: Deactivated SCell Measurement – OD-SSB; SCell activation-AO-SSB

Observation 5: The measured SSB and the SSB used for SCell activation can be different.

In RAN1 reply LS, the spatial relation is the same between AO-SSB and OD-SSB.

Thus, after UE performs measurement based on any type of SSB, the cell shall be believed as a known cell even if the SSB used in SCell activation is different with the SSB in measurement.

Proposal 16: In Case 2, the Scell is believed as a known cell if

- UE has sent a valid L3-RSRP measurement report with SSB index before SCell activation

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation

- The SSB used in SCell activation has the same SSB index as the SSB measured can be detectable

Aspect 3: SSB-less SCell scenario

RAN4 also introduced the SSB-less SCell activation requirement for intra-band and inter-band collocated scenario. UE follows the SCell activation command to activate the SCell without SSB. In our understanding, UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation phase. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 17: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

- Others

Type of OD-SSB based SCell activation

In last meeting, there is an open issue about which type of OD-SSB based SCell activation will be defined.

Issue 1-5-1: The type of OD-SSB based SCell activation FFS: RAN4 to define OD-SSB based SCell activation delay requirements for the following one or more scenarios. - Option 1: Direct SCell activation at SCell addition - Option 2: Multiple SCells activation - Option 3: Deactivated PUCCH SCell activation - Option 4: Rel-17 Fast SCell Activation - Option 5: Rel-18 SCell Activation with the L3 reporting during activation

In RAN1 #118 meeting, RAN1 made the agreement to support OD-SSB based direct SCell activation. Thus, we propose RAN4 to define OD-SSB based direct SCell activation requirement.

RAN1 #118 Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

In RAN2 #127 meeting, RAN2 made the agreement to support OD-SSB based multiple SCells activation. At the same time, we understand a typical use case about OD-SSB transmission is multiple SCells activation in FR2. NW can indicate OD-SSB transmission only in one cell. After UE finishing the deactivated SCell measurement, NW can trigger multiple OD-SSB SCells activation based on known cell condition. Thus, we propose RAN4 to define OD-SSB based multiple SCells activation requirement at least for the scenarios all the SCells being activated are OD-SSB SCells.

RAN2 #127bis Agreement NW should be able to send OD-SSB indication for multiple SCells simultaneously by a MAC CE.

However, in RAN4 #112 meeting, RAN4 had the agreement about the scope and discussion priority as follow.

Issue 1-1-1: OD-SSB work plan Agreement: - RAN4 to first focus on the MAC CE based single SCell activation case as starting point. - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed.

Thus, we proposed to define OD-SSB based direct SCell activation at SCell addition once RAN4 concludes the MAC-CE based single SCell activation. The detail requirement can be discussed in the CR directly. In our understanding, fast SCell activation is mainly based on A-TRS and TRS by aperiodic CSI-RS other than SSB. Thus, RAN4 doesn’t need to define OD-SSB based fast SCell activation, but it’s fine to update the known condition based on OD-SSB.

Proposal 18: RAN4 to define the following OD-SSB based SCell activation requirement once RAN4 concludes the MAC-CE based single SCell activation. The details can be discussed in the CR directly.

- Direct SCell activation at SCell addition requirement

- Deactivated PUCCH SCell activation

- Rel-18 SCell Activation enahancement with the L3 reporting during activation

Proposal 19: RAN4 not to define the OD-SSB based Rel-17 fast SCell activation requirement, but only update the known cell condition based on OD-SSB.

Considering the work load, we suggest to limit the scope with all SCells being activated are OD-SSBs’ SCells and doesn’t impact the WI completion.

Proposal 20: RAN4 to discuss the OD-SSB based multiple SCells activation to limit the scope with all SCells being activated are OD-SSBs’ SCells if times allowed and doesn’t impact the WI’s completion.

- Summary

In this contribution, we have discussed the OD-SSB requirements. Based on the discussions, we have made following proposals and observations:

Observation 1: When OD-SSB based deactivated SCell measurement colliding with the frequency layer with AO-SSB, the CSSF of OD-SSB equals 1. Longer delay is expected for the frequency layer being measured.

Observation 2: When two OD-SSB based deactivated SCell measurement fast windows are non-overlapping, the CSSF for each OD-SSB based SCell equals 1.

Observation 3: When two OD-SSB based deactivated SCell measurement fast windows are overlapping, the CSSF for each OD-SSB based SCell equals 2.

Observation 4: UE can combine the measurement results if the always-on SSB and OD-SSB are on the same frequency with the same spatial relation.

Observation 5: The measured SSB and the SSB used for SCell activation can be different.

Proposal 1: L1 measurements are not supported for deactivated Scell. Instead, RAN4 can define OD-SSB based L1 measurement during unknown Scell activation or after the Scell activated.

Proposal 2: Legacy measurement accuracy requirements should also apply for OD-SSB.

Proposal 3: UE shall always follow legacy requirement based on MeasCycleSCell for the neighbour cell measurements regardless of OD-SSB based neighbour cell exists or not.

Proposal 4: The OD-SSB based cell identification before deactivated SCell measurement is unnecessary at least in the following scenarios.

- Case 1: No always-on SSB on the cell

  - Scenario 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band

  - Scenario 2: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [X]s and then the OD-SSB is activated again with the same spatial reception parameter

- Case 2: Always-on SSB on the cell

Proposal 5: The OD-SSB based fast L3 measurement and cell identification for deactivated Scell should be as follow.

  - UE to prioritize the measurement for Scells with activated OD-SSB in the fast window

  - The measurement within the fast window is based on OD-SSB periodicity regardless of measCycleSCell and DRX cycle

  - Lower bound is not needed for measurement within the fast window

Proposal 6: The OD-SSB based deactivated SCell measurement and cell identification delay requerement in fast mode are

  - FR1 measurement period: ceil(5 x Kp) x OD-SSB periodicity x CSSFOD-SSB

  - FR1 PSS/SSS detection period: ceil(5 x Kp) x OD-SSB periodicity x CSSFOD-SSB

  - FR2 measurement period: ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x OD-SSB periodicity x CSSFOD-SSB

  - FR2 PSS/SSS detection period: ceil(Mmeas_period_w/o_gaps x Kp x Klayer1_measurement) x OD-SSB periodicity x CSSFOD-SSB

Proposal 7: Longer measurement delay of other frequency layers with AO-SSBs only are expected due to OD-SSBs based fast measurement window.

Proposal 8: The CSSF of OD-SSB based deactivated SCell shall only account the number of frequency layers with activated OD-SSBs within its fast measurement window.

- The CSSF equals 1 if non-overlapping with other OD-SSBs transmission during the fast measurement window.

- The CSSF equals N+1 if overlapping with N other layres’ OD-SSB fast measurement window.

Proposal 9: The time window is defined from UE performing OD-SSB based measurement in a fast mode until the time point = min(Treport, Tminimum requirement period).

- Treport is the latest valid OD-SSB based L3 measurement reporting occasion

- Tminimum OD-SSB meas. period is as follow.

        - A detected cell: OD-SSB based measurement period

        - A newly detectable cell: OD-SSB based cell identification period

Proposal 10:  RAN4 to discuss whether to introduce OD-SSB based L3 measurement reporting.

Proposal 11: After OD-SSB deactivation, RAN4 to define a time interval T2 for OD-SSB reactivation which equals to the cell detectable time [X]s.

- When NW re-activates/re-configures the OD-SSB transmission within T2, UE follows legacy deactivated SCell measurement requirement.

- Otherwise, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

Proposal 12: In Case 2, if the serving cell is unknown, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, UE can perform deactivated SCell measurement with a slow mode. Otherwise, UE will follow legacy deactivated SCell measurement requirement.

Proposal 13: In case 2, RAN4 to further support a new scenario 4 based on RAN1 reply LS.

- Scenario 4: AO-SSB and OD-SSB are with same SSB carrier frequency, same periodicity and different time offset

Proposal 14: In Case 2, the measurement requirement is defined based on combining the AO-SSB and OD-SSB if the periodicity of the union of AO-SSB and OD-SSB equals any defined SSB periodicity(10ms, 20ms, 40ms, 80ms, 160ms) and

- AO-SSB and OD-SSB are on the same frequency with the same spatial relation

- Whether and how to combine AO-SSB and OD-SSB is up to UE implementation

Proposal 15: RAN4 to introduce the SSB transmission uncertainty duration TFirstOD-SSB, TFirstOD-SSB_MAX which is the time to the end of the first actual transmitted complete OD-SSB burst after HARQ processing and MAC-CE parsing time similar as legacy TFirstSSB, TFirstSSB_MAX.

- TFirstOD-SSB_MAX is based on max SSB periodicity other than SMTC.

Proposal 16: In Case 2, the Scell is believed as a known cell if

- UE has sent a valid L3-RSRP measurement report with SSB index before SCell activation

- The measured/reported SSB has the same spatial relation as the SSB used in Scell activation

- The SSB used in SCell activation which has the same SSB index as the SSB measured can be detectable

Proposal 17: UE shall follow the OD-SSB based SCell activation procedure once NW indicates the OD-SSB transmission during the SCell activation. Otherwise, UE shall follow the SSB-less SCell activation procedure.

Proposal 18: RAN4 to define the following OD-SSB based SCell activation requirement once RAN4 concludes the MAC-CE based single SCell activation.

- Direct SCell activation at SCell addition requirement

- Deactivated PUCCH SCell activation

- Rel-18 SCell Activation enahancement with the L3 reporting during activation

Proposal 19: RAN4 not to define the OD-SSB based Rel-17 fast SCell activation requirement, but only update the known cell condition based on OD-SSB.

Proposal 20: RAN4 to discuss the OD-SSB based multiple SCells activation to limit the scope with all SCells being activated are OD-SSBs’ SCells if times allowed and doesn’t impact the WI’s completion.

- References

- R4-2420098, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson

- R1-2410917, “Reply LS on SSB relation in On-demand SSB and SSB adaptation on Scell”, Ericsson

- Backup

The summary of SCell activation requirement impact due to the OD-SSB.

FR1

| # Case | # To-be-activated target SCell | # Rules | # Legacy Requirement | # OD-SSB impact |
| --- | --- | --- | --- | --- |
| # #1 | # FR1 known Scell with meas. period <= 2400ms | # Known, measurement period | # TFirstSSB | # TFirstOD-SSB |
| # #2 | # FR1 known Scell with meas. period > 2400ms |  | # TFirstSSB_MAX+ Trs | # TFirstOD-SSB_MAX+ Trs |
| # #3 | # FR1 unknown Scell with active serving cell(s) on the same band | # Unknown, with ssb-PositionInBurst, TCI indication | # TFirstSSB_MAX + TSMTC_MAX+ Trs |  |
| # #4 | # FR1 unknown Scell without active serving cell(s) on the same band |  | # TFirstSSB_MAX + TSMTC_MAX+ 2Trs |  |
| # #5 | # FR1 unknown Scell without TCI indication in MAC-CE, and with SP CSI-RS for CSI reporting | # Purely Unknown | # TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP, measure + TL1-RSRP,report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2, 3 |
| # #6 | # FR1 unknown Scell without TCI indication in MAC-CE, and with P CSI-RS for CSI reporting |  | # 3ms + TFirstSSB_MAX + TSMTC_MAX + Trs + TL1-RSRP,measure + TL1-RSRP,report + max(THARQ + Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay) |  |
| # #7 | # FR1 with active serving cell(s) on the same band | # SSB-less | # 3ms | # Not needed |

FR2

| # Case | # To-be-activated target SCell | # Rules | # Legacy Requirement | # OD-SSB impact |
| --- | --- | --- | --- | --- |
| # #8 | # FR2 with active serving cell(s) on the same band with configured SMTC | # Active serving cell in the same band(SSB-less) | # TFirstSSB+ 5ms | # TFirstOD-SSB + 5ms |
| # #9 | # FR2 with active serving cell(s) on the same band but without configured SMTC, and the RS(s) of Scell being activated is QCL-TypeD with RS of one active serving cell on that FR2 band |  | # 3ms | # Not needed |
| # #10 | # FR2 known Scell without active serving cell on the same band, and with SP-CSI-RS for CSI reporting | # First active FR2 Scell # P1: Known and unknown # P2: SP-CSI-RS, P-CSI-RS | # 3ms + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2 |
| # #11 | # FR2 known Scell without active serving cell on the same band, and with P-CSI-RS for CSI reporting |  | # max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ) |  |
| # #12 | # FR2 unknown Scell without active serving cell on the same band, and with SP-CSI-RS for CSI reporting |  | # 6ms + TFirstSSB_MAX + 15*TSMTC_MAX + 8*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) | # Aspect 2, 3 |
| # #13 | # FR2 unknown Scell without active serving cell on the same band, and with P-CSI-RS for CSI reporting |  | # 6ms + TFirstSSB_MAX + 23*TSMTC_MAX + 12*Trs + TL1-RSRP, measure + TL1-RSRP, report + THARQ + max(Tuncertainty_MAC + TFineTiming + 2ms, Tuncertainty_SP) |  |