3GPP TSG-RAN4 Meeting #114	R4-2501313

# Athens, Greece, 17th Feb 2025 - 21st Feb 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0725 | rev | - | Current version: | 17.15.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_demod_enh2-Perf) CR for 38.101-4 on Correction to FRC tables |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_demod_enh2-Perf |  |  |  |  |  | Date: |  |  | 2025-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Wrong configuration in parameter column of Table A.3.2.2.1-4 The FRC R.PDSCH.1-4.2 TDD does not match naming convention used in 38.101-4 where the FRC name match the table number. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrected the information in parameter column of Table A.3.2.2.1-4 Renamed R.PDSCH.1-4.2 TDD to R.PDSCH.1-5.1 TDD and corrected the reference in Table 5.2.2.2.20-6 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong configuration in parameter column of Table A.3.2.2.1-4 is not corrected and R.PDSCH.1-4.2 TDD still does not conform to the normal convention used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.2.20, A.3.2.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change 1>

##### 5.2.2.2.20	Minimum requirements for PDSCH with inter cell CRS interference

<Unchanged parts omitted>

Table 5.2.2.2.20-6: Minimum performance for Rank 1 without the assistance of network signaling on LTE channel bandwidth

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-5.1 TDD | 20 / 15 | 16QAM, 0.48 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 12.3 |
| 2-2 | R.PDSCH.2-29.1 TDD | 20 / 30 | 16QAM, 0.48 | FR1.30-1 | TDLA30-10 | 4x2, ULA Low | 70 | 11.7 |

# <End of change 1>

# <Start of change 2>

#### A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

<Unchanged parts omitted>

Table A.3.2.2.1-4: PDSCH Reference Channel for TDD CRS interference mitigation for NR scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-4.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} |  | 12 |  |  |  |  |
| Allocated slots per 2 frames |  | 7 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} |  | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 26632 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | CBs | N/A |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | CBs | 4 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A |  |  |  |  |
| For Slots i = 10, 11 | Bits | 53424 |  |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,9,12,…,19} | Bits | 55968 |  |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 9.3212 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with PBCH/PSS/SSS on the interference LTE cell |  |  |  |  |  |  |

Table A.3.2.2.1-5: PDSCH Reference Channel for TDD CRS interference mitigation for NR scenario for inter-RAT measurement enabled

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-5.1 TDD |  |  |  |  |
| Channel bandwidth | MHz | 20 |  |  |  |  |
| Subcarrier spacing | kHz | 15 |  |  |  |  |
| Allocated resource blocks | PRBs | 106 |  |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slots i = {2,12,22,32} |  | N/A |  |  |  |  |
| For Slots i= {0,20} |  | N/A |  |  |  |  |
| For Slots in meauresment gao i= {1,2,3,4,5,6} |  | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} |  | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} and i≠{0,1,2,20} |  | 12 |  |  |  |  |
| Allocated slots per 4 frames |  | 16 |  |  |  |  |
| MCS table |  | 64QAM |  |  |  |  |
| MCS index |  | 13 |  |  |  |  |
| Modulation |  | 16QAM |  |  |  |  |
| Target Coding Rate |  | 0.48 |  |  |  |  |
| Number of MIMO layers |  | 1 |  |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slots i = {2,12,22,32} |  | N/A |  |  |  |  |
| For Slots i= {0,20} |  | N/A |  |  |  |  |
| For Slots in meauresment gao i= {1,2,3,4,5,6} |  | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} |  | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} and i≠{0,1,2,20} |  | 12 |  |  |  |  |
| Overhead for TBS determination |  | 0 |  |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slots i = {2,12,22,32} | Bits | N/A |  |  |  |  |
| For Slots i= {0,20} | Bits | N/A |  |  |  |  |
| For Slots in meauresment gao i= {1,2,3,4,5,6} | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} and i≠{0,1,2,20} |  | 26632 |  |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slots i = {2,12,22,32} | Bits | N/A |  |  |  |  |
| For Slots i= {0,20} | Bits | N/A |  |  |  |  |
| For Slots in meauresment gao i= {1,2,3,4,5,6} | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} and i≠{0,1,2,20} |  | 24 |  |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slots i = {2,12,22,32} | CBs | N/A |  |  |  |  |
| For Slots i= {0,20} | CBs | N/A |  |  |  |  |
| For Slots in meauresment gao i= {1,2,3,4,5,6} | CBs | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} | CBs | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} and i≠{0,1,2,20} |  | 4 |  |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slots i = {2,12,22,32} | Bits | N/A |  |  |  |  |
| For Slots i= {0,20} | Bits | N/A |  |  |  |  |
| For Slots in meauresment gao i= {1,2,3,4,5,6} | Bits | N/A |  |  |  |  |
| For Slots i, if mod(i, 5) = {3,4} | Bits | N/A |  |  |  |  |
| For Slots i = {10,11,30,31} | Bits | 47936 |  |  |  |  |
| For Slots i, if mod(i, 5) = {0,1,2} and i≠{0,1,2,11,20,31} | Bits | 49184 |  |  |  |  |
| Max. Throughput averaged over 4frames | Mbps | 11.9844 |  |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 4 frames Note 3: No user data is scheduled on slots with PBCH/PSS/SSS on the interference LTE cell Note 4: No user data is scheduled on slots used for measurement |  |  |  |  |  |  |

# <End of change 2>