3GPP TSG-RAN WG4 Meeting #114	R4-2501321

# Athens, GR, 17th – 21st Feburary, 2025

Agenda Item:	6.18.2

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN n252 UE RF requirements

Document for:	Approval

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to specify UE RF core requirements for fulfilling the urgent deployment demand.

In the contribution, the consideration for NR-NTN n252 UE RF requirements is provided for discussion.

- Discussion

2.1		Spectrum WI scope for an NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)

First, regarding UE RF requirements, the objective of the WI RF core part is shown below for reference.

- Specify a new NR-NTN FDD band with a UE transmitting at 2000 – 2020 MHz and SAN transmitting at 2180-2200 MHz; - Perform necessary co-existence analysis, for example UE-UE coexistence, for adjacent band per RAN4 scope; - Consider whether assumptions for coexistence should be re-evaluated for the NTN scenario - Support channel bandwidths and SCS as presented in Table 4.1-1 below; - Mandatory support of channel raster points at step of 10 kHz; - Support UE Power class 3 (+23dBm); - Introduce the corresponding SAN and UE RF core requirements; - Introduce the corresponding RRM requirements. Table 4.1-1: Channel bandwidth and SCS system parameters. SCS (kHz); Channel bandwidth (MHz). 15; 5; 10; 15; 20. 30; 10; 15; 20. 60; 10; 15; 20. The specification work of this WI shall leverage the studies and requirements for NR NTN n256, n255, n254, where applicable. All UE requirements specified as part of this WI shall be Release-independent from Rel-17.

2.2		UE RF requirements

2.2.1	NR-NTN UE RF REFSENS for n252

In TS 38.101-5 [3], NR NTN UE reference sensitivity is shown below for reference.

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

It is observed that n252 DL frequency range (i.e., 2180-2200 MHz) is within n256 DL frequency range (i.e., 2170-2200 MHz). Thus, it is expected that band n252 can have similar RX REFSENS performance as band n256.

Observation 1: It is expected that band n252 can have similar RX REFSENS performance as band n256.

Proposal 1: Discuss whether table below could be used as band n252 REFSENS.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |
| n252 | 30 |  | -96.6 | -94.6 | -93.5 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |

2.3.2	Out-of-band Blocking for n252

It is observed that n256 has different out-of-band blocking requirements because of using the same duplexer filter to cover bands of n65 and n256.

Table 7.6.3-2: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Currently, the new duplexer filter would be used for bands n256 and n252.

Observation 2: It is expected that the same duplexer filter would be used for both bands n256 and n252.

Proposal 2: Discuss whether table below could be used for band n252 out-of-band blocking requirements.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band n252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

3 Conclusion

In this paper, our views on NT-NTN band n252 UE RF requirements are provided.

Observation 1: It is expected that band n252 can have similar RX REFSENS performance as band n256.

Proposal 1: Discuss whether table below could be used as band n252 REFSENS.

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |
| n252 | 30 |  | -96.6 | -94.6 | -93.5 |
|  | 60 |  | -97.0 | -94.9 | -93.7 |

Observation 2: It is expected that the same duplexer filter would be used for both bands n256 and n252.

Proposal 2: Discuss whether table below could be used for band n252 out-of-band blocking requirements.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2523 | Finterferer (CW) | MHz | -70 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -95 < f – FDL_low ≤ -70 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 95 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 3: Band n252 lower frequency ranges are modified to enable specific implementations. |  |  |  |  |  |

5 Reference

- RP-241661, “New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2420465, “WF on requirements for NR IoT NTN bands”, Inmarsat

- TS 38.101-5, “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”