# 3GPP TSG-RAN WG4 Meeting #114 　　　　　　　　　　　　　　　　　	R4-2501322

Athens, GR, 17th  – 21st  February, 2025

Source: 	NTT DOCOMO INC.

Title:	Views on increasing MOP tolerance for NTN HD-FDD Redcap

Agenda item:	7.29.2.1

Document for:	Discussion

- Introduction

RAN#105 approved Rel-19 revised WID on Non-Terrestrial Networks (NTN) for NR Phase 3[1]. As one of remaining issues, RAN4#112, RAN4#112bis and RAN4#113 discussed a proposal for increased maximum output power by increasing nominal power level or upper tolerance for NTN HD-FDD Redcap UE. RAN4#112bis agreed that nominal power is 23dBm. This paper provides our views on the remaining part, i.e., the value of power tolerance for maximum output power

- Discussion

Background:

- Previous RAN4 meetings discussed a proposal for increased maximum output power by increasing nominal power level or upper tolerance for NTN HD-FDD Redcap UE. The background is that insertion loss may be reduced due to simpler front-end design compared to FD-FDD. RAN4#112bis agreed that nominal power is 23dBm [2]. Remaining part is the value of power tolerance for maximum output power. As a reference, the related issue captured in the WF [3] is shown below:

## Issue 1-1: PC3 output power for (e)RedCap Agreement: - FFS whether and how a potential new scheme to increase output power is addressed in specifications - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap - If confirmed, no further work is needed. - Alt 2: Power increase by network controlled optional new signalling (e.g capability) - Alt 3: Change in power tolerances - E.g. Upper tolerance exception to UE in specific regions via table note. - FFS whether solution is common for HD-FDD and FD-FDD - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB.

Discussion:

- At present, no regulations have been established for NR NTN (e)Redcap in Japan, and therefore, there are no direct restrictions on current technical specifications. However, in cases where different tolerances are required for NTN Redcap compared to the conventional NR NTN (i.e., non-Redcap NTN), it is anticipated that some evaluation and adjustment would be required when new regulations are created. We would like to note that upper tolerance of 2dB is widely used for all operating bands including NTN and TN in current specifications.

- It is expected that a coexisting study with already existing systems will be conducted considering the worst-case conditions for both the conventional NR NTN and NTN Redcap HD-FDD. From the point of view of coexistence study with other systems, increasing MOP tolerance is same as increasing MOP itself. If there is a system exists with stringent interference conditions, there may be significant challenges in introducing NTN Redcap HD-FDD with relaxed upper tolerance.

- In case that serious coexistence issue is identified, control methods such as P-max will be required. However, P-max is applied to nominal PC, in our understanding, and thus becomes less effective when tolerance is increased. For example, assuming regulatory limit is 25dBm including upper tolerance, when P-max is set as 23dBm, NTN Redcap HD-FDD may transmit with its MOP up to 25.8dBm considering 2.8dB upper tolerance, which would exceed the regulatory limit. To avoid this, one way is to restrict upper bound of maximum output power by setting P-max to 22dBm so that we confirm UE transmitted power is below regulatory limit for NTN HD-FDD Redcap, this would result in unnecessarily restricting the transmit power to a lower level for conventional NR NTN systems.

- Given these considerations, while the direction of increasing transmit power is good to us in general, it is preferable to adopt more refined methods such as improving MPR and introducing HPUE PC from our perspective.

Proposal: For PC3 output power for HD-FDD (e)RedCap, apply the same power level and tolerance level with non-RedCap NR NTN UE.

- To address the issues mentioned above, multiple solutions were discussed during the previous meeting. One proposed solution is putting a NOTE in the maximum output power table, saying the upper tolerance limit would only be raised in regions where such an adjustment is allowed. The second solution suggests that the tolerance limit would be increased only when UE receive an explicit indication from the BS, in addition to the UE capability indication from UE to BS.

- Regarding the first proposed solution, we have several concerns. Firstly, the way how a UE will recognize whether the region allows the increased upper power tolerance are not clearly defined. Consequently, it remains ambiguous how to validate the UE behavior as a part of the 3GPP requirements. Given that this matter, as previously mentioned, has potential implications related to regulations, it would be challenging for us to agree this approach until these points are clarified.

- In contrast, the second proposed solution presents clear UE behavior and is preferable from our perspective. A similar approach has already been established in the power boosting for Pi/2-BPSK, and we can refer to this framework.

- As noted, from our perspective, it is preferable to define tolerances in the traditional manner; however, if RAN4 agrees to introduce the increased upper tolerance limit, it is essential to agree the solution of the indication from the BS as a package.

Observation 1: Regarding putting a NOTE approach, what is the exact UE behavior and how it will be verified are unclear.

Observation 2: Regarding BS indication approach, it presents clear UE behavior.

Observation 3: If RAN4 agrees to introduce the increased upper tolerance limit, it is essential to agree the solution of the indication from the BS as a package.

- Conclusion

Here we summarize our proposals:

Proposal: For PC3 output power for HD-FDD (e)RedCap, apply the same power level and tolerance level with non-RedCap NR NTN UE.

Observation 1: Regarding putting a NOTE approach, what is the exact UE behavior and how it will be verified are unclear.

Observation 2: Regarding BS indication approach, it presents clear UE behavior.

Observation 3: If RAN4 agrees to introduce the increased upper tolerance limit, it is essential to agree the solution of the indication from the BS as a package.

4	References

[1] RP-241789, “Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3”. Thales, CATT

[2] R4-2416586, “Way Forward for [112bis][310] NR_NTN_Ph3_UE_RF”, Qualcomm

[3] R4-2419897, “Way Forward for [113][310] NR_NTN_Ph3_UE_RF”, Qualcomm