3GPP TSG-RAN WG4 Meeting #114	R4-2501330

# Athens, GR, 17th – 21st Feburary, 2025

Agenda Item:	6.18.1

Source:	MediaTek Inc.

Title:	Discussion on UE-to-UE coexistence of n252/B252 UL interference with n2/B2 DL and n25/B2 DL

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the spectrum WID on introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) was approved in [1]. One of the main objectives is to perform necessary UE-UE co-existence analysis for adjacent bands (e.g., n2 and n25).

In the contribution, the consideration for UE-to-UE coexistence of n252/B252 UL interference with n2/B2 DL and n25/B25 DL is provided for discussion.

- Discussion

2.1		Previous Meeting Agreements

In RAN4#113, the approved WF [2] provided the guidance for following works. The partial contents are listed below for reference.

Issue 1-1-2: TN-NTN Isolation Distance - Offline Agreement: - The TN-NTN coexistence assumption of 1.5 km separation between NTN UE and edge of the TN coverage from TR 38.863 has been used to guarantee the minimal performance on both TN and NTN networks. Nevertheless, this might not represent a typical deployment, and an NTN UE might still be able to operate within this isolation distance - Revision of the general TN-NTN coexistence assumptions is not in scope of this WI

Issue 1-1-3: UE to UE Coexistence between n252 UL and n2/b2/n25/b25 DL - Tentative Agreement: - Agree to two different additional spurious emission requirements, each associated to an NS value and corresponding A-MPR, for protection of band b25/n25/b2/n2 - FFS: -30 dBm/MHz for relaxed value. - FFS: This emission level applies and the associated A-MPR is allowed whether or not the NS value is signalled. Also, the protection level should a note equivalent to Note 15 in 38.101-1 which says “These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.” - FFS: [-40] dBm/MHz for the stricter value based on current b23 specification - The requirement is applicable in every country where b25/n25/b2/n2 and n252 are applicable Specify asymmetric channel bandwidths with 5, 10 and [15] MHz UL CHBW for band n252.

2.2		TN-NTN Isolation Distance

If TN-NTN coexistence assumption of 1.5 km separation between NTN UE and edge of the TN coverage from TR 38.863 is not used, the ACIR will be quite higher.

In TR 38.863

#Scenario1 simulation result on NTN Coex newSBand #Scenario1 simulation result on NTN Coex newSBand

Observation 1: Isolation is necessary to protect NTN system in terms of NR-NTN UE ACS or NTN NB-IoT UE ACS.

2.3		UE-to-UE Coexistence

2.3.1	Configuration for evaluating the n252 UE-to-UE Coexistence between n252 UL and n2/b2/n25/b25 DL

To evaluate UE-to-UE coexistence requirements between n252 UL and n2/n25 DL, the configuration for testing channels is shown below.

Emission requirement of -30dBm/MHz and -40dBm/MHz for channel bandwidth 5MHz and 10MHz. - Emission below 1995MHz - for n25/B25 - To evaluate A-MPR for the following Fc. - for 5MHz channel bandwidth, Fc: 2002.5, 2007.5, 2012.5, 2017.5 (MHz) - for 10MHz channel bandwidth, Fc; 2005, 2010, 2015 (MHz) - Emission below 1990MHz - for n2/B2 - For n2/B2, do a similar A-MPR evaluation.

Since the FE filter is not considered, the preliminary measured A-MPR results are for reference only.

2.3.2 	N252 UL A-MPR values for N25 DL and N2 DL when UL 10MHz CBW is used.

2.3.2-1 	Consider NTN-UE to TN-UE co-existence requirement = -40dBm/MHz

Observation 2: Regarding UL CBW of 10MHz and co-existence requirement of -40dBm/MHz, the n252 A-MPR values for n25 DL and n2 DL are shown in Table 1 for reference.

| Table 1 | Co-existence requirement = -40dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2005, 10 | [10.2] | [4.9] |
| 2010, 10 | [4.9] | [1.3] |
| 2015, 10 | [1.4] | 0 |

2.3.2-2 	Consider NTN-UE to TN-UE co-existence requirement = -30dBm/MHz

Observation 3: Regarding UL CBW of 10MHz and co-existence requirement of -30dBm/MHz, the n252 small A-MPR values for n25 DL and n2 DL are shown in Table 2 for reference.

| Table 2 | Co-existence requirement = -30dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2005, 10 | [3.2] | [0] |
| 2010, 10 | [0] | 0 |
| 2015, 10 | 0 | 0 |

2.3.3 	N252 UL A-MPR values for N25 DL and N2 DL when UL 5MHz CBW is used.

2.3.3-1 	Consider NTN-UE to TN-UE co-existence requirement = -40dBm/MHz

Observation 4: Regarding UL CBW of 5MHz and co-existence requirement of -40dBm/MHz, the n252 medium A-MPR values for n25 DL and n2 DL are shown in Table 3 for reference.

| Table 3 | Co-existence requirement = -40dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2002.5, 5 | [5.4] | [0] |
| 2007.5, 5 | [0] | 0 |
| 2012.5, 5 | 0 | 0 |
| 2017.5, 5 | 0 | 0 |

2.3.3-2 	Consider NTN-UE to TN-UE co-existence requirement = -30dBm/MHz

Observation 5: Regarding UL CBW of 5MHz and co-existence requirement of -30dBm/MHz, the very small A-MPR values of n252 for n25 DL and n2 DL are shown in Table 4 for reference.

| Table 4 | Co-existence requirement = -30dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2002.5, 5 | [0.5] | 0 |
| 2007.5, 5 | 0 | 0 |
| 2012.5, 5 | 0 | 0 |
| 2017.5, 5 | 0 | 0 |

2.3.4 	B252 UL A-MPR values for B25 DL and B2 DL when UL 180kHHz CBW is used.

Observation 6: Regarding UL CBW of 0.2MHz and co-existence requirement of -40dBm/MHz and -30dBm/MHz, the very small A-MPR values of B252 for B25 DL and B2 DL are shown in Table 5 and Table 6 respectively for reference.

| Table 5 | Co-existence requirement = -40dBm/MHz |  |
| --- | --- | --- |
| B252 UL Fc, CBW (MHz) | B252 A-MPR (dB) For B25 DL | B252 A-MPR (dB) For B2 DL |
| 2000.1, 0.2 | [0] | 0 |

| Table 6 | Co-existence requirement = -30dBm/MHz |  |
| --- | --- | --- |
| B252 UL Fc, CBW (MHz) | B252 A-MPR (dB) For B25 DL | B252 A-MPR (dB) For B2 DL |
| 2000.1, 0.2 | 0 | 0 |

Proposal 1: As for NTN-UE to TN-UE coexistence between n252/B252 UL and n2/b2/n25/b25 DL, further discuss the spurious emission requirement and the associated A-MPR values.

3 Conclusion

In this paper, our views on A-MPR values for UE-to-UE coexistence requirements between n252/B252 UL and n2/n25/B2/B25 DL are provided.

Observation 1: Isolation is necessary to protect NTN system in terms of NR-NTN UE ACS or NTN NB-IoT UE ACS.

Observation 2: Regarding UL CBW of 10MHz and co-existence requirement of -40dBm/MHz, the n252 A-MPR values for n25 DL and n2 DL are shown in Table 1 for reference.

Observation 3: Regarding UL CBW of 10MHz and co-existence requirement of -30dBm/MHz, the n252 small A-MPR values for n25 DL and n2 DL are shown in Table 2 for reference.

Observation 4: Regarding UL CBW of 5MHz and co-existence requirement of -40dBm/MHz, the n252 medium A-MPR values for n25 DL and n2 DL are shown in Table 3 for reference.

Observation 5: Regarding UL CBW of 5MHz and co-existence requirement of -30dBm/MHz, the very small A-MPR values of n252 for n25 DL and n2 DL are shown in Table 4 for reference.

Observation 6: Regarding UL CBW of 0.2MHz and co-existence requirement of -40dBm/MHz and -30dBm/MHz, the very small A-MPR values of B252 for B25 DL and B2 DL are shown in Table 5 and Table 6 respectively for reference.

Proposal 1: As for NTN-UE to TN-UE coexistence between n252/B252 UL and n2/b2/n25/b25 DL, further discuss the spurious emission requirement and the associated A-MPR values.

| Table 1 | Co-existence requirement = -40dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2005, 10 | [10.2] | [4.9] |
| 2010, 10 | [4.9] | [1.3] |
| 2015, 10 | [1.4] | 0 |

| Table 2 | Co-existence requirement = -30dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2005, 10 | [3.2] | [0] |
| 2010, 10 | [0] | 0 |
| 2015, 10 | 0 | 0 |

| Table 3 | Co-existence requirement = -40dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2002.5, 5 | [5.4] | [0] |
| 2007.5, 5 | [0] | 0 |
| 2012.5, 5 | 0 | 0 |
| 2017.5, 5 | 0 | 0 |

| Table 4 | Co-existence requirement = -30dBm/MHz |  |
| --- | --- | --- |
| N252 UL with full RBs Fc, CBW (MHz) | N252 A-MPR (dB) For N25 DL | N252 A-MPR (dB) For N2 DL |
| 2002.5, 5 | [0.5] | 0 |
| 2007.5, 5 | 0 | 0 |
| 2012.5, 5 | 0 | 0 |
| 2017.5, 5 | 0 | 0 |

| Table 5 | Co-existence requirement = -40dBm/MHz |  |
| --- | --- | --- |
| B252 UL Fc, CBW (MHz) | B252 A-MPR (dB) For B25 DL | B252 A-MPR (dB) For B2 DL |
| 2000.1, 0.2 | [0] | 0 |

| Table 6 | Co-existence requirement = -30dBm/MHz |  |
| --- | --- | --- |
| B252 UL Fc, CBW (MHz) | B252 A-MPR (dB) For B25 DL | B252 A-MPR (dB) For B2 DL |
| 2000.1, 0.2 | 0 | 0 |

5 Reference

- RP-241661, “New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)”, DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- R4-2420465, “WF on requirements for NR IoT NTN bands”, Inmarsat