# 3GPP TSG-RAN WG4 Meeting #114	R4-2501337

# Athens, GR, 17th – 21st February, 2025

Agenda item:	7.26.4.2

Source:	China Telecom

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, RRM core requirements for LP-WUS/WUR was discussed and the WF was approved [1].

In this paper, we present our views on RRM core requirements for LP-WUS/WUR issues captured in the WF [1] and summary [2].

- Discussion

General aspects

Status in the summary [2]:

Issue 1-1-2: Threshold for switch between different cases - Proposals - P1: Up to RAN2 decision (CMCC) - P2: LR measurement can be used to check the criteria for neighbor cell measurement triggering/relaxation (in case #1). LR measurement result shall be comparable to MR measurement result or shall be equivalent to MR measurement result with certain offset/margin (e.g., LR threshold is MR threshold + offset/margin). (Apple) - P3: (Huawei) - For switch from Case #1 to Case #3 based on LR measurement. - Option 1: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ - Option 2: Use existing thresholds for SS-RSRP and SS-RSRQ, and define offsets for LP-RSRP and LP-RSRQ - For switch from Case #3 to Case #1 based on LR or both LR and MR measurements. - Option 1: LR measurement only - Option 2: both LR and MR measurements, and UE stops neighbor cell measurement when both measurements are below their corresponding thresholds - P4: Need to discuss whether case 2 will be considered or not first (LG) - P5: New LP-based threshold(s) based on LP-RSRP or LP-RSRQ used for switch into case 1 from other cases or exit from case 1 to other cases are needed (ZTE vivo) Recommendations: Check RAN2’s progress on this issue. Discuss whether new LP-based threshold(s) used for switch into case 1 from other cases or exit from case 1 to other cases are needed. Whether other cases include case 2 or not depends on issue 1-1-1, i.e., whether case 2 will be considered or not.

The threshold for switch between different cases is under discussion by RAN2, RAN4 should wait for the conclusions from RAN2.

Proposal 1: Threshold for switch between different cases is up to RAN2 decision.

Status in the summary [2]:

Issue 1-1-3: Measurement requirements to be specified for LP-WUR - Proposals - P1: Do not define requirements for OFDM-based LR serving cell measurement based on LP-SS; RAN4 defines measurement requirements for the following: (xiaomi CMCC ZTE Apple oppo) - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state; - Same requirement shall be applied for OOK-based LP-WUR serving cell measurement based on LP-SS with and without overlaid OFDM sequence at Idle/Inactive state. (Apple) - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - P2: The measurement requirements of OFDM based LP-WUR based on LP-SS with or without overlaid OFDM sequence(s) will be the same as that of OOK based LP-WUR based on LP-SS. (Huawei vivo) Background: RAN4 110bis agreement - At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - Other related requirements are FFS RAN1 118bis meeting agreement: Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 - From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences Recommendations: May need further check RAN1’s understanding on “not intended to introduce new RAN4 requirements specific to overlaid sequences”

It was agreed to support overlaid OFDM sequence(s) for LP-SS in RAN1#118bis meeting, and from RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences. Therefore, it’s preferred not to define requirements for OFDM-based LR serving cell measurement based on LP-SS. RAN4 defines measurement requirements for the following:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state;

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Proposal 2: Do not define requirements for OFDM-based LR serving cell measurement based on LP-SS; RAN4 defines measurement requirements for the following:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state;

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state.

Status in the summary [2]:

Issue 1-1-10: Considerations on higher priority frequency layer - Proposals - P1: Support relaxed higher priority frequency layer search (xiaomi CMCC CT ZTE vivo Ericsson) - P1-1 (CMCC) - When fulling offloading case happens (Case#1), the relaxed higher priority layer measurement requirement can be applied, i.e., per higher priority layer within 1h - When partial offloading case happens (Case#3), open to further discuss which higher priority layer measurement requirement should be applied - P1-2: For case 1 and case 3, when the serving cell is above threshold for inter-frequency neighbour cell measurement, existing relaxed requirements K2*Thigher_priority_search are re-used for higher priority frequency search. (vivo Ericsson) - P2: Higher priority layer neighbor cell measurement shall also be OFF for case #1. (Apple) - P3: UE should perform measurement for higher priority frequency layers, if configured (Huawei) - If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements - If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed. Recommendations: Could check P2 firstly, i.e., for case 1, whether higher priority frequency layer search will be performed or not.

In our views, higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

Proposal 3: Higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Status in the summary [2]:

Issue 1-2-1: Accuracy requirements - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (xiaomi LG CMCC CT ZTE Apple Ericsson Huawei MTK) - P2: If “using a margin to reflect accuracy performance of LP-WUR in core specs”, is used for LP-WUR, for MR, the following options could be considered (vivo) - Option 1: For MR, margin is also introduced when MR serving cell measurement result comparing with a threshold, this margin will be based on legacy MR accuracy requirements. - Option 2: For MR, margin is not also introduced when MR serving cell measurement result comparing with a threshold, as that of the legacy specs. - P3: Conclude final accuracy requirement details after simulations have been collected and compared (Nokia) Recommendations: The question of P2 is based on the following table, for example entry conditions for case 1 where both MR threshold and optional LR threshold. When a margin is defined for LP-WUR, how to handle the MR, will a margin be defined for MR as well?

For accuracy requirements, since there is no measurement reporting to network for LP-WUR measurement at RRC_IDLE/INACTIVE state, it’s proposed to reflect the accuracy performance as a margin in the core requirement, no dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states.

Proposal 4: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

MR RRM relaxation

Status in the WF [1] and the summary [2]:

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell for case 3 Agreement: Scaling factor(s) is/are >= 4. Further down-select from - Option 1: 1 fixed scaling factor - Option 2: multiple values configured by NW - Option 3: multiple values, scaling factor value depends on DRX cycle length (QC MTK apple xiaomi vivo) DRX cycle length [s]; Scaling factor. 0.32; TBD. 0.64; TBD. 1.28; TBD. 2.56; TBD. Issue 1-3-2: On Neighbour cell and serving cell relaxation factor Agreement: Same relaxation factor(s) applies to serving and neighbour cell measurements in idle/inactivate state. Issue 1-3-3: On multiple scaling factors - Proposals - P1: Multiple value on scaling factor should be avoided unless there is a clear purpose and a clear way on how to implement it. (vivo) - P2: If not possible to down-selection as single value, multiple values can be considered with delta on threshold (Samsung) - P3: Consider multiple stages of RRM relaxation, with different scaling factors (e.g., 8 in 1st stage and 16 in 2nd stage) depending on some conditions. (MTK) Recommendations:

In our views, we support to define multiple values for MR RRM relaxation scaling factor since different scaling factors achieve different power saving gains for different scenarios/conditions. Multiple values configured by NW or depended on DRX cycle length are open for us and can be further discussed.

Proposal 5: Define multiple values for MR RRM relaxation scaling factor, multiple values configured by NW or depended on DRX cycle length are open to be discussed.

LP-WUR CONNECTED mode

Status in the summary [2]:

Issue 1-4-1: LP-WUR at CONNECTED mode - Proposals - P1: No RRM objectives is expected for connected mode in this WI. (CATT xiaomi vivo LG ZTE Apple MTK) - P2: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress. Paging reception requirement impact can be discussed after further input from RAN2 and RAN1 (Samsung) - P3: Postponed until more progress in other groups (Ericsson Huawei) - P4: RAN4 to discuss interruption requirements on the WUS monitoring in Connected mode, e.g., when the WUS MO collides with a MG. (Qualcomm) - Postpone the discussion on this issue (vivo) Recommendations:

In our views, no RRM objective is expected for connected mode in this WI.

Proposal 6: No RRM objective is expected for connected mode in this WI.

- Conclusion

The contribution presents our views on Rel-19 RRM core requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: Threshold for switch between different cases is up to RAN2 decision.

Proposal 2: Do not define requirements for OFDM-based LR serving cell measurement based on LP-SS; RAN4 defines measurement requirements for the following:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state;

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state.

Proposal 3: Higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

Proposal 4: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 5: Define multiple values for MR RRM relaxation scaling factor, multiple values configured by NW or depended on DRX cycle length are open to be discussed.

Proposal 6: No RRM objective is expected for connected mode in this WI.

- References

- R4-2420100, WF on RRM requirements for NR_LPWUS, vivo, RAN WG4 Meeting #113, 18 – 22 November, 2024.

- R4-2418284, Topic summary for [113][226] NR_LPWUS, vivo, RAN WG4 Meeting #113, 18 – 22 November, 2024.