3GPP TSG RAN WG4 Meeting #114	R4-2501342

Athens, GR, 17th – 21st February, 2025

Agenda item:			7.29.4.1

Source:	vivo

Title:	Discussion on RRM requirements on (e)RedCap for R19 NR NTN Phase 3

Document for:	Discussion

- Introduction

In the last meeting, RAN4 further analyzed the potential RRM impacts on Rel-19 NR NTN WI, which mainly included the objectives of downlink coverage enhancements, support of regenerative payload and Redcap NTN. The considerations from companies and the latest progress on (e)RedCap NTN were summarized in WF [1]. In this contribution, we would like to provide our further analysis on aspects on RRM requirements on (e)RedCap NTN.

- 2.	Discussion

  - General consideration on RRM requirements

    - What RRM requirements are defined for (e)RedCap UE with FR1-NTN

      - Requirements to be further determined

#### Issue 1-1-2: The Requirements to be further determined <Way Forward>: - For SDT requirements in RRC_INACTIVE state, - Proposal 1 (CATT, QC, OPPO, HW): RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements. - Proposal 1a (CATT): TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - Proposal 1b (LG): TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission. - Proposal 1c (CMCC): For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Proposal 1d (vivo): For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable) - Proposal 2 (ZTE): CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be: Measurement; FR1. RSRP1; (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)). RSRP2; (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2. Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.. - For MDT requirements in RRC_IDLE and RRC_INACTIVE state - Proposal 1 (CATT, Samsung, QC, OPPO, Ericsson, vivo, HW, MTK): - Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN as the same situation for (e)RedCap UE in TN system. - Proposal 1a (CATT): Deprioritize to define. - Proposal 2 (CMCC): - Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap over NTN, if the related features are supported in RAN1/2 spec. - Proposal 3 (ZTE): - RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

For SDT requirements, in RAN4#112 meeting, it was agreed that PUR requirements on IoT NTN can be referred.

At the last meeting, there was an update in the TS 38.133 spec for conditions of TA validation for transmission using PUR [2].

#### 4.6A.3.3 Requirements on TA validation for transmission using PUR The UE is allowed to transmit using PUR provided that TA is valid when the following conditions are fulfilled: - either pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as confirmed by lower layers, as defined in clause 5.3.3.19 in TS 36.331 [2]. The UE shall determine the uplink timing, TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

As it is shown above, TA can be considered valid for PUR only if:

-	Either pur-TimeAlignmentTimer is not configured or

-	pur-TimeAlignmentTimer is running as confirmed by lower layers.

We believe that SDT for Redcap NTN can follow the same logic without consideration of the RSRP-based TA validation. One difference is, according to the description for the existing TS 38.213 spec, only one case in which cg-SDT-TimeAlignmentTimer is running is specified in TA Validation for CG-SDT. And for the cg-SDT-TimeAlignmentTimer, it is configured mandatorily.

TS38.213 ### 5.27.2 TA Validation for CG-SDT RRC configures the following parameters for TA validation for CG-SDT: - cg-SDT-RSRP-ChangeThreshold: RSRP threshold for the increase/decrease of RSRP for time alignment validation. The MAC entity shall, upon the reception of CG-SDT configuration: 1> store the current RSRP of the downlink pathloss reference for TA validation as defined in TS 38.331 [5] clause 5.7.17. The MAC entity shall consider the TA of the initial CG-SDT transmission with CCCH message to be valid when the following conditions are fulfilled: 1> The RSRP values for the stored downlink pathloss reference and the current downlink pathloss reference are valid according to TS 38.133 [11]; and 1> Compared to the stored downlink pathloss reference RSRP value, the current RSRP value of the downlink pathloss reference calculated as specified in TS 38.133 [11] has not increased/decreased by more than cg-SDT-RSRP-ChangeThreshold, if configured; and 1> cg-SDT-TimeAlignmentTimer is running.

Accordingly, for the requirements on CG-SDT for Redcap NTN, the description of TA validation can be adapted as:

- the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA, NTA,adjcommon, and NTA,adjUE value as specified in subclause 7.1C provided that

  - cg-SDT-TimeAlignmentTimer is running

Proposal 1: For SDT requirements in RRC_INACTIVE state, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the validation condition is adapted as:

- the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA, NTA,adjcommon, and NTA,adjUE value as specified in subclause 7.1C provided that

  - cg-SDT-TimeAlignmentTimer is running

We also noticed that we reached one related agreement in Issue 2-1-1 RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN in the last meeting, it was agreed that

#### Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Online agreement < Agreement >: - For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE.

To align with what we proposed in Proposal 1, for 1Rx Redcap NTN UE, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 will be not considered for RRC_INACTIVE mode. From this perspective, the agreement needs to be reverted as:

•	For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

Proposal 2: For SDT requirements on Redcap NTN in RRC_INACTIVE state, RAN4 to revert the agreement reached in RAN4#113 and modify it as:

- For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

And for the Minimization of Drive Tests (MDT), currently we already have the corresponding requirements defined for NTN scenario. From the deployment point of view, the data logged for MDT in NTN and (e)RedCap NTN may not be quite different. The necessity of further supporting MDT capability for Redcap UE over NTN is unconvinced.

Proposal 3: RAN4 not to define MDT requirements in RRC_IDLE and RRC_INACTIVE state

    - Features to be considered for (e)RedCap UE with FR1-NTN

#### Issue 1-2-1: Relaxed measurement Online agreement < Agreement >: - For supported types of relaxations for (e)RedCap UE with FR1-NTN, - Only to support R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for cell re-selection. - RAN4 to further discuss the thresholds and specific relaxation factor.

For supporting TN R16 relaxation (i.e., low mobility) and TN R17 relaxation (e.g., stationary), for information, we pasted the requirements for these two parts in the spec as below:

For stationary Table 4.2B.2.9.2-1: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UEs fulfilling stationary criterion for 1 Rx RedCap UE DRX cycle length [s]; Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles); Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles); Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles). 0.32; 11.52 x M2 x K3 (36 x M2 x K3); 1.28 x M2 x K3 (4 x M2 x K3); 5.12 x M2 x K3 (16 x M2 x K3). 0.64; 17.92 x K3 (28 x K3); 1.28 x K3 (2 x K3); 5.12 x K3 (8 x K3). 1.28; 32 x K3 (25 x K3); 1.28x K3 (1 x K3); 6.4 x K3 (5 x K3). 2.56; 58.88 x K3 (23 x K3); 2.56 x K3 (1 x K3); 7.68 x K3 (3 x K3). Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_Intra_RedCap_Relax is expected. Note 2: K3 = 6 is the measurement relaxation factor applicable for UE fulfilling the stationaryMobilityEvaluation [2] criterion.. For low mobility Table 4.2B.2.9.9-1: Tdetect,NR_Intra_RedCap_Relax, Tmeasure,NR_Intra_RedCap_Relax and Tevaluate,NR_Intra_RedCap_Relax for UEs fulfilling low mobility criterion for 1 Rx RedCap UE DRX cycle length [s]; Tdetect,NR_Intra_RedCap_Relax [s] (number of DRX cycles); Tmeasure,NR_Intra_RedCap_Relax [s] (number of DRX cycles); Tevaluate,NR_Intra_RedCap_Relax [s] (number of DRX cycles). 0.32; 11.52 x M2 x K1 (36 x M2 x K1); 1.28 x M2 x K1 (4 x M2 x K1); 5.12 x M2 x K1 (16 x M2 x K1). 0.64; 17.92 x K1 (28 x K1); 1.28 x K1 (2 x K1); 5.12 x K1 (8 x K1). 1.28; 32 x K1 (25 x K1); 1.28 x K1 (1 x K1); 6.4 x K1 (5 x K1). 2.56; 58.88 x K1 (23 x K1); 2.56 x K1 (1 x K1); 7.68 x K1 (3 x K1). Note 1: M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. Note 2: K1 = 3 is the measurement relaxation factor applicable for UE fulfilling the lowMobilityEvaluation [2] criterion..

At the last meeting, we raised our concern about only supporting relaxation for Red-cap NTN R16, and here we will further analyze it in more detail.

First of all, according to the existing design of RAN2, the measurement relaxation for TN does not distinguish between TN and NTN scenarios. When the network is configured with stationary criteria, the measurement for TN cell will be relaxed according to the K=6 factor when the corresponding relaxation conditions are met. For redcap NTN UE, if the stationary scenario is not supported and corresponding requirements are not defined, which specific relaxed requirements UE will follow is not clear, e.g., whether to relax according to the relaxation factor (K=3) of low mobility or completely depends on the UE implementation. From this perspective, we think it's clearer and simpler to have well-defined requirements for the stationary scenario.

Secondly, one issue raised by some companies in the past meeting is that, due to the near-far effect in NTN, the variation of RSRP may not be obvious when UE is in the center of the cell and at the edge of the cell. It caused that these two scenarios are quite similar even though the thresholds are configured differently. As for the differences between low mobility and stationary scenarios, we have the following observations:

For the configurable values of RSRP threshold, the following configuration information shows that the RSRP configurable values in the two scenarios are not the same. In particular, the stationary scenario supports an additional 2dB value.

relaxedMeasurement-r16 SEQUENCE { lowMobilityEvaluation-r16 SEQUENCE { s-SearchDeltaP-r16 ENUMERATED { dB3, dB6, dB9, dB12, dB15, spare3, spare2, spare1}, t-SearchDeltaP-r16 ENUMERATED { s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5, spare4, spare3, spare2, spare1} } OPTIONAL, -- Need R cellEdgeEvaluation-r16 SEQUENCE { s-SearchThresholdP-r16 ReselectionThreshold, s-SearchThresholdQ-r16 ReselectionThresholdQ OPTIONAL -- Need R } OPTIONAL, -- Need R combineRelaxedMeasCondition-r16 ENUMERATED {true} OPTIONAL, -- Need R highPriorityMeasRelax-r16 ENUMERATED {true} OPTIONAL -- Need R } OPTIONAL -- Need R ]], [[ cellEquivalentSize-r17 INTEGER(2..16) OPTIONAL, -- Cond HSDN relaxedMeasurement-r17 SEQUENCE { stationaryMobilityEvaluation-r17 SEQUENCE { s-SearchDeltaP-Stationary-r17 ENUMERATED {dB2, dB3, dB6, dB9, dB12, dB15, spare2, spare1}, t-SearchDeltaP-Stationary-r17 ENUMERATED {s5, s10, s20, s30, s60, s120, s180, s240, s300, spare7, spare6, spare5, spare4, spare3, spare2, spare1} }, cellEdgeEvaluationWhileStationary-r17 SEQUENCE { s-SearchThresholdP2-r17 ReselectionThreshold, s-SearchThresholdQ2-r17 ReselectionThresholdQ OPTIONAL -- Need R } OPTIONAL, -- Need R combineRelaxedMeasCondition2-r17 ENUMERATED {true} OPTIONAL -- Need R } OPTIONAL -- Need R ]] }

In addition to the difference in configurable values of RSRP threshold, as highlighted above, the time period over which the Srxlev variation evaluated for stationary criterion and low mobility criterion can also be configured differently. For stationary scenario, it additionally supports 240s and 300s. This design aims to extensionally support the stationary scenario with little variation in RSRP over long periods of time.

From the above two observations on the configuration of these two scenarios, it can be seen that the low mobility and stationary scenarios are not completely the same. Stationary scenario should not be excluded just because the RSRP variation may be small in NTN. In addition, from the flexibility of the configuration perspective, we still see the benefit of supporting both low mobility and stationary scenarios for Redcap NTN UE.

Third, when both low mobility and stationary scenarios are configured, if the stationary scenario is not supported for NTN-capable UE, the following situations will occur.

  - TN and NTN cells meet the threshold corresponding to the stationary criterion, and TN measurements will be relaxed according to the relaxation factor defined for stationary scenario. And for NTN, if

    - Case 1: the measurements for NTN are relaxed according to the relaxation factor defined for low mobility scenario, then the measurement period for NTN cells will be shorter than the measurement period for TN cells

    - Case 2: The way the NTN cell measurement is relaxed depends on the UE implementation, and no explicit measurement requirements will be defined

Specifically, it is not expected for case 1 and case 2 which will make the measurement delay requirements uncertain or inconsistent for TN/NTN.

Based on the above analysis, we still see the need to support stationary scenarios and reuse R17 stationary for relaxed measurement requirements. Therefore, we have the following proposal:

Proposal 4: RAN4 to define both R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) and Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding relaxed requirements defined in Rel-16 TN relaxation and Rel-17 RedCap relaxation can be the baseline.

  - Reduction in the number of UE Rx branches

#### Issue 2-1-5: Satellite switching with re-sync Online agreement < Agreement >: - For Satellite switching with re-sync requirements, - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy Satellite switching with re-sync requirements for normal UEs (including both hard satellite switch with re-sync and soft satellite switch with re-sync). - For hard Satellite switching with re-sync requirements, for 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs. - Define the relaxation on Tsearch by extending one more [SSB or SMTC] periodicity on the top of Tfirst_SSB: - Tsearch = Tfirst_SSB + [TSSB or TRS] ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite FFS: whether to define soft Satellite switching with re-sync requirements for 1Rx (e)RedCap UEs with FR1-NTN bands.

For Satellite switching with re-sync, the highlighted part was left for further confirmation. According to the design of RAN2 and the previous discussion on this part of R18, the network will provide SSB information of the target satellite, and UE will track SSB according to the provided SSB offset and PDD. Therefore, Tsearch is that the time is based on SSB periodicity of the source satellite and the corresponding value will be Tfirst_SSB rather than Trs.

RAN2#123bis 1. At least soft satellite switching, network provides SSB information of target satellite to UE. FFS on the details: options include e.g. indicating a time offset/information or indicating a different SSB index for the target satellite (FFS for Hard satellite switch) RAN2#124 1. At least for soft switch, there needs to be an “SSB time offset” between the source and the target satellite. “SSB time offset” is specified as a new IE, with the same format as “offset” in SSB-MTC4 2. Target satellite SSB tracking is handled autonomously by the UE based on the provided SSB time offset

For supporting 1 Rx antenna, we can consider one more SSB periodicity on the top of Tfirst_SSB. This rule can be applied for both hard satellite switch with re-sync and soft satellite switch with re-sync. To be specific, the relaxation on Tsearch for 1 Rx antenna is defined as follows:

- Relax by TSSB: Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite

TS38.133 clause 6.1C.3.2.2 Interruption time for hard satellite switch with re-sync the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + Tprocessing + T∆ + Tmargin ms Otherwise, no interruption time requirement is applied. Where: - Tsearch is the time required to search the target NR SAN cell assuming the target cell is not already known when UE starts synchronizing with target satellite. If the target cell Es/Iot ≥ -2 dB, then Tsearch = [Tfirst_SSB] ms. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - Tfirst_SSB is the time to the end of the first complete SSB burst of target satellite, the location of which is determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the [SSB-TimeOffset reference point as defined in 38.331 [2]] to UE.

Proposal 5: For Satellite switching with re-sync requirements (including both hard satellite switch with re-sync and soft satellite switch with re-sync),

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB:

  - Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite

#### Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN Online agreement < Agreement >: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - RAN4 to reuse the legacy mobility requirements in RRC_IDLE / RRC_INACTIVE state defined for normal UEs with FR1-NTN in high level. - Depend on the supported UE capabilities (e.g., multiple SMTC), the existing NR NTN requirements may need be revised. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - RAN4 to reuse the same offset of 1Rx RedCap UEs in TN compared to 2Rx (e)RedCap UEs. - For Qrxlevmin and Qqualmin: -1 dB offset can be reused. - For the measurements of intra- and inter-frequency NR cells, - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline. - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked. - In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria. - For 1 Rx RedCap by a margin of at least: - 6 dB in FR1 for reselections based on ranking or - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities. - RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring. - 6 NR inter-frequency carriers can be reused. - The parameters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)RedCap UE, and the requirements for both will be the same. - For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE. - Define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands. - FFS: How to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to further discuss how to define the cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx and 2 Rx (e)RedCap UE after issue1-1-1.

For supporting the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands, the issue of how to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands was left in the last meeting. In our understanding, for 2Rx, the existing enhanced cell reselection measurement requirements can be reused. The focus is on the discussion of the requirements for 1Rx.

According to discussions in the previous release, for Redcap UE, the core requirement does not distinguish between 1Rx and 2Rx, but distinguishes between RSRP thresholds in performance. From this perspective, it is an option for redcap NTN to consider reusing the existing enhanced cell reselection requirement for NTN LEO without distinguishing between 1Rx and 2Rx.

On the other hand, our concern for enhanced measurement mainly lies in that, compared with the normal requirement, the number of measurement samples for enhanced measurement is reduced to 1. For 1Rx, it is not sure whether the measurement accuracy can be satisfied when the current enhanced cell reselection requirement for NTN LEO is directly applied.

For this issue, we are open to discussing which is the better way to handle the enhanced requirement for 1Rx:

- Option 1: reuse the existing enhanced cell reselection measurement requirements for both 1Rx and 2 Rx (e)RedCap UE

- Option 2: for 1Rx RedCap NTN UE, consider increasing measurement samples defined in enhanced cell reselection measurement requirements. The actual number of samples can be further determined.

Proposal 6: On how to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands,

- For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements

- For 1 Rx (e)RedCap NTN UE, RAN4 to

  - Option 1: Reuse the existing enhanced cell reselection measurement requirements

  - Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined

  - The impact of HD-FDD

#### Issue 3-1-1: General HD-FDD related impact <Way Forward>: - Proposal 1 (CATT, Xiaomi, Samsung, QC, CMCC, ZTE, Ericsson, HW): - The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario. - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions. - Proposal 1a (CATT, HW, MTK): - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects: - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions. - Proposal 1c (CMCC): - Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline. - Wait for further RAN1 progress which may impact Scheduling availability requirement.

As for the impact of HD-FDD, according to the progress of RAN1#119 discussion, regarding case 3, was reached a consensus on how to handle the collision. The default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized so that no additional delay or scheduling restrictions will be introduced. For case 4, RAN1 is still discussing the specific default priority rule, whether FFS to prioritize UL or prioritize DL, and RAN4 needs to wait for RAN1's further conclusion.

RAN1#119 Agreement For Rel-19 HD-FDD (e)Redcap UE in RRC connected mode, for collision case 3, - Handling of collision with Type-0/0A/1/2-PDCCH CSS in RRC-Connected mode is left to UE implementation whether to prioritize UL or prioritize DL with the constraint in the following note. - For other use cases, default priority rule for collision case 3 in RRC-Connected mode is that DL is prioritized. - Network is allowed to indicate UL overriding DL for all the other use cases above - This is signaled by RRC configuration Note: UE shall comply to the following existing procedure in 38.331: UEs in RRC_CONNECTED shall monitor for SI change indication in any paging occasion at least once per modification period if the UE is provided with common search space, including pagingSearchSpace, searchSpaceSIB1 and searchSpaceOtherSystemInformation, on the active BWP to monitor paging, as specified in TS 38.213 [13], clause 13. Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL

Proposal 7: For HD-FDD related impact,

- The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE in NTN scenario.

- RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reach further conclusions.

- Summary

Proposal 1: For SDT requirements in RRC_INACTIVE state, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the validation condition is adapted as:

- the UE is allowed to transmit using CG-SDT using the timing derived using the latest available NTA, NTA,adjcommon, and NTA,adjUE value as specified in subclause 7.1C provided that

  - cg-SDT-TimeAlignmentTimer is running

Proposal 2: For SDT requirements on Redcap NTN in RRC_INACTIVE state, RAN4 to revert the agreement reached in RAN4#113 and modify it as:

- For SDT requirements on Redcap NTN, the condition of +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx TN UE shall be removed.

Proposal 3: RAN4 not to define MDT requirements in RRC_IDLE and RRC_INACTIVE state

Proposal 4: RAN4 to define both R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) and Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding relaxed requirements defined in Rel-16 TN relaxation and Rel-17 RedCap relaxation can be the baseline.

Proposal 5: For Satellite switching with re-sync requirements (including both hard satellite switch with re-sync and soft satellite switch with re-sync),

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the delays for Satellite switching with re-sync should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tsearch by extending one more SSB periodicity on the top of Tfirst_SSB:

  - Tsearch = Tfirst_SSB + TSSB ms if UE is only required to support 1 Rx antenna, where TSSB is the periodicity of the SSB of the source satellite

Proposal 6: On how to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands,

- For 2 Rx (e)RedCap NTN UE, RAN4 to reuse the existing enhanced cell reselection measurement requirements

- For 1 Rx (e)RedCap NTN UE, RAN4 to

  - Option 1: Reuse the existing enhanced cell reselection measurement requirements

  - Option 2: Consider increasing measurement samples based on the current cell reselection measurement enhanced requirements. The actual number of samples can be further determined

Proposal 7: For HD-FDD related impact,

- The legacy requirements and applicable conditions can be as the baseline for defining requirements of RedCap UE in NTN scenario.

- RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reach further conclusions.

- References

- R4-2420104, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT

- R4-2420185, CR on core maintenance for R18 IoT NTN, MediaTek inc.