3GPP TSG RAN WG4 Meeting #114	R4-2501345

Athens, GR, 17th – 21st February, 2025

Agenda item:			5.21.3

Source:	vivo

Title:	Discussion on LS on soft satellite switch with re-sync

Document for:	Discussion

- Introduction

In RAN2 #128, RAN2 discussed SMTC issue for the soft satellite switch and proposed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. For this solution, RAN2 raised some questions for further confirmation from RAN1/RAN4, and the detailed content was shown in LS [1]. In this paper, we will our views on the request proposed by RAN2.

- 2.	Discussion

For reference, the main content of the LS was shown as follows:

For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. To be specific, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite. RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

For the solution of applying a temporarily shifted SSB pattern during [t-ServiceStart, t-Service] illustrated as follows, we will analyze it from both network and UE perspectives.

Fig 1. The solution of applying a temporarily shifted SSB pattern

From network perspective, the main steps of the process include:

- Step 1: During [t-ServiceStart, t-Service], the network transmits the SSB of the target satellite based on a temporary SSB pattern, which shifts a specific offset based on the SSB pattern of the source satellite

- Step 2: After t-Service, the network transmits SSB of the current serving satellite according to the SSB pattern of the previous source satellite

From UE perspective, the main steps of the process include:

- Step 1: during [t-ServiceStart, t-Service], UE performs the following pre-synchronization process

  - Step 1-1. Cell search: according to the ssb-TimeOffset indicated by the network and the difference between propagation delay of the source satellite and the target satellite

  - Step 1-2. Fine time tracking

- Step 2: After t-Service, UE adjusts the SSB location of the receiving service cell according to the DL timing maintained in the first step and some potential PDD change of serving satellite information for subsequent processing

According to the above main steps from the perspective of network and UE, it can be seen that:

First, the change of SSB pattern (e.g., SSB offset) at t-Service is based on NW implementation. The network and UE are aligned to this temporary change, and it is workable for UE to adjust the implementation accordingly.

Second, in addition to the above temporary adjustments, there may be some potential PDD changes after the t-service. For this part, UE can also be calculated based on ephemeris information and its UE position. Thus, UE can detect the SSB of the serving satellite at the appropriate location.

From the above two points, we believe that RAN2's solution of a temporarily shifted SSB pattern is feasible. Specifically, for the assumption on the pre-acquired DL timing started between [t-ServiceStart, t-Service] with target satellite, it can also be confirmed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

Reply to RAN2 that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

- Summary

- Proposal 1: Reply to RAN2 that the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

- References

- R2-2410978, LS on soft satellite switch with re-sync, Huawei