3GPP TSG RAN WG4 Meeting #114		R4- 2501347

Athens GR, Feb.17th – 21st, 2025

Source: 	Nokia

Title: 	NR NTN HPUE

Agenda item:       7.10.2.3.1

Document for:     Discussion

# 1	Introduction

Based on the WF on IoT-NTN HPUE RF requirements from RAN4 #113 meeting [1]. In this paper, we’d like to share our views:

RAN4 WF from #113 meeting: Issue 1-1: Basic principles for co-ex conclusion Agreement: For co-ex conclusion discussion, to follow similar principles in deriving ACIR as documented in TR 38.863 in Rel-17. It is noted that the averaged ACIR for the most stringent case in each scenario would be derived by taking the average among the interpolated ACIR results derived from each company’s results for that case. Moreover, the following considerations are adopted to deal with major disputes for the selected cases’ results in each scenario: - If the required ACIR results, from the contributor who did not participate or their results is still not well-aligned in calibration table, has a difference larger than 10 dB with most others, this result can be not considered in the discussion. - If the required ACIR results, from one contributor, has a difference larger than 10 dB with most others, this result can be not considered in the discussion.. Issue 1-2: TN deployment scenario for co-ex conclusion Agreement: For co-ex study result conclusion discussion, in principle, to use Urban results instead of Rural results which aligns with previous agreement and worst case. Issue 1-3: TN BS antenna type for co-ex conclusion Agreement: For co-ex study result conclusion discussion, in principle, to use AAS results instead of non-AAS results as worst case. Issue 1-4: Wrap-around outside TN cells, and Issue 1-5: Isolation distance between NTN UE and TN cluster edge Agreement: Companies can provide the simulation results with or without wrap-around, and TR will capture the results with or without wrap-around separately. - With wrap-around: Enable wrap-around assuming 1.5km isolation. - Without wrap-around: Disable wrap-around assuming X isolation. - FFS whether X should be 0km or 0/1.5km. ## Sub-topic 1-2 ACIR/ACLR discussion from co-ex study Issue 1-8 to Issue 1-10: Required ACIR/ACLR for from co-ex study Agreement: Based on the results collected till RAN4#113, observations as following for the required ACLR from co-existence study evaluation perspective: For NR-NTN PC2: - Less than 22 dB with 1.5km isolation distance assumed (baseline assumption) - [26.5 dB] with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For NR-NTN PC1.5: - Less than 22.9 dB with 1.5km isolation distance assumed (baseline assumption) - [30.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For NR-NTN PC1: - Less than 23.7 dB with 1.5km isolation distance assumed (baseline assumption) - [28.3/32.7] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For IoT-NTN PC2: - Less than 21.1 dB with 1.5km isolation distance assumed; - Less than [20.3] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case For IoT-NTN PC1: - Less than 21.9 dB with 1.5km isolation distance assumed; - Less than [24.2] dB with 0km isolation distance assumed. - Interested companies are encouraged to provide results on this case # Topic #2: HPUE Tx requirements Issue 2-1: MOP Agreement: Re-use NR TN TS 38.101-1 nominal MOP and +2/-3 tolerance for PC2 for NR-NTN Non-Redcap HPUE Issue 2-2: MPR/A-MPR Agreement: MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation. Issue 2-3: SEM Way forward: - Follow previous agreement that re-use existing SEM from PC3 to other PCs as starting point. - Further discuss if SEM can be relaxed for the best power gains for NTN with co-ex study outcomes. Issue 2-4: ACLR Agreement in #112bis meeting: Issue 2-5: ACLR Agreement: - Co-existence study results and conclusion is an important factor for deriving out-band emission requirements - Check the derived ACLR for A-MPR and additional spurious emission against regulatory information.. Way forward: - Further discuss the ACLR requirements for NR-NTN HPUE - Take into account the co-ex study outcomes in this #113 meeting with previous agreements.

# 2	Discussion

2.1	Discussion for isolation distance and co-existence of TN and NTN

An isolation distance of 1.5km between TN and NTN is agreed as captured in R4-2417122. This is illustrated in Figure 1. This agreed isolation distance is used for all simulations conducted to determine RAN4 requirements needed to ensure co-existence. However, this is an assumption for the study and there is no solution to ensure this in real deployments.

Figure 1 Isolation distance between TN and NTN (38.863 V18.1)

For the real implementation of TN and NTN network, this is an issue for the overall coverage of TN and NTN. If the 1.5 isolation distance is determined around a city, then there is no operator can plan TN cell coverage neither NTN coverage within such 1.5 isolation distance. Within the isolation distance at the vicinity of a city, say 30 million population, people are not expected to receive any signal from the TN/NTN, it will have very significant impact for the people travel back and forth of this city. This is not acceptable in 5G/6G era.

Observation 1: No operator can plan the TN/NTN coverage within 1.5 isolation distance. This is not acceptable in 5G/6G era.

The physical TN cell coverage is irregular shape based on the map. It follows the residential area, industrial area, and other populations physical distribution. Therefore, it is very difficult to guarantee the 1.5 isolation distance in the real deployment of TN and NTN networks. The operator may need to enlarge the 1.5 isolation distance to make sure the TN cells will not be interfered, and this is not acceptable neither. The only solution for now is that, disable the Tx of NTN UE when NTN UE travels into the physical locations of TN cell coverage. Or NTN UEs need to adjust the Tx transmission and follow all the TN specification.

Observation 2: It is very difficult to deploy TN and NTN network in reality to guarantee 1.5 isolation distance between real TN coverage and real NTN coverage since the TN cell coverage is not regular shape, it based on location of population distribution.

Proposal 1: Zero isolation distance is suggested for TN and NTN co-existence study.

Proposal 2: A possible solution to guarantee the TN signal from being interfered by NTN signals is to disable the Tx of NTN UE when NTN UE travels into the physical locations of TN cell coverage. Or NTN UEs need to adjust the uplink transmission and follow all the TN specification.

2.2	Discussion for simulation of co-existence of TN and NTN

In the previous meeting, there were neglectable difference between the simulation results when wrapround is enabled and disable. However, the wrapround tool in Matlab (https://uk.mathworks.com/help/5g/ug/nr-interference-modeling-with-toroidal-wrap-around.html) means to facilitate the simulation in terms of saving resource and time, the simulation results should not be significantly different when wrapround is enabled. The simulation of the UEs at the TN cell border when wrapround is enabled may need to be reconsidered.

When wrapround is enabled, the 1.5 isolation distance between TN and NTN network will be different and can be illustrated as that in Figure 2. The TN cells at the border of the 1.5 isolation distance are not the same TN cells. The TN cells at the border of isolation distance when wrapround is disabled are around the center of TN coverage area when wrapround is enabled. The UEs used for simulation when wrapround is disabled should not be used when wrapround is enabled, the UEs in the new cells that are at the new border of isolation distance need to be selected for simulation.

Figure 2 isolation distance redraws with wrapround enabled

Observation 3: Simulation results between wrapround is enable and disable should not be significantly different.

Proposal 3: The 1.5 isolation distance between TN and NTN network when wrapround is enabled may need to be re-considered and can be illustrated as that in Figure 2. The UEs used for simulations need to be re-selected from the new TN cells at the border of new isolations distance.

# 3	Conclusions

In our contribution, related uplink 3Tx topics are discussed. Here is the list of our proposals:

Observation 1: No operator can plan the TN/NTN coverage within 1.5 isolation distance. This is not acceptable in 5G/6G era.

Observation 2: It is very difficult to deploy TN and NTN network in reality to guarantee 1.5 isolation distance between real TN coverage and real NTN coverage since the TN cell coverage is not regular shape, it based on location of population distribution.

Proposal 1: Zero isolation distance is suggested for TN and NTN co-existence study.

Proposal 2: A possible solution to guarantee the TN signal from being interfered by NTN signals is to disable the Tx of NTN UE when NTN UE travels into the physical locations of TN cell coverage. Or NTN UEs need to adjust the uplink transmission and follow all the TN specification.

Observation 3: Simulation results between wrapround is enable and disable should not be significantly different.

Proposal 3: The 1.5 isolation distance between TN and NTN network when wrapround is enabled may need to be re-considered and can be illustrated as that in Figure 2. The UEs used for simulations need to be re-selected from the new TN cells at the border of new isolations distance.

# References

- R4-2420403, WF on NR NTN HPUE_v02_mod