# 3GPP TSG-RAN WG4 Meeting #114	R4-2501363

Athens, Greece, 17th – 21st February, 2025

Source:			Ericsson

Title:			7 MHz channel bandwidth – BS impacts

Agenda item:		7.9.3

Document for:		Approval

# Introduction

In last RAN#106, a new WI ([1]) has been approved to specify 7 MHz channel bandwidth for NR bands n5 and n26.

In this contribution, we look at the impacts to TS 38.104 when introducing 7MHz channel bandwidth.

# Discussion

## Open issues

### Support of 7 MHz channel bandwidth – scope

The scope of the new WI ([1]) doesn’t mention explicitly for which BS type (1-C, 1-H and/or 1-O) the 7MHz channel bandwidth shall be specified.

Nevertheless, it clarifies that two bands, n5 and n26, shall support the 7 MHz channel bandwidth, both bands being in the 800MHz. As was done for the RMR n100 band (900MHz) when 3 MHz channel bandwidth was introduced, we would then make the following proposals to save some effort:

Proposal1: Restrict the support of 7 MHz channel bandwidth to FDD bands below 1 GHz.

Proposal2: Do not specify 7 MHz channel bandwidth RF requirements for BS OTA requirements (7 MHz channel BW will then not be supported by BS type 1-H and BS type 1-O).

### NB-IoT support

The new WI ([1]) doesn’t mention either if NB-IoT should be supported in the 7 MHz channel bandwidth.

When NB-IoT was introduced, all channel bandwidths specified in a band were assumed being able to support NB-IoT, the only restrictions were:

- For 3 and 5 MHz channel bandwidth and guard operation: NB-IoT guard band support was either not possible or extremely challenging.

- For 1.4 MHz channel bandwidth and NB-IoT guard band/in-band operations: NB-IoT support was not possible as this would puncture one RB, leaving only 5 RBs for LTE.

It would then make sense to support NB-IoT operation in NR in-band for 7 MHz channel bandwidth as well, when the NR band is supporting NB-IoT operation.

Proposal3: NB-IoT operating in NR in-band is also supported within 7 MHz channel bandwidth.

### IAB and NCR

The IAB and NCR specifications (TS 38.174 and TS 38.106) are not listed as impacted specifications in the new WI ([1]). Nevertheless:

- TS 38.106 refers to TS 38.101-1 when specifying the NCR-MT channel bandwidths per operating band, meaning that, if 7 MHz channel bandwidth is added to TS 38.101-1, it would be also supported by NCR. Therefore, some NCR requirements (e.g. ACLR) would also need to be updated, adding 7 MHz channel bandwidth.

- Similarly, TS 38.174 refers to TS 38.104 and TS 38.101-1 when specifying the IAB-DU and IAB-MT channel bandwidths per operating band, with the same consequence on the need to update some IAB requirements (e.g. ACLR) if 7 MHz channel bandwidth should also be supported.

This would require some additional effort while the intention was most likely to not include 7 MHz channel bandwidth support to IAB and NCR types of nodes, as those specifications were not listed in the WI.

Instead, to minimize the effort, we would do the following proposal:

Proposal4: Clarify in the NCR and IAB specifications (TS 38.106 and TS 38.174) that 7 MHz channel bandwidth is not supported for those 2 types of nodes.

## Specification impact overview

Focusing on the BS RF parameters, the following Table 2 gives an overview of the impacts in the NR specifications (TS 38.104) when introducing 7 MHz channel bandwidth.

Table 1: BS impacts overview (conducted requirements)

| Requirements | Any impact? | Comments |
| --- | --- | --- |
| 6. Conducted transmitter requirements |  |  |
| 6.1 General | No |  |
| 6.2 Base station output power | No |  |
| 6.3 Output power dynamics | No |  |
| 6.4 Transmit ON/OFF | No |  |
| 6.5 Transmitted signal quality | No |  |
| 6.6 Unwanted emissions | No |  |
| 6.6.1 General | No |  |
| 6.6.2 Occupied bandwidth | No |  |
| 6.6.3 ACLR | Yes | 7 MHz shall be added to Table 6.6.3.2-1, Table 6.6.3.2-2a and Table 6.6.3.2-3 |
| 6.6.4 Operating Band Unwanted Emissions | No |  |
| 6.6.5 Transmitter spurious emission | No |  |
| 6.7 Transmitter intermodulation | No |  |
| 7. Conducted receiver characteristics |  |  |
| 7.1 General | No |  |
| 7.2 Receiver sensitivity level | Yes | 7 MHz shall be added to Table 7.2.2-1, Table 7.2.2-2 and Table 7.2.2-3, 3rd row (with 5, 10 and 15 MHz) |
| 7.3 Dynamic range | Yes | New row(s) with 7 MHz requirements shall be added in Table 7.3.2-1, Table 7.3.2-2 and Table 7.3.2-3 And Table 7.3.2-1a, Table 7.3.2-2aand Table 7.3.2-3a if NB-IoT should be supported. |
| 7.4 In-band selectivity and blocking | Yes | 7 MHz shall be added to Table 7.4.1.2-1, Table 7.4.1.2-2, Table 7.4.2.2-1, Table 7.4.2.2-2 and Table 7.4.2.2-2 A new row for 7MHz NB interferer offset shall be added to Table 7.4.2.2-3 |
| 7.5 Out of band blocking | No |  |
| 7.6 Receiver spurious emissions | No |  |
| 7.7 Receiver intermodulation | Yes | A new row for 7MHz interferer and NB interferer shall be added to Table 7.7.2-2 and Table 7.7.2-4 |
| 7.8 In channel selectivity | Yes | 7 MHz (15kHz SCS only) shall be added to Table 7.8.2-1, Table 7.8.2-2 and Table 7.8.2-3 And Table 7.8.2-1a, Table 7.8.2-2a and Table 7.8.2-3a if NB-IoT should be supported. |

Refer to our other contribution for an overview of the systems impacts and the UE specification impacts ([2]).

Proposal5: Agree with the system impacts listed in Table 1.

# Conclusion

In this contribution, we gave an overview of the BS specifications impacts when introducing 7 MHz channel bandwidth. We made the following observations and proposals:

Proposal1: Restrict the support of 7 MHz channel bandwidth to FDD bands below 1 GHz.

Proposal2: Do not specify 7 MHz channel bandwidth RF requirements for BS OTA requirements (7 MHz channel BW will then not be supported by BS type 1-H and BS type 1-O).

Proposal3: NB-IoT operating in NR in-band is also supported within 7 MHz channel bandwidth.

Proposal4: Clarify in the NCR and IAB specifications (TS 38.106 and TS 38.174) that 7 MHz channel bandwidth is not supported for those 2 types of nodes.

Proposal5: Agree with the system impacts listed in Table 1.

# References

- RP-243323, New WID on 7 MHz Channel Bandwidth for n26 and n5, China Telecom, T-Mobile USA

- R4-2501361, 7 MHz channel bandwidth – system parameters and UE RF impacts, Ericsson