3GPP TSG-RAN WG4 Meeting #114							R4-2501401

Athens, Greece, 17th to 22nd  February 2025

Agenda Item:	7.10.2.1

Source:	Ericsson

Title:	On Co-existence simulation results for NR-NTN and NB-IoT-NTN HP UE

Document for:	Information

# 1	Introduction

In the RAN plenary meeting RAN#104, there was an agreement for a new Rel-19 RAN4 led WI Enhanced requirements and test methodology for NR and IoT NTN [1]. The baseline assumptions agreed in RAN4#113 meeting have been captured in WF [2].

This contribution provides co-existence simulation results for the Case 4 when TN UL is the victim when isolation distance assumption is 0 km.

# 2	Discussion

In the following sections, we present our simulation results based on the agreed simulation parameters in WF [2].

## 2.1	Case 4 – TN UL as victim and NR NTN/ NB-IoT NTN UL as aggressor

In this sub-section, we share the simulation results between NR-NTN/ NB-IoT NTN HPUE interfering TN UL with Power class 2,1.5 and 1. In this scenario, NTN HPUE is transmitting as aggressor and TN gNB receiving as victim. The two NTN scenarios considered are GEO and LEO1200.

The isolation distance assumption is 0 km as per agreements in WF [2] between NTN UEs and the edge of TN cluster. The ISD for Urban Macro is 750m. NTN UEs are placed at the NTN cluster edge. This section is further divided based on the assumption of with/ without wrap around functionality in order to evaluate performance impact.

Some of the other assumptions are as following:

- With isolation distance of 0km

- Statistics are based on Option 2: TN sectors which have NTN UEs at their edges

- NR NTN/ IoT-NTN HPUE ratio is 100%

- NTN UE NF is 9 dB

- NB-IoT NTN UL UEs and SCS – 9 UEs for 15 kHz

#### 2.1.1	With wrap around

##### NR NTN; TN BS type – AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 28 | 30 | 32 | 34 | 36 | 38 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 3 | 3 | 2 | 2 | 1 | 0 |  |
|  |  |  | 5%-tile | 7 | 5 | 4.5 | 4 | 1 | 1 |  |
|  | PC1.5 | Throughput Loss | Average | 5 | 4 | 3 | 2 | 2 | 1 | 1 |
|  |  |  | 5%-tile | 12 | 11 | 6 | 5 | 4 | 2 | 1 |
|  | PC1 | Throughput Loss | Average | 6 | 5 | 4 | 3 | 2 | 2 | 1 |
|  |  |  | 5%-tile | 22 | 12 | 11 | 6 | 5 | 4 | 2 |
| LEO1200 | PC2 | Throughput Loss | Average | 4 | 3 | 2 | 1 | 1 |  |  |
|  |  |  | 5%-tile | 8 | 5 | 3 | 3 | 3 | 3 |  |
|  | PC1.5 | Throughput Loss | Average | 5 | 4 | 3 | 2 | 2 | 1 | 1 |
|  |  |  | 5%-tile | 10 | 8 | 6 | 5 | 3 | 2 | 2 |
|  | PC1 | Throughput Loss | Average | 7 | 5 | 4 | 3 | 2 | 2 | 1 |
|  |  |  | 5%-tile | 12 | 10 | 8 | 6 | 5 | 3 | 2 |

##### 2.1.1.2	NR NTN; TN BS type – Non AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 28 | 30 | 32 | 34 | 36 | 38 | 40 | 42 | 44 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 2 | 1 | 1 | 1 | 1 | 0 | 0 |  |  |
|  |  |  | 5%-tile | 10 | 6 | 4.5 | 2.1 | 2.1 | 2.1 | 2.1 |  |  |
|  | PC1.5 | Throughput Loss | Average | 3 | 2 | 2 | 1 | 1 | 1 | 1 |  |  |
|  |  |  | 5%-tile | 11 | 10 | 10 | 10 | 2 | 2 | 2 |  |  |
|  | PC1 | Throughput Loss | Average | 4 | 3 | 2 | 2 | 1 | 1 | 1 |  |  |
|  |  |  | 5%-tile | 12 | 11 | 10 | 10 | 10 | 2 | 2 |  |  |
| LEO1200 | PC2 | Throughput Loss | Average | 3 | 2.4 | 1.9 | 1.5 | 1.1 | 0.9 | 0.6 |  |  |
|  |  |  | 5%-tile | 8.4 | 7.6 | 6.4 | 5.2 | 4.8 | 3.5 | 3.5 |  |  |
|  | PC1.5 | Throughput Loss | Average |  |  |  | 2.1 | 1.6 | 1.3 | 1 | 0.7 | 0.5 |
|  |  |  | 5%-tile |  |  |  | 7.6 | 6.2 | 4.8 | 4.8 | 3.5 | 2.5 |
|  | PC1 | Throughput Loss | Average |  |  |  | 2.7 | 2.1 | 1.6 | 1 | 1 | 1 |
|  |  |  | 5%-tile |  |  |  | 8.2 | 7.6 | 6.2 | 4.8 | 4.8 | 3.5 |

##### 2.1.1.3	NB-IoT NTN; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 26 | 28 | 30 | 32 | 34 | 36 | 38 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | SNR Loss | Average | 0.9 | 0.7 | 0.6 | 0.5 | 0.4 | 0.3 |  |  |
|  |  |  | 5%-tile | 3.6 | 2.6 | 1.9 | 1.0 | 0.9 | 0.8 |  |  |
|  | PC1.5 | SNR Loss | Average | 1.1 | 0.9 | 0.8 | 0.6 | 0.5 | 0.4 |  |  |
|  |  |  | 5%-tile | 6.3 | 4.5 | 2.9 | 2.2 | 1.2 | 0.9 |  |  |
|  | PC1 | SNR Loss | Average |  |  | 0.9 | 0.8 | 0.6 | 0.5 | 0.4 | 0.4 |
|  |  |  | 5%-tile |  |  | 4.5 | 2.9 | 2.2 | 1.2 | 0.9 | 0.8 |
| LEO1200 | PC2 | SNR Loss | Average | 0.8 | 0.6 | 0.5 | 0.4 | 0.3 | 0.2 |  |  |
|  |  |  | 5%-tile | 1.6 | 1.2 | 0.9 | 0.5 | 0.3 | 0.3 |  |  |
|  | PC1.5 | SNR Loss | Average | 1.1 | 0.9 | 0.7 | 0.6 | 0.5 | 0.4 |  |  |
|  |  |  | 5%-tile | 2.4 | 1.8 | 1.5 | 0.9 | 0.8 | 0.3 |  |  |
|  | PC1 | SNR Loss | Average |  |  |  |  |  |  |  |  |
|  |  |  | 5%-tile | 3.6 | 2.5 | 1.8 | 1.5 | 0.9 | 0.7 |  |  |

#### 2.1.2	Without wrap around

##### 2.1.2.1	NR NTN; TN BS type – AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 28 | 30 | 32 | 34 | 36 | 38 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 5,0 | 4,0 | 3,0 | 2,0 | 2,0 | 1,0 |  |
|  |  |  | 5%-tile | 6,0 | 6,0 | 6,0 | 4,0 | 2,0 | 2,0 |  |
|  | PC1.5 | Throughput Loss | Average | 6,0 | 5,0 | 4,0 | 3,0 | 2,0 | 1,0 |  |
|  |  |  | 5%-tile | 10,0 | 9,0 | 7,0 | 6,0 | 6,0 | 3,0 |  |
|  | PC1 | Throughput Loss | Average |  |  | 5,00 | 4,00 | 3,00 | 2,00 | 2,00 |
|  |  |  | 5%-tile |  |  | 10,00 | 7,00 | 6,00 | 6,00 | 3,00 |
| LEO1200 | PC2 | Throughput Loss | Average |  |  | 3,0 | 3,0 | 2,0 | 1,0 |  |
|  |  |  | 5%-tile |  |  | 10,0 | 5,0 | 2,0 | 1,0 |  |
|  | PC1.5 | Throughput Loss | Average |  |  |  |  | 3,0 | 2,0 | 2,0 |
|  |  |  | 5%-tile |  |  |  |  | 9,0 | 3,0 | 2,0 |
|  | PC1 | Throughput Loss | Average |  |  |  |  | 4,00 | 3,00 | 2,00 |
|  |  |  | 5%-tile |  |  |  |  | 13,00 | 9,00 | 3,00 |

##### 2.1.2.2	NR NTN; TN BS type – Non AAS; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 28 | 30 | 32 | 34 | 36 | 38 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | Throughput Loss | Average | 2,0 | 2,0 | 1,0 | 1,0 | 1,0 | 1,0 | 0,0 |
|  |  |  | 5%-tile | 8,0 | 7,0 | 4,0 | 1,0 | 1,0 | 1,0 | 0,0 |
|  | PC1.5 | Throughput Loss | Average | 3,0 | 2,0 | 2,0 | 1,0 | 1,0 | 1,0 | 1,0 |
|  |  |  | 5%-tile | 13,00 | 9,00 | 7,00 | 5,00 | 1,00 | 1,00 | 1,00 |
|  | PC1 | Throughput Loss | Average | 4,00 | 3,00 | 2,00 | 2,00 | 1,00 | 1,00 | 1,00 |
|  |  |  | 5%-tile | 15,00 | 13,00 | 9,00 | 7,00 | 5,00 | 1,00 | 1,00 |
| LEO1200 | PC2 | Throughput Loss | Average |  | 3,0 | 3,0 | 2,0 | 2,0 | 1,0 | 1,0 |
|  |  |  | 5%-tile |  | 8,0 | 5,0 | 3,0 | 3,0 | 3,0 | 3,0 |
|  | PC1.5 | Throughput Loss | Average |  | 5,0 | 4,0 | 3,0 | 2,0 | 2,0 | 1,0 |
|  |  |  | 5%-tile |  | 19,0 | 10,0 | 7,0 | 3,0 | 3,0 | 3,0 |
|  | PC1 | Throughput Loss | Average |  | 6,00 | 4,00 | 4,00 | 3,00 | 2,00 | 2,00 |
|  |  |  | 5%-tile |  | 33,00 | 19,00 | 9,00 | 7,00 | 3,00 | 3,00 |

##### 2.1.2.3	NB-IoT NTN; Deployment Scenario – Urban

| GEO | Power Class |  | Required ACIR (dB) | 20 | 22 | 24 | 26 | 28 | 30 | 32 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC2 | SNR Loss | Average | 2.3 | 1.9 | 1.5 | 1.2 | 0.9 | 0.7 |  |  |
|  |  |  | 5%-tile | 2.5 | 2,0 | 1.3 | 1.1 | 0.7 | 0.5 |  |  |
|  | PC1.5 | SNR Loss | Average |  |  | 2.1 | 1.7 | 1.3 | 1.1 | 0.8 | 0.6 |
|  |  |  | 5%-tile |  |  | 2.1 | 1.5 | 1.1 | 0.8 | 0.6 | 0.5 |
|  | PC1 | SNR Loss | Average |  |  | 2.6 | 2.1 | 1.7 | 1.3 | 1.1 | 0.8 |
|  |  |  | 5%-tile |  |  | 3.2 | 2.2 | 1.5 | 1.1 | 0.9 | 0.6 |
| LEO1200 | PC2 | SNR Loss | Average |  |  | 1.3 | 1.1 | 0.9 | 0.7 | 0.6 | 0.5 |
|  |  |  | 5%-tile |  |  | 1.8 | 1.4 | 1,0 | 0.8 | 0.6 | 0.4 |
|  | PC1.5 | SNR Loss | Average |  |  | 1.8 | 1.5 | 1.2 | 1,00 | 0.8 | 0.6 |
|  |  |  | 5%-tile |  |  | 3.4 | 2.2 | 1.5 | 1.2 | 1,00 | 0.8 |
|  | PC1 | SNR Loss | Average |  |  | 1.8 | 1.5 | 1.2 | 1,00 | 0.8 | 0.6 |
|  |  |  | 5%-tile |  |  | 5.2 | 3.4 | 2.2 | 1.5 | 1.3 | 1,00 |

## 2.2	Summary of ACIR requirements for PC2, PC 1.5 and PC 1

Table 2.2-1: Summary of ACIR requirements for PC2, 1.5 and 1 with isolation distance of 1.5km and 0km

| ACIR (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Case 4 |  | PC 2 | PC 1.5 | PC 1 |
| With isolation distance of 1.5km (based on results presented in R4-2418995 [3]) | With wrap-around | GEO | NR NTN and TN BS with AAS | 11 | 13 | 15 |
|  |  |  | NR NTN and TN BS with Non-AAS | 3 | 6 | 8 |
|  |  |  | NB-IoT NTN | NA | 1 | 3 |
|  |  | LEO1200 | NR NTN and TN BS with AAS | 9 | 13 | 15 |
|  |  |  | NR NTN and TN BS with Non-AAS | 3 | 5 | 7 |
|  |  |  | NB-IoT NTN | 1 | 4 | 5 |
|  | Without wrap-around | GEO | NR NTN and TN BS with AAS | 3 | 7 | 9 |
|  |  |  | NR NTN and TN BS with Non-AAS | NA | 2 | 4 |
|  |  |  | NB-IoT NTN | NA | 2 | 4 |
|  |  | LEO1200 | NR NTN and TN BS with AAS | 6 | 9 | 11 |
|  |  |  | NR NTN and TN BS with Non-AAS | NA | 2 | 4 |
|  |  |  | NB-IoT NTN | NA | 0 | 2 |
| With isolation distance of 0km | With wrap-around | GEO | NR NTN and TN BS with AAS | 30 | 34 | 38 |
|  |  |  | NR NTN and TN BS with Non-AAS | 31 | 35 | 37 |
|  |  |  | NB-IoT NTN | 32 | 35 | 37 |
|  |  | LEO1200 | NR NTN and TN BS with AAS | 31 | 35 | 37 |
|  |  |  | NR NTN and TN BS with Non-AAS | 35 | 37 | 39 |
|  |  |  | NB-IoT NTN | 29 | 31 | 33 |
|  | Without wrap-around | GEO | NR NTN and TN BS with AAS | 33 | 37 | 39 |
|  |  |  | NR NTN and TN BS with Non-AAS | 31 | 34 | 36 |
|  |  |  | NB-IoT NTN | 27 | 31 | 33 |
|  |  | LEO1200 | NR NTN and TN BS with AAS | 34 | 37 | 39 |
|  |  |  | NR NTN and TN BS with Non-AAS | 32 | 35 | 37 |
|  |  |  | NB-IoT NTN | 28 | 32 | 34 |

The ACIR requirements for all combinations within Case 4 are more stringent when isolation distance assumption between the NTN UE and edge of TN cluster is 0km.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The ACIR requirements for all combinations within Case 4 are more stringent when isolation distance assumption between the NTN UE and edge of TN cluster is 0km.

# References

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair (Huawei)

- R4-2420403,” WF on NR NTN HPUE”, 3GPP RAN4#113, November 2024

- R4-2418995,” On Co-existence simulation results for NR-NTN and NB-IoT-NTN HP UE”, Ericsson, 3GPP RAN4#113, November 2024