3GPP TSG-RAN WG4 Meeting #114	R4-2501402

Athens, Greece, 17-21 February 2025

Source:	Nokia

Title:	On remaining LTM requirements

Agenda item:	5.23.1

Document for:	Discussion

Introduction

In this contribution we discuss some of the remaining open issues for Rel-18 LTM.

TLTM_RRC-processing

Despite continuing the discussion for multiple meetings, RAN4 has not been able to make final conclusion about how the UE capability of fast RRC processing works in case TCI state activation command or PDCCH order is sent for more candidate cells than the UE pre-processing capability supports.

As we have proposed in the previous meetings, in our view the most recent TCI state activation commands and PDCCH orders matter in this scenario and TLTM-RRC-processing = 0 if the candidate cell is one of the most recent candidate cells for which TCI state is activated or PDCCH order is sent.

TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states or PDCCH-ordered PRACH transmission within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

Furthermore, since this proposal still leaves open the scenario when the network does not send TCI state activation command or PDCCH order for any candidate cells, and the number of candidate cells in the configuration exceeds UE capability to pre-process, we additionally propose:

TLTM_RRC-processing =0, if the target cell is listed as one of the first Min(maxNumberConfigs-r18,  maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells in the LTM candidate cell configuration.

TfirstRS

Regarding the conditions for TfirstRS and TRSproc being 0, we continue to propose that the following condition would apply both for the case when UE is and is not configured with LTM L1 measurements:

-	The time between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].

The condition for Tfirst-RS = 0 “The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].” applies also for the case when the UE is configured with L1 measurements.

Fine DL synchronization after TCI state activation

Furthermore, related to TfirstRS, we would like to once more bring up the limitation of UE being required to keep track of the fine DL timing synchronization of an activated TCI state only for 160 ms in most scenarios, when TCI state activation is based on L1 measurements, or for [480] ms if TCI state activation is based on L3 measurements. Although we understand that this limitation applies to some UEs, we would like to check with the group whether it is reasonable to assume that all UEs have the same limitation. This is because from network point of view, network is based on the minimum requirements forced to follow the worst case scenario i.e. to send cell switch command within the 160 (or [480]) ms, as sending a cell switch command after this time will lead to earlier TCI state activation potentially becoming useless. If some UE would be able to continue keeping track of the DL sync after this, this is in no way visible to the network and hence such UEs cannot benefit from this ability. Hence, we would like to check with the group whether there would be support for defining a UE capability that allows the network to know if the UE is able to keep track of the DL timing without limitations (until cell switch) or at least for a longer period than 160/[480] ms as the minimum requirement states.

RAN4 to discuss whether there would be support for defining a UE capability to indicate network that the UE can keep track of fine DL synchronization of an activated TCI state for longer than 160 ms (even if the measurement period is longer than 160 ms) with L1 measurements, or for longer than [480 ms] with L3 measurements.

Early TCI state activation

In the last meeting we proposed in our CR R4-2417682 to update the early TCI state activation delay requirements with the details of UL TCI states. However, this proposal was not discussed during the meeting.

The requirement for TCI state activation delay currently only mentions “DL TCI state activation” or “TCI state activation”. However, when the UE receives an early TCI state activation command, the command contains one or more joint DL/UL TCI states or pairs of separate DL and UL TCI states. Hence, the requirement leaves the UL TCI state activation part completely unclear.

RAN4 agreed that PL-RS tracking that is in legacy part of the UL TCI state activation delay is expected to be done when UE has performed L1 or L3 measurement for the candidate cell and hence UL TCI state activation is not visible in the delay. However, the UL TCI state is still contained in the command and added on the active TCI state list at this point.

LTM TCI state activation MAC-CE activates one or more joint DL/UL TCI states or pairs of separate UL and DL TCI states for a candidate cell. However, RAN4 early TCI state activation delay requirement does not mention UL TCI state at all.

While it may be clear for RAN4 experts that the delay takes only into account the DL synchronization part, for anyone outside the RAN4 discussions it may appear confusing that UL TCI state is not mentioned in the requirement, although the MAC-CE is to activate both DL and UL TCI states (joint or separate). Therefore, we propose to clarify the requirement for early DL TCI state activation so that it specifically mentions also UL TCI state, while the delay requirement is to stay the same and contain only the DL synchronization part. Additionally, we think it needs to be clarified that the known TCI state condition applies only for the DL TCI state, as the current requirement is mixing the terms DL TCI state and TCI state. We have given a text proposal for this in our CR.

Clarify in the early TCI state activation delay requirement that the activation applies for both UL and DL TCI state (joint or separate), without changing the agreed delay.

RAN1 LS

RAN1 has sent LS R1-2410911 to RAN4 to ask about the scenario when RACH transmission on one candidate cell collides with SSB reception on another candidate cell:

1. Overall Description: On R18 LTM, for unpaired spectrum, RAN1 identified possible collision between valid RA occasion (RO) and SSB associated with different cells in time domain, which is due to that RO validation is performed per cell. The collision could happen when the different cells are in the same frequency band or in different frequency band. It is unclear to RAN1 on whether/how to handle the collision between an SSB and a valid RO associated with different cells. 2. Actions: To RAN4: RAN1 respectfully asks RAN4 on whether/how to handle the collision between an SSB and a valid RO as described above and to provide answer for it.

The scenario here is that network sends PDCCH order to the UE considering candidate 1. The time slot the UE is to use for preamble transmission on candidate 1 is overlapping with SSB of candidate 2. RAN1 wants to know how to handle such collision in case the UE cannot both measure and transmit the preamble at the same time.

It is obvious that preamble transmission should be prioritized in such scenario. If the UE does not send the preamble, this leads to random access error. When PDCCH order is sent, network has likely already made the cell switch decision, as cell switch command would need to be sent very soon after the PDCCH order for the TA to remain useful. Hence, prioritizing measurement on another candidate cell in this case does not make sense.

Reply to LS: PDCCH ordered preamble transmission on a candidate cell is to be prioritized over an overlapping SSB measurement on another candidate cell, if the UE is not able to perform both.

It is unclear from the LS whether RAN1 is worried about L1 or L3 measurements, but in both cases the likelihood of such event happening and especially having an impact on the cell switch decision is quite small. As the network would likely not send PDCCH order without a plan to send cell switch command to the same cell, the UE missing one SSB for another candidate cell does in practice not have a meaningful impact on the cell switch decision. Furthermore, if before PDCCH order, a TCI state has been activated only for the candidate cell the PDCCH order considers, L1 measurement requirements do not even apply for other cells and hence in this case there is no requirement impact. For L3 measurements, we doubt that missing one SSB has a big impact considering the measurements are anyways averaged over time. Therefore, we do not see that this scenario needs to be taken into account in RAN4 requirements.

The scenario in RAN1 LS has very little or no impact on cell switch decision in practice. Hence, no need to take this scenario into account in RAN4 requirements.

Conclusion

The following observations and proposals were made in this paper:

Proposal 1: TLTM_RRC-processing =0 applies to the LTM candidates with the most recently activated TCI states or PDCCH-ordered PRACH transmission within UE capability maxNumberStoredConfigCells-r18 and maxNumberConfigs-r18.

Proposal 2: TLTM_RRC-processing =0, if the target cell is listed as one of the first Min(maxNumberConfigs-r18,  maxNumberStoredConfigCells-r18 – number of serving cells) candidate cells in the LTM candidate cell configuration.

Proposal 3: The condition for Tfirst-RS = 0 “The time gap between the latest PDCCH ordered RACH preamble transmission on the target cell and the cell switch command is not more than [160 ms].” applies also for the case when the UE is configured with L1 measurements.

Proposal 4: RAN4 to discuss whether there would be support for defining a UE capability to indicate network that the UE can keep track of fine DL synchronization of an activated TCI state for longer than 160 ms (even if the measurement period is longer than 160 ms) with L1 measurements, or for longer than [480 ms] with L3 measurements.

Observation 1: LTM TCI state activation MAC-CE activates one or more joint DL/UL TCI states or pairs of separate UL and DL TCI states for a candidate cell. However, RAN4 early TCI state activation delay requirement does not mention UL TCI state at all.

Proposal 5: Clarify in the early TCI state activation delay requirement that the activation applies for both UL and DL TCI state (joint or separate), without changing the agreed delay.

Proposal 6: Reply to LS: PDCCH ordered preamble transmission on a candidate cell is to be prioritized over an overlapping SSB measurement on another candidate cell, if the UE is not able to perform both.

Proposal 7: The scenario in RAN1 LS has very little or no impact on cell switch decision in practice. Hence, no need to take this scenario into account in RAN4 requirements.