3GPP TSG RAN WG4 Meeting #114	R4-2501414

# Athens, Greece, 17 Feb. – 21 Feb., 2025

Agenda item:			7.17.2.1

Source:	vivo

Title:	Further discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

In the RAN#105 meeting, the revised WID on enhancements for NR and MR-DC RRM requirements was approved [1]. The objectives for RAN4 in the WID are as following

- FR2-1 SSB based L3 measurement delay reduction for connected mode - For UE supporting multiple-Rx simultaneous reception on single carrier: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - Rx beam sweeping factor - The CSSF requirement optimization is defined assuming a UE not operating with multiple-Rx based simultaneous L3 measurement: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2 - Note: Rel-19 CSSF optimization applies for CA/DC scenario independently of the UE support of multi-Rx capabilities. - Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In the last RAN4 meeting, the topic of FR2-1 SSB based L3 measurement delay reduction for connected model is discussed and the way forwards are captured in [2]. In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

- Discussion

- 2.1 L3 measurement delay reduction by optimizing Rx BSF for UE supporting multiple-Rx simultaneous reception

In the last meeting, following issues are captured in the WF

Issue 1-1-1: Conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF:

Agreement: - RAN4 to introduce serving cell’s signal quality (i.e., based on RSRP and/or RSRQ threshold(s)) based triggering condition to activate/deactivate multi-Rx for L3 measurement. Agreement: - Not support UE mobility status based triggering condition in Rel-19.

In the previous meetings, the former agreements were achieved and introducing serving cell’s signal quality threshold is agreed. Regarding the detail of the serving cell quality threshold, it is preferred that the “not at cell edge” threshold used for Rel-16 UE power saving is reused.

Regarding the serving cell quality threshold, it is preferred that the “not at cell edge” threshold used for Rel-16 UE power saving is reused.

The following options are available for further discussion.

- Option 3(Nokia, Ericsson): Use L3 event-triggered reporting conditions for triggering FBS

- Option 4(Apple): RAN4 to introduce UE power consumption based triggering condition to activation/deactivate multi-Rx for L3 measurement

  - Opponent to option 4 (Intel): Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced

- Option 5(Apple): RAN4 to introduce UE QoS based triggering condition to activate/deactivate multi-Rx for L3 measurement

- Option 6(Nokia): Conditional handover configuration is used to trigger FBS.

  - Option 6a (Ericsson): For CHO, FBS is applied during Tmeasure after the condition for handover is met.

- Option 7(Xiaomi): Activating/Deactivating FBS for L3 measurements (intra/inter-frequency) can upon the conditions without additional NW signalling (e.g. DRX configuration).

- Option 8(Intel): Conditions for UE to apply multi-Rx reception and reduced beam sweeping factor include the below aspects:

  - Multi-Rx simultaneous reception active, following Rel-18 definitions as much as reasonable, including,

    - Group based beam reporting is configured for the UE not long ago prior to the expected L3 reporting,

  - Measurement configurations meet processing limitations, including,

    - Single carrier is configured,

    - Measurement periodicity is large enough compared to the number of SSB to be measured in a burst,

    - Existing side conditions for L3 measurements.

  - Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions.

- Option 9 (Ericsson): BFD and/or RLM status may trigger UE to enable FBS.

  - If the UE meets the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE hasn’t the demand to conduct FBS. On the contrary, if the UE doesn’t meet the criteria of relaxed BFD and/or RLM, it indeed implicitly indicates the UE may have the demand to conduct FBS.

  - If beam failure or radio link failure is detected, FBS may be applied.

- Option 10 (QC): Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS

  - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports

  - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline

- Option 11(CATT): Send LS to RAN2 about the criteria/thresholds for triggering Multi-Rx operation on L3 measurements and the corresponding UE behaviors after the whole framework is decided.

Based on the current status of WI, it is preferred to design solution based on current agreement and whether to consider any further options could be discussed at later stage.

For conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF, it is preferred to design solution based on current agreement and whether to consider any further options could be discussed at later stage.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” :

- For issue 1-1-2, the following recommended WF are available.

- Recommended WF

For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”:

- Option 2 (LGE, OPPO, Ericsson, MTK, CTC):  UE indication to the NW when UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time).

  - Option 2a (LGE, OPPO, NTT DCM(scenario 1/2/3/4))Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point.

- Option 3 (HW, Samsung, Ericsson): Apart from the threshold conditions, UE is allowed to fall back to normal measurement mode (i.e. single panel) after UE has reported qualified measurement results, since NW already has the candidate target cells for potential HO or SCell modification.

- Option 7 (Samsung, Ericsson): Introduce validity timer configuration with certain ranges via dedicated RRC signalling

- Among these options, option 3 is preferred. If there is no consensus on option 3, it is preferred to only consider threshold conditions as the exit conditions.

Option 3 is preferred. If there is no consensus on option 3, it is preferred to only consider threshold conditions as the exit conditions.

Issue 1-2-1: Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

Agreement:

For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability:

- FFS: details of the capability

Issue 1-2-2: Delay requirements of L3 measurement delay reduction by optimizing Rx BSF

In the RAN4#112 meeting, following scenarios were at least agreed to use the L3 measurement delay reduction by optimizing Rx BSF.

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

- Handover

- RRC Re-establishment/RRC Connection Release with Redirection

Take SSB based Intra-frequency measurement without MG as an example, in the current spec, beam sweeping factors related value M are corresponding to different power class in measurement delay requirement.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36. Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36. MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

It can be observed that the M value for FR2-1 in PC 3 is 24. In fact, the M value is a compromise value, which includes the BSF value and the measurement samples (e.g., M = BSF*sample). We can not derive the real value of BSF from the M since the sample value is unknow. If sample=4, the BSF value =6 and if sample=3, BSF value=8. Some company mentioned that Rx BSF can be reduced to 2, 4 or 6 or use Rel-18 BSF value as baseline

| 30. NR_FR2_multiRX_DL | [30-2] | Fast beam sweeping for layer-1 measurement | Supports beam sweeping factor reduction for SSB-based layer-1 measurement. | Per band | FR2-1 only | Candidate values for Component 2: {2,4,6} for FR2-1 Note: It is only supported for power class 3. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- |

Reuse {2,4,6} is a feasible way but may not applicable to L3 measurement. Based on the above analysis, we have the following approaches to consider this issue.

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

RAN4 to consider the following approaches as solutions:

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

Issue 1-2-3: FFS: requirement at transition between the normal L3 measurement and FBS L3 measurement

- Option 1 (CATT, Apple):

  - Do not define the requirement at transition between the normal L3 measurement and FBS L3 measurement.

- Option 2 (Ericsson): Regarding the transition time,

  - For the case from non-FBS to FBS, RAN4 specify a delay from the time instant when the condition is met.

- FBS activation delay shall at the least includes below items.

  - When the UE receives a RRC message implying FBS the UE shall be ready to start the L3 measurements with FBS operation within DFBS seconds from the end of the last TTI containing the RRC command.

DFBS = TRRC + TEvent_FBS + Tmeasure + Tprocessing_FBS

  - Where:

    - TRRC is the RRC procedure delay defined in clause 12 in TS 38.331 [2].

    - TEvent_FBS is the delay uncertainty which is the time from when the UE successfully decodes the RRC command until a condition exists at the measurement reference point which will trigger the FBS.

    - Tprocessing_FBS is for UE processing and preparing FBS operation before UE applying FBS.

  - For the case from FBS to non-FBS, the necessity of a delay during transition is unclear since the fallback solution isn’t fixed.

For the requirement for the transition between normal L3 measurement and FBS L3 measurement, option 1 is preferred.

- Summary

In this contribution, we provided our views on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. Based on above analysis, following proposals are present.

- Regarding the serving cell quality threshold, it is preferred that the “not at cell edge” threshold used for Rel-16 UE power saving is reused.

For conditions to trigger UE to activate L3 measurement delay reduction by optimizing Rx BSF, it is preferred to design solution based on current agreement and whether to consider any further options could be discussed at later stage.

Option 3 is preferred. If there is no consensus on option 3, it is preferred to only consider threshold conditions as the exit conditions.

RAN4 to consider the following approaches as solutions:

- Introduce the UE capability to indicate the reduced M value (M=sample*BSF), e.g., 12, 14, 16, etc.

- Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc.

- Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4.

For the requirement for the transition between normal L3 measurement and FBS L3 measurement, option 1 is preferred.

- References

- RP-242380, Revised WID: NR Radio Resource Management (RRM) Phase 5, RANP 105

- R4-2420093, WF on RRM requirements for NR_RRM_Ph5_Part1, RAN4 113, Apple