3GPP TSG-RAN WG4 Meeting #114	R4-2501427

Athens, GR, 17th – 21nd February, 2025

Source:	Samsung

Title:	Further Discussion and simulation results on MIMO layer evaluation for 6Rx UE

Agenda item:	7.1.1.3.2

Document for:	Discussion

- Introduction

In the RAN#103 meeting, the WID [1] for “UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4” has been approved, in which “6Rx for handheld and FWA” is an important objective for both core and performance parts. This is an enhancement for receiver in addition to Rel-15 4Rx and Rel-18 8Rx improvements (which are specified for fixed wireless access (FWA) types of devices). The 6Rx in Rel-19 is targeting for handheld UE considering 6Rx would be possible and useful to improve DL coverage and throughputs for the large size handheld UEs, e.g., the foldable smartphone. In the RAN#104 meeting, the WID has been updated [2].

Objective of Core part WI 6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution. Objective of Performance part WI 6Rx for handheld and FWA UE - Specify the performance requirements to enable 6Rx on higher frequency bands (>2.5GHz) targeting at support of smartphone for NR FR1 single carrier scenario - Investigate and if necessary, specify RLM test cases to support 6Rx - Specify UE PDSCH demodulation performance and CSI requirements to support 6 Rx - Support 4 MIMO layers at least, and follow the conclusion in core part for whether to support 6 MIMO layers - Specify the SDR requirements to support 6Rx

From RAN4#112 meeting, companies started to provide simulation results to verify the performance of 6 MIMO layers and try to align simulation assumptions. The latest link level simulation assumptions alignment results achieved in RAN4#113 meeting [3], although there are still several unreasonable options proposed for some key parameters.

In RAN4#113 meeting, interested companies [4~13] provided link level simulation results for the performance comparison between 6 MIMO layers and 4 layers, some limited gain could be observed in [4~8] under certain conditions, no gain of 6 MIMO layers could be observed in [9~13]. A summary [14] of companies’ simulation results shows generally the gain of 6 MIMO layers could be only observed with high SNR (more than 30dB) and small Tx EVM (no more than 1.25%).

In this contribution, we provide more analysis and simulation results according the WF [3] in last RAN4#113 meeting.

- Discussion and simulation results

  - The logic between handheld UE and FWA

In last RAN4#113 meeting, companies’ opinions on the performance requirements (if introduced) for 6 MIMO layers for handheld UE and FWA diverged as below.

Issue 2-1-4: Way forward for performance requirements for 6-layers for handheld UE and FWA Way Forward: RAN4 to further discuss the following options. - Option 1: Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices. - Option 2: RAN4 to introduce 6-layer performance requirements for handheld and FWA devices. - Option 3: RAN4 shall conclude that whether to introduce 6 layers for both FWA and handheld UE depends on the outcome of 6 MIMO layers evaluation on handheld UE. - Option 4: From WF in R4-2417209, companies to further discuss the following aspect of 6-layer support. - If the handheld feasibility is confirmed, then the handheld and FWA devices will share the same 6-layer requirements. - If the handheld feasibility is not confirmed, 6-layer requirements will be specified for FWA device only. - Option 5: Do not specify 6-layer requirements for handheld and FWA devices in Rel-19.

In which, Option 2 and Option 5 are proposals about “introduce” or “not introduce” which should be pending on the 6 MIMO layer gain evaluation conclusion later. Option 1 and Option 3 are similar, both of them are further explanation of the objective descriptions in the WID [2]. It is clearly mentioned the objective is targeting at the support of handheld UE or smartphone. 6 MIMO layers can be supported only when both the gain and feasibility are verified, not only the feasibility. At last, once the Rx requirements for 6 MIMO layers are specified for handheld UE, these requirements can also be applicable for FWA devices, herein the FWA device is in a subordinate position to the handheld UE. Hence, the logic of the WID should be “if the Rx requirements for 6 MIMO layers are specified for handheld UE, then these requirements can also be applicable for FWA device; on the other hand, if the Rx requirements for 6 MIMO layers are not specified for handheld UE, then no related requirements could be used for FWA device as well.” Therefore, the second bullet of Option 4 is inconsistencies with the WID.

Observation 1: The logic for the objective of 6Rx described in the WID is “if the Rx requirements for 6 MIMO layers are specified for handheld UE, then these requirements can also be applicable for FWA device; on the other hand, if the Rx requirements for 6 MIMO layers are not specified for handheld UE, then no related requirements could be used for FWA device as well.”

According above observations, we think these is no need to discuss the 6 MIMO layers feasibility for FWA devices at all. Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices.

Proposal 1: Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices.

  - UE antenna correlation matrix

Both UE antenna correlation and BS antenna correlation matrix can impact the channel independent. It’s no doubt that the performance of 6 MIMO layers could outperform 4 MIMO layers if the channel uncorrelation promised. However, once correlation between antennas are introduced, the performance of 6 MIMO layers can decrease compared with 4 MIMO layers. UE antenna correlation matrix should be highly related with the product design, different product design may correspond to different antenna correlation matrix. In our view, this is the reason why RAN4 has recieved some contributions focusing on foldable devices which may understate the antenna correlation with a larger form factor than normal handheld UE. However, on top of the larger space of the foldable UEs, it also should be noted that there would be a lot of other restrictions to support the advanced form factor. Even for the the foldable devices on the market, according to our experiences, it’s not easy to make RF path between two sides due to the hinge and both side full screen for example. Hence we take two antenna deployments as reference in Figure 2-1, which are surely practical layouts of 6Rx for foldable Ues. The ECC antenna correlation matrix A and B for foldable devices corresponding to Figure 2-1 are used for our simulation.

Observation 2: For realistic antenna correlation factors of the foldable devices, it also should be noted that there would be a lot of other restrictions to support the advanced form factor on top of the larger space of the foldable UEs.

UE antenna correlation matrix A

UE measured ECC (Envelop Correlation Coefficient) matrix A for N41 based on antenna deployment in Figure 2-1 (a).

1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00

UE antenna correlation matrix B

UE measured ECC matrix B for N77 based on antenna deployment in Figure 2-1 (b).

1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00

(a)                                           (b)

Figure 2-1. Antenna deployment examples for foldable UE

Meanwhile, considering low UE antenna correlation matrix could improve the performance of 6 MIMO layers, in order to provide sufficient simulation results based on various conditions, we also introduce the UE correlation matrix proposed by OPPO in our simulation assumptions.

UE antenna correlation matrix C

UE measured ECC matrix B from OPPO.

  - BS antenna correlation matrix

As mentioned above, the final channel correlation not only come from UE receive antennas but also may come from BS transmit antennas and wireless channels. Therefore, the BS antenna correlation matrix should be considered for the 6 MIMO layers performance evaluation as well. Ideal BS antenna correlation matrix (no correlation between different BS transmit antennas) could be used for simualtion results alignment, but it may be hard to promise by real product. In addition, BS antenna correlation matrix with some correlations between different BS transmit antennas could be defined since it’s more realistic.

BS antenna correlation matrix A

Identity matrix assuming no correlation between different BS transmit antennas.

1001000000001001000000000000000000000000000000001001000000001001

BS antenna correlation matrix B

Refer to the channel spatial correlation matrix defined in 38.101-4 B.2.3.1, the BS antenna correlation matrix could be described as below.

1α149α449α949α1649α2549α3649αα149*1α149α449α949α1649α2549α3649α449*α149*1α149α449α949α1649α2549α949*α449*α149*1α149α449α949α1649α1649*α949*α449*α149*1α149α449α949α2549*α1649*α949*α449*α149*1α149α449α3649*α2549*α1649*α949*α449*α149*1α149α*α3649*α2549*α1649*α949*α449*α149*1

RAN4 defines α as 0.3 for ULA Medium correlation and 0.9 for ULA high correlation. Here we set α as 0.05 in order to introduce a really low correlation coefficient between BS transmit antennas.

  - DMRS configurations

In addition, since 4 MIMO layers and 6 MIMO layers corresponding to different number of DMRS ports, which may impact the overhead and final throughput performance. Hence the configuration of DMRS ports is a key factor need to be discussed.

For the DMRS configurations, lastest wayforward as below.

- DMRS Configuration: - Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) from Annex A table. - Consider DMRS Configuration Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table. - Consider DMRS Configuration Option 1: Rel-18 1+1(6L), Rel-15 1+1(4L) and Option 3: Rel-15 1+1(4L), Rel-15 2+2(6L) from Annex A table. - Consider additional DMRS Configuration Option 1a: Rel-18 1+1(6L), Rel-18 1+1(4L)

Per our understanding and the description of DMRS configurations described in 38.211 and 38.331, DMRS type 1 with one additional OFDM symbol for DMRS ports and maxLength=1 is the typical DMRS configuration (Rel-15 1+1 DMRS symbols) for 4 MIMO layers as Figure 2-2 (a), it is straightforward to configure maxLength=2 in order to support 6 MIMO layers as Figure 2-2 (b), hence we think Option 3 is the typical configuration for real network.  For Option 1 with Rel-18 1+1(6L), it only could be supported when the feature of Rel-18 enhanced DMRS is supported, however Rel-18 enhanced DMRS has not been supported by most UE devices in the real network.

Proposal 2: For DMRS configurations, use the typical configuration Option 3 (Rel-15 1+1(4L), Rel-15 2+2(6L)).

At the same time, considering some contributions could observe some 6 MIMO layer performance gain based on Rel-18 1+1 (6L) or Rel-15 2+0 (6L)  configurations, we also apply Rel-18 1+1 (6L) DMRS configurations in our following simualation to verify 6 MIMO layer performance adequately. According the specification defined in 38.212, DMRS ports should be configured as Figure 2-2.

38.212, DMRS ports configurations

- Rel-15 typical 4 ports configuration

- Rel-15 typical 6 ports configuration

- Rel-18 6 ports configuration

Figure 2-2. DMRS ports configurations for Rel-15 DMRS and Rel-18 DMRS

  - TxEVM assumption

The value of TxEVM can impact the throughput performance, especially at the high SNR scenarios the impct is non-negligible.

About the TxEVM for simulation alignment, several options are proposed as below in last RAN4#113 meeting.

Issue 2-1-1: Tx EVM assumptions for 6-layer feasibility evaluation Way Forward: RAN4 to further discuss the following options. - Option 1: Use Tx EVM = 1% - Option 2: Use Tx EVM = 3% for all of modulation order in MCS Table 2 for performance evaluation - Option 3: Consider TxEVM of 6% for 64QAM and TxEVM 3% for 256QAM

3GPP 38.104 defines RF core requirements for BS Tx EVM as 8% for 64QAM and 3.5% for 256QAM.  RAN4 demodulation performance evaluation [15][16] are based on tightened values as below considering the real BS implementation. If other values proposed for BS Tx EVM, it should be based on justification and the specification for BS Tx EVM should be updated accordingly.

3GPP 38.104 BS Tx EVM requirements

Tx EVM assumption for DL performance evaluation in RAN4 [15][16]

Rel-15 DL performance evaluation assumption - TX EVM = 6% for QPSK/16QAM/64QAM - TX EVM = 3% for 256QAM Rel-18 8Rx DL performance evaluation assumption - TX EVM for 64QAM: 6% - TX EVM for 256QAM: 3%

Since there is no plan to do the feasibility study for tightening Tx EVM, we think it is a good selection to follow the performance evaluation assumption used before, i.e. use Tx EVM of 6% for 64QAM and Tx EVM 3% for 256QAM.

Proposal 3: For the Tx EVM used for performance evaluation assumption, use Tx EVM of 6% for 64QAM and Tx EVM 3% for 256QAM.

Moreover, considering some contributions could observe some 6 MIMO layer performance gain based on Tx EVM 0% or 1% or 1.25%, we will also add more simualtion cases using 0% Tx EVM in our following simualation to verify 6 MIMO layer performance sufficiently.

  - Other parameters

Besides the UE correlation matrix, BS correlation matrix, DMRS configurations and Tx EVM assumptions, the max number of rank should also be considered since more options exist.

For the max number of rank, the description in the WID [2] mentioned “Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers”, hence we should know the max number of rank could be 4 or 6, not including 5, which means the max number of rank as 5 is out of scope.

Observation 3: The description in the WID mentioned supporting the max number of rank as 4 and even could be 6 once the gain and feasibility study verified, the max number of rank as 5 is out of scope.

  - Simulation assumptions

Our initial simulation results [17] which based on XP low/medium (even the correlation factor downgrades from 0.6 to 0.1) and further simulation results [18] which based on the measured UE ECC correlation matrixes show that the performance of 6 MIMO layers case is hard to outperform 4 MIMO layers. In this contribution, we provide our further analysis and simulation results for 6Rx receiver based on the agreed simulation assumptions in last RAN4#113 meeting.

As there are many options for different parameters in the feasibility study link level simulation assumption [3], it’s necessary to narrow down some options. Our simulations are based on Table 2-1.

Table 2-1 test parameters for 6Rx

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| Bandwidth |  | 40MHz |
| SCS |  | 30kHz |
| Antenna configuration |  | { 8Tx, 6Rx} |
| Propagation channel |  | TDLA30-10 |
| Rank |  | Fixed, rank 4 and 6 |
| MCS |  | Fixed MCS17 Adaptive MCS (target BLER 10%) MCS Table 2 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | Specific to each Reference channel |
|  | PDSCH aggregation factor | 1 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
|  | Resource allocation type | Type 0 |
|  | RBG size | Config2 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Position of the first DMRS for PDSCH mapping type A | 2 |
|  | Antenna ports indexes | {1000-1003} for Rank 4 tests (1+1) {1000-1003, 1004, 1006} for Rel-15 Rank 6 tests (2+2) {1000-1003, 1008, 10010} for Rel-18 Rank 6 tests (1+1) |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 1 for rank <= 4 2 for rank > 4 1 for case “R18DMRS” of rank > 4 |
|  | Precoding | Random PMI (Rel-15 Type 1 codebook) for 8Tx |
| Codebook configuration | CodebookType | typeI-SinglePanel |
|  | CodebookMode | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) | (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| UE correlation matrix |  | Matrix A: 1.000.220.221.000.300.580.600.150.150.100.050.150.300.600.580.151.000.150.151.000.150.150.150.100.150.050.100.150.150.150.150.101.000.300.301.00 Matrix B: 1.000.100.101.000.250.350.500.100.100.050.050.100.250.500.350.101.000.100.101.000.100.150.100.050.100.050.050.100.100.100.150.051.000.150.151.00 Matrix C: |
| BS correlation matrix |  | Matrix A: 1001000000001001000000000000000000000000000000001001000000001001 Matrix B: α as 0.05 1α149α449α949α1649α2549α3649αα149*1α149α449α949α1649α2549α3649α449*α149*1α149α449α949α1649α2549α949*α449*α149*1α149α449α949α1649α1649*α949*α449*α149*1α149α449α949α2549*α1649*α949*α449*α149*1α149α449α3649*α2549*α1649*α949*α449*α149*1α149α*α3649*α2549*α1649*α949*α449*α149*1 |
| Tx EVM |  | 0% and 3% |

  - Simulation results

Based on the test parameters in Table 2-1, our simulation results are summarized in:

Figure 2-1 - Figure 2-2 for limited BS correlation with fixed MCS level with different UE correlation Matrixes and different Tx EVM values.

Figure 2-3 - Figure 2-4 for ideal BS correlation with LA with different UE correlation Matrixes and different Tx EVM values.

Figure 2-5 - Figure 2-6 for limited BS correlation with LA with different UE correlation Matrixes and different Tx EVM values.

Figure 2-1. Performance evaluation for 6Rx with MCS17, BS correlation matrix B and Tx EVM = 0%

Figure 2-2. Performance evaluation for 6Rx with MCS17, BS correlation matrix B and Tx EVM = 3%

Figure 2-3. Performance evaluation for 6Rx with LA, BS correlation matrix A and Tx EVM = 0%

Figure 2-4. Performance evaluation for 6Rx with LA, BS correlation matrix A and Tx EVM = 3%

Figure 2-5. Performance evaluation for 6Rx with LA, BS correlation matrix B and Tx EVM = 0%

Figure 2-6. Performance evaluation for 6Rx with LA, BS correlation matrix B and Tx EVM =3%

From our simulation results, for both ideal BS antenna correlation matrix A (zero BS antenna correlation considered) and non-ideal BS antenna correlation matrix B, for three UE antenna correlation martix A, B, C, for Tx EVM 0% and 3%, for 6Rx with 6 MIMO layers cases, there is no performance gain could be observed compared with 6Rx with 4 MIMO layers cases.

Based on the same BS antenna correlation matrix, the performance of three UE antenna correlation matrixes do not have obvious difference, the performance of UE antenna correlation matrix C is slightly better than UE antenna correlation matrix B, and the performance of UE antenna correlation matrix B is slightly better than UE antenna correlation matrix A. This phenomenon is in line with expectations, since the correlation coefficient of UE antenna correlation matrix A is higher than UE antenna correlation matrix B, and the correlation coefficient of UE antenna correlation matrix B is higher than UE antenna correlation matrix C.

Observations from our simulation results could be summarized as:

Observation 4: For both ideal BS antenna correlation matrix A (zero BS antenna correlation considered) and non-ideal BS antenna correlation matrix B, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

Observation 5: For three UE antenna correlation matrixes A, B and C, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

Observation 6: For both Tx EVM 0% and 3%, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

From above observations, it seems it’s hard to get the benefit using 6Rx with 6 MIMO layers.

Simulation results analysis

According to our simulation results and observations above, 6 MIMO layers could not get benefit even at high SNR value range, even with zero BS antenna correlation assumed and even with 0% TxEVM applied. In addition, for the practical deployment, we can easily assume that it is hard to promise zero BS antenna correlation factor and 0% TxEVM. As a consequence of this performance evaluation, for both handheld UE and FWA device with 6Rx receiver antennas, we propose not to introduce requirements for 6 MIMO layers.

Proposal 4: For both handheld UEs and FWA device with 6Rx antennas, there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results based on realistic UE antenna correlation matrixes.

- Conclusion

In this contribution, we provide our observations from WID and specifications, simulation assumptions, simulation results analysis, and views on “6Rx for handheld and FWA” topic.

The observations and proposals could be summarized as:

Observation 1: The logic for the objective of 6Rx described in the WID is “if the Rx requirements for 6 MIMO layers are specified for handheld UE, then these requirements can also be applicable for FWA device; on the other hand, if the Rx requirements for 6 MIMO layers are not specified for handheld UE, then no related requirements could be used for FWA device as well.”

Proposal 1: Once the gain and feasibility for handheld UE with 6 MIMO layers are verified, then 6 MIMO layers related requirements defined for handheld UE could be applicable for FWA devices. Otherwise, no 6 MIMO layers related requirements should be defined for neither handheld UE nor FWA devices.

Observation 2: For realistic antenna correlation factors of the foldable devices, it also should be noted that there would be a lot of other restrictions to support the advanced form factor on top of the larger space of the foldable UEs.

Proposal 2: For DMRS configurations, use the typical configuration Option 3 (Rel-15 1+1(4L), Rel-15 2+2(6L)).

Proposal 3: For the Tx EVM used for performance evaluation assumption, use Tx EVM of 6% for 64QAM and Tx EVM 3% for 256QAM.

Observation 3: The description in the WID mentioned supporting the max number of rank as 4 and even could be 6 once the gain and feasibility study verified, the max number of rank as 5 is out of scope.

Observation 4: For both ideal BS antenna correlation matrix A (zero BS antenna correlation considered) and non-ideal BS antenna correlation matrix B, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

Observation 5: For three UE antenna correlation matrixes A, B and C, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

Observation 6: For both Tx EVM 0% and 3%, there is no performance gain could be observed for 6Rx with 6 MIMO layers cases compared with 6Rx with 4 MIMO layers cases.

Proposal 4: For both handheld UEs and FWA device with 6Rx antennas, there is no need to introduce requirements for 6 MIMO layers since no benefit could be observed from our simulation results based on realistic UE antenna correlation matrixes.

- Reference

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#103 meeting, Huawei

[2] RP-241656, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, RAN#104 meeting, Huawei

[3] R4-2420376, WF on 6Rx UE requirements, RAN4#113 meeting, AT&T

[4] R4-2417916, R19 Simulation results of 6Layer performance, RAN4#113 meeting, OPPO

[5]	R4-2418011, Simulation Results for on MIMO layer evaluation for 6Rx UE, RAN4#113 meeting, Nokia

[6]	R4-2418437, Discussion on the demodulation performance requirements for 6Rx devices, RAN4#113 meeting, Qualcomm

[7]	R4-2419165, Discussion on MIMO layer evaluation for 6Rx UE, RAN4#113 meeting, ZTE

[8]	R4- 2419550, Discussion on the MIMO layers for 6Rx UE, RAN4#113 meeting, Ericsson

[9]	R4-2419029, On MIMO layer evaluation for 6Rx UE, RAN4#113 meeting, Huawei, HiSilicon

[10] R4-2418619, Discussion and simulation results on MIMO layer evaluation for 6Rx UE, RAN4#113 meeting, Samsung

[11] R4-2418595, Views on Maximum Number of MIMO Layers for 6Rx UEs, RAN4#113 meeting, Apple

[12] R4-2418188, Updated simulation results of MIMO layer evaluation for 6Rx UE, RAN4#113 meeting, Vivo

[13] R4-2417561, Feasibility evaluation of MIMO layer for 6Rx UE, RAN4#113 meeting, MediaTek

[14] RP-242942, RAN4 study status for 6 layer under 6Rx objective, RAN#106 meeting, Samsung

[15] R4-1811394, Way forward on NR UE PDSCH demodulation requirements and General aspects, RAN4#88 meeting, Intel Corporation

[16] R4-2309806, WF for 8Rx UE performance requirements, RAN4#107 meeting, Huawei, HiSilicon

[17] R4-2412877, Discussion and initial simulation results on MIMO layer evaluation for 6Rx UE, RAN4#112 meeting, Samsung

[18] R4-2415681, Discussion and simulation results on MIMO layer evaluation for 6Rx UE, RAN4#112bis meeting, Samsung