3GPP TSG-RAN WG4 Meeting #114	R4-2501428

Athens, GR, 17th – 21nd February, 2025

Source:	Samsung

Title:	Discussion on spatial channel model for demodulation performance requirements

Agenda item:	7.16.2

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#108 in June 2025. The objectives are as below:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In last RAN4#113 meeting, High level TR skeleton for 38.753 and initial work split have been agreed, and obvious progresses have been achieved for test cases, parameter assumptions, CDL methodologies and AAV configurations [2]. However, until now the discussion mainly focuses on TR 38.827. There are still several issues need to be further clarified and discussed for the CDL modelling, and the discussion for TDL modelling also need to be started.

In this contribution, we share our views and analysis on the open issues for CDL-based model and TDL-based model.

- Discussion

  - Common for all Methodologies

In RAN4#113 meeting, initial simulation assumptions are achieved for SCM simulation result alignment and comparison among companies. Companies are invited to provide SNR working points or curves for different cases. However, we still need some metrics to measure the quality of simulation results alignment. About the metrics for alignment and comparison, there is no agreement [2].

Metrics for comparison Contributions welcome at RAN4#114.

In current PDSCH demodulation performance requirements, SNR working point at 30% or 70% fraction of maximum throughput is selected as the test metric. For the PMI reporting requirements, gamma value defined as the throughput ratio (following PMI vs random PMI) for the SNR working point at 90% fraction of maximum throughput is the test metric.

In order to check the alignment of simulation results, the metrics for alignment and comparison should be as many as possible besides full curves. Hence, we propose to evaluate the alignment of SNR working point at 30%, 50%, 70% fraction of maximum throughput.

Proposal 1: For the metrics for simulation results alignment and comparison, propose to evaluate the alignment of SNR working point at 30%, 50%, 70% fraction of maximum throughput.

In last RAN4#113 meeting, for SU-MIMO PMI cases, companies agreed to use wideband PMI for typeI-SP following and random cases, furthermore, the periodicity of CSI-RS resource need to be clarified thus companies could provide simulation results based on the same configuration.

Proposal 2: For SU-MIMO PMI cases, the periodicity of CSI-RS resource need to be clarified thus companies could provide simulation results based on the same configuration, we propose to use 5ms.

  - TDL Based Methodologies

TDL parameters for performance comparsion

Considering current TDL models defined in TS38.101-4 will be used as the baseline for CDL performance comparision, the correlation matrix for baseline TDL model need to be clarified since no final argeement until now.

The TDL parameters mentioned in WF of RAN4#112 meeting [4]

Tap Delay Line Parameters Agreement: The following Legacy TDL models for comparison: - Option 1: TDLC 300-100 ULA Low - Option 2: TDLA 30-10 ULA Low - FFS: TDLC 300-100 medA - FFS: TDLC 300-100 medB - FFS: TDLB 100

The Doppler parameters mentioned in WF of RAN4#112-bis meeting [3]

UE Speed Agreement Interested companies to use 30 km/h and/or 3 km/h for UE speed/velocity. TDL doppler values to be used for simulation: - 100 Hz for 30 km/h - 10 Hz for 3 km/h Assumption: 3.5 GHz carrier to be used for simulations (aligned with FR1)

Meanwhile, CDL model introduces the correlation cofficient for polarizations between BS and UE side as expjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ.

Proposal 3: Clarify the correlation matrix used for each company’s simulation of the baseline TDL model defined in TS38.101-4.

TDL Model Extension

The motivation of the SID [1] is current TDL model does not have spatial property for RAN4 demodulation/CSI reporting requirements, which are needed for some scenarios e.g. High rank, PMI reporting cases. This target could also be achieved by extending current TDL model through introducing spatial clusters as mentioned in [6][7] multi-cluster TX-RX beam steering with TDL model. The principles of both TDL extensation methods [6][7] are similar.

Observation 1: Multi-cluster TX-RX beam steering with TDL model method could also achieve the target for introducing spatial property, which is easier than CDL model since no antenna patterns and antenna combining weight for multiple antennas needed.

Since TDL based channel modeling is a parallel approach with CDL based channel modeling, and we don’t have enough time to discsuss it in past meetings. Before the simulation campaign for extended TDL channel modeling, one suitable extended TDL modeling should be discussed and selected as the starting point, and key parameters for this model should also aligned, such as the number of clusters, the angle of beams per cluster.

Proposal 4: Since TDL based channel modeling is a parallel approach with CDL based channel modeling, one suitable extended TDL modeling should be discussed and selected as the starting point, and key parameters for this model should also be aligned, such as the number of clusters, the angle of beams per cluster.

  - CDL Based Methodologies

After RAN4#113 meeting, several issues for CDL based channel modeling approach need further discussion as below.

- Angles of Departure and Angles of Arrival Way Forward: Option 1: Replace the mean angle in the last position of departure and arrival angles generated equation by the desired angle as . Set mf,desired based on the LOS direction between the TX and RX Option 2: Following 38.827 (i.e., mf,desired = mf,model). - BS Antenna Virtualiser Way Forward: Broadside for BS AAV options except fully connected. Max receive power for standard fully connected AAV. Interested companies to study with AAV to use max receive power of single strongest direction. - SNR Definition with CDL Agreement: Define normalised receive signal power as the following: The sum of signal power is normalised across all Rx RF chains using a single normalisation factor. The normalisation factor is calculated considering the long term average PDP for the spatial CDL channel and AAV [spatial response/combining gain]. The normalisation factor will be aligned and can be included in TR 38.753. Interested companies are welcome to provide measured normalization factor for CDL channel with different AAV options in next meeting.

The generation of departure and arrival angles

According current description in 38.827, the departure and arrival angles are generated using below equation

Where n,AOA+ cASAm denotes the arrival angles with ray offsets, and  is the mean angle of the original channel model table calculated using  in NLOS case and the LOS angle  in LOS case. Therefore, the technical effects of above equation are only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angel ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle, e.g., Uma CDL-C scaled departure/arrival angles in Figure 2-5. Then the drawbacks mentioned in 38.901 section 7.7.5.1 are still exist, the directions are limited/fixed by specific angle table, such as Table 7.2.1-8 for UMa CDL-C at 3.5 GHz. If we want to set UE at a predefined angle, for example, for MU MIMO pairing with several UEs simulation, it is not easy to implement using current formula. Hence, we prefer to replace the  in the last position by the desired/wanted direction angle μϕ,desired.

Figure 2-1. Scaled angles with ray offsets defined for Uma CDL-C in 38.827

Observation 2: The technical effects of current departure and arrival angles generation equation in 38.827 is only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angle ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle instead of desired angles.

Proposal 5: Replace the mean angle  in the last position of departure and arrival angles generated equation by the desired angle μϕ,desired as ϕn,m,AOA,scaled=ASdesiredASmodeϕn,AOA+cASAαm-μϕ,model+μϕ,desired

SNR Definition with CDL

With the introduction of the BS antenna radiation pattern in CDL based channel modelling, the normalization should be applied by a normalization factor and “The normalization factor is calculated considering the long-term average PDP for the spatial CDL channel”, otherwise an additional gain will be added to the power of channel.

For NLOS scenario, the normalization factor is calculated by

P=n=0N-1Frx,u,θθn,ZOA,φn,AOAFrx,u,φθn,ZOA,φn,AOATexp0κn-1exp0κn-1exp0exp0Ftx,s,θθn,ZOD,φn,AODFtx,s,φθn,ZOD,φn,AOD2

For LOS scenario, the normalization factor is calculated by

P=n=1N-1Frx,u,θθn,ZOA,φn,AOAFrx,u,φθn,ZOA,φn,AOATexp0κn-1exp0κn-1exp0exp0Ftx,s,θθn,ZOD,φn,AODFtx,s,φθn,ZOD,φn,AOD2+Frx,u,θθ0,ZOA,φ0,AOAFrx,u,φθ0,ZOA,φ0,AOAT100-1Ftx,s,θθ0,ZOD,φ0,AODFtx,s,φθ0,ZOD,φ0,AOD2

Taking UMa CDL-C at 3.5 GHz as an example, Figure 2-2 shows the power CDF comparison with TDL channel model. In order to compare the performance for CDL and TDL at the same level, this normalization for long-term PDP and BS antenna radiation pattern is essential.

Figure 2-2. power CDF comparison between TDL and CDL (Uma CDL-C in 38.827)

Besides this normalization, since different AAV configurations as below are planned for CDL based channel modelling simulation campaign, a normalization method should be discussed and defined to remove the AAV or beamforming gain in order to compare the throughput performance with different AAV options at the similar SNR level.

AAV Configurations for 4Tx CSI-RS ports:

- Option 1: with Antenna Array Virtualisation (AAV) using subarray configuration

  - (M,N,P,Ms,Ns)  = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X)

  - (M,N,P,Ms,Ns)  = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y)

- Option 2: with AAV without subarray (fully connected)

  - (M,N,P,Ms,Ns) = (8,8,2,8,8) for 8Tx and 4Tx CSI-RS Ports

- Option 3: with AAV with the following configuration:

  - (M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports

Proposal 6: A normalization method should be discussed and defined to remove the AAV or beamforming gain in order to compare the throughput performance with different AAV options at the similar SNR level.

  - Comparison of methodologies

In our companion contribution [7], we provide our initial simulation results for CDL and baseline TDL comparison, with 4 ports 4 layers.

For doppler 10Hz simulations

Averaged AAV/beamforming gain:

- 7.41dB for CDL-option1X

- 1.96dB for CDL-option 1Y

SNR working points:

SNR working points at 30%, 50%, 70% fractions of maximum throughput could be summarized as below table.

| Fraction of maximum throughput | TDLC300-10 Low corr | CDL option1X | CDL option1Y | CDL option3 |
| --- | --- | --- | --- | --- |
| 30% | 5.4 dB | 7.8 dB | 6.0 dB | 5.4 dB |
| 50% | 10.1 dB | 14.0 dB | 14.0 dB | 14.0 dB |
| 70% | 13.4 dB | 14.8 dB | 15.2 dB | 14.9 dB |

For doppler 100Hz simulations

Averaged AAV/beamforming gain:

- 6.71dB for CDL-option1X

- 0.63dB for CDL-option1Y

SNR working points:

SNR working points at 30%, 50%, 70% fractions of maximum throughput could be summarized as below table.

| Fraction of maximum throughput | TDLC300-100 Low corr | TDLC300-100 XPMed corr | CDL option1X | CDL option1Y | CDL option3 |
| --- | --- | --- | --- | --- | --- |
| 30% | 5.4 | 6.7 | 8.4 | 3.8 | 2.7 |
| 50% | 10.5 | 13.4 | 12.0 | 8.0 | 8.0 |
| 70% | 15.2 | 17.6 | 16.8 | 18.4 | 18.1 |

- Conclusion

In this contribution, we provide analysis and views on SCM topic.

The observations and proposals could be summarized as:

Proposal 1: For the metrics for simulation results alignment and comparison, propose to evaluate the alignment of SNR working point at 30%, 50%, 70% fraction of maximum throughput.

Proposal 2: For SU-MIMO PMI cases, the periodicity of CSI-RS resource need to be clarified thus companies could provide simulation results based on the same configuration, we propose to use 5ms.

Proposal 3: Clarify the correlation matrix used for each company’s simulation of the baseline TDL model defined in TS38.101-4.

Observation 1: Multi-cluster TX-RX beam steering with TDL model method could also achieve the target for introducing spatial property, which is easier than CDL model since no antenna patterns and antenna combining weight for multiple antennas needed.

Proposal 4: Since TDL based channel modeling is a parallel approach with CDL based channel modeling, one suitable extended TDL modeling should be discussed and selected as the starting point, and key parameters for this model should also be aligned, such as the number of clusters, the angle of beams per cluster.

Observation 2: The technical effects of current departure and arrival angles generation equation in 38.827 is only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angle ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle instead of desired angles.

Proposal 5: Replace the mean angle  in the last position of departure and arrival angles generated equation by the desired angle μϕ,desired as ϕn,m,AOA,scaled=ASdesiredASmodeϕn,AOA+cASAαm-μϕ,model+μϕ,desired

Proposal 6: A normalization method should be discussed and defined to remove the AAV or beamforming gain in order to compare the throughput performance with different AAV options at the similar SNR level.

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] R4-2419782, Way Forward for [113][320]NR_SCM, RAN4#113 meeting, Nokia

[3] R4-2416591, Way Forward Minutes for [112bis][318] NR_SCM, RAN4#112-bis meeting, Nokia

[4] R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112 meeting, Nokia

[5] R4- 2415382, Discussion on Spatial Channel Modelling, RAN4#112-bis meeting, MediaTek Inc.

[6] R4-2419895, Discussion on spatial channel model for demodulation requirements, RAN4#113 meeting, Huawei, HiSilicon

[7] R4-24XXXXX, Initial simulation results on spatial channel model, RAN4#114 meeting, Samsung