# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501450

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the remaining issues for NR NTN phase 3

Agenda Item:	7.29.2.1

Document for:	Approval

Introduction

In the RAN4#113 meeting, the following way forwards for NR NTN phase 3 were captured in the Tdoc [1].

- Issue 1-1: PC3 output power for (e)RedCap Agreement: - FFS whether and how a potential new scheme to increase output power is addressed in specifications - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap - If confirmed, no further work is needed. - Alt 2: Power increase by network controlled optional new signalling (e.g capability) - Alt 3: Change in power tolerances - E.g. Upper tolerance exception to UE in specific regions via table note. - FFS whether solution is common for HD-FDD and FD-FDD - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB.

In this paper, we’d like to discuss this issue.

Discussion

Referring to the contribution [2], companies observed that the HD-FDD RF front-end insertion loss is lower than the FD-FDD front-end insertion loss and can improve the Rx REFSENS and TX maximum output power (MOP) simultaneously since FDD duplexer may not be implemented for HD-FDD RedCap UE.

Observation 1: the HD-FDD RF front-end insertion loss is lower than the FD-FDD front-end insertion loss since FDD duplexer may not be implemented for HD-FDD RedCap UE.

In the WF, there is an open issue whether the solution is common for HD-FDD and FD-FDD. However, based on the observation above, it’s obvious that the assumed RF architectures for HD-FDD and FD-FDD are different. For NR NTN FD-FDD, we haven’t met the similar issue and RAN4 is discussing the HPUE in a separate WI.

Observation 2: As RAN4 haven’t touched the similar issue (insertion loss decrease) for NR NTN FD-FDD, it’s very hard for RAN4 to consider a common solution for both HD-FDD and FD-FDD.

According to the ETSI EN 301 908-13 [3], the way to address this issue is to take Alt3: i.e. to increase Upper tolerance exception to NR NTN HD-FDD RedCap UE.

Proposal 1: take Alt3 to address this issue: i.e. to increase Upper tolerance exception to NR NTN HD-FDD RedCap UE.

Summary

Observation 1: the HD-FDD RF front-end insertion loss is lower than the FD-FDD front-end insertion loss since FDD duplexer may not be implemented for HD-FDD RedCap UE.

Observation 2: As RAN4 haven’t touched the similar issue (insertion loss decrease) for NR NTN FD-FDD, it’s very hard for RAN4 to consider a common solution for both HD-FDD and FD-FDD.

Proposal 1: take Alt3 to address this issue: i.e. to increase Upper tolerance exception to NR NTN HD-FDD RedCap UE.

- References

[1] R4-2419897, Way Forward for [113][310] NR_NTN_Ph3_UE_RF, Qualcomm Inc.

[2] R4-2419541, Discussion on Rel-19 NR-NTN RedCap UE RF requirements, MediaTek Inc.

[3] ETSI EN 301 908-13, IMT cellular networks: Harmonised Standard for access to radio spectrum; Part 13: Evolved Universal Terrestrial Radio Access (E-UTRA) User Equipment (UE)