# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501451

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on Rel-19 ATG UE RF with DL inter-band CA_n3-n39

Agenda Item:	7.12.1.1

Document for:	Approval

Introduction

In the RAN4#110bis meeting, RAN4 started to discuss the Rel-19 ATG enhancement. One high level way forward [1] were approved. In our contribution [2], we shared some initial considerations for this topic.

In the RAN4#112 meeting, RAN4 reached a lot of consensus on this topic below, which are captured into the WF [3].

In the RAN4#112bis meeting, RAN4 reached some consensus in the approved WF [4].

In the RAN4#113 meeting, RAN4 updated the MSD test parameters for CA_n3A-n39A with n39 UL based on the approved WF [5].

Issue 3-3: MSD for CA_n3A-n39A with n39 UL Agreement: - Define the MSD value with 40dBm output power - FFS on how to scale MSD for other output power level. - Following the parameters in the table below. UL band; DL band; UL Fc; UL BW; SCS of UL band; UL RB Allocation; DL Fc; DL BW; MSD; Cross-band Interference source. (MHz); (MHz); (kHz); LCRB; (MHz); (MHz); (dB). n39; n3; 1900; 30; 15; 160 (RBstart=0); 1817.5; 25; [TBD]; ACLR2.

In this paper, we’d like to our views on how to scale MSD for different output power level for DL CA_n3-n39 of ATG UE.

Discussion

We’d like to provide two exemplary RF architectures below to allow the transition guard band between DL band n3 and band n39.

figure 1 Common antenna RF architecture for DL CA_n3-n39 with UL in either band n3 or n39

Figure 2 separate antenna RF architecture for DL CA_n3-n39 with UL in either band n3 or n39

For the RF architectures in figures, only partial frequency range can be implemented for the filter(s) in DL band n3 or band n39 or both DL band n3 and n39.

Based on the way forward [3], companies are encouraged to provide the inputs for the frequency gap in the filters.

In China, the spectrum allocation in band n3 and n39 is shown below.

table 1 Band 3 UL spectrum allocation in China

| Band 3 UL |  |  |
| --- | --- | --- |
| CMCC | CU | CTC |
| 1710~1735MHz | 1735~1765MHz | 1765~1785MHz |

table 2 Band 3 DL and band 39 UL/DL spectrum allocation in China

| Band 3 DL |  |  | Band 39 UL/DL |  |
| --- | --- | --- | --- | --- |
| CMCC | CU | CTC | No allocation | CMCC |
| 1805~1830MHz | 1830~1860MHz | 1860~1880MHz | 1880~1885MHz | 1885~1915MHz |

Observation 1: 5MHz frequency gap was reserved for the band 3 and 39 spectrum allocation in China.

Observation 2: From ATG operator’s perspective, the frequency separation between band 3 DL and band 39 UL is about 55MHz.

Thus, we checked the MSD based on 30MHz aggressor located in 1885~1915MHz and 25MHz victim located in 1805~1830MHz, which is the MSD case due to ACLR2 cross band isolation. The evaluation is shown in the table 3. The antenna isolation is assumed as 20dB. The rejection for band n3 Rx filter at 1885~1915MHz and n39 Tx filter rejection at 1805~1830MHz are assumed as 50dB. Since there is no volumn limitation and power consumption limitation in the airplane, the assumed performance for these parameters are better than the handheld UE. For handheld UE, band n39 Tx filter can provide 45dB attenuation at least in frequency range 1805~1830MHz and band n3 Rx filter can only provide 20dB attenuation in frequency range 1885~1915MHz. However, from demands’ perspective, the band n3 Rx filter has to provide 50dB attenuation in order to avoid blocking from band n39 Tx signal especially for common antenna RF architecture.

table 3 main parameters for MSD evaluation

| Parameters | Values (set 1) | Values (set 2) |
| --- | --- | --- |
| Antenna Isolation | 20dB | 20dB |
| The rejection for band n3 Rx filter at 1885~1915MHz | 50dB | 30dB |
| n39 Tx filter rejection at 1805~1830MHz | 50dB | 50dB |

The MSD values for different output power and different parameters sets are summarized below.

table 4 MSD values for different parameter sets

| Output power | MSD (dB) for Parameters set 1 |  | MSD (dB) for Parameters set 2 |  | Requirement sets |
| --- | --- | --- | --- | --- | --- |
|  | Common antenna | Separate antenna | Common antenna | Separate antenna |  |
| 23 dBm | 3.22 | 0.62 | 3.61 | 0.64 | Requirement set A |
| 30 dBm | 4.9 | 2.33 | 5.69 | 2.68 | Requirement set B |
| 35 dBm | 7.38 | 5.65 | 10.34 | 7.33 | Requirement set C |
| 40 dBm | 11.12 | 10.38 | 18.17 | 15.16 | Requirement set D |

For output power ranging from 36dBm to 40dBm, the requirement set D is applicable.

For output power ranging from 31dBm to 35dBm, the requirement set C is applicable.

For output power ranging from 24dBm to 30dBm, the requirement set B is applicable.

For 23dBm output power, the requirement set A is applicable.

Proposal 1: For ATG UE, RAN4 can specify the following MSD requirements for DL band n3 with band n39 UL interreference under different output powers considering operator’s spectrum allocation and 5MHz frequency gap in 1880~1885MHz.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | ACLR2 and >ACLR2 |

table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | MSD (dB) for Parameters set 1 |  | MSD (dB) for Parameters set 2 |  | Requirement sets |
| --- | --- | --- | --- | --- | --- |
|  | Common antenna | Separate antenna | Common antenna | Separate antenna |  |
| 23 dBm | 3.22 | 0.62 | 3.61 | 0.64 | Requirement set A |
| 24 ~ 30 dBm | 4.9 | 2.33 | 5.69 | 2.68 | Requirement set B |
| 31 ~ 35 dBm | 7.38 | 5.65 | 10.34 | 7.33 | Requirement set C |
| 36 ~ 40 dBm | 11.12 | 10.38 | 18.17 | 15.16 | Requirement set D |

Summary

Observation 1: 5MHz frequency gap was reserved for the band 3 and 39 spectrum allocation in China.

Observation 2: From ATG operator’s perspective, the frequency separation between band 3 DL and band 39 UL is about 55MHz.

Proposal 1: For ATG UE, RAN4 can specify the following MSD requirements for DL band n3 with band n39 UL interreference under different output powers considering operator’s spectrum allocation and 5MHz frequency gap in 1880~1885MHz.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n39 | n3 | 1900 | 30 | 15 | 160 (RBstart=0) | 1817.5 | 25 | MSD Table X | ACLR2 and >ACLR2 |

table X CA_n3-n39 with band n39 UL MSD requirements for different output powers

| Output power | MSD (dB) for Parameters set 1 |  | MSD (dB) for Parameters set 2 |  | Requirement sets |
| --- | --- | --- | --- | --- | --- |
|  | Common antenna | Separate antenna | Common antenna | Separate antenna |  |
| 23 dBm | 3.22 | 0.62 | 3.61 | 0.64 | Requirement set A |
| 24 ~ 30 dBm | 4.9 | 2.33 | 5.69 | 2.68 | Requirement set B |
| 31 ~ 35 dBm | 7.38 | 5.65 | 10.34 | 7.33 | Requirement set C |
| 36 ~ 40 dBm | 11.12 | 10.38 | 18.17 | 15.16 | Requirement set D |

- References

[1] R4-2406594	WF on Rel-19 ATG UE requirements, CMCC

[2] R4-2405344	General discussion on Rel-19 ATG UE RF, Huawei, HiSilicon

[3] R4-2414322 WF on Rel-19 ATG UE RF requirements, CMCC

[4] R4-2417121	WF on UE RF for ATG enhancement, CMCC

[5] R4-2420338 WF on NR ATG UE RF, CMCC