# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501454

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the full NTN L band definition

Agenda Item:	6.20.1

Document for:	Approval

Introduction

In the RAN4#112 meeting, RAN4 reached the consensus on band plan for this WI captured in the WF [2].

Offline Agreement: - Proposed band numbering as follows: Table 1: NR-NTN satellite bands in FR1-NTN NTN satellite operating band; Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high; Duplex mode. n253; 1668MHz – 1675MHz; 1518MHz – 1525MHz; FDD. n251; 1626.5MHz – 1660.5MHz; 1518MHz – 1559MHz; FDD. n250; 1668MHz – 1675MHz; 1518MHz – 1559MHz; FDD. NOTE: NTN satellite bands are numbered in descending order from n256..

There are some open issues on how to implement one duplexer to cover all the NTN L bands, e.g. Rx out-of-band blocking requirements and Protection of Radio Astronomy stations. In this paper, we’d like to share our views on these open issues.

Discussion on out-of-band blocking requirements

Referring to the band n255, the general out-of-band blocking requirements were specified below.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

As we discussed in previous paper [1], it’s possible to implement one duplexer (DL: 1518 – 1559 MHz, UL: 1626.5 – 1675 MHz) to cover all four L-bands, i.e. n255, n253, n251 and n250. Thus, when RAN4 specify the out-of-band blocking requirements, such one duplexer implementation should be allowed.

Proposal 1: to introduce the following out-of-band blocking requirements for band n253, n251 and n250.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n251, n250 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n253 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 94 | -85 < f – FDL_low ≤ -60 or 94 ≤ f – FDL_high < 119 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 119 ≤ f ≤ 12750 |

Discussion on Protection of Radio Astronomy stations

In the RAN4#112bis meeting, we have the following way forward [6] for one common duplexer implementation.

- Issue 3-2-1: UE Duplexer - Agreement: - Further discuss whether implementation with one duplexer is feasible, considering the requirements to protect Radio Astronomy in the 1660-1668MHz range between the two UL.

In the RAN4#113 meeting, we have the following way forward [3] for Protection of Radio Astronomy stations.

Issue 3-1-5: Requirements and Mechanisms on Protection of Radio Astronomy stations - Further study mechanisms to implement protection of Radio Astronomy stations, at least the following options could be considered: - TX Disable, considering possible impact on backwards compatibility - RB restrictions - NS Issue 3-1-6: Signalling of proximity to Radio Astronomy stations - Further study how to determine proximity of NTN UE to Radio Astronomy stations

After checking the ETSI EN 301 681 [4], the requirements to protect Radio Astronomy service operation are listed below.

In order to protect Radio Astronomy service operation, the MES shall be able to have its transmissions disabled. The requirements to protect Radio Astronomy service operation are independent with the duplexer implementation. For the transmission disable mechanism, if RAN4 agrees to introduce this mechanism into 3GPP, it may have other working groups’ impacts.

Observation 1: For the transmission disable mechanism, if RAN4 agrees to introduce this mechanism into 3GPP, it may have other working groups’ impacts.

As SAN may cover very large area, it’s very hard for SAN to know whether NTN UE is close to Radio Astronomy stations operating in 1660~1660.5 MHz and 1668 ~ 1670MHz or not. That means SAN can’t send the transmission disable and enable command properly. As current NR NTN RB scheduling is flexible enough, no need to design the special transmission disable and enable command and NTN UE can just follow SAN RB scheduling.

Observation 2: As current NR NTN RB scheduling is flexible enough, no need to design the special transmission disable and enable command and NTN UE can just follow SAN RB scheduling.

Summary

Proposal 1: to introduce the following out-of-band blocking requirements for band n253, n251 and n250.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n251, n250 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n253 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 94 | -85 < f – FDL_low ≤ -60 or 94 ≤ f – FDL_high < 119 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 119 ≤ f ≤ 12750 |

Observation 1: For the transmission disable mechanism, if RAN4 agrees to introduce this mechanism into 3GPP, it may have other working groups’ impacts.

Observation 2: As current NR NTN RB scheduling is flexible enough, no need to design the special transmission disable and enable command and NTN UE can just follow SAN RB scheduling.

- References

[1] R4-2419067, Discussion on the full NTN L band definition, Huawei, HiSilicon

[2] R4-2414287, WF on Rel-19 FR1 NTN bands, Moderator (Inmarsat)

[3] R4-2420465, WF on requirements for NR IoT NTN bands, Moderator (Inmarsat)

[4] ETSI EN 301 681, Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES) of Geostationary mobile satellite systems, including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) under the Mobile Satellite Service (MSS), operating in the 1,5 GHz and 1,6 GHz

frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU

[5] R4-2321813, WF on IoT_NTN_extLband, Moderator (Inmarsat)

[6] R4-2417206, WF on NTN and IoT-NTN bands, Moderator (Inmarsat)