# 3GPP TSG-RAN WG4 Meeting # 114	R4-2501455

# Athens, Greece, 17th – 21st February, 2025

Source: 	Huawei, HiSilicon

Title: 	Discussion on the NTN S band definition

Agenda Item:	6.18.2

Document for:	Approval

Introduction

In the RAN4#113 meeting, RAN4 reached some consensus in the way forward [1], but there are some remaining open issues e.g. out-of-band blocking requirements and FCC requirements for protection of aeronautical radionavigation-satellite service. In this paper, we’d like to share our views on these issues in detail.

Protection of aeronautical radionavigation-satellite service

After checking 47 CFR Part 25 [2], the following regulatory requirements related to S-band (2000 – 2020 MHz) are cited below.

25.265 Acceptance of interference in 2000-2020 MHz. (a) MSS receivers operating in the 2000-2020 MHz band must accept interference from lawful operations in the 1995-2000 MHz band, where such interference is due to: (1) The in-band power of any operations in 1995-2000 MHz (i.e., the portion of transmit power contained in the 1995-2000 MHz band); or (2) The portion of out-of-band emissions contained in 2000-2005 MHz

25.216 Limits on emissions from mobile earth stations for protection of aeronautical radionavigation-satellite service. (e) The e.i.r.p density of emissions from mobile earth stations with assigned uplink frequencies between 1990 MHz and 2025 MHz shall not exceed −70 dBW/MHz, averaged over any 2 millisecond active transmission interval, in frequencies between 1559 MHz and 1610 MHz. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1559 MHz and 1605 MHz shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval. The e.i.r.p. of discrete emissions of less than 700 Hz bandwidth from such stations between 1605 MHz and 1610 MHz manufactured more than six months after FEDERAL REGISTER publication of the rule changes adopted in FCC 03-283 shall not exceed −80 dBW, averaged over any 2 millisecond active transmission interval.

The requirements are similar to the ASE specified in clause 6.5.3.3.2 of TS38.101-5 for “NS_02N”.

Frequency range (MHz); Channel bandwidth / Spectrum emission limit1 (dBm); Measurement bandwidth; NOTE. 5 MHz, 10 MHz, 15 MHz, 20 MHz. 1559≤ f < 1605; -50; 700 Hz; Averaged over any 2 millisecond active transmission interval. 1605≤ f ≤ 1610; -50 + 24/5 (f-1605); 700Hz. 1559 ≤ f < 1605; -40; 1MHz; Averaged over any 2 millisecond active transmission interval. 1605≤ f ≤ 1610; -40 + 24/5 (f-1605); 1MHz. NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna..

For the emission requirements, it seems that the frequency separation between UL 2000 – 2020 MHz and protected frequency range 1559 ~ 1610 MHz is large enough, so there is no need additional power reduction to comply with this requirements. But it can be reflected in the UE RF requirements.

Proposal 1: For new S-band n252, the following additional spurious emission requirements for 2	Protection of aeronautical radionavigation-satellite service can be specified.

Additional spurious emission requirements for band n252

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Out-of-band blocking requirement

In the contribution [3], companies proposed that one duplexer may cover all the bands n252, n256 and NR band n1/n65. The proposal is good enough to reuse the same component to cover more bands as many as possible. Thus, the out-of-band blocking requirements for band n252 can be similar to the band n256. The FDL_high is same between band n256 and n252, but there are 10MHz difference for the FDL_low. The Out-of-band blocking requirements for band n256 are defined below.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Thus, it’s necessary to reflect the difference of FDL_low in the spec, and there is no need to change the frequency range of FDL_high part.

Proposal 2: To introduce the following out-of-band blocking requirements for band n252.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256/n252 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Summary

Proposal 1: For new S-band n252, the following additional spurious emission requirements for 2	Protection of aeronautical radionavigation-satellite service can be specified.

Additional spurious emission requirements for band n252

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |  |
| 1559≤ f < 1610 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1559 ≤ f < 1610 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Proposal 2: To introduce the following out-of-band blocking requirements for band n252.

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n2521 | Finterferer (CW) | MHz | -110 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -155 < f – FDL_low ≤ -110 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 155 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256/n252 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

- References

[1] R4-2420465, WF on requirements for NR IoT NTN bands, Moderator (Inmarsat)

[2] 47 CFR Part 25, https://www.ecfr.gov/current/title-47/chapter-I/subchapter-B/part-25

[3] R4-2419519, Discussion on NR-NTN n252 UE RF requirements, MediaTek Inc.