3GPP TSG-RAN WG4 Meeting #114		R4-2501462

Athens, GR, 17th – 21st February, 2025

Agenda item:		7.22.3.1

Source:		Samsung

Title:	Discussion on RRM requirements of UE-initiated beam management in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of Rel-19 MIMO Phase 5 are described in [1] including:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In previous RAN4 meetings, we have already discussed the RRM requirements of UE-initiated/event-driven beam management and latest agreements have been captured in WF [2]. In this contribution, we continue to discuss the open issues of RRM requirements for UE-initiated/event-driven beam management.

- Discussion

In RAN4#113bis meeting, the agreements and open issues are captured in WF [2]. We will discuss the open issues.

Issue 1-2: How to define measurement period (evaluation time / measurement time) in Event triggered measurement reporting delay? < Agreement> Agreement: For specifying the RAN4 delay requirement for event 2 - Measure both the current beam and new beam after the starting point, FFS the details. - Note: this does not imply to define the delay requirement based on the sum of the current beam measurement period and new beam measurement period, which is pending on further RAN1 discussion. - Start the RAN4 RRM discussion for FR2. - Send LS to RAN1 to ask whether FR1 is covered as well. FFS the exact wording in the LS.

In legacy L3 event triggered measurement reporting delay, the measurement time is one L3 measurement period.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2.

There were proposals from some companies which want to reuse the same principle by replacing Tidentify intra with index or T identify intra without index with legacy L1 measurement period of candidate beam.

However, as we pointed out the difference by new L1 event-driven beam management and legacy L3 measurement:

In current L3 measurement, the time period and measurement period are based SMTC (+) DRX cycle.

In RAN2 38.331, the measurement is configured by multiple MO. For each MO, the smtc is configured.

MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2

to ensure that, in the measConfig associated with a CG:

-     for all SSB based measurements there is at most one measurement object with the same ssbFrequency;

-    for that ssbFrequency, the measurement window according to the smtc1 configured by the MCG includes the measurement window according to the smtc1 configured by the SCG, or vice-versa, with an accuracy of the maximum receive timing difference specified in TS 38.133 [14].

In 38.331, there are multiple events in L3 measurement like:

Event A1:   Serving becomes better than absolute threshold;

Event A2:   Serving becomes worse than absolute threshold;

Event A3:   Neighbour becomes amount of offset better than PCell/PSCell;

Event A4:   Neighbour becomes better than absolute threshold;

Event A5:   PCell/PSCell becomes worse than absolute threshold1 AND Neighbour/SCell becomes better than another absolute threshold2;

Event A6:   Neighbour becomes amount of offset better than SCell;

Take L3 Event A3 as example: Neighbor becomes amount of offset better than PCell/PSCell. The measurement is based on samples of SMTC. The SSB periodicity can be the same or different from SMTC. But the measurement is based on SMTC.

The measurement time are based on the SMTC period for both cells.

However, in L1 measurement, the period is T_SSB if take SSB as the example. If the UE evaluates and “UE is ready to transmit the report” which means the L1-RSRP will be triggered by UE side for event-2/1/7 is met (reporting criteria are fulfilled).

Until now, RAN1 agreed three events:

Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam. Event-1: Quality of the current beam is worse than a certain threshold. RAN1#118bis Working Assumption The following working assumption in RAN1#118 is revised in red. On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th Q-th best quality. - M Q is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

UE is configured what is included in the L1-RSRP e.g. current beam L1-RSRP (whether current beam is included in the report, it is configured by RRC) + N new beams

On UE-initiated/event-driven beam reporting, regarding L1-RSRP report format Option-3 depending on Event-2, the candidate value of ‘N’ at least comprises {1, 2, 3, 4} - FFS: additional candidate value(s) of {5, 6, 7, 8} FFS: If ‘N’ is not RRC configured, only one L1-RSRP and CRI/SSBRI are reported by default.

If the UE in configured with N beams like event-2 (RRC configured) and event-7, the condition is met by compare the current beam and new beams. That is what we agreed in last meeting in high level principle: measure both the current beam and new beam after the starting point

In last RAN1#119 meeting, RAN1 send an LS to RAN4 in [3]:

In RAN1#119, regarding UE-initiated/event-driven beam management for Rel-19 NR MIMO Ph5, the following agreement on the evaluation periodicity for determining Event-2 instance was reached: Agreement Regarding for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured], at least Alt-1 is supported for the single-CC beam reporting (for case that the CSI report configuration and RS for the current beam and new beam(s) are in the same CC). - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS. - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS. - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s). - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}. - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s). Note: There is the same periodicity for the new beam RS(s). FFS: Down-selection among above Alt(s) for cross-CC beam reporting.

According to latest RAN1 LS, RAN1 only support at least Alt-1 which means the periodicity of current beam RS should the same as new beam RS(s). Alt-2 hasn’t been supported in RAN1.

In MIMO, for this case, the RS for current beam and new beam(s) are in the same CC. In addition, the types of RSs of current beam and new beams are the same.

The evaluation periodicity can reuse the legacy L1-RSRP measurement period.

Observation 1: RAN1 only support Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

Proposal 1: According to latest RAN1 support, the evaluation periodicity can reuse the legacy L1-RSRP measurement period.

It is still open of whether to specify requirements cover both no time window based or with time window based.

Issue 1-3: Whether RAN4 to specify requirements for case 1 or cover both case1 and case2? Case 1: no time window based Case 2: with time window based, M is configurable < Agreement> Agreement: It is RAN4 common understanding that no additional filtering applies for case 2 compared to case 1, if RAN4 will define requirement for case 2.

RAN1 introduced a time window

RAN1#117 Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

The procedure is shown as:

For the basic feature, which means as long as one measurement of L1-RSRP of the new beam becomes better than current beam, UE initiated beam report.

The configurable M counter, can avoid the instantaneous results to guarantee the better performance with valid counter. In addition, it can avoid like ping-pong behavior if NW use this event-triggered L1-RSRP results.

Proposal 2: RAN4 to specify requirements for both basic feature and within a time window configured.

For the case of no time window based, as mentioned in P1, the legacy L1-RSRP measurement period can be reused.

For the current beam, which is the beam with the indicated TCI state.

For the new beams, they are explicit configured.

RAN1#118. Agreement Regarding explicit RS configuration for new beam measurement for Event 2, at least Option-1 is supported - Option-1: The RS(s) for new beam(s) are explicitly configured in one RS resource set associated with an CSI reporting configuration - If legacy UE capability signaling cannot be reused, introduce a UE capability signaling of indicating the maximum number of the configured RS(s) in the RS resource set. - FFS: The RS in the RS resource set can be updated by MAC-CE

Proposal 3: For the case of no time window based, the L1 evaluation time is defined as:

FR2+SSB based+event-2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index in indicated TCI state and SSB-index configured in [TBD]

FR2+SSB based+event-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index in indicated TCI state.

For event-7, it is a little complicated. The condition is met by compare the beam conditions of new beam and RS derived from the activated TCI state with the Q-th best quality.

In Rel-17 unified TCI states, the activated TCI states can be up to 8.

If it is sorted by Q-th best, which means UE shall perform the measurement of all activated TCI state.

In addition, there is no conclusion of whether the RS periodicity of RSs in activated TCI states are the same or not.

Observation 2: For event-7, no conclusion of whether the RS periodicity of RSs in activated TCI states are the same or not.

Observation 3: The measurement should cover all activated TCI state and new beam (s).

Proposal 4: For the case of no time window based, the L1 evaluation time is defined as:

FR2+SSB based+event-7

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*max(TSSB_active_0, …TSSB_active_i, TSSB_active_X, TSSB_new)) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX, TSSB_active_0, …TSSB_active_i, TSSB_active_X, TSSB_new)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

TSSB_active_i = ssb-periodicityServingCell is the periodicity of the SSB-Index in i-th active TCI state.

Proposal 5: For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event.

Issue 1-4: Reporting delay requirements <Way forward > - Option 1: (Qualcomm) - Tgap + TL1-RSRP-measure; TL1-RSRP-measure in Issue 1-2; The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on; In test cases, set Tgap = 0; - Option 2: (Nokia) - The maximum delay requirement for the first PUCCH channel is determined in relation to the reception time instant of the RS that triggered the UEIBM event. - Option 3: (Xiaomi) - If multiple beams will be measured, total measurement time will be maximum measurement period between multiple beams. - Option 4: (Samsung) - Need further RAN1 progress for RSs relationship between current beam and new beams. - Option 5: (CMCC) - The event triggered measurement reporting delay shall be less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Period + TSSB_time_index. The measurement reporting delay excludes a delay which is caused by no UL resources being available for UE to send the measurement report on - Option 6: (Apple) - The total reporting delay is Tfirst-RS-NB + TL1-RSRP-NB + TPUCCH, Add TPucch in reporting delay - Option 7: ( MediaTek) - The total reporting delay is TL1-RSRP_measure + Tfirst UL channel, TL1-RSRP_measure is base on SSB or CSI-RS - Event evaluation period for a SSB or CSI-RS is same as their respective measurement period - Option 8: (Ericsson) - Event triggered measurement reporting delay for SSB is TL1-RSRP_event triggered_measurement_Period_SSB (ms) + TTime to first UL channel and event triggered measurement delay for CSI-RS is TL1-RSRP_event triggered_measurement_Period_CSI-RS (ms) + TTime to first UL channel - RAN4 to discuss components and criteria for defining TTime to first UL channel.

Proposal 6: After we achieved the agreement of L1 evaluation time, the reporting delay can be specified as: T_L1 evaluation time + T_time_first_pucch.

There is proposal from option 5, the L1 measurement time shall also added TSSB_time_index. In this case, unlike LTM, the current beam and new beams are in the same CC.

Proposal 7: No need to add TSSB_time_index

Issue 1-7: CSI-RS based L1-RSRP for inter-cell BM <Way forward > - Option 1: (Qualcomm, Nokia, Samsung, MediaTek) - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case. - Option 2: (CMCC) - For reporting delay for CSI-RS based UE-initiated/event-driven beam reporting, it is proposed to define L1 measurement reporting delay requirements, and FFS whether legacy CSI-RS based L1 measurement delay requirements can be reused. - Option 3: (Apple, Huawei) - For UE perspective, for CSI-RS based BM, there is no differentiation for intra-cell and inter-cell. CSI-RS based L1-RSRP measurement requirements applies to both intra-cell and inter-cell without differentiation. - Option 4: (ZTE) - RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - RAN4 to use principles of SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Option 5: (Ericsson) - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario and inter-cell beam management in FR2 - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements

For the measurement of CSI-RS based L1-RSRP, because there is no other CSI-RS resource configuration. There is just CSI-RS resource configured from serving cell.

Proposal 8: Similar approach of TL1-RSRP_Measurement_Period_CSI-RS  in section 9.5.4.2 can be reused as the principle in SSB based UEIBM.

Issue 1-8: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM <Way forward > - Option 1: (Qualcomm, Nokia, Samsung, Apple, Huawei) - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM - Option 2: (vivo) - Regarding the impact to know conditions of TCI state activation from event-triggered L1 reporting, RAN4 waits further RAN1/2 conclusions. - Option 3: (ZTE) - Need to define the condition of known cell for UEIBM. The definition in ICBM can be starting point. Issue 1-9: Whether to specify new unified TCI switching delay requirements? <Way forward > - Option 1: (Qualcomm, Nokia, Samsung (For known conditions for TCI states), Apple, ZTE, Ericsson) - The existing requirements for TCI state switch delay should be reused for UE-initiated / event-triggered beam management. - Option 2: (Samsung) - No unknown case is applied if the TCI states switching is based on UEIBM. - Option 3: (ZTE) - Need to decide whether to define L1-RSRP requirements for unknown cell

After the UE sent L1 report to gNB, gNB can use the L1 report results to indicate UE to switch to a new beam or not. Until now, there is no any difference whether the L1 reports are UE triggered or NW based. The only difference is when the target TCI state is unknown in the TCI state switching. In legacy requirements, L1-RSRP time is added in the TCI state switching delay. However, in event-triggered L1 measurement, the target TCI is always known if it remains detectable. For unknown case of the switching, it has less usage in event trigger L1 reporting.

Proposal 9: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy.

Proposal 10: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM.

For event-2/event-1, the measurement is based on current beam. The RS of current beam is coming from indicated TCI state. If UE received TCI state switching based on MAC-CE or DCI, it means the indicated TCI state has been changed. The measurement of L1 evaluation time should be changed after the MAC-CE command or DCI indicator.

Proposal 11: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI.

Issue 1-10: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward > - Option 1: (Nokia) - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. - Option 2: (Xiaomi, Samsung, Apple, Ericsson, MediaTek) - Reuse current beam sweeping factor for UE/event triggered beam reporting

The fast beam sweeping in rel-18 multi-RX is applicable when gNB configures groupBasedBeamReporting and within the single carrier.

In the WID, the UEIBM is targeting FR2 and sTRP communication.

Proposal 12: No enhancement for fast beam sweeping.

Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward > - Option 1: - The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements. - Option 2: - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements. - Option 3: - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, reusethe existing L1-RSRP accuracy requirement. - Other options are not precluded.

If the measurement time is reused similar L1-RSRP measurement period, and the side condition is the same, there is no reason to define a different accuracy requirement.

Proposal 13: The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements.

Issue 1-11: Capabilities for Support of Event Triggering and Reporting Criteria? <Way forward > - Option 1: (Nokia) - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. - Option 2: (Apple) - Capabilities shall be defined by RAN1, to be discussed later if any RAN4 specific UE capabilities need to be added. - Option 3: (Ericsson) - Define UE capability for total number of events supported for UE-initiated beam management across single and multiple TRP. - Option 4: (MediaTek) - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency. Issue 1-12: Compatibility of capabilities with LTM <Way forward > - Option 1: (Nokia) - RAN4 to define UE capabilities in the context of UE initiated beam reporting towards the later part of the work item. - Option 2: (Samsung, Apple) - Do not combine capabilities across different L1 measurements. - Option 3: (Ericsson) - RAN4 to wait for further progress in RAN1/2 for the UE capability discussions and compatibility between MIMO and mobility. - Option 4: (MediaTek) - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency.

Event triggered L1 measurement reporting will be introduced in Rel-19 MIMO and Rel-19 mobility separately. However, the motivation and procedures are different. For example, one is for FR2 different beams, while the other is for FR1 and FR2 beams for candidate cells. The Events are also different. In addition, there might be different UE capabilities to support either of them or both of them.

Proposal 14: Define separate capabilities for MIMO and mobility.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for UE-initiated/event-driven beam management in Rel-19 MIMO phase 5:

Observation 1: RAN1 only support Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s).

Proposal 1: According to latest RAN1 support, the evaluation periodicity can reuse the legacy L1-RSRP measurement period.

Proposal 2: RAN4 to specify requirements for both basic feature and within a time window configured.

Proposal 3: For the case of no time window based, the L1 evaluation time is defined as:

FR2+SSB based+event-2

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index in indicated TCI state and SSB-index configured in [TBD]

FR2+SSB based+event-1

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index in indicated TCI state.

Observation 2: For event-7, no conclusion of whether the RS periodicity of RSs in activated TCI states are the same or not.

Observation 3: The measurement should cover all activated TCI state and new beam (s).

Proposal 4: For the case of no time window based, the L1 evaluation time is defined as:

FR2+SSB based+event-7

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*max(TSSB_active_0, …TSSB_active_i, TSSB_active_X, TSSB_new)) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX, TSSB_active_0, …TSSB_active_i, TSSB_active_X, TSSB_new)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note: TDRX is the DRX cycle length. TReport is configured periodicity for reporting. |  |

TSSB_active_i = ssb-periodicityServingCell is the periodicity of the SSB-Index in i-th active TCI state.

Proposal 5: For within a time window configured case, the L1 evaluation time is defined as multiple times of L1 evaluation time in basic to fulfil M counter is met for the event.

Proposal 6: After we achieved the agreement of L1 evaluation time, the reporting delay can be specified as: T_L1 evaluation time + T_time_first_pucch.

Proposal 7: No need to add TSSB_time_index

Proposal 8: Similar approach of TL1-RSRP_Measurement_Period_CSI-RS  in section 9.5.4.2 can be reused as the principle in SSB based UEIBM.

Proposal 9: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy.

Proposal 10: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM.

Proposal 11: The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI.

Proposal 12: No enhancement for fast beam sweeping.

Proposal 13: The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements.

Proposal 14: Define separate capabilities for MIMO and mobility.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2420096, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung

[3] R1-2410914, LS to RAN4 on event-instance evaluation periodicity for UE-initiated/event-driven beam management, RAN1