3GPP TSG-RAN WG4 Meeting #114		R4-2501464

Athens, GR, 17th – 21st February, 2025

Agenda item:		7.23.3.1

Source:		Samsung

Title:	Views on L1 CLI measurement requirements

Document for:		Discussion

- Introduction

The objectives of Rel-19 “Evolution of NR duplex operation: Sub-band full duplex (SBFD)” described in latest WID [1] which includes L1 based UE-to-UE CLI measurement and reporting.

In previous RAN4 meetings, the group achieved some agreements and latest WF is approved in [2]. There are still some open issues in [2]. In this contribution, we continue to discuss the open issues of RRM requirements for L1 CLI measurement.

- Discussion

UE-to-UE CLI handling

In last RAN4#113 meeting, RAN4 has agreed to specify L1 CLI measurement requirements for Method#1 and Method#2 as listed in WF [2]:

#### Issue 1-1-2: Measurement methods - Agreements (from Thu AH session) - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

In addition, according the RAN1 further agreements in RAN1#119:

Agreement For L1 UE-to-UE CLI measurement and reporting, Method#3 is supported - Method #3: UE measures RSSI within UL subband - A UE cannot be configured to perform measurement using Method #1 and Methods#3 in the same OFDM symbol. Measurement resource(s) corresponding to Methods #1 and #3 shall not be associated with the same CSI-ReportConfig.

Proposal 1: RAN4 to further specify L1 CLI measurement and reporting for:

- Method #3: UE measures RSSI within UL subband

In last RAN4#113 meeting, we provided our analysis on measurement period and side condition, the measurement period requirements are still open in WF[2]:

#### Issue 1-1-6: Measurement period - Agreements (from Wed AH session) - For L1-SRS-RSRP, further discuss the following options based on simulation results: - Option 1: requirement specified with [1] sample measurement assumption - Option 2: requirement specified based on [1] and [3] samples according to NW configuration, e.g. timeRestrictionForChannelMeasurement - For L1-CLI-RSSI, - Confirm measurement period requirements are defined based on 1 sample, and - Clarify in the requirements that the measurement is based on DL timing.

In previous meeting, we provided the analysis of the L1-SRS-RSRP are sensitive to the residual timing error especially for inter-cell scenario. The number of samples will be related to the different cases.

Case 1: 4.67us for FR1, 3.67us for FR2 (inter-cell case) Case 2: 1.67us for FR1, 0.67us for FR2 (intra-cell case)

An LS was sent to RAN1 to check the scope of L1 CLI measurement.

RAN1 sent a reply LS for the L1-CLI measurement in [3]:

When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

From RAN1 perspective, there is no distinction between the intra-cell UE-to-UE L1 CLI measurement and inter-cell UE-to-UE L1-CLI measurement, but intra-cell scenario can be prioritized.

We also update our simulation results for case 2 in TDL-A in the appendix, in addition, further case 1 simulation results are also update for more PRBs configurations.

The trend is similar as the observation as before:

Observation 1: For case 2, intra-cell case, with the number of samples for measurement increased, the improvement is not so larger, it is less than 0.5dB by extended to 3 samples to 1 sample.

However, for inter-cell case, one sample measurement degraded a lot especially in FR2 deployments.

Observation 2: For inter-cell case, one sample measurement degraded a lot especially in FR2 deployments.

In last meeting, there is no agreements on whether to have single set measurement period or different sets measurement period.

By our preference, prioritized intra-cell scenario for single set of requirements with one shot measurement is enough.

Proposal 2: Prioritized intra-cell scenario for single set of requirements:

- one shot measurement is enough, X=1 sample

#### Issue 1-1-9: Scheduling and measurement restriction - Agreements (from Thu AH session) - For scheduling restriction, - For L1-SRS-RSRP, - For FR1, R16 requirements are re-used as baseline, FFS exact number of restricted symbols considering the impact of timing assumption - For FR2, FFS whether R16 requirements are re-used as baseline, considering the impact of Rx beam determination, and timing assumption - For L1-CLI-RSSI, FFS whether R16 requirements are re-used as baseline, considering the measurement is based on DL timing. - For measurement restriction, - Requirements for L1 CLI measurement do not apply when the L1 CLI measurement resource collides with SMTC window, SSB or CSI-RS configured for RLM, BFD, CBD or L1-RSRP measurement or measurement gaps. - FFS whether need to consider the exception cases where L1 CLI measurement can be performed with other measurement in parallel.

For L1-SRS-RSRP, for FR1, the scheduling restriction should consider the timing between SRS arriving timing and DL timing of the serving cell.

The key factor is Tc*N_TA_offset and propagation delay. By calculation of NTA offset in Table 7.1.2-2 and Tc = 0.509ns plus 1.67us (FR1) and 0.67us (FR2).

| SCS | symbol length |
| --- | --- |
| 15kHz | 66.67us |
| 30kHz | 33.33us |
| 60kHz | 16.67us |
| 120kHz | 8.33us |
| 240kHz | 4.17us |

Proposal 3: For scheduling restriction, the UE is not expected to receive PDSCH on SRS-RSRP measured OFDM symbols, and on Y data symbols before L1-SRS-RSRP measured OFDM symbols:

- Y = 1 for 15kHz/30kHz for FR1,

- Y = 2 for 60kHz for FR1

- Y = 1 for 60kHz for FR2

- Y = 2 for 120kHz for FR2

#### Issue 1-1-4: Measurement resources - Recommended WF - Further discuss the issue

Unlike L3 CLI measurement, it should be based on L1 CSI framework.

According to RAN1 previous agreements:

RAN1#118bis Agreement Extend CSI-ResourceConfig to include two CLI measurement resource set lists for SRS-RSRP and CLI-RSSI measurements based on Rel-16 SRS-ResourceConfigCLI and rssi-ResourceConfigCLI defined in MeasObjectCLI for L3 based SRS-RSRP and CLI-RSSI measurement. - The resourceType for the two new CLI measurement resource set lists can be set to periodic, semi-persistent or aperiodic - The number of periodic/semi-persistent CLI measurement resource sets is restricted to 1 in a CSI-ResourceConfig as in current specifications. - Note: No new usage needs to be defined for SRS resource sets RAN1#118 Agreement For frequency resource allocation of a CLI-RSSI measurement resource, the following are supported - Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband - Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands - FFS: Number of resources that can be configured - FFS: UE behavior for measurement Agreement For L1 UE-to-UE CLI measurement and reporting, the following are supported - Wideband CLI-RSRP reporting - Wideband CLI-RSSI reporting FFS: Subband CLI-RSSI reporting RAN1#118bis Agreement For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported. - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband. RAN1#119 Conclusion There is no RAN1 consensus on the following: - Subband CLI-RSSI reporting for L1 CLI-RSSI measurement in Rel-19.

Proposal 4: For measurement resource type, RAN4 to specify periodic, semi-persistent or aperiodic resource types.

Proposal 5: For CLI-RSRP reporting, RAN4 to specify the requirements based on wideband CLI-RSRP reporting.

Proposal 6: For CLI-RSSI reporting, RAN4 to specify the requirements based on wideband CLI-RSSI reporting.

#### Issue 1-1-7: Measurement reporting - Recommended WF - Further discuss the issue

According to RAN1 status, only aperiodic reporting is agreed at least. There is no further conclusion to support periodic and semi-persistent reporting.

Proposal 7: Unlike L3 CLI measurement in legacy requirements, for measurement reporting, RAN4 to specify the requirements only for aperiodic reporting.

- Conclusion

In this contribution, we provide our proposals for L1 CLI measurement:

Proposal 1: RAN4 to further specify L1 CLI measurement and reporting for:

- Method #3: UE measures RSSI within UL subband

Observation 1: For case 2, intra-cell case, with the number of samples for measurement increased, the improvement is not so larger, it is less than 0.5dB by extended to 3 samples to 1 sample.

Observation 2: For inter-cell case, one sample measurement degraded a lot especially in FR2 deployments.

Proposal 2: Prioritized intra-cell scenario for single set of requirements:

- one shot measurement is enough, X=1 sample

Proposal 3: For scheduling restriction, the UE is not expected to receive PDSCH on SRS-RSRP measured OFDM symbols, and on Y data symbols before L1-SRS-RSRP measured OFDM symbols:

- Y = 1 for 15kHz/30kHz for FR1,

- Y = 2 for 60kHz for FR1

- Y = 1 for 60kHz for FR2

- Y = 2 for 120kHz for FR2

Proposal 4: For measurement resource type, RAN4 to specify periodic, semi-persistent or aperiodic resource types.

Proposal 5: For CLI-RSRP reporting, RAN4 to specify the requirements based on wideband CLI-RSRP reporting.

Proposal 6: For CLI-RSSI reporting, RAN4 to specify the requirements based on wideband CLI-RSSI reporting.

Proposal 7: Unlike L3 CLI measurement in legacy requirements, for measurement reporting, RAN4 to specify the requirements only for aperiodic reporting.

- Reference

[1] RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2420097, WF on RRM requirements for NR_duplex_evo, Huawei

[3] R1-2410890, Reply LS on the scenarios of UE-to-UE CLI measurement report in SBFD operation, RAN1

Appendix: simulation results update:

Case 2 in AWGN:

| SRS symbol number = 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR | SCS | SRS BW | Sample | Samsung |  |  |  |  |  |
|  | (kHz) | (PRBs) |  | SNR (dB) |  |  |  |  |  |
|  |  |  |  | 1 dB |  | 2 dB |  | 3 dB |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.6231 | 0.552 | -0.5483 | 0.4903 | -0.4832 | 0.4339 |
|  |  |  | 2 | -0.4373 | 0.417 | -0.3887 | 0.3716 | -0.3422 | 0.3281 |
|  |  |  | 3 | -0.3226 | 0.3686 | -0.2812 | 0.3251 | -0.2469 | 0.2871 |
|  |  | 48 | 1 | -0.4686 | 0.4039 | -0.4102 | 0.3564 | -0.3583 | 0.3142 |
|  |  |  | 2 | -0.3421 | 0.3674 | -0.3271 | 0.2334 | -0.291 | 0.2032 |
|  |  |  | 3 | -0.2873 | 0.2249 | -0.2674 | 0.1973 | -0.2298 | 0.1747 |
|  | 30 | 24 | 1 | -0.6231 | 0.552 | -0.5483 | 0.4903 | -0.4832 | 0.4339 |
|  |  |  | 2 | -0.4373 | 0.417 | -0.3887 | 0.3716 | -0.3422 | 0.3281 |
|  |  |  | 3 | -0.3226 | 0.3686 | -0.2812 | 0.3251 | -0.2469 | 0.2871 |
|  |  | 48 | 1 | -0.6217 | 0.3137 | -0.4102 | 0.3564 | -0.3583 | 0.3142 |
|  |  |  | 2 | -0.49 | 0.2198 | -0.3066 | 0.2551 | 0.3142 | 0.2032 |
|  |  |  | 3 | -0.4074 | 0.1831 | -0.2363 | 0.192 | -0.2298 | 0.1747 |
|  | 60 | 24 | 1 | -0.6231 | 0.552 | -0.6231 | 0.552 | -0.4832 | 0.4339 |
|  |  |  | 2 | -0.4373 | 0.417 | -0.4532 | 0.4312 | -0.3422 | 0.3281 |
|  |  |  | 3 | -0.3226 | 0.3686 | -0.3115 | 0.3622 | -0.2469 | 0.2871 |
|  |  | 48 | 1 | -0.4686 | 0.4039 | -0.4102 | 0.3564 | -0.3583 | 0.3142 |
|  |  |  | 2 | -0.3668 | 0.2703 | -0.3271 | 0.2334 | -0.291 | 0.2032 |
|  |  |  | 3 | -0.3122 | 0.223 | -0.2674 | 0.1973 | -0.2298 | 0.1747 |
| FR2 | 60 | 24 | 1 | -0.6231 | 0.552 | -0.5483 | 0.4903 | -0.4832 | 0.4339 |
|  |  |  | 2 | -0.4373 | 0.417 | -4.4576 | 0.3987 | -0.3422 | 0.3281 |
|  |  |  | 3 | -0.3226 | 0.3686 | -0.2781 | 0.3204 | -0.2469 | 0.2871 |
|  |  | 48 | 1 | -0.4686 | 0.4039 | -0.4102 | 0.3564 | -0.3583 | 0.3142 |
|  |  |  | 2 | -0.3668 | 0.2703 | -0.3271 | 0.2334 | -0.291 | 0.2032 |
|  |  |  | 3 | -0.3122 | 0.223 | -0.2674 | 0.1973 | -0.2298 | 0.1747 |
|  | 120 | 24 | 1 | -0.6231 | 0.552 | -0.5483 | 0.4903 | -0.4832 | 0.4339 |
|  |  |  | 2 | -0.4373 | 0.417 | -0.3887 | 0.3716 | -0.3422 | 0.3281 |
|  |  |  | 3 | -0.3226 | 0.3686 | -0.2812 | 0.3251 | -0.2469 | 0.2871 |
|  |  | 48 | 1 | -0.4686 | 0.4008 | -0.4102 | 0.3564 | -0.3583 | 0.3142 |
|  |  |  | 2 | -0.3976 | 0.3587 | -0.3271 | 0.2334 | -0.291 | 0.2032 |
|  |  |  | 3 | -0.3023 | 0.218 | -0.2674 | 0.1973 | -0.2298 | 0.1747 |

Case 1+TDLA

| SRS symbol number = 1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FR | SCS | SRS BW | Sample | Samsung |  |  |  |  |  |
|  | (kHz) | (PRBs) |  | SNR (dB) |  |  |  |  |  |
|  |  |  |  | 1 dB |  | 2 dB |  | 3 dB |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.8132 | 0.6336 | -0.7148 | 0.5567 | -0.6139 | 0.4819 |
|  |  |  | 2 | -0.5452 | 0.4329 | -0.4765 | 0.3771 | -0.4186 | 0.3362 |
|  |  |  | 3 | -0.4471 | 0.382 | -0.3943 | 0.3258 | -0.3493 | 0.2851 |
|  |  | 48 | 1 | -2.061 | -0.7977 | -2.061 | -0.7977 | -1.9505 | -0.8699 |
|  |  |  | 2 | -1.8284 | -0.9283 | -1.8284 | -0.9283 | -1.7815 | -0.9827 |
|  |  |  | 3 | -1.7674 | -0.9935 | -1.7674 | -0.9935 | -1.7182 | -1.0446 |
|  | 30 | 24 | 1 | -2.3082 | -0.4716 | -0.9074 | -0.5743 | -2.0275 | -0.667 |
|  |  |  | 2 | -2.0192 | -0.7045 | -1.8894 | -0.7899 | -1.808 | -0.8434 |
|  |  |  | 3 | -1.8347 | -0.8674 | -1.7749 | -0.9074 | -1.7234 | -0.9696 |
|  |  | 48 | 1 | -3.3235 | -1.7287 | -3.179 | -1.8193 | -3.0721 | -1.898 |
|  |  |  | 2 | -3.0504 | -1.9224 | -2.9789 | -1.9781 | -2.9221 | -2.0409 |
|  |  |  | 3 | -2.9789 | -2.0033 | -2.8973 | -2.0701 | -2.8309 | -2.1131 |
|  | 60 | 24 | 1 | -2.3223 | 0.6263 | -2.2415 | 0.5601 | -2.6354 | -0.7862 |
|  |  |  | 2 | -2.1223 | 0.5183 | -2.0518 | 0.4398 | -2.2357 | -0.954 |
|  |  |  | 3 | -2.0477 | 0.4264 | -2.0068 | 0.3925 | -2.164 | -1.1295 |
|  |  | 48 | 1 | -3.12 | -1.4979 | -2.992 | -1.5791 | -2.889 | -1.6721 |
|  |  |  | 2 | -2.8654 | -1.7118 | -2.7705 | -1.7768 | -1.6721 | -1.8231 |
|  |  |  | 3 | -2.733 | -1.7776 | -2.6748 | -1.8286 | -1.8231 | -1.8802 |
| FR2 | 60 | 24 | 1 | -0.8365 | 0.638 | -0.7144 | 0.5548 | -0.6285 | 0.4879 |
|  |  |  | 2 | -0.6675 | 0.418 | -0.5944 | 0.4748 | -0.5093 | 0.3347 |
|  |  |  | 3 | -0.5266 | 0.3761 | -0.4608 | 0.3174 | -0.4167 | 0.268 |
|  |  | 48 | 1 | -2.2356 | -0.6339 | -2.1181 | -0.7248 | -2.0308 | -0.8122 |
|  |  |  | 2 | -2.0091 | -0.7763 | -1.8937 | -0.8433 | -1.8121 | -0.9146 |
|  |  |  | 3 | -1.8411 | -0.8625 | -1.7747 | -0.936 | -1.729 | -0.9967 |
|  | 120 | 24 | 1 | -2.2721 | -0.4831 | -2.1403 | -0.5775 | -2.0271 | -0.6616 |
|  |  |  | 2 | -1.9927 | -0.69 | -1.8885 | -0.7696 | -0.6616 | -0.8142 |
|  |  |  | 3 | -1.7893 | -0.8079 | -1.7353 | -0.8633 | -1.6706 | -0.928 |
|  |  | 48 | 1 | -3.3973 | -1.7477 | -3.2655 | -1.8383 | -3.1665 | -1.9103 |
|  |  |  | 2 | -3.178 | -1.9746 | -3.0721 | -2.0224 | -2.9795 | -2.0653 |
|  |  |  | 3 | -3.0096 | -2.0374 | -2.9284 | -2.0855 | -2.8602 | -2.127 |