3GPP TSG-RAN WG4 Meeting #114		R4-2501466

Athens, GR, 17th – 21st February, 2025

Agenda item:		7.29.4.1

Source:		Samsung

Title:	Discussion on RRM requirements of RedCap in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In RAN4#113 meeting, we have already discussed all the RRM requirements of RedCap in NTN and achieved some agreements in [1]. There are still some open issues in [1]. In this contribution, we continue to discuss the FFS parts of RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands.

- Discussion

In RAN4#113 meeting, RAN4 achieved agreements in [1]. We will discuss remaining open issues as below.

#### Issue 1-1-1: Whether to define measurement requirement for inter-RAT (NR NTN to E-UTRAN TN) in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN? <Way Forward>: - Proposal 1 (CATT, Xiaomi, LG, Samsung CMCC, Apple, Ericsson, HW, ZTE): - RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN. - Proposal 2 (vivo): - RAN4 not to define the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

In Rel-18 NTN, the measurements of inter-RAT E-UTRAN cells with TN carrier has been supported for normal UEs. From the real scenario, the inter-RAT for co-existence between NTN FR1 and LTE are also possible for operation. From the requirements view, for 2Rx (e)RedCap UE, there is no such difference between 2Rx RedCap UE and Rel-17 and Rel-18 handheld UEs.

In addition, for Redcap UE, the UE shall measure and detect E-UTRA cells as well as specified in 38.133:

The requirements in this clause apply to inter-RAT E-UTRAN FDD measurements and E-UTRA TDD measurements. When the measurement rules indicate that inter-RAT E-UTRAN cells are to be measured, the UE shall measure RSRP and RSRQ of detected E-UTRA cells in the neighbour frequency list at the minimum measurement rate specified in this clause.

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN.

#### Issue 1-1-2: The Requirements to be further determined <Way Forward>: - For SDT requirements in RRC_INACTIVE state, - Proposal 1 (CATT, QC, OPPO, HW): RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements. - Proposal 1a (CATT): TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - Proposal 1b (LG): TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission. - Proposal 1c (CMCC): For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Proposal 1d (vivo): For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable) - Proposal 2 (ZTE): CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be: Measurement; FR1. RSRP1; (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)). RSRP2; (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2. Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.. - For MDT requirements in RRC_IDLE and RRC_INACTIVE state - Proposal 1 (CATT, Samsung, QC, OPPO, Ericsson, vivo, HW, MTK): - Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN as the same situation for (e)RedCap UE in TN system. - Proposal 1a (CATT): Deprioritize to define. - Proposal 2 (CMCC): - Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap over NTN, if the related features are supported in RAN1/2 spec. - Proposal 3 (ZTE): - RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

From LTE and NR, the driving testing has been introduced. The typical use scenario is to use it for network optimization.

In Rel-16 NR, MDT has been introduced as an optional feature.

For (e)Redcap UE, it can support to log such measurement in RRC_IDLE/RRC_INACTIVE case. However, typically, the log especially for long time log for example: time-stamped RSRP/RSRQ will cause the UE power consumption. That is why we think (e)RedCap UE can use for MDT but not typical usage. That is also why no RAN4 requirements for Rel-17/Rel-18 (e)RedCap UE for MDT requirements in TN in Rel-17/Rel-18.

Proposal 2: Follow same principle, No RAN4 requirements for Rel-17/Rel-18 (e)RedCap UE for MDT requirements in TN in Rel-17/Rel-18. Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE in NTN FR1 as the same situation for (e)RedCap UE in TN system due to UE power consumption.

#### Issue 1-2-2: eDRX configuration Online agreement < Agreement >: - For earth-moving LEO scenario: - All eDRX cycle cannot be applied for RedCap/eRedCap UEs in NTN and not to define requirements for eDRX in earth-moving beam LEO scenario. - For quasi-earth fixed LEO scenario: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - Requirements are applicable for 2.56s and 5.12s eDRX cycle - FFS for 10.24s eDRX cycle and larger than 10.24s eDRX cycle

In last meeting, we achieved the agreements for LEO earth moving scenario.

For RedCap UEs, the smallest eDRX is 2.56s:

Table 4.2B.2.3-2: Tdetect,NR_Intra_RedCap, Tmeasure,NR_Intra_RedCap and Tevaluate,NR_Intra_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Intra_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x M2 (1 x M2) | 0.64 x M2 (2 x M2) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| Note 1: The number of DRX cycles in this table corresponds to the DRX cycles within PTWs, when PTW is configured. Note 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. Note 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. Note 4: The lower bound of PTW length is derived based on Tevaluate,NR_Intra_RedCap*DRX_cycle1.28*1.28. Note 5: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms; otherwise M2=1. |  |  |  |  |  |

Table 4.2B.2.4-2: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |

For earth-fixed beam LEO deployment, most important factor is the dwell time. In previous discussion for the TR, the typical dwell time is about 1~10min.

In last meeting, we have already agreed 2.56s and 5.12s eDRX are feasible. Therefore, the requirements are applicable for 2.56s and 5.12s eDRX cycle.

For 10.24s eDRX and larger than 1024s eDRX, they are still FFS.

According to the tables above, for 10.24s eDRX, the longest detection time is up to 235.52s. Although there are no agreements for the dwell time in previous releases. However, during the discussion and values provided by companies, the typical dwell time is about 1~10min in real satellite deployments. Compare to longest dwell time 10min=600s, UE shall be able to reselect to a new satellite when finish the detection+measurement, it is too short for NW to provide regular service and hard to guarantee the performance. The UE will always perform detection/measurement and change the served satellites, there is no room for the data transmission kept between satellites and UE.

For eDRX even larger than 10.24s, the situation goes worse. When UE hasn’t finished the detection/measurement, the satellite has moved out of the dwelling.

Proposal 3: For earth-fixed beam LEO deployment, 10.24s eDRX cycle and larger than 10.24s eDRX cycle cannot be applied. RedCap UEs are not required to meet the requirements.

#### Issue 1-2-4: Multiple SMTCs Online agreement <Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

In Rel-17 NTN, 2 SMTC is default and mandatory for UE. 4 SMTC-s are optional UE capability.

In Rel-17 NTN, UE can measure multiple LEO satellites in parallel within an SMTC in RRC_Idle and RRC_Connected modes if UE with the capability.

From technical view, the parallel measurement can also be supported. However, the key concern for RedCap UE is the complexity and power consumption. From UE perspective, deprioritize parallel measurement of multiple LEO satellites within an SMTC for RedCap UE in NTN.

Proposal 4: Do not support parallel measurements on multiple NGSO satellites within as SMTC for RedCap UE in NTN.

#### Issue 2-1-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN - Define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands. - FFS: How to define the enhanced cell reselection measurement requirements for 1Rx and 2Rx (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to further discuss how to define the cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for 1Rx and 2 Rx (e)RedCap UE after issue1-1-1.

Proposal 5: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands,

- For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

Proposal 6: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands,

- For 2Rx UE, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands.

- For 1RX UE, reuse the same margin of RSRP/RSRQ as for 2Rx.

#### Issue 2-4-1: L1/L3 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, HW): RAN4 to reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level. - Depend on the supported UE capabilities (e.g., multiple SMTC),, the existing NR NTN requirements may need be revised. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, ZTE, HW): The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter. - Not to extend the lower boundary. - Proposal 1a (QC): Extended delays for PSS/SSS detection. - Proposal 1b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples. - Proposal 2 (CATT, [Samsung?], CMCC, HW): The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter. - Extend the lower boundary from ‘120 ms’ to ‘160ms’. - Proposal 2a (QC): Extended delays for SSB index identification (PBCH decoding). - Proposal 2b (OPPO): Support to relax measurement requirements including extending the measurement delays with more measurement samples. - Proposal 3 (CATT, CMCC, HW): The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs. - Reuse the same extensions for 1Rx RedCap UE in TN network. - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples for T SSB_measurement_period_inter. - Proposal 4 (CATT, CMCC, ZTE, HW): The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same. - Proposal 5 (CATT, CMCC, HW): The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

The L3 measurement are still open. As legacy requirements, For L3 measurements for (e)RedCap UE with FR1-NTN, these requirements should be specified:

- Intra frequency measurements without measurement gaps

- Intra-frequency measurements with measurement gaps

- Inter-frequency measurement with measurement gaps

- Inter frequency measurements without measurement gaps

From high level principle, for 2Rx RedCap UE, reuse the legacy FR1-NTN requirements in high level (FFS on whether parallel measurement is allowed for RedCap UE)

For 1Rx RedCap, requirements difference comparation to 2Rx UE, reuse the same principle in TN system for RedCap UE which means additional samples for extension. Use the same number of additional samples as that in TN.

Proposal 7:

- For 2Rx (e)RedCap UEs, reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level.

- For 1Rx (e)RedCap UEs, reuse the same principle in TN system for RedCap UE which means additional samples for extension. Use the same number of additional samples as that in TN:

    - The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

      - Not to extend the lower boundary.

    - The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

      - Extend the lower boundary from ‘120 ms’ to ‘160ms’.

    - The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

      - Extend the lower boundary from ‘120 ms’ to ‘160ms’.

    - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

    - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

#### Issue 2-5-1: L1/L3 measurement accuracy for 1Rx/2Rx (e)RedCap UEs with FR1-NTN <Way Forward>: - For 2Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Samsung, CMCC, ZTE): RAN4 to reuse the measurement accuracy requirements for normal UEs defined in NTN. - For 1Rx (e)RedCap UEs with FR1-NTN bands, - Proposal 1 (CATT, Xiaomi, Samsung, [QC?], CMCC, ZTE, [OPPO?], HW): RAN4 to relax the L1/L3 measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs. - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused. - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB. - L1-RSRP: relax by 3dB.

Proposal 8: For accuracy requirements:

- For 2Rx (e)RedCap UEs, reuse Rel-17 NTN requirements.

- For 1Rx (e)RedCap UEs, reuse the same relaxation for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP in TN:

  - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

  - L1-RSRP: relax by 3dB.

#### Issue 3-1-1: General HD-FDD related impact <Way Forward>: - Proposal 1 (CATT, Xiaomi, Samsung, QC, CMCC, ZTE, Ericsson, HW): - The legacy requirements and applicable conditions can be as baseline for defining requirements of RedCap UE in NTN scenario. - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions. - Proposal 1a (CATT, HW, MTK): - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN: - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state - Handover interruption time - Random access - SA: RRC Connection Release with Redirection - Minimum requirement for L1 indication for RLM and LR - MAC-CE/DCI/RRC based uplink spatial relation switch delay - Scheduling availability of UE performing intra/inter measurements - Scheduling availability of UE during L1-RSRP measurement - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects: - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions. - Proposal 1c (CMCC): - Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline. - Wait for further RAN1 progress which may impact Scheduling availability requirement.

In RedCap in TN system, the RRM requirements are specified for HD-FDD UEs:

#### 4.2B.2.6 Maximum interruption in paging reception for RedCap ## 6.1D Handover for RedCap ### 6.2.2B Random access for RedCap ### 6.2.3A SA: RRC Connection Release with Redirection for RedCap ## 8.1B Radio Link Monitoring for RedCap ## 8.5B Link Recovery Procedures for Redcap ## 8.12A Uplink spatial relation switch delay for RedCap #### 9.3B.7.3/9.2B.5.3 Scheduling availability of UE during intra/inter-frequency measurements #### 9.5B.6 Scheduling availability of UE during L1-RSRP measurement

Proposal 9:

- To use legacy TN requirements as baseline, RAN4 to specify the requirements for HD-FDD UEs for the table above.

In legacy requirements such as scheduling availability, the restriction is applied because DL measurement is prioritized over UL transmission. However, in RAN1 discussion, not all the priority rules have been agreed as shown:

RAN1#119 meeting Agreement The collision case 4 (Dynamically scheduled DL reception collides with dynamic scheduled UL transmission) is not considered as an error case for Rel-19 HD-FDD (e)Redcap UE in RRC connected mode. - FFS whether to prioritize UL or prioritize DL.

Proposal 10: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for (e)RedCap UE with NR NTN operating in FR1-NTN bands:

Proposal 1: RAN4 to define measurement requirements for inter-RAT (NTN to E-UTRAN TN) measurement in IDLE/INACITVE mode for (e)RedCap UE with FR1-NTN.

Proposal 2: Follow same principle, No RAN4 requirements for Rel-17/Rel-18 (e)RedCap UE for MDT requirements in TN in Rel-17/Rel-18. Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE in NTN FR1 as the same situation for (e)RedCap UE in TN system due to UE power consumption.

Proposal 3: For earth-fixed beam LEO deployment, 10.24s eDRX cycle and larger than 10.24s eDRX cycle cannot be applied. RedCap UEs are not required to meet the requirements.

Proposal 4: Do not support parallel measurements on multiple NGSO satellites within as SMTC for RedCap UE in NTN.

Proposal 5: For enhanced cell re-selection for (e)RedCap UE with FR1-NTN bands,

- For 2Rx/1Rx, the Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh are reused the legacy FR1-NTN requirements. In addition, The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.

Proposal 6: For cell re-selection measurement requirements of inter-RAT E-UTRAN cells with TN carrier for (e)RedCap UE with FR1-NTN bands,

- For 2Rx UE, the existing cell reselection requirements in Rel-18 (e)NTN can be reused for (e)RedCap 2Rx UE with FR1-NTN bands.

- For 1RX UE, reuse the same margin of RSRP/RSRQ as for 2Rx.

Proposal 7:

- For 2Rx (e)RedCap UEs, reuse the legacy FR1-NTN measurement requirements defined for normal UEs in high level.

- For 1Rx (e)RedCap UEs, reuse the same principle in TN system for RedCap UE which means additional samples for extension. Use the same number of additional samples as that in TN:

    - The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

      - Not to extend the lower boundary.

    - The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

      - Extend the lower boundary from ‘120 ms’ to ‘160ms’.

    - The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

      - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

      - Extend the lower boundary from ‘120 ms’ to ‘160ms’.

    - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

    - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Proposal 8: For accuracy requirements:

- For 2Rx (e)RedCap UEs, reuse Rel-17 NTN requirements.

- For 1Rx (e)RedCap UEs, reuse the same relaxation for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP in TN:

  - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

  - L1-RSRP: relax by 3dB.

Proposal 9:

- To use legacy TN requirements as baseline, RAN4 to specify the requirements for HD-FDD UEs for the table above.

Proposal 10: For scheduling availability, need further RAN1 progress for detailed requirements of HD-FDD UEs.

- Reference

[1] R4-2420104, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT