3GPP TSG-RAN WG4 Meeting #114		R4-2501467

Athens, GR, 17th – 21st February, 2025

Agenda item:		7.29.4.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In RAN4#113 meeting, we have already discussed the RRM requirements of “DL coverage enhancements” and “regenerative payload”. There are still some open issues for DL coverage enhancement in latest WF [1].

In this contribution, we continue to discuss the open issues of RRM requirements for DL coverage enhancement.

- Discussion

In RAN4#113 meeting, the RRM impacts of objective #1 DL coverage enhancement are still open in [1] as below:

Agreement (online): For SSB periodicity larger than 160ms if supported by other WGs: - Whether RAN4 will define the RRM requirements for SSB periodicity larger than 160ms is depending on whether it is supported by WGs.

We discussed in previous meeting, if SSB periodicity will be larger than 160ms like 320ms, many RRM requirements are needed to update.

In last RAN plenary RAN#106 meeting:

RP-243113 SSB periodicity extension on R19 NR NTN CATT, CSCN, CAICT, Baicells, HONOR, Lenovo, OPPO, Spreadtrum, TCL, xiaomi Replaces Provide our views on Rel-19 NTN SSB periodicity extension RAN chair: who wants to discuss proposal 1 here in RAN? 7 companies RAN chair: who wants to discuss proposal 1 in RAN1? 14 companies RAN chair: so discuss proposal 1 in RAN1 and you can discuss how to revise the WID, result will be in RP-243262 The document was noted. Replaced by

Observation 1: In latest revised WID (RP-243300) [2], the maximum of 160ms SSB periodicity is supported. There is no further conclusion to support 320ms SSB periodicity.

Proposal 1: RAN4 to specify the RRM requirements to consider maximum of 160ms SSB periodicity.

Based on P1, the detailed RRM impacts by SSB periodicity extension are listed as below:

### Issue 1-1: SSB periodicity enhancement Agreement (online): For SSB periodicity up to 160ms: - No further discussion on the impact on initial access from the perspective of RRM requirement - Discuss whether to Modify the default values for SSB and SMTC periodicities for the following if the UE is not provided with SMTC configuration or measurement object on the corresponding frequency by NW - RRC Re-establishment: TSMTC,i assumption - RRC connection release with redirection to NR: Trs assumption - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync: Trs assumption - Further discuss the details on how to update the requirement. - Further discuss the following for SSB periodicity > 20ms: - Cell Reselection: for the SSB-periodicity of 160ms, whether the values for Tmeasure_NR_xxx_enh need to be updated for DRX Cycles equal to 0.32 s and other DRX cycles - Paging interruption in RRC Idle/Inactive: whether/how to adjust the requirements when SSB periods =160 ms in NGSO scenarios - RRC re-establishment: RRC re-establishment requirements for Serving cell SSB Ês/Iot < -8dB - Handover, Conditional handover, RACH-less HO and satellite switch with re-sync - Further discuss whether and how to update the requirement

In existing Rel-17 NTN requirements:

##### 6.2C.1.2.1 UE Re-establishment delay requirement TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

For Re-establishment, the UE should perform similar behaviour as initial access process in idle mode. In legacy initial access, RAN1 specified it as 20ms.

In last RAN1#118bis meeting, the agreement for SSB periodicity extension has been approved as:

Agreement For NR NTN, support extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is at least 160ms. - FFS: whether 320ms can be supported as the maximum of the additional default value instead of or in addition to 160ms.

Proposal 2: For Re-establishment delay requirement, update the default value of: TSMTC,I to 160ms because UE should perform similar behavior as initial access process.

For existing RRC Connection Release requirements and series of HO requirements, copied as below:

### 6.2C.3 SA: RRC Connection Release with Redirection for Satellite Access Trs is the SMTC periodicity of the target NR cell if the UE has been provided with an SMTC configuration for the target cell in the redirection command, otherwise Trs is the SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing configured for the RRC connection release with redirection. If the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise,

### 6.1C.1 NR SAN Handover Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. If the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5 ms assuming the SSB transmission periodicity is 5 ms. There is no requirement if the SSB transmission periodicity is not 5 ms. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2 prior to the handover command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell. In addition, other similar procedures like CHO, RACH-less HO and satellite switch

For RRC connection Release and handover requirements, by our understanding, it is not similar as that in Re-establishment delay requirement.

In RRC connection release, the legacy requirement is about the UE behavior when the target SMTC is not known to UE after release. The default value is 20ms.

In HO requirements, UE usually measure the target cell with a configured SMTC configuration. If this information is not provided, 5ms SSB is assumed to be transmitted. If the SSB is not transmitted every 5ms, no requirements.

In NTN DL coverage scenario, although the SSB periodicity still can be other values which is less than 160ms. However, to support the better thousands of beams, the SSB periodicity of 160ms is more realistic in NTN system.

Proposal 3: For RRC connection Release and series of HO requirements, update the default value of: Trs to 160ms in NTN DL coverage scenario.

In Rel-17 NTN cell-reselection requirements:

Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1. 0.32; 1; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 2: The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment.. Table 4.2C.2.3-2: Tdetect,NR_Intra_enh, Tmeasure,NR_Intra_enh and Tevaluate,NR_Intra_enh DRX cycle length [s]; Tdetect,NR_Intra_enh [s] (number of DRX cycles); Tmeasure,NR_Intra_enh [s] (number of DRX cycles); Tevaluate,NR_Intra_enh [s] (number of DRX cycles). 0.32; 2.56 x M2 (8 x M2)Note 1; 0.32 x M3 (1 x M3) Note 1; 0.96 x M4 (3 x M4) Note 1. 0.64; 5.12 (8); 0.64 (1); 1.92 (3). 1.28; 8.96 (7); 1.28 (1); 3.84 (3). 2.56; 58.88 (23); 2.56 (1); 7.68 (3). NOTE 1: When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 2, M3 = M4 = 2.5.

For the UE not support enhanced RRM requirements, the M2 is not so tight as M3 for the UE support enhanced RRM requirements.

In RRC_IDLE mode/Inactive modes, the cell reselection requirements are based on the DRX and SMTC. The SMTC period can be the same or different from SSB period. However, in SMTC duration time, if there is always no SSB, which in this case, the real SSB transmission periodicity is greater than the SMTC configured, it has no contribution for the detection/measure/evaluate. More samples are expected.

Proposal 4: For cell reselection requirements, RAN4 can discuss and specify further more samples or additional scaling factor if SSB periodicity is too greater than the SMTC.

For cell DTX/DRX, the agreement is listed in WF as below:

### Issue 1-3: RRM impacts due to Satellite Beam Hopping (e.g. Cell DTX/DRX) Agreement: RAN4 to wait for more progress from RAN1/2 on the detailed solutions to adopt the concept of Rel-18 cell DTX/DRX based satellite beam hoping support before discussing whether and how to modify the relevant RRM requirements.

In latest RAN#128 agreements:

Agreements:

1.	For cell level DTX for NTN DL coverage enhancement, RAN2 currently understands that there will be only two states: "cell DTX On period" and "Cell DTX Off period" (FFS if the UE behaviour will be different from the NES states or the same). FFS on Cell DRX states.

In the WID, the bullet shows: Rel-18 network energy saving techniques should be considered as baseline in the system level study. In Rel-18, cell level DTX are introduced. However, cell level DTX doesn’t affect DL measurement RSs.

Observation 2: By latest RAN2 agreements, at least the cell DTX off period will be considered.

Observation 3: In Rel-18 NES, no DL measurement impacts due to cell level DTX.

Observation 4: Although RAN2 still has FFS if the UE behavior will be different from the NES states or the same, by our understanding, in this NTN hopping beam scenarios, most beams may be in off states which means no any DL signals including DL measurement RSs due the power issue of RF panels in satellite.

Proposal 5: RAN4 to discuss the RRM impacts by introducing the cell DTX in NTN DL coverage enhancements.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for DL coverage enhancement:

Observation 1: In latest revised WID (RP-243300) [2], the maximum of 160ms SSB periodicity is supported. There is no further conclusion to support 320ms SSB periodicity.

Proposal 1: RAN4 to specify the RRM requirements to consider maximum of 160ms SSB periodicity.

Proposal 2: For Re-establishment delay requirement, update the default value of: TSMTC,I to 160ms because UE should perform similar behavior as initial access process.

Proposal 3: For RRC connection Release and series of HO requirements, update the default value of: Trs to 160ms in NTN DL coverage scenario.

Proposal 4: For cell reselection requirements, RAN4 can discuss and specify further more samples or additional scaling factor if SSB periodicity is too greater than the SMTC.

Observation 2: By latest RAN2 agreements, at least the cell DTX off period will be considered.

Observation 3: In Rel-18 NES, no DL measurement impacts due to cell level DTX.

Observation 4: Although RAN2 still has FFS if the UE behavior will be different from the NES states or the same, by our understanding, in this NTN hopping beam scenarios, most beams may be in off states which means no any DL signals including DL measurement RSs due the power issue of RF panels in satellite.

Proposal 5: RAN4 to discuss the RRM impacts by introducing the cell DTX in NTN DL coverage enhancements.

- Reference

[1] R4-2420105, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm Incorporated

[2] RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, THALES, CATT