3GPP TSG-RAN WG4 Meeting #114	R4-2501477
Athens, Greece, 17th – 21nd February, 2025

Agenda item:	7.29.2.1

Source: 	OPPO

Title: 	Discussion on RedCap UE RF requirements with NR NTN operating in FR1-NTN bands

Document for:	Discussion

- Introduction

In RAN4 #113 meeting, the RedCap UE RF requirements with NR NTN operating in FR1-NTN bands were discussed and some agreements are approved in WF [1]:

Agreements: Issue 1-1: PC3 output power for (e)RedCap - Agreement: - FFS whether and how a potential new scheme to increase output power is addressed in specifications - Alt 1:Further confirm whether rel-18 power boost is already applicable for NR NTN including (e)RedCap - If confirmed, no further work is needed. - Alt 2: Power increase by network controlled optional new signalling (e.g capability) - Alt 3: Change in power tolerances - E.g. Upper tolerance exception to UE in specific regions via table note. - FFS whether solution is common for HD-FDD and FD-FDD - If no solution agreed, PC3 tolerance will remain unchanged to +/- 2 dB. Issue 1-2: HD-FDD refsens for 2 Rx - Agreement: Further discussion on potential improvement beyond TN HD-FDD 2 RX REFSENS level is limited to maximum 0.2 dB improvement based on average of inputs to RAN4. Issue 1-3: Simultaneous operation with GNSS - Agreement: - Simultaneous GNSS and NR-NTN operation for (e)Redcap cannot be guaranteed in some occasions for some NTN bands for some specific UE implementations. - Send LS to RAN2 requesting common IDC solution for (e)RedCap NR NTN and non-RedCap NR NTN which further improves the existing IDC framework to be more suitable for NR NTN – GNSS co-existence. Final solution is up to RAN2. Issue 2-1: UL capacity enhancements - Agreement: No RF impact on inter-slot OCC over 2 slots.

This contribution further discussed the UE RF requirements for RedCap NTN.

- Discussion

Issue 1-1: PC3 output power for (e)RedCap

From RAN4 NR NTN UE Spec, the requirements of MRP and configured transmitted power for NR NTN reused the requirements for NR TN directly. And for NR TN (e)Redcap, it also reused the general requirements for NR TN directly.

From the point of signalling, if no special definition, the UE capability for NR TN also apply to NR NTN UE. Therefore, the UE capability of rel-18 power boost for NR TN is applicable for NR NTN including (e)Redcap. Correspondingly, the UE capability of rel-18 power boost for NR TN is applicable for NR NTN including (e)Redcap naturally as long as the NR NTN Redcap can access the NTN network and the NTN network can recognize this capability.

So, whether rel-18 power boost for NR TN is applicable to NR NTN including (e)Redcap depends on whether the NTN network can recognize this capability.

Proposal 1: The UE capability of rel-18 power boost for NR TN is applicable for NR NTN including (e)Redcap, no further work is needed.

Issue 1-2: HD-FDD refsens for 2 Rx

In the previous RAN4 meeting, for HD-FDD REFSENS for 2 Rx, 0.5 dB tightening was agreed for n256, however, the tightening amount for n254 and n255 are still under discussion.

For below 6GHz, the baseline REFSENS level was calculated by the equation below:

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM – diversity gain

We can get the values of baseline REFSENs with 2Rx for different channel bandwidth, here just list some channel bandwidth in Table 2-1:

Table 2-1 baseline REFSENs with 2Rx

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| Baseline | 15 | -100 | -96.8 | -95 | -93.8 |

And we can further compare the REFSENS of FDD band n1 with that of TDD band n39 as shown in Table 2-2:

Table 2-2 REFSENs with 2Rx for n1 and n39

| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n1 | 15 | -100 | -96.8 | -95 | -93.8 |
| n39 | 15 | -100 | -96.8 | -95 | -93.8 |

We can see the REFSENS for both of FDD band n1 and TDD band n39 equal to the baseline REFSENS, since there is no duplexer for TDD bands, it demonstrates that the baseline REFSENs didn’t consider any impact from Tx noise due to the limitation of duplexer isolation for FDD bands.

Therefore, the REFSENs of FDD bands operating in HD-FDD for 2Tx should not be lower than the baseline REFSENs. HD-FDD 2 RX REFSENS of n254 can adopt 0.5 dB tighten as n256 operating in HD-FDD for 2Tx, and HD-FDD 2 RX REFSENS of n255 don’t adopt any tighten, since its FD-FDD 2 RX REFSENS are the baseline values.

HD-FDD 2 RX REFSENS for n254, n255, n256 can adopt the same REFSENS values with n256 operating in HD-FDD for 2Tx as below Table 2-3.

Table 2-3 HD-FDD REFSENS for 2 Rx

| Operating Band | SCS (kHz) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |
| --- | --- | --- | --- | --- | --- |
| n256 n255 | 15 | -100.0 | -96.8 | -95.0 | -93.8 |
|  | 30 |  | -97.1 | -95.1 | -94.0 |
|  | 60 |  | -97.5 | -95.4 | -94.2 |
| n254 | 15 | -100.0 | -96.8 | -95.0 |  |
|  | 30 |  | -97.1 | -95.1 |  |
|  | 60 |  | -97.5 | -95.4 |  |

Proposal 2: The REFSENS of FDD bands operating in HD-FDD for 2Tx should not be lower than the baseline REFSENS calculated by the equation.

Proposal 3: HD-FDD 2 RX REFSENS of n254 adopt 0.5 dB tighten as n256 operating in HD-FDD for 2Tx.

Proposal 4: HD-FDD 2 RX REFSENS of n255 adopt 0 dB tighten compared to FD-FDD 2 RX REFSENS of n255.

- Conclusion

This paper discussed the HD-FDD REFSENS for 2 Rx and proposed:

Proposal 1: The UE capability of rel-18 power boost for NR TN is applicable for NR NTN including (e)Redcap, no further work is needed.

Proposal 2: The REFSENS of FDD bands operating in HD-FDD for 2Tx should not be lower than the baseline REFSENS calculated by the equation.

Proposal 3: HD-FDD 2 RX REFSENS of n254 adopt 0.5 dB tighten as n256 operating in HD-FDD for 2Tx.

Proposal 4: HD-FDD 2 RX REFSENS of n255 adopt 0 dB tighten compared to FD-FDD 2 RX REFSENS of n255.

- Reference

[1] R4-2419897, Way Forward for [113][310] NR_NTN_Ph3_UE_RF, Qualcomm Inc., RAN4#113 2024-11