3GPP TSG-RAN WG4 Meeting #114                                   R4-2501479

Athens, Greece, 17th – 21st February, 2025

Source: 	MediaTek Inc.

Title: 	Discussion on UE RF requirements and system parameters for 7MHz CBW

Agenda Item:	7.9.2

Document for:	Approval

- Introduction

In RAN#106, a new WI [1] was agreed upon to introduce a new channel bandwidth of 7 MHz in NR, with the RAN4-related objectives shown as follows. In this paper, we will provide our views on spectrum utilization and UE RF requirements for 7 MHz NR FR1 CBW.

- Specify the generic core requirements to support 7 MHz NR FR1 channel bandwidth - Introduce the channel bandwidth 7MHz - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements for UL and DL - Specify the band specific core requirements to introduce 7 MHz for NR band n5, n26. - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed: - MPR (relative bandwidth criteria) - Additional Maximum Power Reduction (A-MPR) - Enable optional UE support of 7 MHz channel bandwidth from Rel-15 onwards in a release independent manner.

- Discussion

2.1 Spectrum utilization

When introducing new CBWs, it is essential to first define the spectrum utilization. This topic has been extensively discussed in RAN4. Initially, during the early stages of NR, the spectrum utilization for new CBWs was determined through simulations to meet EVM and SEM requirements. Subsequently, mathematical methods were employed to define the spectrum utilization for new CBWs.

Table 1 summarizes the spectrum utilization and guard bands with different CBWs for FR1 with 15 kHz SCS. As shown in the table, the spectrum utilization percentage increases monotonically with the CBW. Consequently, the spectrum utilization percentage for 7 MHz should fall between that of 5 MHz and 10 MHz. Based on this and our evaluation, we propose that the NRB for 7 MHz with 15 kHz SCS be set at 36 RBs, which corresponds to 92.57% spectrum utilization.

Table 1 Spectrum utilization for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 7 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 36 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| SU(%) | 90 | 90 | 92.6 | 93.6 | 94.80 | 95.40 | 95.76 | 96.00 | 96.69 | 97.20 | 96.80 | 97.20 | 94.80 | 95.40 | 95.76 | 96.00 | 96.69 |
| Guard band (kHz) | 142.5 | 242.5 | 252.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 | 712.5 | 692.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 |
| 30 | N/A | 11 | N/A | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Proposal 1: RAN4 to consider NRB =36 with SCS=15kHz for 7MHz channel bandwidth.

2.2 UE RF requirements

Table 2 reviews each UE Tx and Rx RF requirements individually to determine which ones need modification with the introduction of the 7 MHz NR FR1 CBW.

Table 2 Specification impact for 7 MHz NR FR1

| NR UE Tx/Rx requirement | NR 7 MHz |
| --- | --- |
| 6.2 Transmitter power |  |
| 6.2.1 UE maximum output power | No impact expected. |
| 6.2.2 MPR | No impact expected. |
| 6.2.3 A-MPR | Has specification impact to some bands. |
| 6.2.4 Configured transmitted power | No impact expected. |
| 6.3 Output power dynamics |  |
| 6.3.1 Minimum output power | 7 MHz shall be added in Table 6.3.1-1, same column as for 5 MHz CBW. |
| 6.3.2 Transmit OFF power | 7 MHz shall be added to Table 6.3.2-1, same column as for 5 MHz CBW. |
| 6.3.3 Transmit ON/OFF time mask | No impact expected. |
| 6.3.4 Power control | No impact expected. |
| 6.4 Transmit signal quality |  |
| 6.4.1 Frequency error | No impact expected. |
| 6.4.2 Transmit modulation quality | No impact expected. |
| 6.5 Output RF spectrum emissions |  |
| 6.5.1 Occupied bandwidth | 7 MHz shall be added to Table 6.5.1-1 |
| 6.5.2.2 Spectrum emission mask | Dedicated general SEM for 7 MHz needed. |
| 6.5.2.3 Additional Spectrum emission mask | No impact expected.Not needed for bands n5 and n26 |
| 6.5.2.4.1 NR ACLR | 7 MHz shall be added to Table 6.5.2.4.1-1, same column as for 5 MHz CBW. |
| 6.5.2.4.2 UTRA ACLR | No impact expected. |
| 6.5.3.1 General spurious emissions | 7 MHz shall be added to Table 6.5.3.1-1, same column as for 5 MHz CBW. |
| 6.5.3.2 Spurious UEtoUE co-ex | No impact expected. |
| 6.5.3.3 Additional spurious emissions | 7 MHz need to be listed |
| 6.5.4 Transmit intermodulation | No impact expected. |
| 7.3.2 Reference sensitivity | Add REFSENS requirement for 7MHz channel bandwidth. |
| 7.4 Max input level | 7 MHz shall be added to Table 7.4-1, same column as for 5 MHz CBW. |
| 7.5 Adjacent channel selectivity | 7 MHz shall be added, same column as for 5 MHz CBW. |
| 7.6 Blocking characteristics |  |
| 7.6.2 In-band blocking | 7 MHz shall be added to Table 7.6.2-1, same column as for 5 MHz CBW. |
| 7.6.3 Out of band blocking | 7 MHz shall be added to Table 7.6.3-1 |
| 7.6.4 Narrow band blocking | 7 MHz shall be added to Table 7.6.4-1 |
| 7.7 Spurious response | 7 MHz shall be added to Table 7.7-1, same column as for 5 MHz CBW. |
| 7.8 Intermodulation characteristics | 7 MHz shall be added to Table 7.8.2-1 |

Proposal 2: Table 2 is used for UE RF requirements to support 7MHz NR FR1 CBW.

- 3	Conclusion

In this contribution, we provide our views on spectrum utilization and UE RF requirements for 7 MHz NR FR1 CBW. The proposals are summarized as follows:

Proposal 1: RAN4 to consider NRB =36 with SCS=15kHz for 7MHz channel bandwidth.

Proposal 2: Table 2 is used for UE RF requirements to support 7MHz NR FR1 CBW.

- 4	References

- RP-243302, “7 MHz Channel Bandwidth for n26 and n5”, China Telecom, T-Mobile USA.