3GPP TSG-RAN WG4 Meeting #113	R4-2501485

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Maintenance on soft satellite switching and LS reply to R2-2410978

Agenda item:	5.21.3

Document for:	Discussion

Introduction

This document aims at providing a reply to the LS sent by RAN2 (R2-2410978) on the Soft Satellite switch with re-sync. The soft satellite switch with re-synchronization was introduced in Rel-18 as part of the NR_NTN_enh work item, and aims at providing early measurements and minimized interruption time towards the target satellite before the synchronization with same PCI is concluded.

Discussion

LS Reply

In their LS to RAN4, RAN2 has discussed a solution to introduce a temporarily shifted SSB pattern during the interval between t-serviceStart (when the target satellite initiates transmission) and t-service (when the source satellite stops transmitting), which is a modification of the previous mechanism of this feature.

More specifically, RAN2 has provided the input that can be seen below (note: the highlights are our own, and are there to facilitate the discussion in this document):

For soft satellite switching with re-synchronization, to ensure non-colliding SSB with same PCI at the UE side, RAN2 has discussed one possible solution to introduce a temporarily shifted SSB pattern during [t-ServiceStart, t-Service]. To be specific, the target satellite transmits SSB during [t-ServiceStart, t-Service] which is not overlapping with SSB from the source satellite. At t-Service, the new serving satellite SSB pattern (e.g., SSB offset) is restored to the same as the previous source satellite. RAN2 wonders whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

First of all, it is important to comment the part in yellow, where RAN2 indicates that the target satellite transmits SSB during the “soft transition window” which is non overlapping with the SSB from the source satellite.

As the feature relates to a change in the transmitting satellite while the PCI is maintained, the fact that source and target satellites “transmit non-overlapping SSBs” is not very significant from the receiver point of view.  Both signals have to go through different propagation paths, which will cause them both to suffer from different propagation delays and doppler.  This effect is already considered in RAN4 specifications, which ensures the requirements are only applicable provided that both SSBs can be “separated” at UE side (see highlighted part in yellow below):

TS 38.133, Clause 6.1C.3.2.3 During the time period from t-seviceStart to t-service, scheduling restriction as defined in clause 9.2C.5.3 is allowed, with the exception that the locations of SSB symbols of target satellite where scheduling restriction applies are determined by the periodicity and location of SSB of the source satellite, the ssb-TimeOffset and the difference between propagation delay of the serving satellite and the target satellite counted from the [SSB-TimeOffset reference point as defined in TS 38.331 [2] to UE. UE is allowed to skip measurements for other cells and satellites than the target satellite and source satellite from t-seviceStart to the satellite switch completion. The requirement in this clause applies and UE is required to keep the connection (DL and UL) with the source NGSO satellite, under the following conditions: - SSBs from the two satellites are spaced apart from each other at least by 1 OFDM symbol in the time domain at UE Rx side.

Therefore,

From the requirements point of view, the signals of source and target satellite shall be separated at the receiving end, regardless of what happens at transmitters end.

The offset introduced in the time between source and target satellite is only necessary if it ensures complete separation at the receiving end.

For the part marked in blue in the text of the LS from RAN2, which corresponds to the changes RAN2 wants to introduce, there is the intent to “move” the SSB of the target satellite to restore it to “the same as previous source satellite”.

As we’ve discussed above, from the UE point of view, the SSB time will never be the same when received from the source and target satellites, due to the propagation delay. However, the RAN2 LS mention the change on “SSB Pattern”, which indicates an understanding of the ssb-TimeOffset that might be different from the one previously held by RAN4.

It is important to note that, according to TS 38.213, clause 4.1,  the SSBs of a given cell may only be transmitted in certain positions of the frame. In more details for FR1, which is the scope of the present feature, there are the following possibilities:

Table 1. Allowed OFDM Symbol positions for the SSB location in a frame

| Case | SCS [kHz] | Frequency Range | Symbols |
| --- | --- | --- | --- |
| A | 15 | Below 3 GHz | 2, 8, 16, 22 |
| A | 15 | Above 3 GHz | 2, 8, 16, 22, 30, 36, 44, 50 |
| B | 30 | Below 3 GHz | 4, 8, 16, 20 |
| B | 30 | Above 3 GHz | 4, 8, 16, 20, 32, 36, 44, 48 |
| C | 30 | Below 3 GHz | 2, 8, 16, 22 |
| C | 30 | Above 3 GHz | 2, 8, 16, 22, 30, 36, 44, 50 |

Therefore, the ssb-TimeOffset feature cannot be used to change the pattern (i.e. the OFDM symbols) in which the SSB is transmitted, as they are pre-coded in specification. Pick, for example, case A, where a SSB transmission may be performed in OFDM symbols 2 and 16. To “move” the SSB transmitted in symbol 2 (SSB index 0), it is not just “offsetting” it by 2 subframes, as it would cause the SSB index to change. The best way to achieve that would be by modifying the ssb-PositionsInBurst in the target cell compared to the source cell.  The SSB index are related to the DMRS signal generation and the calculation of the half frame. This operation cannot be done without clearly going against the specification on TS 38.213.

The parameter “ssb-TimeOffset” cannot offset the location of the SSB in the frame, according to TS 38.213.

In our interpretation, the ssb-TimeOffset can only be accounted in time, as in, the offset in time between the transmission point of SFN0 for source and target cell. As was indicated in Option 1 of the RAN2 discussion document in  [2].

For the last part of the LS analysis, the following action was asked by RAN2:

ACTION: RAN2 kindly asks RAN4 and RAN1 whether the pre-acquired DL synchronization started between [t-ServiceStart, t-Service] with target satellite can be assumed as maintained if the SSB pattern (e.g., SSB offset) is changed (determined by ssb-TimeOffset) at t-Service.

It is our interpretation that the SSB pattern cannot be changed, for the reasons explained above. Cell defining SSB cannot be changed as this alters the identification of the subframe count by the UE. This action is only possible if the “shifted pattern” SSB is a  non-cell Defining SSB.

Based on the observations above, we propose the following:

Reply to RAN2 that UEs are not capable of handling change on SSB patterns, as they are pre-coded in specification.

Indicate to RAN2 that “moving” SSB patterns would require non-cell defining SSBs to be transmitted between t-serviceStart and t-service

Approve the LS reply text provided in the annex A.

High mobility scenarios for timing advance

In RAN4 specifications, we have agreed that UEs are allowed to perform autonomous adjustment of timing advance, in order to keep fine tracking of the uplink transmission timing. However, as discussed during Release 17, it was important to consider that there is some uncertainty related to the physical information estimated by the UE, in special the UE position estimated by GNSS, which might fluctuate over time due to variations on UE actual position or in the signals used to estimate the GNSS position. Therefore, in order to prevent a multitude of “back and forth” adjustments, we have agreed in RAN4 to introduce some requirements over the UE autonomous adjustments:

7.1C.2.1 Gradual timing adjustment When the transmission timing error between the UE and the reference timing exceeds Te_NTN then the UE shall adjust the timing such that timing error is to within Te_NTN. The reference timing shall be NTA+NTA-offset+NTA,adjcommon+NTA,adjUE×Tc before the downlink timing of the reference cell. All adjustments made to the UE uplink timing shall follow these rules: 1) The maximum amount of the magnitude of the timing change, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon between the previous transmission and the current transmission, in one adjustment shall be Tq_NTN. 2) The minimum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last one second, shall be Tp_NTN per second. 3) The maximum aggregate adjustment rate, apart from a change of NTA,adjUE due to satellite position update and NTA,adjcommon during the last 200ms, shall be Tq_NTN per 200 ms. Where, the maximum autonomous time adjustment step Tq_NTN and the aggregate adjustment rate Tp_NTN are specified in Table 7.1C.2.1-1. Table 7.1C.2.1-1: Tq_NTN Maximum Autonomous Time Adjustment Step and Tp_NTN Minimum Aggregate Adjustment rate Frequency Range; SCS of uplink signals (kHz); Tq_NTN; Tp_NTN. FR1-NTN; 15; 5.5*64*Tc; 5.5*64*Tc. 30; 5.5*64*Tc; 5.5*64*Tc. 60; N/A; N/A. FR2-NTN; 60; 2.5*64*Tc; 2.5*64*Tc. 120; 2.5*64*Tc; 2.5*64*Tc. NOTE: Tc is the basic timing unit defined in TS 38.211 [6].

As indicated by our highlights, RAN4 specs indicate that only changes on the UE specific part due to satellite position update are not counted as part of the gradual adjustment requirements, which means that the changes in UE position are bound to the requirements above.

Changes in timing caused by updates on UE position are limited by maximum and minimum adjustment rate requirements.

Another important aspect to consider is that airborne devices have been considered an example of application for NTN since Rel-17. For example, in Release 18, and operation in FR2-NTN, a special type of UE was introduced (VSAT) but it was accounted and presented in the WID that two types of devices should be considered (and one of the example cases in the WID is the airborne device):

Fixed VSAT: VSAT used in FSS system at given position; the given position may be a specified fixed point or any fixed point within specified areas. NOTE: Mobile VSAT is excluded from this definition Mobile VSAT: VSAT on moving platform, and which can be further declined in three types: airborne, maritime or land based. NOTE: Mobile VSAT can be also referred to as ESIM or ESOMP

For FR1-NTN, the values currently defined for Tp and Tq are equal to 0,18 us (microseconds), whereas for FR2-NTN the values currently defined for Tp and Tq are equal to 0,08 us (microseconds). These values shall account for twice the changes in the distance between the UE and the base station.

Assuming a speed of light approximated by 300,000 km/s, such correction factors would correspond to distances of respectively 26,8 and 12,2 meters. When applying these values in point 3 above, that would force the variation in distance to be smaller than 134 m in FR1-NTN or 60 m in FR2-NTN every second. This translates into a change in distance caused by a relative speed of 483 km/h (FR1-NTN) or 219 km/h (FR2-NTN). Such maximum speeds of adjustments are not enough to cover for airplane speeds (up to 900 km/h and beyond).

Based on the analysis above, we propose:

For Mobile VSAT, if the mobile VSAT is travelling above 200 km/h, Tp and Tq shall be adjusted.

Conclusion

The paper has analyzed the need for maintenance on mobility requirements for VSAT UE and provided a detailed analysis on the LS submitted by RAN2 in R2-2410978

The following Observations and Proposals were made:

Observation 1: From the requirements point of view, the signals of source and target satellite shall be separated at the receiving end, regardless of what happens at transmitters end.

Observation 2: The offset introduced in the time between source and target satellite is only necessary if it ensures complete separation at the receiving end.

Observation 3: The parameter “ssb-TimeOffset” cannot offset the location of the SSB in the frame, according to TS 38.213.

Proposal 1: Reply to RAN2 that UEs are not capable of handling change on SSB patterns, as they are pre-coded in specification.

Proposal 2: Indicate to RAN2 that “moving” SSB patterns would require non-cell defining SSBs to be transmitted between t-serviceStart and t-service

Proposal 3: Approve the LS reply text provided in the annex A.

Observation 4: Changes in timing caused by updates on UE position are limited by maximum and minimum adjustment rate requirements.

Proposal 4: For Mobile VSAT, if the mobile VSAT is travelling above 200 km/h, Tp and Tq shall be adjusted.

- References

- R2-2410978, “LS on soft satellite switch with re-sync”, Huawei, RAN2 #128, Orlando, US, Nov. 2024

- R2-2410971, Report of [AT128][301][R18 NR NTN] SMTC issues (Huawei), Huawei, HiSilicon, RAN2 #128, Orlando, US, Nov. 2024

- Annex

3GPP TSG-RAN WG4 Meeting #113	R4-25xxxxx

Athens, Greece, Feb 17th – Feb 21st, 2025

Title:	LS reply on Soft Satellite Switch with re-sync

Response to:	R2-2410978

Release:	Rel-18

Work Item:	NR_NTN_enh-Core

Source:	RAN4

To:	RAN2

Cc:	RAN1

Contact person:	XXX@XXX.com

Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# 1	Overall description

RAN4 has received the LS R2-2410978 and appreciates the sharing of raised concerns by RAN2. In this LS RAN2 has asked RAN4 to evaluate whether the DL synchronization acquired during the Soft Satellite switch with re-sync could be maintained at the target satellite, if the SSB pattern was modified after t-Service.

RAN4 would like to indicate that the previous understanding in the group of the applicability of ssb-TimeOffset was the same as indicated in Option 1 in R2-2410971.

Upon debating the topic, RAN4 has identified that the SSB patterns are pre-coded in specification in what regards the OFDM symbols in which they are transmitted, and therefore they cannot be changed. The SSB indexes are used to obtain the DL synchronization and to obtain the right subframe count and altering their pattern would have consequences on this procedure. Only non-cell defining SSBs can have their patterns (relative positions in the frame) changed.

# 2	Actions

To RAN1 and RAN2

ACTION: 	RAN4 kindly asks RAN2 to take into consideration the response above.

# 3	Dates of next TSG RAN WG 1 meetings

TSG RAN WG4 Meeting #114bis 	07 – 11 April 2025 Wuhan, China

TSG RAN WG4 Meeting #115		19 – 23 May 2025 Malta