3GPP TSG-RAN WG4 Meeting #113	R4-2501486

Athens, Greece, Feb 17th – Feb 21st, 2025

Source:	Nokia

Title:	Discussing requirements for RedCap support in NR NTN

Agenda item:	7.29.4.1

Document for:	Discussion

Introduction

In [1], RAN plenary has approved a new WI for NTN, which will progress the effort of specification for NTN systems in 3GPP technologies in Rel-19 (NR_NTN_PH3). The most recent version of the WID can be found in [2]. It includes a handful of objectives such as:

- Downlink Coverage Enhancements

- Uplink Capacity/Coverage Enhancements

- Signaling of the intended service area of a broadcast service (MBS broadcast)

- Support of regenerative payload for NTN systems

- Support of Rel-17 and Rel-18 RedCap and (e)RedCap UEs in NTN systems

This document is related to the 5th topic on the list above: RedCap deployments over NTN. The discussions for this objective were initiated in the previous meeting RAN4 #112. And as the outcome of the discussion a way-forward (WF) document was approved which contains a set of agreements and other way-forward (for further discussion) guidance for currently identified open issues [3].  The discussions further ensued in RAN4 #113, which culminated in the WF found in [4].

In this document we provide our views, related to the RAN4 impact of the introduction of RedCap over NTN considering the scope of this WI and/or the open issues already identified in the WF.

Discussion

In the present document we will discuss some of the open issues presented in the previous WF [4], as listed below:

- Issue 1-1-2 The Requirements to be further determined

- Issue 1-2-4: Multiple SMTCs

- Issue 1-2-8: Other features

- Issue 2-4-1 Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

- Issue 3-2-1: The impact on NR Handover due to HD-FDD

- Issue 4: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

Requirements to be further determined

RAN4 has discussed SDT and MDT requirements for RedCap UEs over NTN, and several different views were captured in the WF, as provided in the box below:

Issue 1-1-2: The Requirements to be further determined <Way Forward>: - For SDT requirements in RRC_INACTIVE state, - Proposal 1 (CATT, QC, OPPO, HW): RAN4 to remove the RSRP-based TA validation in CG-SDT RRM requirements. - Proposal 1a (CATT): TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - Proposal 1b (LG): TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission. - Proposal 1c (CMCC): For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN. - Proposal 1d (vivo): For SDT requirement in RRC_INACTIVE state for (e)RedCap NTN, RAN4 to follow the same requirements on TA validation for the PUR requirements for IoT NTN (when stable) - Proposal 2 (ZTE): CG-SDT shall be used for RedCap UEs in NTN whatever it’s the GSO or NGSO for 1Rx and 2Rx RedCap UEs, and the requirements can be: Measurement; FR1. RSRP1; (T1 – min(800ms, M1*TDRX)) ≤ T1’ ≤ (T1 + min(800ms, M1*TDRX)). RSRP2; (T2 – min(800ms, M1*TDRX)) ≤ T2’ ≤ T2. Note: The UE is not required to meet the requirements for 2.56s DRX cycle length for earth-moving LEO deployment.. - For MDT requirements in RRC_IDLE and RRC_INACTIVE state - Proposal 1 (CATT, Samsung, QC, OPPO, Ericsson, vivo, HW, MTK): - Do not specify Minimization of Drive Tests (MDT) requirements in RRC_IDLE state and RRC_INACTIVE state for (e)RedCap UE with FR1-NTN as the same situation for (e)RedCap UE in TN system. - Proposal 1a (CATT): Deprioritize to define. - Proposal 2 (CMCC): - Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for RedCap over NTN, if the related features are supported in RAN1/2 spec. - Proposal 3 (ZTE): - RAN4 shall consider NTN cell size and RedCap UE capability and then decide whether to define relevant requirements.

For SDT, most companies seem to agree that RSRP-TA validation is no longer valid for NR over NTN – due to the limited near-far effect in NTN and the instability of the TA and its association with a validity timer and ephemeris - and it shall be removed from the feature mechanism. However, for IoT NTN this decision was taken by RAN2 and it was later validated by RAN4. As this has impact on the description of RAN2 features, it cannot be solely decided by RAN4.

RAN4 to confirm with RAN2 whether the RSRP-based TA validation in CG-SDT shall be removed from RRM requirements.

Regarding whether to adopt similar validation criteria as currently being adopted by RAN2 for PUR in IoT NTN, this decision is out of scope of RAN4 as this mechanism needs to be confirmed by other RAN groups, and captured in their respective specifications.

Confirm with RAN2 whether similar validation criteria as used for PUR in IoT NTN can be used for CG-SDT in NTN.

Another feature being discussed was the minimization of drive tests (MDT) and whether it should be specified for RedCap over NTN or not. Given the number of open issues, the size of the work item and the total workload, this feature does not seem to be a priority for discussion, therefore we propose:

Do not specify MDT for RedCap over NTN

Other features

In the previous WF, a series of requirements were left for further discussion on whether they could be optionally supported by RedCap UEs. Those requirements are described in Issue 1-2-8 of the WF and provided in two sets:

Issue 1-2-8: Other features <Way Forward>: - Proposal 1 (CMCC): For the optional NTN features which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether RAN4 need to define requirements should be further discussed. - Set 1: propose to be optionally supported by Redcap over NTN - SMTC adjustment in idle/inactive - Location-based measurement report trigger - Enhanced RRM requirements for measurements in IDLE and INACTIVE modes - The number of target LEO satellites the UE can monitor per carrier including serving satellite - Further discuss the max number - Set 2: FFS - Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell - Parallel measurements on multiple NGSO satellites within a SMTC

We will first discuss the features provided in Set 1.

- SMTC adjustment in idle/inactive: We see this feature as a paramount feature for the correct functioning of NTN measurements. In connected mode the NW may recur to TA reports to adjust the measurement objects configuration for helping the UEs to track the neighbor cells belonging to different satellites. In Idle mode, this is not possible. If this feature is not present, the UE will not be capable to adjust the SMTC window to measure a neighbor satellite as the differential propagation delay changes over time compared to the serving satellite. This feature, therefore, has to be made mandatory.

SMTC adjustment in idle/inactive is mandatory in NTN and shall be kept mandatory for RedCap.

- Location-based measurement report trigger: It is an optional feature for NR over NTN, and shall be kept optional for RedCap UEs, as it might facilitate the mobility procedures when the near-far effect is not very pronounced, which is likely to be the case in NTN.

- Enhanced RRM requirements for measurements in Idle and Inactive: Likewise, this is also an optional feature in NR over NTN, and shall be kept optional for RedCap devices. Although the feature might produce enhanced results in earth-moving cell scenarios, we could compromise in not defining RRM requirements for this feature related to redcap devices.

Location-based measurement trigger is kept optional in RedCap over NTN, and RAN4 will define requirements for this feature.

Enhanced RRM requirements for measurements in IDLE and INACTIVE modes is kept optional in Redcap over NTN.

The discussion of set 2 and regarding the number of satellites to be measured by UE is provided in this document in sections 2.3 and 2.4

Multiple SMTCs

RAN4 has discussed whether to adopt the current set of capabilities existent for NR over NTN to RedCap UEs over NTN. During this discussion a problem has been raised regarding the adoption of multiple configured SMTCs per RedCap UE.  Companies have agreed that the RAN4 will define RRM requirements for multiple SMTC-s for a single frequency carrier for RedCap UE, but no consensus was reached regarding whether a RedCap UE is capable of supporting the feature of parallel measurements on multiple NGSO satellites within a SMTC:

Online agreement <Agreements>: - RAN4 to define RRM requirements for multiple SMTC-s for a single frequency carrier for (e)RedCap UEs. - For whether to support parallel measurements on multiple NGSO satellites within a SMTC (Feature 25-5) for (e)RedCap UEs? - Option 1 (MTK, Xiaomi, Samsung, ZTE, Ericsson, LG, QC): Not support - Option 2 (HW, Nokia): FFS - Further discuss the supported number of SMTCs, the number of cells and SSB, the number of supported satellites, the proximity distance between two SMTCs. - Further discuss the specific measurement requirements, i.e, whether and how to extend measurement period. - Further discuss other R17 NTN UE capabilities (i.e, Feature 25-2/25-3) related to 'in parallel'.

In general, most capabilities available for NR UEs are also available for RedCap UEs, unless stated otherwise. The limitation of RedCap capabilities sometimes is related to limited technical feasibility due to the reduced capabilities in such devices.

Although RedCap UEs are expected to possess limited capabilities compared to a NR UE, some RedCap UEs are provided with 2RX antennas, and are capable of implementing more complex receivers. The capability for parallel measurements on multiple NGSO within one SMTC is optional and is not expected to be implemented by every UE. But there is no technical reason to prevent this capability of being implemented by a terminal. Therefore, we propose:

Support for parallel measurements on multiple NGSO satellites can be supported by RedCap UEs.

Another related discussion regards the number of satellites to be measured per RedCap device. NTN UEs might be capable to measure at least two satellite per available SMTC in a single frequency, including the serving satellite. Therefore, the total number of satellites to be measured must be at least equal to N+1, where N is the number of SMTCs configured for measurement in this frequency layer.

RedCap UEs shall support at least one NGSO satellite per SMTC configured in a frequency layer in addition to the serving satellite.

The total number of supported satellites for measurement in a frequency layer  is equal to N+1 where N is the number of configured SMTCs, if the serving satellite is to be measured in this frequency layer and N if the serving satellite is not to be measured in this frequency layer.

The impact on NR Handover due to HD-FDD

When analysing the impact of HD-FDD on mobility requirements (HO, CHO, Satellite Switch with re-sync), several companies have proposed that:

“For RedCap NTN UE with HD-FDD, the requirements are met provided that the SSB is available at the UE once every SMTC period during Tsearch”

This requirement is derived from early HD-FDD applicability in terrestrial networks. But there is a technical concern in adapting this requirement directly to NTN scenarios.

Differently from TN, in NTN scenarios, if UEs support more than one satellite per SMTC, and therefore, if the UE cannot support simultaneously measurements from two different satellites (feature 25-5) within the same SMTC, then the SMTC is to be shared by multiple satellites (K_multi factor). This indicates that there might be further scheduling restriction when the UE is measuring neighbor satellites. Therefore, depending on how “availability” is defined, it cannot be guaranteed that one SSB will be available at the UE once every SMTC period.

For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side.

Others

The WF is currently debating the best way to incorporate RedCap over NTN specifications within the existing TS 38.133 document. Two options are being considered:

- New Suffix: Creating a new suffix (e.g., "E") for RedCap over NTN specifications.

- New Subclauses: Adding new subclauses to the existing NTN clauses within TS 38.133.

To minimize the specification effort and maintain a concise document, we propose inserting RedCap subclauses as subsections within the existing NTN section (suffix C). This approach ensures a coherent and complete specification while keeping the document size manageable.

Adopt RedCap subclauses under NTN sections (under Suffix C).

Another point of discussion in the WF is the applicability of RedCap requirements under NTN needs further discussion. For a UE to support RedCap over NTN, it must support both RedCap and NTN features.

- Option A: Declare support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

- Option B: Declare explicitly – via a new capability – the support to RedCap over NTN.

Both options have their pros and cons (flexibility versus capability signalling overhead). The first option allows for less signalling overhead and less capabilities declaration. However, it is less flexible as it does not allow a UE that is NTN capable to be TN Redcap capable only. It also ensues from option B that if a UE supports RedCap over NTN, it must support RedCap over TN, which might not be always the case.

In order to leverage the most flexibility on the capability indication, we propose to adopt option B.

Introduce new capability to declare support over RedCap over NTN.

Conclusion

This paper has analysed some of the open issues provided in the WF for this WI [4], and the following set of observation and proposals are made:

Proposal 1: RAN4 to confirm with RAN2 whether the RSRP-based TA validation in CG-SDT shall be removed from RRM requirements.

Proposal 2: Confirm with RAN2 whether similar validation criteria as used for PUR in IoT NTN can be used for CG-SDT in NTN.

Proposal 3: Do not specify MDT for RedCap over NTN

Proposal 4: SMTC adjustment in idle/inactive is mandatory in NTN and shall be kept mandatory for RedCap.

Proposal 5: Location-based measurement trigger is kept optional in RedCap over NTN, and RAN4 will define requirements for this feature.

Proposal 6: Enhanced RRM requirements for measurements in IDLE and INACTIVE modes is kept optional in Redcap over NTN.

Proposal 7: Support for parallel measurements on multiple NGSO satellites can be supported by RedCap UEs.

Proposal 8: RedCap UEs shall support at least one NGSO satellite per SMTC configured in a frequency layer in addition to the serving satellite.

Proposal 9: The total number of supported satellites for measurement in a frequency layer  is equal to N+1 where N is the number of configured SMTCs, if the serving satellite is to be measured in this frequency layer and N if the serving satellite is not to be measured in this frequency layer.

Proposal 10: For NTN UEs with HD-FDD, the requirements are met provided that the SSB of at least one of the satellites to be measured in this SMTC period is available at the UE side.

Proposal 11: Adopt RedCap subclauses under NTN sections (under Suffix C).

Proposal 12: Introduce new capability to declare support over RedCap over NTN.

- References

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2414037, "The WF on RRM requirements for Rel-19 NR NTN phase3", Moderator (CATT), RAN4 #112, Maastricht, Netherlands, Aug. 19-23, 2024.

- R4-2420104, “WF on RRM requirements for NR_NTN_Ph3_Part1”, Orlando , US, Nov 18 – 22, 2024