3GPP TSG-RAN WG4 Meeting #114	R4-2501498

Athens, Greece, 17 February – 21 February 2025

Agenda Item:	7.10.4

Source:	Ericsson

Title:	TE-emulated channel model for NTN UE tests

Document for:	Discussion

# 1	Introduction

RAN4#113 agreed with the way forward on the TE-emulated channel modeling for NGSO NTN testing [1]. In the WF, RAN4 first focus on the time varying Doppler shift and propagation delay models.

This contribution presents on the satellite propagator model and how to derive the time varying Doppler shift and propagation delay.

# 2	NGSO NTN test environment

Figure 1	Assumed test environment for NGSO NTN UE testing.

Figure 1 illustrates an overview of the NGSO NTN test environment. During the test, DUT (UE) estimates the Doppler shift and propagation delay based on the satellite ephemeris information and UE’s location information at a given time. The UE location information is pre-configured or provided via AT command, according to TS 38.508 (See C.1). The estimated Doppler shift and propagation delay are used to perform the pre-compensation before the baseband processing in the NTN UE.

As agreed in RAN4#113, the TE calculates the satellite position and velocity based on the satellite propagator model. To reduce the computational complexity, the TE may update the satellite position/velocity every Tsat (sec). However, since the TE needs to apply to the Doppler shift and propagation delay every slot to the SAN transmit signals before the fading simulator, we assume the TE updates the Doppler shift and propagation delay by interpolation every slot. We also assume the TE knows the configured UE location information, given by longitude, latitude, and altitude.

During the test, the TE need to signal the determined satellite position/velocity information to the DUT (UE) with SIB19 ephemerisInfo-r17 for NR or SIB31 ephemerisInfo-r17 for IoT-NTN periodically, every Tsib (sec), together with epochTime.

Figure 2 summarizes the update timing of the satellite position/velocity, ephemerisInfo, Doppler shift and propagation delay. We assume, Tsib=10.24 (sec), and Tsat < Tsib.

Figure 2	Update timing of satellite position/velocity, epehmerisInfo, Doppler shift, and propagation delay.

# 3	Evaluation of satellite position/velocity estimation models

## 3.1	Satellite position/velocity estimation based on the equation of motion

RAN4#113 agreed to use the Kepler’s equation to determine the satellite position/velocity at time t, however some companies concerned the computational complexity with the Kepler’s equation because it requires many trigonometric functions and iterative algorithm.

As an alternative method to determine the satellite position/velocity, it was proposed to use the equations of motion for a satellite [2], given by

r=-GMr3r=-μr3r

where μ=GM is the gravitational parameter for Earth, r=rxryrz is a satellite position vector, and  r=rxryrz is a second-order differential of r, that is, the acceleration vector. This is an ordinary differential equation and thus the satellite motion is determined by its initial position and velocity. Historically Johannes Kepler had stated three laws about planetary motion first, and then Issac Newton shows that these laws are derived from the equation of motion. Therefore, we can say that the use equation of motion does not contradict the agreement in [3] to use Keplerian propagator model.

Observation 1:	Kepler’s lows of planetary motion can be derived from the equation of motion.

The equation of motion is given in an inertial frame as follows:

rECI=vECI=-GMrECI3rECI=-μrECI3rECI vECI=rECI

Since the initial satellite position/velocity is given in ECEF format, we need to rewrite this equation in ECEF as follows:

rECEF=-μrECEF3rECEF+2ΩE×rECEF+ΩE×ΩE×rECEF

=-μrECEF3rECEF+2ΩE×vECEF+ΩE×ΩE×rECEF

where, ΩE=[00ωE], and × means outer product of vectors. From this equation, it is found that the acceleration of satellite at time t in ECEF,  rtECEF is the function of the satellite position and velocity at time t, that is, rtECEF and vtECEF.

The differential equations are expressed as follows:

rtECEF=frtECEF,vtECEF=-μrt3rt,xECEF+2ωEvt,yECEF+ωE2rt,xECEF-μrt3rt,yECEF-2ωEvt,xECEF+ωE2rt,yECEF-μrt3rt,zECEFT

Where rt=rt,xECEF2+rt,yECEF2+rt,zECEF2

We can derive satellite position/velocity vectors at time t in ECEF by solving these differential equations. As proposed by [2], these equations can be solved by the fourth-order Runge-Kutta method, as shown in A.1.2. The benefits of this method compared with the Kepler’s equation in A.1.1 are:

- this method does not need to use iterative algorithm, i.e., Newton’s method, for which it is easier to estimate the computation time in TE, and

- this method directly estimates the satellite position/velocity in Earth-fixed frame; on the other hand, Kepler’s equation needs to convert between Inertial frame (ECI) and Earth-fixed frame (ECEF).

## 3.2	Comparison of satellite position/velocity estimation methods

This section evaluates both the satellite position/velocity estimation methods discussed in 3.1. For evaluation, we have also compared the satellite position/velocity vectors with GMAT [4] using the ephemeris information specified in TS 38.521-5 Table 6.4.1_2.4.3-1 (LEO-600, maximum positive Doppler, See B.2) over 1,000 seconds.

Figure 3	Comparison of position vector among GMAT, Kepler’s equation, and equation of motion.

Figure 4	Comparison of velocity vector among GMAT, Kepler’s equation, and equation of motion.

Figure 3 and Figure 4 compare the position and velocity vectors, respectively, among GMAT, Kepler’s equation, and equation of motion. The left figures show each element of position/satellite vectors (x, y, z), and the right figures show the difference between Kepler’s equation and GMAT, and the difference between equation of motion and GMAT. Although these figures compare the vectors over 1,000 seconds, it is sufficient to compare up to ~500 seconds if we consider the elevation angles more than 10 degrees. It is observed from the figures that position/velocity vectors calculated by both Kepler’s equation and equation of motion have well aligned each other, and both results are also well aligned with GMAT.

Observation 2:	Satellite position/velocity estimation by Kepler’s equation in A.1.1 is well aligned with the GMAT results.

Observation 3:	Satellite position/velocity estimation by the equation of motion in A.1.2 is well aligned with the GMAT results.

Based on the comparison results, we propose to consider both the methods in RAN4, and it is up to TE vendors which method is used to implement the varying Doppler shift and propagation delay for NGSO tests.

Proposal 1:	Specify both the satellite position/velocity calculation methods in TS38.101-5/TS36.102; based on the Kepler’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test.

# 4	Other open issues

## 4.1	Initial ephemeris information

One of the remaining open issues is the initial satellite position/velocity.

- Option 1: Consider the NGSO (LEO-600) satellites (maximum positive Doppler) parameters specified in TS 38.521-5 Table 6.4.1_2.4.3-1 and the NGSO (LEO-1200) satellites (maximum positive Doppler) parameters specified in TS 38.508-1 Table 5.6.2.1-3 as a starting point to specify the TE-emulated Doppler shift and propagation delay model. - GMAT

Our preference is to reuse the ephemeris information specified by TS38.521-5, targeting the maximum positive Doppler tests. Figure 5 shows the Doppler shift and propagation delay using LEO-600 maximum positive Doppler test parameters, and Figure 6 shows the corresponding elevation angles.

Figure 5	Doppler shift and propagation delay for LEO-600 maximum positive Doppler scenario.

Figure 6	Elevation angle for LEO-600 maximum positive Doppler scenario.

Since this parameter set can cover the elevation angles above 10 degrees, RAN4 can derive the initial position/velocity vectors according to the scenario. For example, if we need the parameters staring from the elevation angles of 30 degrees, we can set the parameters as shown in Table 1.

Table 1	Example of initial ephemeris information stating from elevation angles of 30 degrees.

| Parameters | Unit | Values |
| --- | --- | --- |
| positionX-r17 | km | -3451.1273 |
| positionY-r17 | km | 5703.5635 |
| positionZ-r17 | km | 2071.0809 |
| velocityVX-r17 | km/s | 0.87360 |
| velocityVY-r17 | km/s | -2.06700 |
| velocityVZ-r17 | km/s | 7.21231 |

We think this option not only meets the purpose of the objective in the WI but avoid different parameter setting per TE vendor.

Proposal 2:	Reuse the ephemeris information specified by TS38.521-5 to specify the initial ephemeris information for time varying Doppler shift and propagation delay.

## 4.2	Doppler shift and propagation delay updates in TE

As it is shown in Figure 2, the TE updates Doppler shift and propagation delay every slot using interpolation, where the TE calculates the satellite position/velocity every Tsat seconds. RAN4 discussed the two interpolation methods: linear interpolation and cubic spline interpolation.

Figure 7 and Figure 8 compares the two interpolation methods for Doppler shift and propagation delay updates, respectively, with LEO-600 maximum positive Doppler scenario, where it is assumed Tsat=1.00 (sec). The right figures show the differences between two methods. It is observed the differences between two interpolation methods are very small, ~0.7Hz for Doppler shift, and ~0.035 us for propagation delay.

Figure 7	Doppler shift update in TE (LEO-600 maximum positive Doppler scenario).

Figure 8	Propagation delay update in TE (LEO-600 maximum positive Doppler scenario).

Although it is up to TE implementation how often it calculates the satellite position/velocity using the propagator algorithm discussed in 3, considering the SIB ephemeris information updates every 10.24 sec, RAN4 should recommend updating the satellite position/velocity at least every 1 second.

Proposal 3:	TE should update the satellite location/velocity at least every 1 second.

Proposal 4:	With the assumption the UE updates the satellite location/velocity at least every 1 second, the TE may update Doppler shift and propagation delay every slot by linear interpolation or cubic spline interpolation. It is up to TE implementation which interpolation method is used.

## 4.3	Impact on RAN4 demodulation requirements

- RAN4 NTN UE demodulation requirements specification need to configure the period of ephemeris information update. - RAN4 discuss further the existing NR NTN and IoT-NTN UE demodulation requirements can apply or it needs more margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

As illustrated in Figure 1, UE receives the ephemeris information every Tsib=10.24 seconds. Based on the satellite ephemeris information, the UE estimate the Doppler shift and propagation delay in between the ephemeris information updates.

Figure 9 and Figure 10 show the interpolated Doppler shift and propagation delays at UE, respectively, with LEO-600 maximum positive Doppler scenario, where it is assumed Tsib=10.00 (sec). We have evaluated three interpolation methods: linear (1st order) interpolation, quadratic (2nd order) interpolation, and cubic spline interpolation. The lower figures show the difference (mismatch) between the Doppler shift/propagation delay generated by TE and Doppler shift/propagation delay estimated by UE, where it is assumed TE generates the Doppler shift and propagation delay with the cubic spline interpolation with Tsat=1 (sec) (same as Figure 7 and Figure 8). The right lower figures emphases the difference among three interpolation methods.

Figure 9	Doppler shift estimation in UE (LEO-600 maximum positive Doppler scenario).

Figure 10		Propagation delay estimation in UE (LEO-600 maximum positive Doppler scenario).

It is observed from the figures that the errors with linear interpolation are ~70 Hz for Doppler shift and ~3.5 us for propagation delay. We think it is NOT negligible considering the Tx frequency error of 0.1ppm (=200Hz @ 2GHz) and the cyclic prefix of 4.7us for SCS=15kHz.

On the other hand, if we assume UE uses the quadratic interpolation or cubic spline interpolation, the mismatch from TE is very small, that is, ~2Hz/0.024us for the quadradic interpolation and <<1Hz/0.01us for the cubic spline interpolation methods.

Observation 4:	Mismatch between TE and UE regarding the Doppler shift and propagation delay is not negligible if it is assumed UE uses the linear interpolation method to estimate the Doppler shift and propagation delay. UE may not pass the existing NTN UE demodulation requirements.

Observation 5:	Mismatch between TE and UE regarding the Doppler shift and propagation can be negligible if it is assumed UE uses the quadradic interpolation or cubic spline interpolation methods to estimate the Doppler shift and propagation delay. UE should be able to pass the existing NTN UE demodulation requirements.

Although WID [5] states that RAN4 will specify the PDSCH demodulation performance requirements, we think RAN4 can reuse the existing NTN PDSCH demodulation requirements even if we apply the time varying Doppler shift and propagation delay models.

NTN testing for NGSO […] Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements Specify the applicability of the tests under the TE-emulated channel model

Proposal 5:	RAN4 should assume UE uses the second-order or higher-order interpolation method to estimate the Doppler shift and propagation delay. With this assumption, RAN4 can reuse the existing NTN UE demodulation requirements specified in TS 38.101-5 and TS 36.102.

# 5	Summary

Observations:

Observation 1:	Kepler’s lows of planetary motion can be derived from the equation of motion.

Observation 2:	Satellite position/velocity estimation by Kepler’s equation in A.1.1 is well aligned with the GMAT results.

Observation 3:	Satellite position/velocity estimation by the equation of motion in A.1.2 is well aligned with the GMAT results.

Observation 4:	Mismatch between TE and UE regarding the Doppler shift and propagation delay is not negligible if it is assumed UE uses the linear interpolation method to estimate the Doppler shift and propagation delay. UE may not pass the existing NTN UE demodulation requirements.

Observation 5:	Mismatch between TE and UE regarding the Doppler shift and propagation can be negligible if it is assumed UE uses the quadradic interpolation or cubic spline interpolation methods to estimate the Doppler shift and propagation delay. UE should be able to pass the existing NTN UE demodulation requirements.

Proposals:

Proposal 1:	Specify both the satellite position/velocity calculation methods in TS38.101-5/TS36.102; based on the Kepler’s equation and equation of motion. It is up to TE implementation which method is used to derive the varying Doppler shift and propagation delay for NGSO test.

Proposal 2:	Reuse the ephemeris information specified by TS38.521-5 to specify the initial ephemeris information for time varying Doppler shift and propagation delay.

Proposal 3:	TE should update the satellite location/velocity at least every 1 second.

Proposal 4:	With the assumption the UE updates the satellite location/velocity at least every 1 second, the TE may update Doppler shift and propagation delay every slot by linear interpolation or cubic spline interpolation. It is up to TE implementation which interpolation method is used.

Proposal 5:	RAN4 should assume UE uses the second-order or higher-order interpolation method to estimate the Doppler shift and propagation delay. With this assumption, RAN4 can reuse the existing NTN UE demodulation requirements specified in TS 38.101-5 and TS 36.102.

# References

- R4-2419786, “Way Forward for [113][322] NTN_testing_NGSO_channel_model”, Samsung.

- R4-2417672, “Numerical solution of equations of satellite motion under the ECEF coordinate system”, Anritsu Corporation.

- R4-2416556, “Way Forward for [112bis][321] NTN_testing_NGSO_channel_model”, Samsung.

- General Mission Analysis Tool. https://sourceforge.net/projects/gmat/

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, RAN4 chair.

# Appendix

## A.1	Satellite position/velocity estimation

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| r0ECEF=r0,xECEFr0,yECEFr0,zECEF | km | Initial satellite position state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| v0ECEF=v0,xECEFv0,yECEFv0,zECEF | km/s | Initial satellite velocity state vector at time 0 in Earth-centered earth-fixed frame (ECEF) |
| t | second | Time to derive the satellite position and velocity |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vtECEF=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| μ | Gravitational parameter for Earth | 3.986004418 x 105 | km2/s2 |
| ωE | Earth angular speed | 7.2921151467 x 10-5 | rad/s |

### A.1.1	Keplerian model based estimation

#### Step 1	Derive the orbital parameters (a, e, i, Ω, ω, M0)

##### Step 1-0	Convert the initial position/velocity state vectors from ECEF format to ECI format

Note we assume the x-axis of ECI and ECEF the same for simplicity.

r0=r0ECEF=r0,xECEFr0,yECEFr0,zECEF

v0=v0,xv0,yv0,z=v0ECEF+00ωE×r0ECEF

=v0,xECEF-ωEr0,yECEFv0,yECEF+ωEr0,xECEFv0,zECEF

##### Step 1-1	Position magnitude (r), velocity magnitude (v), and orbital angular momentum (h)

r=r0=r0,x2+r0,y2+r0,z2

v=v0=v0,x2+v0,y2+v0,z2

vr=v0⋅r0r=r0,xv0,x+r0,yv0,y+r0,zv0,zr

h=hxhyhz=r×v=r0,yv0,z-r0,zv0,yr0,zv0,x-r0,xv0,zr0,xv0,y-r0,yv0,x

h=h=hx2+hy2+hz2

##### Step 1-2	Inclination (INC, i)

K=[001]

i=cos-1h⋅Kh=cos-1hzh

Note the range of INC is between 0 and π (radian).

##### Step 1-3	Right Ascension of the Ascending Node (RAN, Ω)

n=nxnynz=K×h=-hyhx0

n=n=nx2+ny2+nz2

Ω=cos-1nxn,ny≥02π-cos-1nxnny<0

Note the range of RAN is between 0 and 2π (radian).

Note Ω is called as ‘Longitude of ascending node’ in TS 38.331 ephemerisInfo-r17.

##### Step 1-4	Eccentricity (ECC, e), Semi-major axis (SMA, a), Period (P)

e=exeyezT=1μv2-μrr0-r0⋅v0v0=1μv2-μrr0,xr0,yr0,zT-r⋅vrv0,xv0,yv0,zT

e=e=ex2+ey2+ez2

a=h2μ(1-e2)

Period of the satellite around earth, P (sec), is given by:

P=2πa3μ

##### Step 1-5	Argument of Periapsis (AP, ω)

ω=cos-1e⋅n e⋅n=cos-1exnx+eyny+eznzenx2+ny2+nz2,ez≥02π-cos-1exnx+eyny+eznzenx2+ny2+nz2,ez<0

Note the range of AP is between 0 and 2π (radian).

##### Step 1-6	Mean Anomaly at time 0 (MA, M0)

True Anomaly at time 0 (ν0):

ν0=cos-1e⋅r0e⋅r=cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr≥02π-cos-1exr0,x+eyr0,y+ezr0,ze⋅rvr<0

Eccentric Anomaly at time 0 (E0):

E0=2tan-11-e1+etanv02

Mean Anomaly at time 0 (M0):

M0=E0-esinE0

Note the range of M0 is between 0 and 2π (radian).

#### Step 2	Determine the satellite position and velocity at time t (sec)

##### Step 2-1	Mean Anomaly at time t (Mt):

Mt=M0+2πtP=M0+tμa3

##### Step 2-2	Derive Eccentric Anomaly at time t (Et) by solving Kepler’s equation with Newton’s method

Build the Kepler’s equation between Mt and Et:

Mt=Et-esinEt

Step 2-2-1: Set Et(0)=Mt

Step 2-2-2: Calculate fEt(n)=Et(n)-esinEtn-Mt

Step 2-2-3: Calculate f'Et(n)=dfEt(n)dEt(n)=1-ecosEt(n)

Step 2-2-4: Update Et(n+1) from Et(n), fEt(n), and f'Et(n) , as follows:

Et(n+1)=Et(n)-fEt(n)f'Et(n)=Et(n)-Et(n)-esinEt(n)-Mt1-ecosEt(n)

Step 2-2-5: If Et(n+1)-Et(n)<ϵ, then set Et=Et(n+1) and go to Step 2-3. Otherwise, go to Step 2-2-2 by setting n := n+1.

Note ϵ=10-6, for example.

Note n=0, 1, 2, …

##### Step 2-3 Derive True Anomaly at time t (νt)

νt=2tan-11+e1-etanEt2

Note the range of νt is between 0 and 2π (radian).

##### Step 2-4	Convert the orbital elements to the state vector in ECI frame

Convert to the state vector in perifocal frame,

rt,xpqwrt,ypqwrt,zpqw=rtcosνtsinνt 0=h2μ11+ecosνtcosνtsinνt0=a1-e21+ecosνtcosνtsinνt0

vt,xpqwvt,ypqwvt,zpqw=μh-sinνt e+cosνt0=μa1-e2-sinνt(e+cosνt)0

Convert the state vector from perifocal frame to ECI.

rt,xECIrt,yECIrt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001rt,xpqwrt,ypqwrt,zpqw

vt,xECIvt,yECIvt,zECI=cosΩ-sinΩ0sinΩcos(Ω)00011000cosi-sini0sin(i)cos(i)cosω-sinω0sinωcosω0001vt,xpqwvt,ypqwvt,zpqw

Convert the state vector from ECI to ECEF.

rt,xECEFrt,yECEFrt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos(-ωEt)0001rt,xECIrt,yECIrt,zECI

vt,xECEFvt,yECEFvt,zECEF=cos-ωEt-sin-ωEt0sin-ωEtcos-ωEt0001vt,xECIvt,yECIvt,zECI-0-ωE0ωE00000rt,xECEFrt,yECEFrt,zECEF

Table 2 shows our calculation shows our calculation results for t=1 (sec) with the initial position/velocity vector using TS38.508 Table 5.6.2.1-2.

Table 2	Calculation results to derive satellite position/velocity vector at t=1 (sec) using TS38.508 Table 5.6.2.1-2.

| r0ECEF | [-3315.1053, 5396.4261, 2941.9442] |
| --- | --- |
| v0ECEF | [1.3182, -2.88984, 6.84654] |
| r0ECI | [-3315.1053, 5396.4261, 2941.9442] |
| v0ECI | [0.9246864, -3.13158129, 6.84654] |
| r1ECI | [-3314.17867368, 5393.29136118, 2948.78901695] |
| v1ECI | [0.92856604, -3.13789573, 6.84309258] |
| r1ECEF | [-3313.78537986, 5393.53302056, 2948.78901695] |
| v1ECEF | [1.32163985, -2.89631839, 6.84309258] |

### A.1.2	Equation of Motion based estimation

##### Step 1: Initialization

n:=0

rnΔt=rnΔt,xrnΔt,yrnΔt,z=r0,xECEFr0,yECEFr0,zECEF=r0ECEF

vnΔt=vnΔt,xvnΔt,yvnΔt,v=v0,xECEFv0,yECEFv0,zECEF=v0ECEF

Set time step size of position/velocity updates. Set Δt=1 (sec), for example, if the satellite position/velocity is updated every 1 second.

##### Step 2	Solve the equation of motion with the 4th-order Runge-Kutta method.

k1,r=k1,r,xk1,r,yk1,r,z=vnΔtΔt

k1,v=k1,v,xk1,v,yk1,v,z=frnΔt,vnΔtΔt

k2,r=k2,r,xk2,r,yk2,r,z=vnΔt+12k1,vΔt

k2,v=k2,v,xk2,v,yk2,v,z=frnΔt+12k1,r,vnΔt+12k1,vΔt

k3,r=k3,r,xk3,r,yk3,r,z=vnΔt+12k2,vΔt

k3,v=k3,v,xk3,v,yk3,v,z=frnΔt+12k2,r,vnΔt+12k2,vΔt

k4,r=k4,r,xk4,r,yk4,r,z=vnΔt+k3,vΔt

k4,v=k4,v,xk4,v,yk4,v,z=frnΔt+k3,r,vnΔt+k3,vΔt

rn+1Δt=rn+1Δt,xrn+1Δt,yrn+1Δt,z=rnΔt+16k1,r+2k2,r+2k3,r+k4,r

vn+1Δt=vn+1Δt,xvn+1Δt,yvn+1Δt,z=vnΔt+16k1,v+2k2,v+2k3,v+k4,v

Where

frt,vt=frt,x,rt,y,rt,z,vt,x,vt,y,vt,z=-μrt3rt,x+2ωEvt,y+ωE2rt,x-μrt3rt,y-2ωEvt,x+ωE2rt,y-μrt3rt,zT

rt=|rt|=rt,x2+rt,y2+rt,z2

##### Step 3: Determine the satellite position/velocity at time t.

If t=n+1Δt:

rtECEI=rn+1Δt=rn+1Δt, xrn+1Δt, yrn+1Δt, z

vtECEI=vn+1Δt=vn+1Δt, xvn+1Δt, yvn+1Δt, z

Otherwise, set n≔n+1, and go to Step 1.

Table 3 shows our calculation results for t=1 (sec) with the initial position/velocity vector using TS38.508 Table 5.6.2.1-2. It is observed the results are well aligned with the results with Keplerian model based estimation.

Table 3	Calculation results to derive satellite position/velocity vector at t=1 (sec) using TS38.508 Table 5.6.2.1-2.

| r0ECEF | [-3315.1053, 5396.4261, 2941.9442] |
| --- | --- |
| v0ECEF | [1.3182, -2.88984, 6.84654] |
| k1,r=vnΔt Δt | [1.3182, -2.88984, 6.84654] |
| k1,v=frnΔt,vnΔtΔt | [0.00344111, -0.00647986, -0.00344343] |
| k2,r=vnΔt+12k1,vΔt | [1.31992056, -2.89307993, 6.84481828] |
| k2,v=frnΔt+12k1,r,vnΔt+12k1,vΔt | [ 0.00343985, -0.00647839, -0.00344742] |
| k3,r=vnΔt+12k2,vΔt | [1.31991993, -2.8930792, 6.84481629] |
| k3,v=frnΔt+12k2,r,vnΔt+12k2,vΔt | [0.00343985, -0.0064784, -0.00344742] |
| k4,r=vnΔt+k3,vΔt | [1.32163985, -2.8963184, 6.84309258] |
| k4,v=frnΔt+k3,r,vnΔt+k3,vΔt | [0.00343859, -0.00647692, -0.00345141] |
| rn+1Δt=rnΔt+16k1,r+2k2,r+2k3,r+k4,r | [-3313.78537986, 5393.53302056, 2948.78901695] |
| vn+1Δt=vnΔt+16k1,v+2k2,v+2k3,v+k4,v | [1.32163985, -2.89631839, 6.84309258] |
| rtECEI=rn+1Δt | [-3313.78537986, 5393.53302056, 2948.78901695] |
| vtECEI=vn+1Δt | [1.32163985, -2.89631839, 6.84309258] |

## A.2	Varying Doppler shift and propagation delay generation

### A.2.1	Determine UE position

This step calculates the UE position according to the earth angular speed. Since the UE location is configured with a format of latitude, longitude, and altitude, we need to convert the values to ECEF format to align with the satellite position/velocity state vectors.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UElatitude | degree | Initial UE latitude |
| UElontitude | degree | Initial UE longitude |
| UEaltitude | km | Initial UE altitude |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |

Constant parameters:

| Parameters | Description | Values | Unit |
| --- | --- | --- | --- |
| aE | Earth radius | 6378.137 | km |
| eE2 | Square of Earth eccentricity | 6.6943799014 x 10-3 |  |

ϕ=π180UElatitude

λ=π180UElongitude

N=aE1-eE2sin2ϕ

Since it is assumed that UE is stationary during the tests, UE position does not change regardless of time t.

UEt,xECEF=N+UEaltitudecosϕcosλ UEt,yECEF=N+UEaltitudecosϕsinλUEt,zECEF=N1-eE2+UEaltitudesinϕ

### A.2.2	Doppler shift and propagation delay

This step calculates the Doppler shift and propagation delay based on the satellite position/velocity and UE position state vectors calculated in the earlier steps.

Input values:

| Parameters | Unit | Description |
| --- | --- | --- |
| UEt,xECEFUEt,yECEFUEt,zECEF | km | UE position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| rtECEF=rt,xECEFrt,yECEFrt,zECEF | km | Satellite position state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| vt=vt,xECEFvt,yECEFvt,zECEF | km/s | Satellite velocity state vector at time t in Earth-centered earth-fixed frame (ECEF) |
| fc | Hz | Carrier frequency (e.g., 2.0 x 109 for band n256) |

Output values:

| Parameters | Unit | Description |
| --- | --- | --- |
| Doppler | Hz | Doppler shift at time t |
| Delay | sec | Propagation delay at time t |

Constant parameters:

| Parameters | Description | Example of values | Unit |
| --- | --- | --- | --- |
| c | Speed of light | 299792.458 | km/s |

ρtECEF=ρt,xECEIρt,yECEIρt,zECEI=rt,xECEF-UEt,xrt,yECEF-UEt,yrt,zECEF-UEt,z

Range(t)=ρtECEI=ρt,xECEI2+ρt,yECEI2+ρt,zECEI2

RangeRate(t)=ρtECEF⋅vtECEFRnage(t)=ρt,xECEF⋅vt,xECEF+ρt,yECEF⋅vt,yECEF+ρt,zECEF⋅vt,zECEFRange(t)

Delay(t)=Range(t)c

Doppler(t)=-RangeRate(t)×fcc

Additionally, the elevation angle, El (degrees), of the satellite position from the UE viewpoint at time t can be derived as follows:

e=-sinλcosλ0, n=-sinϕcos λ-sinϕsin λcosϕ, u=cosϕcosλcosϕsinλsinϕ

Note e, n, and u are the unit vectors for east, north, and zenith direction at the UE position at time t.

lt,e=ρtECEI⋅e=ρt,xECEI⋅ex+ρt,yECEI⋅ey+ρt,zECEI⋅ez

lt,n=ρtECEI⋅n=ρt,xECEI⋅nx+ρt,yECEI⋅ny+ρt,zECEI⋅nz

lt,u=ρtECEI⋅u=ρt,xECEI⋅ux+ρt,yECEI⋅uy+ρt,zECEI⋅uz

lt,h=lt,e2+lt,n2

Elt=180πtan-1lt,ult,h

Azimuth angle, Az (degrees), is also derived as follows.

Azt=180πtan-1lt,elt,n

## B.1	SIB19 NTN Ephemeris Information for zero Doppler conditions specified in TS 38.508

Table 5.6.2.1-1: SIB19- NR NTN Ephemeris information for GSO satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -16944523 | -22027.8799 (km) |  |
| positionY-r17 | 27362209 | 35570.8717 (km) |  |
| positionZ-r17 | 4019393 | 5225.2109 (km) |  |
| velocityVX-r17 | -336 | -0.02016 (km/s) |  |
| velocityVY-r17 | -208 | -0.01248 (km/s) |  |
| velocityVZ-r17 | 1 | 0.00006 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 35.04 degrees, one-way delay equal to 127.33 ms and Doppler equal to 4.89E-06 ppm |  |  |  |

Table 5.6.2.1-2: SIB19- NR NTN Ephemeris information for NGSO (LEO-600) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2550081 | -3315.1053 (km) |  |
| positionY-r17 | 4151097 | 5396.4261 (km) |  |
| positionZ-r17 | 2263034 | 2941.9442 (km) |  |
| velocityVX-r17 | 21970 | 1.3182 (km/s) |  |
| velocityVY-r17 | -48164 | -2.88984 (km/s) |  |
| velocityVZ-r17 | 114109 | 6.84654 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.61 degrees, one-way delay equal to 2.03 ms and Doppler equal to 0.013 ppm |  |  |  |

Table 5.6.2.1-3: SIB19- NR NTN Ephemeris information for NGSO (LEO1200) satellites (zero Doppler conditions)

| Derivation Path: Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB19-r17 ::= SEQUENCE { |  |  |  |
| ntn-Config-r17 SEQUENCE { |  |  |  |
| epochTime-r17 | Not present |  |  |
| ephemerisInfo-r17 CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2771542 | -3603.0046 (km) |  |
| positionY-r17 | 4509243 | 5862.0159 (km) |  |
| positionZ-r17 | 2455572 | 3192.2436 (km) |  |
| velocityVX-r17 | 20837 | 1.25022 (km/s) |  |
| velocityVY-r17 | -46176 | -2.77056 (km/s) |  |
| velocityVZ-r17 | 109528 | 6.57168 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 89.54 degrees, one-way delay equal to 4.04 ms and Doppler equal to 3.6E-3 ppm |  |  |  |

## B.2	SIB19 NTN Ephemeris Information for zero Doppler conditions for UL frequency error test specified in TS 38.521-5

Table 6.4.1_2.4.3-1: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2717617 | -3532.9021 (km) |  |
| positionY-r17 | 4550419 | 5915.5447 (km) |  |
| positionZ-r17 | 852799 | 1108.6387 (km) |  |
| velocityVX-r17 | 6164 | 0.36984 (km/s) |  |
| velocityVY-r17 | -19424 | -1.16544 (km/s) |  |
| velocityVZ-r17 | 124281 | 7.45686 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 10 degrees, one-way delay equal to 6.44 ms and Doppler equal to 22.65 ppm. |  |  |  |

Table 6.4.1_2.4.3-2: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum negative Doppler)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2199272 | -2859.0536 (km) |  |
| positionY-r17 | 3404229 | 4425.4977 (km) |  |
| positionZ-r17 | 3535794 | 4596.5322 (km) |  |
| velocityVX-r17 | 35394 | 2.12364 (km/s) |  |
| velocityVY-r17 | -74414 | -4.46484 (km/s) |  |
| velocityVZ-r17 | 94682 | 5.68092 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 169.97 degrees, one-way delay equal to 6.60 ms and Doppler equal to -22.62 ppm. |  |  |  |

Table 6.4.1_2.4.3-3: SIB19 for RF tests with NR NTN Ephemeris Information for NGSO (LEO-600) satellites (maximum positive Doppler/2)

| Derivation Path: TS 38.508-1, Table 5.6.3.1-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| EphemerisInfo-r17 ::= CHOICE { |  |  |  |
| positionVelocity-r17 SEQUENCE { |  |  |  |
| positionX-r17 | -2592823 | -3370.6699 (km) |  |
| positionY-r17 | 4245650 | 5519.345 (km) |  |
| positionZ-r17 | 2024520 | 2631.876 (km) |  |
| velocityVX-r17 | 19359 | 1.16154 (km/s) |  |
| velocityVY-r17 | -43278 | -2.59668 (km/s) |  |
| velocityVZ-r17 | 116553 | 6.99318 (km/s) |  |
| } |  |  |  |
| } |  |  |  |
| NOTE 1: Satellite-UE elevation angle equal to 60.25 degrees, one-way delay equal to 2.30 ms and Doppler equal to 11.29 ppm. |  |  |  |

## C.1	UE location (TS 38.508-1)

### 5.6.1 UE location UE shall determine its location during the test using any of the following means. 1. UE location for the test shall be provided to the UE via pre-configured means. During the test the UE location is not expected to change unless explicitly stated as a requirement for the test. 2. Other options such as providing UE location via AT command are not precluded. UE location provided to the UE before the start of the test is as follows for RF Tx/Rx and DEMOD test cases for both GSO and NGSO config is as follows: Longitude: 121.56076999 Latitude: 25.08439333 (NGSO satellites), 55.0 (GSO satellites) Altitude: 0