3GPP TSG-RAN WG4 Meeting #114	R4-2501499

Athens, Greece, 17 February – 21 February 2025

Agenda Item:	7.19.2

Source:	Ericsson

Title:	Discussion on CSI prediction

Document for:	Discussion

# 1	Introduction

RAN4#113 agreed with the way forward on AI/ML for NR air interface [1], where the way forward also captured the several agreements on CSI prediction as follows:

Issue 2-1: Channel model for CSI prediction tests Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test. Issue 2-2: Reference throughput Agreement: - reference throughput is obtained based on random PMI with Rel-15 Type I single panel codebook Issue 2-3: Testing conditions Agreement: - use static conditions as baseline. - FFS on whether to have non-static condition

This contribution continues to discuss the core requirements on CSI prediction.

# 2	Discussion

## 2.1	Test framework

RAN#113 agreed to reuse the test metric of the existing PMI reporting requirements for the AI/ML-based CSI prediction performance requirements, that is, throughput ratio of follow CSI prediction PMI codebook and random Rel-15 Type-I codebook. Since RAN1/RAN2 is still discussing the PHY specification for Rel-19 WI AI/ML for air interface, the detailed test parameters/scenarios, e.g., CSI-RS configuration and codebook configuration, should be discussed when the performance part starts.

Proposal 1:	RAN4 should discuss the detailed test parameters for AI/ML-based CSI prediction test (PMI reporting test with Rel-19 CSI prediction) when the WI performance part starts.

Unlike the legacy non-AI-based CSI reporting, the AI/ML-based CSI reporting performance depends on the dataset used to train the CSI prediction model. Since we don’t expect 3GPP provides the training dataset, each chipset vendor should prepare the dataset and train their CSI prediction model.

For non-AI-based PMI reporting performance, RAN4 usually define one PMI reporting test case per precoder type. It is because we believe the UE applies the same reporting algorithm regardless of the test scenarios (e.g., SNR, PDSCH FRC, channel model). For example, RAN4 usually use the low-dispersion and low-Doppler spread TDL channel models such as TDL-A with delay spread of 30ns and Doppler spread of 5Hz or 10Hz for FR1 PMI reporting tests. Regarding the SNR range, RAN4 usually set SNR corresponding to 90% of the maximum throughput with fixed FRC with 16QAM with code rate of 0.50, which is not very high SNR.

On the other hand, since the AI/ML-based CSI prediction performance depends on the training dataset, even if a UE reports very accurate predicted PMI in one specific scenario, this UE may report PMI that degrades the performance in other scenarios. To ensure the accurate CSI prediction performance in as many scenarios as possible, we propose RAN4 define several PMI reporting requirements. At least RAN4 consider several channel models such as TDLA30 and TDLC300.

Proposal 2:	RAN4 should define more than one AI/ML-based CSI prediction requirements to ensure the generality of the CSI prediction model. At least RAN4 consider several channel models such as TDLA30 and TDLC300.

In RAN4#113, RAN4 agreed to use TDL channel model for AI/ML-based CSI prediction tests. However, RAN4 should consider testing with spatial channel model such as CDL. We understand RAN4 is still discussing the feasibility for performance requirements with CDL channel model in Rel-19 SI. We therefore RAN4 may consider the AI/ML-based PMI reporting requirements with CDL channel model when a suitable model is available.

Proposal 3:	RAN4 should consider defining AI/ML-based CSI prediction requirements using CDL channel model when a suitable CDL model is available, depending on the outcome of SI spatial channel model.

## 2.2	Performance monitoring

RAN1 is going to specify the performance monitoring framework so that the gNB can activate/deactivate the proper CSI reporting functionality, as shown in Figure 1. RAN1 is discussing the detailed performance monitoring framework, e.g., CSI-RS resource configuration, reported quantity, etc.

Figure 1	Performance monitoring of AI/ML-based CSI prediction model.

In our understanding, RAN1 are going to discuss three options for performance monitoring reporting for CSI prediction [3], and RAN1#117 has clarified the options for performance monitoring as follows.

Agreement For the boundary between Type 3 and Type 1 performance monitoring, the difference is whether UE reports performance metric or performance monitoring output to NW, respectively. - The monitoring output is determined based on performance metric, and additionally, baseline and/or threshold criterion if configured. Observation For CSI prediction using UE-sided model, for performance monitoring, at least following specification impacts are additionally identified compared to that has been captured in TR38.843, - Type 1 - Definition/configuration of performance metric - Definition of threshold criterion, if configured - Definition/configuration and report of monitoring output, and corresponding report mechanism - Type 2 - Definition/configuration and report of ground truth CSI, and corresponding report mechanism. - Type 3 - Definition/configuration and report of performance metric, and corresponding report mechanism. For all types of performance monitoring, NW indication to the UE of the decision regarding the monitoring action

In Type 3, the UE calculates the measurement quantity like SGCS or NMSE between the predicted CSI and ground-truth CSI, and then the UE reports the quantity to the measurement quantity to gNB. In Type 1, before the reporting, the UE derives the monitoring output based on the configured threshold, and then the UE reports the monitoring outputs. In Type 2, the UE reports both predicted CSI and/or ground-truth CSI so that the gNB can calculate the metric. We expect RAN1 will decide the performance monitoring quantity and the reporting framework including CSI-RS configuration and CSI computation time (CPU) for performance monitoring.

Since the required RAN4 core requirement depends on the RAN1 agreements, it is early for RAN4 to discuss the detailed core requirements for the performance monitoring. However, from our experience on CSI reporting requirements, we think RAN4 may need at least the following requirements.

- CSI prediction performance monitoring reporting delay requirements

- CSI prediction performance monitoring reporting accuracy requirements

  - For example, accuracy of monitoring quantity such as SGCS/NMSE.

In our understanding, some monitoring quantities may be difficult to verify their accuracies. For example, SGCS/NMSE is derived from the predicted PMI and grand-truth PMI (PMI derived from the measured CSI-RS) at the UE. To evaluate the accuracy, TE need the ground-truth PMI to calculate ideal SGCS/NMSE. However, we are wondering if the TE could get the ground-truth PMI at UE antenna connector. If it is not possible, alternatively, we may check the reported quantities at the PMI reporting performance requirements together with the throughput ratio. For example, we may add the following requirements in the PMI reporting performance requirements.

- Reported SGCS/NMSE shall be more than X at least Y% of the test time (e.g., X=0.7 and Y=80%)

Proposal 4:	RAN4 should wait for RAN1 conclusion on the performance monitoring for CSI prediction. Once RAN1 conclude the performance monitoring framework, RAN4 start to discuss the necessary (RRM) core requirements such as reporting delay and reporting accuracy requirements.

Proposal 5:	Testability of monitoring reporting delay/accuracy needs discussion.

# 3	Summary

Proposals:

Proposal 1:	RAN4 should discuss the detailed test parameters for AI/ML-based CSI prediction test (PMI reporting test with Rel-19 CSI prediction) when the WI performance part starts.

Proposal 2:	RAN4 should define more than one AI/ML-based CSI prediction requirements to ensure the generality of the CSI prediction model. At least RAN4 consider several channel models such as TDLA30 and TDLC300.

Proposal 3:	RAN4 should consider defining AI/ML-based CSI prediction requirements using CDL channel model when a suitable CDL model is available, depending on the outcome of SI spatial channel model.

Proposal 4:	RAN4 should wait for RAN1 conclusion on the performance monitoring for CSI prediction. Once RAN1 conclude the performance monitoring framework, RAN4 start to discuss the necessary (RRM) core requirements such as reporting delay and reporting accuracy requirements.

Proposal 5:	Testability of monitoring reporting delay/accuracy needs discussion.

# References

- R4-2420513, “WF on AI/ML air interface part 1”, Qualcomm Incorporated.

- RP-241610, “Study on spatial channel model for demodulation performance requirements”, Nokia, et al.

- TR 38.843, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”, 3GPP.

# Appendix

Performance monitoring in TR [3].

For CSI prediction using UE side model use case, at least the following aspects have been proposed by companies on performance monitoring for functionality-based LCM: - Type 1: - UE calculates the performance metric(s) - UE reports performance monitoring output that facilitates functionality fallback decision at the network - Performance monitoring output details can be further defined - NW may configure threshold criterion to facilitate UE side performance monitoring (if needed). - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 2: - UE reports predicted CSI and/or the corresponding ground-truth - NW calculates the performance metrics. - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting). - Type 3: - UE calculates the performance metric(s) - UE reports performance metric(s) to the NW - NW makes decision(s) of functionality fallback operation (fallback mechanism to legacy CSI reporting).