# 3GPP TSG-RAN WG4 Meeting # 114                                    R4-2501505

# Athens, GR, 17th – 21st February, 2025

# Title: 	Draft TP introduction of comparison of spatial channel models for SU-MIMO PMI cases

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.16.2

# Document for:	Approval

# 1 Background

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.

# 2 Draft TP

# 6  Comparison of Spatial Channel Models

6.1  PDSCH performance under SU-MIMO scenario(PMI Aspects)

This section provides comparison for different spatial channel modes with simulation assumptions captured in Table 6.1-2.

Table 6.1-2: Simulation assumptions for PMI test cases

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | TDD |
| TDD Slot Configuration Pattern |  | 7D1S2U |
| FR / Carrier frequency |  | FR1,3.5GHz |
| UE speed |  | 3km/h |
| Antenna configuration |  | 8T4R |
| Number of layers |  | 4,2 |
| PMI |  | Follow, Random |
| Waveform |  | CP-OFDM with normal CP |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |
| MCS |  | 13 (64 QAM table) |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PDSCH aggregation factor | 1 |
|  | Resource allocation type | Type 0 |
|  | VRB-to-PRB mapping type | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |
| Codebook configuration | CodebookType | type-I-SP, eType-II |
|  | Codebook configuration | For type-I-SP:(N1,N2,O1,O2) = (4,1,4,1) For eType-II:(N1,N2,O1,O2) = (4,1,4,1) [ paramCombination-r16=5 (L=4, pν=1/4, β=3/4) ] |
| PDSCH DMRS Precoding Configuration |  | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |
|  | Number of CSI-RS ports (X) | 8 CSI-RS Ports (4,1) |
|  | Density (ρ) | 1 |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |
| CQI/RI/PMI delay |  | 6.5ms |
| Number of HARQ Processes |  | 8 |
| Maximum HARQ transmissions |  | 4 |
| Channel Models |  | Option 1X: CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,8,2,8,2) Option 1Y: CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,4,2,8,1) Option 2: CDL-UMa-C channel with (M,N,P,Ms,Ns)=(8,8,2,8,8) Option 3: CDL-UMa-C channel with (M,N,P,Ms,Ns)=(1,4,2,1,1) Enhanced TDL-C channel [TDL-C, XPL, High] |
| UE receiver type |  | MMSE-IRC |
| Test metric |  | Full Throughput Curves |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |
|  | BS Antenna Polarisation | Cross Polarized antenna elements with +/-45 degrees polarization slant angles |
|  | BS Radiation Pattern | Defined Table 7.3-1 in TS 38.901 |
|  | UE Antenna Polarisation | cross-polarized antenna elements with +90/0 degrees polarization slant angles |
|  | UE Antenna Radiation Pattern | Omnidirectional |
|  | Antenna Panel Placement | YZ Plane |

The summary of simulation results are captured in the following.

TBD

Observation: TBD

# 3 Conclusion

This draft TP captures comparison of PDSCH performance under SU-MIMO scenario for different spatial channel model part.