# 3GPP TSG-RAN WG4 Meeting # 114                                    R4-2501508

# Athens, GR, 17th – 21st February, 2025

# Title: 	Discussion on testability and interoperability issues for CSI prediction

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	7.19.2

# Document for:	Approval

# 1 Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. The objectives for RAN4 are shown in following table.

- Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

During the last meeting, some conclusions has been reached for CSI compression and CSI prediction[2]. In the following section, we will provide the detailed discussions for CSI compression and prediction.

# 2 Discussion

Channel model for CSI prediction tests

Agreement: - For channel model for CSI prediction tests, TDL channel models will be used. - FFS on whether CDL channel model is needed for generalization test.

In last meeting, RAN4 discussed the issue of channel model for CSI prediction test. Generally, TDL channel model is used for testing is the basically functionality in RAN4. From the channel modeling perspective, TDL channel model is more easier to establish. Only needs to consider the power of each tap, delay and Doppler. This approach does not require consideration of the other gain, etc, antenna and beamforming gain.

In the CDL channel models, the concept of clusters is introduced. A cluster refers to a group of multi-paths that share similar AOA, AOD and delays. This approach allows the model to more accurately represent the statistical properties of real-world wireless channels by considering not just individual multi-paths but also their aggregated behavior within clusters. This can be particularly useful in scenarios where signal reflections and scattering result in a few dominant clusters rather than a uniform distribution of multi-paths components. Also this design allows for better characterization of spatial correlations.

On the other side, the another issue is about how to derive SNR values in link level simulation. During the CDL channel modeling process, the antenna pattern was taken into consideration. Moreover, if RAN4 wants to model more accurately, the antenna array virtualization also needs to be considered. This approach will bring beamforming gain in signal transmission process, which will result in the transmit power of the signal no longer being equal to 1 in the link level simulation. Without a doubt, this will increase the signal power. However, the legacy CSI requirements testing based on the SNR and throughput. If the transmit signal is no longer being equal to 1, how to derive the SNR value in link level simulation is the critical issue in evaluation and testing.

Observation 1. TDL channel model doesn’t need to any gain in signal transmission.

Observation 2. CDL channel model needs to consider antenna pattern gain, beamforming gain in signal transmission, which will results in the transmit power of the signal no longer being equal to 1 in link level simulation.

Proposal 1. No need to consider CDL channel model for CSI prediction test.

# 3 Conclusion

In this contribution, we give some detailed discussions on testability and interoperability issues for CSI prediction , The conclusions are:

Observation 1. TDL channel model doesn’t need to any gain in signal transmission.

Observation 2. CDL channel model needs to consider antenna pattern gain, beamforming gain in signal transmission, which will results in the transmit power of the signal no longer being equal to 1 in link level simulation.

Proposal 1. No need to consider CDL channel model for CSI prediction test.

# 4 References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface.

- R4-2414323, WF on AI/ML. Qualcomm Incorporated.