3GPP TSG-RAN WG4 Meeting #114	R4-2501514

# Athens, Greece, 17th – 21st Feb, 2025

Title: 	Simulation results on 8Rx performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.18.2

Document for:	Discussion

Background

According to WF R4-2419784 [1], some open issues on 8Rx performance requirements with MMSE-IRC were left. This contribution provides our simulation results.

- Simulation results for inter cell interference

Table 2-1 summarizes the simulation assumptions for inter cell interference

Table 2-1 Simulation assumptions for inter cell interference

| Parameter |  | Unit | Value |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 3 |
|  |  |  | Enabled | Enabled | Enabled for HomoNet Disabled for HetNet |
| Duplex mode |  |  | FDD |  |  |
| Active DL BWP index |  |  | 1 |  |  |
| Physical cell ID |  |  | 0 | 1 | 2 |
| Transmission rank |  |  | 1,2,4 | Random rank with 70% and 30% probability for rank 1 and rank 2 | Random rank with 70% and 30% probability for rank 1 and rank 2 for HomoNet N/A for HetNet |
| Time offset to Cell1 |  | us | N/A | 3 | -1 |
| Frequency offset to Cell 1 |  | Hz | N/A | 300 | -100 |
| Interference Model |  |  | N/A | As specified in B.6.2 in 38.101-4 |  |
| INR (Note 2) |  | dB | N/A | 7.77 for HomoNet 7.58 for HetNet | 2.29 for HomoNet N/A for HetNet |
| PDSCH configuration | Mapping type |  | Type A |  |  |
|  | k0 |  | 0 |  |  |
|  | Starting symbol (S) |  | 2 |  |  |
|  | Length (L) |  | 12 |  |  |
|  | PDSCH aggregation factor |  | 1 |  |  |
|  | PRB bundling type |  | Static |  |  |
|  | PRB bundling size |  | 2 |  |  |
|  | Resource allocation type |  | Type 0 |  |  |
|  | RBG size |  | Config2 |  |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |  |
|  | Number of additional DMRS |  | 1 |  |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |  |
| Number of HARQ Processes |  |  | 4 |  |  |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |  |  |
| Note1: Cell 1 is the serving cell; Cells 2, 3 are interfering cells Note 2: INR is defined in Annex B.6.1 in 38.101-4 |  |  |  |  |  |

Table 2.2 and Figure 2.3 provides simulation results for 8Rx with inter-cell interference.

- Table 2-2: Summary of simulation results for 8Rx with inter-cell interference (HomoNet)

| Propagation conditions | Rank | MCS |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance difference (Baseline receiver vs Simplified receiver) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| TDLC300-100 | Rank1 | MCS17 |  | 5.5 | 9.8 | 4.3 | 7.8 | 11.3 | 3.5 | 2.3 |
| TDLA30-10 | Rank2 |  | MCS17 | 7.3 | 12.1 | 4.8 | 10.8 | 14.2 | 3.4 | 3.5 |
|  | Rank4 |  | MCS13 | 8.6 | 13.7 | 5.1 | 15.2 | 17.2 | 2.0 | 6.6 |

- Table 2-3: Summary of simulation results for 8Rx with inter-cell interference (HetNet)

| Propagation conditions | Rank | MCS |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance difference (Baseline receiver vs Simplified receiver) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| TDLA30-10 | Rank1 | MCS17 |  | 3.6 | 7.1 | 3.5 | 5.3 | 8.5 | 3.2 | 1.7 |
|  | Rank2 |  | MCS17 | 6.4 | 10.7 | 4.3 | 8.9 | 12.8 | 3.9 | 2.5 |
|  | Rank4 |  | MCS13 | 7.8 | 11.9 | 4.1 | 13.4 | 15.9 | 2.5 | 5.6 |

- Simulation results for MU-MIMO

Table 3-1: Simulation assumptions for MU-MIMO sceanrio

| Parameter |  | Unit | Target UE | Co-scheduled UE |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  | FDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 2 |  |
|  | Length (L) |  | 12 |  |
|  | PDSCH aggregation factor |  | 1 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 0 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
|  | Antenna ports indexes |  | 1000 | 1001 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 | 1 |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. | Single Panel Type I, For Rank 1+1: Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook Index, chosen from section 5.2.2.2.1 of TS 38.214 [12].Any column of precoder matrix is not equal to any column of precoder matrix of Target UE. For Rank 2+2 and Rank4+4: Select the precoder to ensure any column of precoder is orthogonal to any column of precoder for the target PDSCH |
| MU-MIMO Beamforming Model |  |  | As specified in B.4.2 in 38.101-4 |  |
| Number of HARQ Processes |  |  | 4 | N/A |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 | N/A |
| Note 1: The DMRS scrambling ID is same for both target UE and Co-scheduled UE. |  |  |  |  |

- Table 3-2 summarizes the simulation results for 8Rx under MIMO scenario:

- Table 3-2: Summary of simulation results for 8Rx with MU-MIMO

|  |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance loss |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| Rank2+2 | MCS17 | 9.3 | Inf | Inf | 13.3 | Inf | Inf | 4.0 |
|  | MCS19 | 10.8 | Inf | Inf | 15.2 | Inf | Inf | 4.4 |
| Rank4+4 | MCS13 | 12.4 | Inf | Inf | Inf | Inf | Inf | Inf |

Figure shows throughput- SNR curve for MU-MIMO

Figure 3-1: Simulation results for MU-MIMO

- Conclusion

This contribution provides our simulation results for 8Rx requirements with interference.

- Reference

[1]  R4-2419784 WF on NR demodulation performance: Phase 5. Nokia