3GPP TSG-RAN WG4 Meeting # 114	R4-2501515

# Athens, Greece, 17th – 21st Feb, 2025

Title: 	Discussion on BS MMSE-IRC requirements with inter-cell interference

Source: 	Huawei, HiSilicon

Agenda item:	7.18.3

Document for:	Discussion

Background

According to WF R4-2419780 [1], some open issues for BS MMSE-IRC requirements are left. This contribution provides our views.

- Discussions

Test requirements definition

There are two options for interference definition criteria: DIP based modelling with SINR based requirements definition and INR based modelling with SNR based requirements definition:

Test requirement definition - Candidate options: - Option 1: FFS whether SINR based definition is needed - Option 2: Use the SNR based requirement definition

Such two criteria are equivalent to each other and have no impact on performance. Considering INR based modelling with SNR based requirements definition criteria has been used in UE MMSE-IRC performance requirements definition. We slightly prefer to be aligned with UE side and consider INR based modelling with SNR based requirements definition.

Proposal 1: Use INR based modelling with SNR based requirements definition.

Number of interferers for 2Rx

One issue is number of interference cells. Candidate options are:

Number of interferers for 2Rx - Option 1: Define BS MMSE-IRC requirements for 2Rx - Option 1A: 1 interferer - Option 1B: 2 interferers - Option 1C: Consider 2 interferers for HomNet and 1 interfere for HetNet - Option 2: Do not need to specify requirements for 2Rx - Companies are encouraged to provide simulation results for 1 or 2 interferers for 2Rx, to check the performance gain.

Based on our results on 2Rx, MMSE-IRC performance gain over MMSE-MRC is around 2.0dB for DIP set1 and 4.0dB for DIP set2, which are enough to define requirements. Hence we slightly prefer to define 2 interference for 2Rx

Proposal 2: RAN4 to consider 2 interference cells for 2Rx

Channel bandwidth

We have following options for channel bandwidth:

- Proposals on the CHBW for requirement definition for CP-OFDM: - Option 1: - For 15kHz SCS, cover 5MHz, 10MHz, 20MHz - For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz - Option 2: - For 15kHz SCS, cover 5MHz, 10MHz - For 30kHz SCS, cover 10MHz, 40MHz - Option 3: minimum CBW - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Option 4: 5/50MHz BW with 15KHz SCS and 10/100MHz BW with 30KHz SCS - Proposals for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz

We prefer to only consider minimum bandwidth for each SCS, i.e. 5MHz SCS for 15kHz SCS and 10MHz for 30kHz SCS. In Rel-15, all bandwidth for each SCS have been verified, it’s expected to reduce the test cases for bandwidth for future Release. It is mentioned that many Rel-18 requirements only cover minimum bandwidth such as ATG and enhanced DMRS, the approach of which can also be applied to Rel-19 MMSE-IRC requirements with interference cell considering bandwidth have negligible impact on performance.

Proposal 3: RAN4 to only consider minimum bandwidth for each SCS, i.e. 5MHz for 15kHz SCS and 10MHz for 30kHz.

MCS

One open issue is MCS, candidate opens are listed below:

- Consider the following options for performance evaluation and as candidates for further down-selection: o For 2Rx:  Option 1: MCS 2  Option 2: MCS 16 o For 4Rx:  Option 1: MCS 16 o For 8Rx:  Option 1: MCS 20  Option 2: MCS 16 - Note: Reuse the same FRC A.3, A.4 and A.5 with Additional DM-RS position = pos1 and 1 transmission layer defined in TS38.104 for MCS 2, MCS 16 and MCS 20 respectively. - Check whether the existing test applicability rule can be reused if different MCS is configured for different Rx configuration.

For fairness, it is better to keep same MCS for different Rx, MCS16 is a good choice which shows good performance gain.

Proposal 4: RAN4 to use MCS16 for all Rx.

Propagation condition

One issue is propagation conditions:

– For serving cell, consider the following options for performance evaluation and as candidates for further down-selection: o Option 1: TDLA30-10 HetNet and TDLB100-400 for HomNet o Option 2: Consider different channel for different MCS  Option 2A: MCS 2 with TDLB100-400, MCS 16 with TDLC300-100, MCS 20 with TDLA30-10 – For the interference cell, use TDLC300-100.

Open2A is same as legacy PUSCH test, which can be reused.

Proposal 5: RAN4 to use MCS 2 (if agreed) with TDLB100-400, MCS 16 (if agreed) with TDLC300-100, MCS 20 (if agreed) with TDLA30-10

Waveform

We have following options for waveform:

Waveform – whether to cover DFT-s-OFDM - Candidate options: - Option 1: Cover both CP-OFDM and DFT-s-OFDM waveform - Option 2: Cover CP-OFDM only

Based on our understanding, DFT-s-OFDM is rarely used in the real deployment, considering the timeline, we prefer to only focus on CP-OFDM.

Proposal 6: RAN4 to only consider CP-OFDM for requirements definition.

Applicability rule for different test scenarios

One open issue is applicability rules, proposals are listed as below:

Applicability rule for different test scenarios - Proposals on applicability for Homnet and Hetnet scenarios: - Proposal 1: HomNet requirements are not applicable for Local Area BS - Proposal 2: HomNet requirements are only applied for Wide area BS, and HetNet requirements are only applied for Home BS and Local BS - Proposals on additional applicability rule: - Proposal 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped - Proposal 2: Introduce the applicability rules that only one of HomNet and HetNet scenarios is tested

LTE applicability rule specifies that HomNet requirements are not applicable for Local Area BS and Home BS, there is no Homo BS defined for NR, so proposal1 is similar to LTE applicability rule which can be reused.

Proposal 7: RAN4 to define the applicability rules that HomNet requirements are not applicable for Local Area BS

To reduce test number, applicability rule that BS declaring to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped can be defined.

Proposal 8: Define the applicability rule that applicability rule that BS declaring to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped

- Simulation results

In this clause we provide our initial simulation results to check the MMSE-IRC performance gain over MMSE-MRC under following assumptions:

Table 3-1: Simulation assumptions

| Assumptions |  | Serving cell | Interference cell 1 | Interference cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | 5MHz/15kHz | 5MHz/15kHz | 5MHz/15kHz |
| Duplex mode |  | FDD | FDD | FDD |
| MCS |  | 2,16,20 | 16QAM random symbols | 16QAM random symbols |
| Propagation conditions: |  | TDLC300-100 Low for MCS2 TDLB100-400 Low for MCS16 TDLA30-10 Low for MCS20 | TDLC300-100 Low | TDLC300-100 Low |
| Antenna configuration |  | 1T2R, 1T4R,1T8R | 1T2R, 1T4R,1T8R | 1T2R, 1T4R,1T8R |
| Interference model |  |  | DIP set1: DIP1=-1.11dB DIP set2: DIP1=-0.43dB | DIP set1: DIP2=-10.91dB DIP set2: DIP2=-13.78dB |
| DM-RS | DM-RS configuration type | 1 | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB | -3 dB |
|  | DM-RS port | {0} | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Receiver assumption |  | MMSE-MRC, MMSE-IRC | N/A | N/A |

The simulation results are captured in Table 3-2:

Table 3-2: Simulation results

| DIP Set | MCS | Antenna configuration | MMSE-IRC (SNR(dB)@70% of maxTP) | MMSE (SNR(dB)@70% of maxTP) | Gain (dB) |
| --- | --- | --- | --- | --- | --- |
| DIP Set1 | MCS2 | 1T2R | 0.5 | 2.8 | 2.3 |
|  | MCS16 | 1T2R | 15.0 | 16.9 | 1.9 |
|  |  | 1T4R | 8.8 | 12.3 | 3.5 |
|  |  | 1T8R | 5.4 | 8.9 | 3.5 |
|  | MCS20 | 1T8R | 7.3 | 10.5 | 3.2 |
| DIP Set2 | MCS2 | 1T2R | 3.0 | 6.9 | 3.9 |
|  | MCS16 | 1T2R | 17.4 | 21.1 | 3.7 |
|  |  | 1T4R | 10.0 | 16.3 | 6.2 |
|  |  | 1T8R | 6.1 | 12.9 | 6.8 |
|  | MCS20 | 1T8R | 7.9 | 14.0 | 6.1 |

- Conclusion

This contribution provides our views on BS MMSE-IRC requirements. The proposals and observations are:

Proposal 1: Use INR based modelling with SNR based requirements definition.

Proposal 2: RAN4 to consider 2 interference cells for 2Rx

Proposal 3: RAN4 to only consider minimum bandwidth for each SCS, i.e. 5MHz for 15kHz SCS and 10MHz for 30kHz.

Proposal 4: RAN4 to use MCS16 for all Rx.

Proposal 5: RAN4 to use MCS 2 (if agreed) with TDLB100-400, MCS 16 (if agreed) with TDLC300-100, MCS 20 (if agreed) with TDLA30-10

Proposal 6: RAN4 to only consider CP-OFDM for requirements definition.

Proposal 7: RAN4 to define the applicability rules that HomNet requirements are not applicable for Local Area BS

Proposal 8: Define the applicability rule that applicability rule that BS declaring to support Wide Area BS, Medium Range BS and pass the tests for HomNet (DIP Set1), the test for HetNet (DIP Set 2) can be skipped

- Reference

[1]   R4-2419780 Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS. China Telecom, MediaTek