3GPP TSG-RAN WG4 Meeting # 114	R4-2501516

# Athens, Greece, 17th – 21st Feb, 2025

Title: 	Simulation results on BS MMSE-IRC requirements with inter-cell interference

Source: 	Huawei, HiSilicon

Agenda item:	7.18.3

Document for:	Discussion

Background

According to WF R4-2419780 [1], we provide our simulation results for BS MMSE-IRC requirements with inter cell interference.

- Simulation results

In this clause we provide our initial simulation results to check the MMSE-IRC performance gain over MMSE-MRC under following assumptions:

Table 3-1: Simulation assumptions

| Assumptions |  | Serving cell | Interference cell 1 | Interference cell 2 |
| --- | --- | --- | --- | --- |
| Bandwidth/SCS |  | 5MHz/15kHz | 5MHz/15kHz | 5MHz/15kHz |
| Duplex mode |  | FDD | FDD | FDD |
| MCS |  | 2,16,20 | 16QAM random symbols | 16QAM random symbols |
| Propagation conditions: |  | TDLC300-100 Low for MCS2 TDLB100-400 Low for MCS16 TDLA30-10 Low for MCS20 | TDLC300-100 Low | TDLC300-100 Low |
| Antenna configuration |  | 1T2R, 1T4R,1T8R | 1T2R, 1T4R,1T8R | 1T2R, 1T4R,1T8R |
| Interference model |  |  | DIP set1: DIP1=-1.11dB DIP set2: DIP1=-0.43dB | DIP set1: DIP2=-10.91dB DIP set2: DIP2=-13.78dB |
| DM-RS | DM-RS configuration type | 1 | 1 | 1 |
|  | DM-RS duration | single-symbol DM-RS | single-symbol DM-RS | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 | pos1 | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 | 2 | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB | -3 dB | -3 dB |
|  | DM-RS port | {0} | {0} | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 | NID0=1, nSCID =0 | NID0=2, nSCID =0 |
| Receiver assumption |  | MMSE-MRC, MMSE-IRC | N/A | N/A |

The simulation results are captured in Table 3-2:

Table 3-2: Simulation results

| DIP Set | MCS | Antenna configuration | MMSE-IRC (SNR(dB)@70% of maxTP) | MMSE (SNR(dB)@70% of maxTP) | Gain (dB) |
| --- | --- | --- | --- | --- | --- |
| DIP Set1 | MCS2 | 1T2R | 0.5 | 2.8 | 2.3 |
|  | MCS16 | 1T2R | 15.0 | 16.9 | 1.9 |
|  |  | 1T4R | 8.8 | 12.3 | 3.5 |
|  |  | 1T8R | 5.4 | 8.9 | 3.5 |
|  | MCS20 | 1T8R | 7.3 | 10.5 | 3.2 |
| DIP Set2 | MCS2 | 1T2R | 3.0 | 6.9 | 3.9 |
|  | MCS16 | 1T2R | 17.4 | 21.1 | 3.7 |
|  |  | 1T4R | 10.0 | 16.3 | 6.2 |
|  |  | 1T8R | 6.1 | 12.9 | 6.8 |
|  | MCS20 | 1T8R | 7.9 | 14.0 | 6.1 |

- Conclusion

This contribution provides our simulation results on BS MMSE-IRC requirements.

- Reference

[1]   R4-2419780 Way Forward for [113] [321] NR_demod_Ph5_Part1_General_BS. China Telecom, MediaTek