3GPP TSG-RAN WG4 Meeting #114	R4-2501518

# Athens, Greece, 17th – 21st Feb, 2025

Title: 	Simulation results on spatial channel models

Source: 	Huawei, HiSilicon

Agenda item:	7.16.2

Document for:	Discussion

Background

In last meeting, WF R4-2419782 [1] related to spatial channel model was approved, this contribution provides our simulation results.

- Simulation results for PDSCH cases

This section provides our simulation results for PDSCH cases, with simulation assumptions in Table 8.2.

Figure 4-1: Normalized throughput-SNR curve for PDSCH cases

Table 4-1 summarizes SNR(dB)@70% of max throughput.

Table 4-1: Summary of SNR (dB)@70% of max TP

| SNR(dB)@70% of max throughput | Without AAV | FixedArray | FixedSubArray | FullConnceted | Enhanced TDL (2 beams for Rank4, 4beams for Rank8) |
| --- | --- | --- | --- | --- | --- |
| 4T4R, Rank4 | 15.1 | 16.5 | 17.2 | 15.5 | 13.9 |
| 8T8R, Rank8 | 22.1 | 16.8 | 22.4 | 22.3 | 18.6 |

- Simulation results for PMI cases

This section provides our simulation results for PMI cases, with simulation assumptions in Table 8.2:

Rank 2 test

Figure 5.1-1: Normalized throughput-SNR curve for Rank2 PMI cases

Table 5.1-1 summarizes SNR(dB) @90% of throughput:

Table 5.1-1: Summary of SNR(dB)@90% of max throughput

| SNR(dB)@90% of max throughput | Without AAV | FixedArray | FixedSubArray | FullConnceted | Enhanced TDL with 2 beams |
| --- | --- | --- | --- | --- | --- |
| Type I Random | 9.4 | 10.3 | 10.4 | 7.9 | 8.2 |
| Type I Follow | 2.6 | 4.8 | 4.3 | 6.9 | 3.5 |
| eType II Follow | 2.4 | 3.7 | 3.3 | 3.2 | 2.5 |

Table 5.1-2 summarizes Gama value, which is the absolute throughput ratio of follow PMI to random PMI at SNR@ 90% of max throughput of follow PMI

Table 5.1-2: Summary of Gama value

| Gama value | Without AAV | FixedArray | FixedSubArray | FullConnceted | Enhanced TDL with 2 beams |
| --- | --- | --- | --- | --- | --- |
| Type I Follow | 2.3 | 1.4 | 1.4 | 1.22 | 1.88 |
| eType II Follow | 2.3 | 2.1 | 2.14 | 1.73 | 2.09 |

Rank 4 test

Figure 5-2: Normalized throughput-SNR curve for Rank4 PMI cases

Table 5.2-1 summarizes SNR(dB) @90% of throughput:

Table 5.2-1: Summary of SNR@90% of max throughput

| SNR(dB)@90% of max throughput | Without AAV | FixedArray | FixedSubArray | FullConnceted | Enhanced TDL with 2 beams |
| --- | --- | --- | --- | --- | --- |
| Type I Random | 20.7 | 20.4 | 22.0 | 18.6 | 18.4 |
| Type I Follow | 13.2 | 14.5 | 14.8 | 16.8 | 10.1 |
| eType II Follow | 15.1 | 16.5 | 16.7 | 14.9 | 13.2 |

Table 5.2-2 summarizes Gama value, which is the absolute throughput ratio of follow PMI to random PMI at SNR@ 90% of max throughput of follow PMI

Table 5.2-2: Summary of Gama value

| Gama value | Without AAV | FixedArray | FixedSubArray | FullConnceted | Enhanced TDL with 2 beams |
| --- | --- | --- | --- | --- | --- |
| Type I Follow | 1.76 | 1.64 | 1.70 | 1.29 | 1.84 |
| eType II Follow | 1.78 | 1.50 | 1.63 | 1.55 | 1.80 |

- Conclusion

This contribution provides our simulation results on spatial channel model.

- Reference

[1]  R4-2419782 Way Forward for [113][320] NR_SCM

- Appendix

Table 8.1: Simulation results for Normalized factor

|  | Without AAV | FixedArray | FixedSubArray | FullConnceted |
| --- | --- | --- | --- | --- |
| 4Tx,4Rx | 0.139 | 0.010 | 0.022 | 0.621 |
| 8Tx,8Rx | 0.136 | 0.011 | 0.018 | 0.559 |

Table 8.2: Simulation assumptions

| Parameter |  | SU-MIMO PDSCH (FR1) | SU-MIMO PMI (FR1) |
| --- | --- | --- | --- |
| Duplex mode |  | TDD |  |
| TDD Slot Configuration Pattern |  | 7D1S2U |  |
| FR / Carrier frequency |  | 3.5GHz |  |
| UE receiver type |  | IRC Baseline |  |
| Number of Tx Ports |  | 8,4 | 8 |
| Number of Rx Ports/Antennas |  | 8,4 | 4 |
| Number of layers |  | 8,4 | 4,2 |
| PMI |  | Single panel type 1, Random | Follow, Random |
| Waveform |  | CP-OFDM with normal CP |  |
| Channel Bandwidth/SCS |  | 40MHz/30kHz |  |
| MCS |  | 13 (64 QAM table) |  |
| PDSCH configuration | Mapping type | Type A |  |
|  | k0 | 0 |  |
|  | Starting symbol (S) | 2 |  |
|  | Length (L) | 12 |  |
|  | PDSCH aggregation factor | 1 |  |
|  | Resource allocation type | Type 0 |  |
|  | VRB-to-PRB mapping type | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size | N/A |  |
| PDSCH DMRS configuration | DMRS Type | Type 1 |  |
|  | Number of additional DMRS | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS | 2 (for rank > 4) 1 (for rank <= 4) |  |
| Codebook configuration | CodebookType | type-I-SP | type-I-SP, eType-II |
|  | Codebook configuration | For 8Tx (N1,N2,O1,O2) = (4,1,4,1) For 4Tx (N1,N2,O1,O2) = (2,1,4,1) | For typeI-SP: (N1,N2,O1,O2) = (4,1,4,1) For eType-II: (N1,N2,O1,O2) = (4,1,4,1) paramCombination-r16=5 (L=4, pν=1/4, β=3/4) |
| PDSCH DMRS Precoding Configuration |  | For Random precoding: for every PRB Bundle (size=2) For Fixed precoding: Wideband PMI | For typeI-SP (random and follow): Wideband PMI For eType-II with follow and random: TBD |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type | Aperiodic |  |
|  | Number of CSI-RS ports (X) | 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer | 8 CSI-RS Ports (4,1) |
|  | Density (ρ) | 1 |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) | (13) |  |
| CQI/RI/PMI delay |  | N/A | 6.5 ms |
| Number of HARQ Processes |  | 8 |  |
| Maximum HARQ transmissions |  | 4 |  |
| Channel Models |  | The purpose of the study item. |  |
| Testing Metric |  | Full Throughput Curves |  |
| Channel Geometry (CDL) | LCS UE | α = 180°, β=0°, γ = 0° |  |
|  | LCS gNodeB | α = 0°, β=10°, γ = 0° |  |
|  | GCS UE | Height = 1.5 m; Azimuth = 0; X Coordinate = 100 m |  |
|  | GCS gNodeB | Height = 25 m; Azimuth = 0; X Coordinate = 0 m |  |
|  | BS Antenna Polarisation | (+45, -45) |  |
|  | BS Radiation Pattern | defined in TR38.901 Table 7.3-1. |  |
|  | UE Antenna Polarisation | (0, 90) |  |
|  | UE Antenna Radiation Pattern | Omnidirectional |  |
|  | Antenna Panel Placement | YZ Plane |  |