3GPP TSG-RAN WG4 Meeting #114	R4-2501520

# Athens, Greece, 17th – 21st Feb, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | 0735 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on 38.101-4: Modification on applicability rules and test setup for 8Rx requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ENDC_RF_FR1_enh2-Perf |  |  |  |  |  | Date: |  |  | 2025-11-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - Applicability rules for Rank8 tests are missing - Indication which receiver type Rank8 tests are applied is missing - Note 2 in Table 5.2A.4.1-7 and 5.2A.4.1-8 is incorrect, Rank 2 requirements in Tables 5.2A.4.1-1 and 5.2A.4.1-4 shall be applied only for 8Rx CCs not 2Rx/4Rx CCs - Note 3 in Table 5.2A.4.1-8 is incorrect since Rank8 tests are only applicable for Baseline Receiver - SDR test, PDCCH precoding configurations for 8Tx are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add applicability rules for Rank8 tests. - Modify the title of Rank8 requirements table to clarify that only Baseline receiver are appliled. - Modify note2 in Table 5.2A.4.1-7 and 5.2A.4.1-8 to “For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-1 and 5.2A.4.1-4 shall be applied for CCs with 8Rx.” - Remove Note 3 in Table 5.2A.4.1-8 - Add precoding configurations for 8Tx for SDR test. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The incorrections will still be existing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3, 5.2.4.1.1, 5.2S.4.1, 5.5A.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR … CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |  |
| --- | --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1)Up to 16 DL carriers 2)Same numerology across carrier for data/control channel at a given time |  |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |  |
| Alternative 64QAM MCS table for PDSCHNew 64QAM MCS table for PDSCH (dl-64QAM-MCS-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |  |
| CQI table with target BLER of 10^-5New CQI table (cqi-TableAlt) | FR1 FDD | PDSCH | Clause 5.2.2.1.5 Clause 5.2.3.1.5 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.5 Clause 5.2.3.2.5 |  |  |
| PDSCH repetitions over multiple slots (pdsch-RepetitionMultiSlots) | FR1 FDD | PDSCH | Clause 5.2.2.1.6 Clause 5.2.3.1.6 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.6 Clause 5.2.3.2.6 |  |  |
| UE PDSCH processing capability #2 (pdsch-ProcessingType2) | FR1 FDD | PDSCH | Clause 5.2.2.1.7 Clause 5.2.3.1.7 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.7 Clause 5.2.3.2.7 |  |  |
| Pre-emption indication for DL (pre-EmptIndication-DL) | FR1 FDD | PDSCH | Clause 5.2.2.1.8 Clause 5.2.3.1.8 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.8 Clause 5.2.3.2.8 |  |  |
| Single DCI based SDM transmission for multi-TRxP (singleDCI-SDM-scheme-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.11 Clause 5.2.3.1.11 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.11 Clause 5.2.3.2.11 |  |  |
| Multi DCI based multi-TRxP support (multiDCI-MultiTRP-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.12 Clause 5.2.3.1.12 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.12 Clause 5.2.3.2.12 |  |  |
| Single DCI based FDM Scheme-A for multi-TRxP(supportFDM-SchemeA-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.13 Clause 5.2.3.1.13 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.13 Clause 5.2.3.2.13 |  |  |
| Single DCI based inter-slot TDM for multi-TRxP (supportInter-slotTDM-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |
| Maximum number of TCI states in Single-DCI based inter-slot TDM (maxNumberTCI-states-r16) | FR1 FDD | PDSCH | Clause 5.2.2.1.14 Clause 5.2.3.1.14 |  | The requirements apply only when maxNumberTCI-states-r16 = 2. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.14 Clause 5.2.3.2.14 |  |  |  |
| DRX Adaptation (drx-Adaptation-r16) | FR1 FDD | PDCCH | Clause 5.3.2.1.3 | If the Test 1-1 in Clause 5.3.2.1.3 is passed, the test coverage can be considered fulfilled without executing Test 1-3 in clause 5.3.2.1.1. |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.3 | If the Test 1-1 in Clause 5.3.2.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.3.2.2.1. |  |
|  | FR1 FDD | PDCCH | Clause 5.3.3.1.3 | If the Test 1-1 in Clause 5.3.3.1.3 is passed, the test coverage can be considered fulfilled without executing Test 1-3 in clause 5.3.3.1.1. |  |
|  | FR1 TDD | PDCCH | Clause 5.3.3.2.3 | If the Test 1-1 in Clause 5.3.3.2.3 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.3.3.2.1. |  |
| Validating P/SP-CSI-RS reception (periodicAndSemi-PersistentCSI-RS-r16) | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 Clause 5.2A.2.3 Clause 5.2A.3.3 | The requirements apply only in case tested UE supporting operations in shared spectrum access and validation of P/SP-CSI-RS reception based on DCI |  |
| Supported UL channels for dynamic channel access mode (ul-DynamicChAccess-r16) or UL channel access for semi-static channel access mode (ul-Semi-StaticChAccess-r16) or both | FR1 TDD | PDSCH | Clause 5.2.2.2.15 Clause 5.2.3.2.15 | The requirements apply only in case tested UE supports one of UL channels for dynamic channel access mode and UL channel access for semi-static channel access mode |  |
| 1024QAM modulation for PDSCH for FR1 (pdsch-1024QAM-FR1-r17 or pdsch-1024QAM-2MIMO-FR1-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-8) Clause 5.2.3.1.1 (Test 1-8) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-12) Clause 5.2.3.2.1 (Test 1-12) |  |  |
|  |  | SDR | Clause 5.5.1 Clause 5.5A.1 | 1024QAM MCS indexes are used only if UE supports 1024QAM for FR1 DL. |  |
| Support of neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS ( CRS-IM-DSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.18 Clause 5.2.3.1.17 | UE can support the feature on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.19 Clause 5.2.3.2.18 |  |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 2-1) Clause 5.2.3.1.18 (Test 2-1) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-1) Clause 5.2.3.2.19 (Test 2-1) |  |  |
| Support of neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (CRS-IM-nonDSS-NWA-15kHzSCS-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.19 (Test 1-1) Clause 5.2.3.1.18 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.1.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.1.19. If the Test 2-1 in Clause 5.2.3.1.18 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.1.18. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-1) Clause 5.2.3.2.19 (Test 1-1) | If the Test 2-1 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.2.2.20. If the Test 2-1 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-1 in clause 5.2.3.2.19. |  |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 2-2) Clause 5.2.3.2.19 (Test 2-2) | The UE can perform CRS-IM when MeasObjectEUTRA IE is configured, and the configured measurement gaps overlap with neighbour LTE cell PBCH position. |  |
| CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signaling on LTE channel bandwidth (crs-IM-nonDSS-NWA-30kHzSCS-r17) | FR1 TDD | PDSCH | Clause 5.2.2.2.20 (Test 1-2) Clause 5.2.3.2.19 (Test 1-2) | If the Test 2-2 in Clause 5.2.2.2.20 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.2.2.20. If the Test 2-2 in Clause 5.2.3.2.19 is passed, the test coverage can be considered fulfilled without executing Test 1-2 in clause 5.2.3.2.19. |  |
| Support for SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH (sfn-SchemeA-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.20 Clause 5.2.3.1.19 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.21 Clause 5.2.3.2.20 |  |  |
| Support for SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH (sfn-SchemeB-r17) | FR1 FDD | PDSCH | Clause 5.2.2.1.21 Clause 5.2.3.1.20 |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.22 Clause 5.2.3.2.21 |  |  |
| Support for PDCCH with intra-slot repetition (mTRP-PDCCH-Repetition-r17) | FR1 FDD | PDCCH | Clause 5.3.2.1.5 Clause 5.3.3.1.4 |  |  |
|  | FR1 TDD | PDCCH | Clause 5.3.2.2.5 Clause 5.3.3.2.4 |  |  |
| Support for TDD-TDD intra-band non-colocated NR-CA deployment (intraBandNR-CA-non-collocated-r18) | FR1 TDD | PDSCH | Clause 5.2A.2.6 | The requirements apply on in case the UE indicates support of 256QAM modulation scheme for PDSCH for FR1 (pdsch-256QAM-FR1) |  |
| Support for MU-MIMO Interference Mitigation advanced receiver (R-ML), when co-scheduled UE(s)’ modulation order is explicitly signalled by DCI index 1-5 in Table 7.3.1.2.2-12 of TS38.212 [10]. (advReceiver-MU-MIMO-r18) | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-1) Clause 5.2.3.1.16 (Test 3-1, Test 4-1) | If the Test 2-2 in Clause 5.2.2.1.16 is passed, the test coverage can be considered fulfilled without executing Test 2-1 in clause 5.2.2.1.16. If the Test 3-2 in Clause 5.2.3.1.16 is passed, the test coverage can be considered fulfilled without executing Test 3-1 in clause 5.2.3.1.16. If the Test 4-2 in Clause 5.2.3.1.16 is passed, the test coverage can be considered fulfilled without executing Test 4-1 in clause 5.2.3.1.16. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-1) Clause 5.2.3.2.17 (Test 3-1, Test 4-1) | If the Test 2-2 in Clause 5.2.2.2.17 is passed, the test coverage can be considered fulfilled without executing Test 2-1 in clause 5.2.2.2.17. If the Test 3-2 in Clause 5.2.3.2.17 is passed, the test coverage can be considered fulfilled without executing Test 3-1 in clause 5.2.3.2.17. If the Test 4-2 in Clause 5.2.3.2.17 is passed, the test coverage can be considered fulfilled without executing Test 4-1 in clause 5.2.3.2.17. |  |
| Support for MU-MIMO Interference Mitigation advanced receiver with modulation order detection or MU-MIMO Interference Mitigation advanced receiver with modulation order detection Enh in 38.306 [14] | FR1 FDD | PDSCH | Clause 5.2.2.1.16 (Test 2-2) Clause 5.2.3.1.16 (Test 3-2, Test 4-2) | Test 4-2 is only applicable for UE supporting ‘MU-MIMO Interference Mitigation advanced receiver with modulation order detection Enh’ in TS38.306 [14]. |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.17 (Test 2-2) Clause 5.2.3.2.17 (Test 3-2, Test 4-2) |  |  |
| Baseline SU-MIMO 8Rx receiver | FR1 FDD | PDSCH | Clause 5.2.4.1.1(Tests 1-1,3-1,5-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.4.2.1(Tests 1-1,3-1,5-1) |  |  |
| Simplified SU-MIMO 8Rx receiver | FR1 FDD | PDSCH | Clause 5.2.4.1.1(Tests 2-1,4-1) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.4.2.1(Tests 2-1,4-1) |  |  |
| Support for enhanced DMRS (pdsch-TypeA-DMRS-r18) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 2-3) Clause 5.2.3.1.1 (Test 4-2) |  |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 2-3) Clause 5.2.3.2.1 (Test 4-2) |  |  |
| Reception of NR PDCCH candidates overlapping with LTE CRS REs (nr-PDCCH-OverlapLTE-CRS-RE-r18) | FR1 FDD | PDCCH | Clause 5.3.2.1.6 Clause 5.3.2.2.6 |  |  |
|  | FR1 TDD | PDCCH | Clause 5.3.3.1.5 Clause 5.3.3.2.5 |  |  |
| Support for Dedicated Spectrum of Less than 5MHz (support-3MHz-ChannelBW-r18) | FR1 FDD | PDCCH | Clause 5.3.2.1.7 (Test 1-1) Clause 5.3.3.1.6 (Test 1-1) |  |  |
|  |  | PBCH | Clause 5.4.2.1 (Test 3-1) Clause 5.4.3.1 (Test 3-1) |  |  |

5.2.4	8RX requirements

5.2.4.1	FDD

5.2.4.1.1	Minimum requirements for PDSCH Mapping Type A

The performance requirements are specified in Table 5.2.4.1.1-3 - Table 5.2.4.1.1-7, with the addition of test parameters in Table 5.2.4.1.1-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.4.1.1-1.

Table 5.2.4.1.1-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 8 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 2-1, 3-1, 4-1, 5-1 |

Table 5.2.4.1.1-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel |
|  | CodebookMode |  | 1 |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (2,1) for 4Tx (4,1) for 8Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,1) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.4.1.1-3: Minimum performance for Rank 2 with Baseline SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.5 |

Table 5.2.4.1.1-4: Minimum performance for Rank 2 with Simplified SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.7 |

Table 5.2.4.1.1-5: Minimum performance for Rank 4 with Baseline SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.1-3.7 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 4x8, ULA Low | 70 | 12.8 |

Table 5.2.4.1.1-6: Minimum performance for Rank 4 with Simplified SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 4-1 | R.PDSCH.1-3.7 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 4x8, ULA Low | 70 | 15.8 |

Table 5.2.4.1.1-7: Minimum performance for Rank 8 with Baseline SU-MIMO 8Rx receiver

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5-1 | R.PDSCH.1-3.8 FDD | 10 / 15 | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.5 |

5.2A.4	8RX requirements

5.2A.4.1	Minimum requirements

For CA with different numbers of DL component carriers, the requirements are defined in Table 5.2A.4.1-7 or Table 5.2A.4.1-8 (dependent on Baseline or Simplified SU-MIMO 8Rx Receiver), the requirements are based on the single carrier requirements for different SCSs and different bandwidth specified in Table 5.2A.4.1-1 ~ Table 5.2A.4.1-6, with the parameters in Table 5.2A-1 ~ Table 5.2A-3 and the downlink physical channel setup according to Annex C.3.1. The performance requirements specified in this sub-clause do not apply for UE single carrier test.

Table 5.2A.4.1-1: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-22.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 15 | R.PDSCH.1-22.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 20 | R.PDSCH.1-22.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 25 | R.PDSCH.1-22.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 30 | R.PDSCH.1-23.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 35 | R.PDSCH.1-23.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 40 | R.PDSCH.1-23.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 45 | R.PDSCH.1-23.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.9 |
| 50 | R.PDSCH.1-23.5 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 14.0 |

Table 5.2A.4.1-2: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-22.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 15 | R.PDSCH.1-22.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 20 | R.PDSCH.1-22.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 25 | R.PDSCH.1-22.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 30 | R.PDSCH.1-23.1 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.2 |
| 35 | R.PDSCH.1-23.2 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 40 | R.PDSCH.1-23.3 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.2 |
| 45 | R.PDSCH.1-23.4 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |
| 50 | R.PDSCH.1-23.5 FDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.4 |

Table 5.2A.4.1-3: Single carrier performance for FDD 15 kHz SCS for CA configurations, Rank8,Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-20.1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.6 |
| 10 | R.PDSCH.1-3.7 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.7 |
| 15 | R.PDSCH.1-20.2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.9 |
| 20 | R.PDSCH.1-20.3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 25 | R.PDSCH.1-20.4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 30 | R.PDSCH.1-21.1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.4 |
| 35 | R.PDSCH.1-21.2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.6 |
| 40 | R.PDSCH.1-21.3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.9 |
| 45 | R.PDSCH.1-21.4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.7 |
| 50 | R.PDSCH.1-21.5 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.5 |

Table 5.2A.4.1-4: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-36.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.3 |
| 10 | R.PDSCH.2-36.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.5 |
| 15 | R.PDSCH.2-36.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 20 | R.PDSCH.2-36.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 25 | R.PDSCH.2-36.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 30 | R.PDSCH.2-36.6 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.6 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 50 | R.PDSCH.2-37.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 60 | R.PDSCH.2-37.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.8 |
| 80 | R.PDSCH.2-37.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 14.0 |
| 90 | R.PDSCH.2-37.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |
| 100 | R.PDSCH.2- 37.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.7 |

Table 5.2A.4.1-5: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 2, Simplified SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-36.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 10 | R.PDSCH.2-36.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 15.9 |
| 15 | R.PDSCH.2-36.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 20 | R.PDSCH.2-36.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 25 | R.PDSCH.2-36.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 30 | R.PDSCH.2-36.6 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.0 |
| 50 | R.PDSCH.2-37.1 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 60 | R.PDSCH.2-37.2 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.1 |
| 80 | R.PDSCH.2-37.3 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |
| 90 | R.PDSCH.2-37.4 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.4 |
| 100 | R.PDSCH.2- 37.5 TDD | 64QAM, 0.5 | TDLC300-100 | 2x8, ULA Medium B | 70 | 16.3 |

Table 5.2A.4.1-6: Single carrier performance for TDD 30 kHz SCS for CA configurations, Rank 8, Baseline SU-MIMO 8Rx Receiver

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-34.1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.7 |
| 10 | R.PDSCH.2-34.2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.6 |
| 15 | R.PDSCH.2-34.3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.9 |
| 20 | R.PDSCH.2-34.4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.8 |
| 25 | R.PDSCH.2-34.5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.8 |
| 30 | R.PDSCH.2-34.6 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.2 |
| 40 | R.PDSCH.2-3.7 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.1 |
| 50 | R.PDSCH.2-35.1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.0 |
| 60 | R.PDSCH.2-35.2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.4 |
| 80 | R.PDSCH.2-35.3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.6 |
| 90 | R.PDSCH.2-35.4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.7 |
| 100 | R.PDSCH.2- 35.5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 23.8 |

Table 5.2A.4.1-7: Minimum performance for multiple CA configurations, Baseline SU-MIMO 8Rx Receiver

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Tables 5.2A.4.1-1, and 5.2A.4.1-3 per CC |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-4, and 5.2A.4.1-6 per CC |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-1, 5.2A.4.1-3, 5.2A.4.1-4, and 5.2A.4.1-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. Note 2: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-1 and 5.2A.4.1-4 shall be applied for CCs with 8Rx. Note 3: For CA Combinations with two 8Rx CCs, Rank 8 requirements in Tables 5.2A.4.1-3 and 5.2A.4.1-6 shall be applied for both CCs. |  |  |

Table 5.2A.4.1-8: Minimum performance for multiple CA configurations, Simplified SU-MIMO 8Rx Receiver

| Test number | CA duplex mode | Minimum performance requirements |
| --- | --- | --- |
| 1 | FDD 15 kHz + FDD 15 kHz | As defined in Tables 5.2A.4.1-2, and 5.2A.4.1-3 per CC |
| 2 | TDD 30 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-5, and 5.2A.4.1-6 per CC |
| 3 | FDD 15 kHz + TDD 30 kHz | As defined in Tables 5.2A.4.1-2, 5.2A.4.1-3, 5.2A.4.1-5, and 5.2A.4.1-6 per CC |
| Note 1: The applicability of requirements for different CA duplex modes, SCSs, CA configurations and bandwidth combination sets is defined in 5.1.1.7. Note 2: For CA combinations between 8Rx and 4Rx or 2Rx, Rank 2 requirements in Tables 5.2A.4.1-2 and 5.2A.4.1-5 shall be applied for CCs with 8Rx, . |  |  |

5.5A	Sustained downlink data rate provided by lower layers

5.5A.1	FR1 CA requirements

<Editor's note: Open issues to be resolved:

Whether same requirements apply for FR1 DC>

The Sustained Data Rate (SDR) requirements in this clause are applicable to the FR1 CA.

The purpose of the test is to verify that the Layer 1 and Layer 2 correctly process in a sustained manner the received packets corresponding to the maximum data rate indicated by UE capabilities. The sustained downlink data rate shall be verified in terms of the success rate of delivered PDCP SDU(s) by Layer 2. The test case below specifies the RF conditions and the required success rate of delivered TB by Layer 1 to meet the sustained data rate requirement.

The test parameters are determined by the following procedure:

-	Select one CA bandwidth combination among all supported CA configurations and set of per component carrier (CC) UE capabilities among all supported UE capabilities that provides the largest data rate in accordance with clause 4.1.2 of TS 38.306 [14].

-	Set of per CC UE capabilities includes channel bandwidth, subcarrier spacing, number of PDSCH MIMO layers, modulation format and scaling factor in accordance with clause 4.1.2 of TS 38.306 [14].

-	When there are multiple sets of CA bandwidth combinations and UE capabilities (channel bandwidth, subcarrier spacing, number of MIMO layer, modulation format, scaling factor) with same largest data rate, select one among sets with the smallest aggregated channel bandwidth.

-	For UE not capable of 1024QAM (pdsch-1024QAM-FR1), for each CC in CA bandwidth combination, use Table 5.5A-5 to determine MCS based on test parameters and indicated UE capabilities.

-	For UE capable of 1024QAM (pdsch-1024QAM-FR1), for each CC in CA bandwidth combination, use Table 5.5A-6 to determine MCS based on test parameters and indicated UE capabilities if the maximum modulation format is 10. If the maximum modulation format (supportedModulationOrderDL) is less than 10, use Table 5.5A-5 to determine MCS based on test parameters and indicated UE capabilities.

The TB success rate shall be higher than 85% when PDSCH is scheduled with MCS defined for the selected CA bandwidth combination and with the downlink physical channel setup according to Annex C.3.1.

The TB success rate is defined as 100%*NDL_correct_rx / (NDL_newtx + NDL_retx), where NDL_newtx is the number of newly transmitted DL transport blocks, NDL_retx is the number of retransmitted DL transport blocks, and NDL_correct_rx is the number of correctly received DL transport blocks.

The common test parameters are specified in Table 5.5A-1. The parameters specified in Table 5.5A-2 are applicable for tests on FDD CCs and parameters specified in Table 5.5A-3 are applicable for tests on TDD CCs.

Unless otherwise stated, no user data is scheduled on slot #0, 10 and 11 within 20 ms for SCS 15 kHz.

Unless otherwise stated, no user data is scheduled on slot #0, 20 and 21 within 20 ms for SCS 30 kHz.

Table 5.5A-1: Common test parameters for FDD and TDD component carriers

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| EPRE ratio of PTRS to PDSCH |  |  | dB | N/A |
| Channel bandwidth |  |  | MHz | Channel bandwidth from selected CA bandwidth combination |
| Common serving cell parameters | Physical Cell ID |  |  | 0 |
|  | SSB position in burst |  |  | First SSB in Slot #0 |
|  | SSB periodicity |  | ms | 20 |
|  | First DMRS position for Type A PDSCH mapping |  |  | 2 |
| Cross carrier scheduling |  |  |  | Not configured |
| Active DL BWP index |  |  |  | 1 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 2) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
| DL BWP configuration #1 | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  |  | Maximum transmission bandwidth configuration as specified in clause 5.3.2 of TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |
|  | Subcarrier spacing |  | kHz | 15 or 30 |
|  | Cyclic prefix |  |  | Normal |
| PDCCH configuration | Slots for PDCCH monitoring |  |  | Each slot |
|  | Symbols with PDCCH |  |  | Symbols #0 |
|  | Number of PRBs in CORESET |  |  | Table 5.5A-4 |
|  | Number of PDCCH candidates and aggregation levels |  |  | 2/AL2 for 15 kHz / 5 MHz and 30 kHz / 15 MHz 2/AL4 for 15 kHz / 10 MHz, 30 kHz / 10 MHz and 30 kHz / 20 MHz 2/AL8 for other greater combinations |
|  | CCE-to-REG mapping type |  |  | Non-interleaved |
|  | DCI format |  |  | 1_1 |
|  | TCI State |  |  | TCI state #1 |
|  | PDCCH & PDCCH DMRS Precoding configuration |  |  | For number of TX = 1: No precoding; For Number of Tx = 2: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of precoder index 0 and 2 For Number of Tx= 4 and 8: Single Panel Type I, Randomized precoder selection for every REG bundle and updated per slot with equal probability of i_1,1 in {1,2,3,5,6,7} and i_2 in {0,2} |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | wideband |
|  | Resource allocation type |  |  | Type 0 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 1 |
|  | Length |  |  | Rank 2/4: 1 Rank 8: 2 |
|  | Antenna ports indexes |  |  | {1000} for 1 Layer CCs {1000, 1001} for 2 Layers CCs {1000 – 1003} for 4 Layers CCs {1000-1007} for 8 Layers CCs |
|  | Number of PDSCH DMRS CDM group(s) without data |  |  | 1 for 1 layer and 2 layers CCs 2 for 4 Layers and 8 Layers CCs |
| PTRS configuration |  |  |  | PTRS is not configured |
| CSI-RS for tracking | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 3 for CSI-RS resource 1,2,3,4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 6 for CSI-RS resource 1 and 3 l0 = 10 for CSI-RS resource 2 and 4 |
|  | Number of CSI-RS ports (X) |  |  | 1 for CSI-RS resource 1,2,3,4 |
|  | CDM Type |  |  | 'No CDM' for CSI-RS resource 1,2,3,4 |
|  | Density (ρ) |  |  | 3 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS periodicity |  | Slots | 15 kHz SCS: 20 for CSI-RS resource 1,2,3,4 30 kHz SCS: 40 for CSI-RS resource 1,2,3,4 |
|  | CSI-RS offset |  | Slots | 15 kHz SCS: 10 for CSI-RS resource 1 and 2 11 for CSI-RS resource 3 and 4 30 kHz SCS: 20 for CSI-RS resource 1 and 2 21 for CSI-RS resource 3 and 4 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #0 |
| NZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 4 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | Same as number of transmit antenna |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size /4)*4 |
|  | QCL info |  |  | TCI state #1 |
| ZP CSI-RS for CSI acquisition | Subcarrier indexes in the PRB used for CSI-RS |  |  | k0 = 0 |
|  | OFDM symbols in the PRB used for CSI-RS |  |  | l0 = 12 |
|  | Number of CSI-RS ports (X) |  |  | 4 |
|  | CDM Type |  |  | 'FD-CDM2' |
|  | Density (ρ) |  |  | 1 |
|  | CSI-RS periodicity |  |  | 15 kHz SCS: 20 30 kHz SCS: 40 |
|  | CSI-RS offset |  |  | 0 |
|  | Frequency Occupation |  |  | Start PRB 0 Number of PRB = ceil(BWP size/4)*4 |
| TCI state #0 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| Maximum number of code block groups for ACK/NACK feedback |  |  |  | 1 |
| Maximum number of HARQ transmission |  |  |  | 4 |
| PUCCH HARQ ACK spaitial bundling |  |  |  | Not configured |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  |  | For number of TX = 1: No precoding; For number of TX > 1: Single Panel Type I; Randomized precoder selection for every PRB bundle and updated per slot, with equal probability of each applicable i1/i2 combination or codebook index, chosen from section 5.2.2.2.1 of TS 38.214 [12]. |
| Symbols for all unused REs |  |  |  | OP.1 FDD as defined in Annex A.5.1.1 OP.1 TDD as defined in Annex A.5.2.1 |
| Propagation condition |  |  |  | Static propagation condition No external noise sources are applied |
| Antenna configuration | 1 layer CCs |  |  | 1x1 or 1x2 or 1x4 |
|  | 2 layers CCs |  |  | 2x2 or 2x4 or 2x8 |
|  | 4 layers CCs |  |  | 4x4 or 4x8 |
|  | 8 layers CCs |  |  | 8x8 |
| Physical signals, channels mapping and precoding |  |  |  | As specified in Annex B.4.1 |
| Es |  |  | dBm/Hz | -112 for MCS indexes in Table 5.5A-5: -110.5 For 1024QAM MCS indexes in Table 5.5A-6: |
| Note 1: UE assumes that the TCI state for the PDSCH is identical to the TCI state applied for the PDCCH transmission Note 2: Point A coincides with minimum guard band as specified in Table 5.3.3-1 from TS 38.101-1 [6] for tested channel bandwidth and subcarrier spacing |  |  |  |  |